3GPP TSG-RAN5 Meeting #98	R5-230652

# Athens, 27th  Feb 2023- 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 1411 | rev |  | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add EPS-UPIP to ATTACH and TAU Request message |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPIP_SEC_LTE-RAN-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | If UE support user plane integrity protection with EPS, UE must indicate EPS-UPIP support and ENDC support in UE network capability IE of ATTACH Request and TAU Request. [24.301 5.5.1.2.2] If the UE supports EPS-UPIP, the UE shall set the EPS-UPIP bit to "EPS-UPIP supported" in the UE network capability IE of the ATTACH REQUEST message. [24.301 5.5.3.2.2] If the UE supports EPS-UPIP, the UE shall set the EPS-UPIP bit to "EPS-UPIP supported" in the UE network capability IE of the TRACKING AREA UPDATE REQUEST message. [33.401 7.3.3] NOTE a: Only EN-DC capable UEs and EN-DC capable eNBs support UP integrity protection. Therefore, the eNB can only activate UP integrity protection with a UE that is EN-DC capable and supports user plane integrity protection with EPS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add EPS-UPIP to ATTACH and TAU Request message |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not Comformant UE may pass the common procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

-	ATTACH REQUEST

This message is sent by the UE to the SS.

Table 4.7.2-4: ATTACH REQUEST

| Derivation Path: 24.301 clause 8.2.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | EMM |  |  |
| Security header type |  | '0000'B | Plain NAS message, not security protected |  |
| Attach request message identity |  | '0100 0001'B | Attach request |  |
| EPS attach type |  | '0001'B | EPS attach | EPS_only |
|  |  | '0010'B | combined EPS/IMSI attach | combined_EPS_IMSI |
| NAS key set identifier |  | Any allowed value |  |  |
| EPS mobile identity |  | Any allowed value |  |  |
| UE network capability |  | Any allowed value |  |  |
| UE network capability |  |  |  | EN-DC |
| All octets with the exception of octet 9, bit 5 |  | Any allowed value |  |  |
| Dual connectivity with NR (DCNR) (octet 9, bit 5) |  | 1 | dual connectivity with NR supported |  |
| UE network capability |  |  |  | NR |
| All octets with the exception of octet 8, bit 8 and octet 9, bit 6 |  | Any allowed value |  |  |
| Extended protocol configuration options (ePCO) (octet 8, bit 8) |  | '1' | Extended protocol configuration options IE supported |  |
| N1 mode supported (N1mode) (octet 9, bit 6) |  | 1 | N1 mode supported |  |
| UE network capability |  |  |  | pc_EPC_RACS |
| All octets with the exception of octet 10, bit 1 |  | Any allowed value |  |  |
| Radio capability signalling optimisation (RACS) capability (octet 10, bit 1) |  | 1 |  |  |
| UE network capability |  |  |  | pc_EPS_UPIP |
| All octets with the exception of octet 4, bit 1 and octet 9, bit 5 |  | Any allowed value |  |  |
| EPS-UPIP supported (octet 4, bit 1) |  | 1 | EPS-UPIP supported |  |
| Dual connectivity with NR (DCNR) (octet 9, bit 5) |  | 1 | dual connectivity with NR supported |  |
| ESM message container |  | PDN CONNECTIVITY REQUEST message to request PDN connectivity to the default PDN |  |  |
| ESM message container |  | ESM DUMMY MESSAGE |  | CIoT_Attach_WithoutPDN |
| Old P-TMSI signature |  | Not present or any allowed value |  |  |
| Additional GUTI |  | Not present or any allowed value |  |  |
| Last visited registered TAI |  | Not present or any allowed value |  |  |
| DRX parameter |  | Not present or any allowed value |  |  |
| MS network capability |  | Not present or any allowed value |  |  |
| Old location area identification |  | Not present or any allowed value |  |  |
| TMSI status |  | Not present or any allowed value |  |  |
| Mobile station classmark 2 |  | Not present or any allowed value |  |  |
| Mobile station classmark 3 |  | Not present or any allowed value |  |  |
| Supported Codecs |  | Not present or any allowed value |  |  |
| Additional update type |  | Not present |  | EPS_only |
| Additional update type |  | Any allowed value |  | EPS_only_with_SMS |
| Additional update type |  | Not present or any allowed value |  | combined_EPS_IMSI or CIoT |
| Voice domain preference and UE's usage setting |  | Not present or any allowed value |  |  |
| Device properties |  | Not present or any allowed value |  |  |
| Old GUTI type |  | Not present or any allowed value |  |  |
| MS network feature support |  | Not present or any allowed value |  |  |
| TMSI based NRI container |  | Not present or any allowed value |  |  |
| T3324 value |  | Not present or any allowed value |  |  |
| T3412 extended value |  | Not present or any allowed value |  |  |
| Extended DRX parameters |  | Not present or any allowed value |  |  |
| UE additional security capability |  | Any allowed value (NOTE 1, NOTE 2) |  | EN-DC OR NR |
| UE status |  | Not present or any allowed value |  |  |
| Additional information requested |  | Not present or any allowed value |  |  |
| UE radio capability ID availability |  | Not present or any allowed value |  | pc_EPC_RACS |
| Requested IMSI offset |  | Not present or any allowed value |  |  |
| NOTE 1: For a UE supporting dual connectivity with NR, if the UE supports one of the encryption algorithms for 5GS in bits 8 to 5 of octet 3, it shall support the same algorithms for E-UTRAN as specified in 3GPP TS 33.401 [41]. NOTE 2: For a UE supporting dual connectivity with NR, if the UE supports one of the integrity protection algorithms for 5GS in bits 8 to 5 of octet 5, it shall support the same algorithms for E-UTRAN as specified in 3GPP TS 33.401 [41]. |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| EPS_only | See the definition below table 4.7.2-1. |
| combined_EPS_IMSI | See the definition below table 4.7.2-1. |
| CIoT_Attach_WithoutPDN | If the UE wants to attach without PDN for CIoT optimization |
| EPS_only_with_SMS | If the UE wants to perform EPS attach with SMS only for CIoT optimization |
| CIoT | If UE supports CIoT Optimisation |
| EN-DC | Used in E-UTRA-NR dual connectivity test cases (TS 38.523-1 [71]) |
| NR | Used in NR test cases (TS 38.523-1 [71]) |

NOTE:	This message is sent integrity protected when a valid security context exists and without integrity protection otherwise.

<End of modified section 1>

<Start of modified section 2>

-	TRACKING AREA UPDATE REQUEST

This message is sent by the UE to the SS.

Table 4.7.2-27: TRACKING AREA UPDATE REQUEST

| Derivation Path: 24.301 clause 8.2.29 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| Protocol discriminator |  | EMM |  |  |  |  |  |
| Security header type |  | '0000'B |  | Plain NAS message, not security protected |  |  |  |
| Tracking area update request message identity |  | '0100 1000'B |  | Tracking area update request |  |  |  |
| EPS update type |  |  |  |  |  |  |  |
| EPS update type Value |  | '000'B |  | TA updating |  | TA_only |  |
|  |  | '001'B |  | Combined TA/LA updating |  | combined_TA_LA |  |
| "Active" flag |  | '0'B |  | No bearer establishment requested |  |  |  |
| NAS key set identifier |  |  |  |  |  |  |  |
| NAS key set identifier |  | The valid NAS key set identifier of the UE |  |  |  |  |  |
| TSC |  | '0'B |  | native security context (for KSIASME) |  |  |  |
| Old GUTI |  | Any allowed value |  |  |  |  |  |
| Non-current native NAS key set identifier |  | Not present |  |  |  |  |  |
| GPRS ciphering key sequence number |  | Not present |  |  |  |  |  |
| Old P-TMSI signature |  | Not present |  |  |  |  |  |
| Additional GUTI |  | Not present |  |  |  |  |  |
| NonceUE |  | Not present |  |  |  |  |  |
| UE network capability |  | Not present or any allowed value |  |  |  |  |  |
| UE network capability |  |  |  |  |  | EN-DC |  |
| All octets with the exception of octet 9, bit 5 |  | Any allowed value |  |  |  |  |  |
| Dual connectivity with NR (DCNR) (octet 9, bit 5) |  | 1 |  | dual connectivity with NR supported |  |  |  |
| UE network capability |  |  |  |  |  | NR |  |
| All octets with the exception of octet 8, bit 8 and octet 9, bit 6 |  | Any allowed value |  |  |  |  |  |
| Extended protocol configuration options (ePCO) (octet 8, bit 8) |  | 1 |  | ePCO supported |  |  |  |
| N1 mode supported (N1mode) (octet 9, bit 6) |  | 1 |  | N1 mode supported |  |  |  |
| UE network capability |  |  |  |  |  | pc_EPC_RACS |  |
| All octets with the exception of octet 10, bit 1 |  | Any allowed value |  |  |  |  |  |
| Radio capability signalling optimisation (RACS) capability (octet 10, bit 1) |  | 1 |  |  |  |  |  |
| UE network capability |  |  |  |  |  | pc_EPS_UPIP |  |
| All octets with the exception of octet 4, bit 1 and octet 9, bit 5 |  | Any allowed value |  |  |  |  |  |
| EPS-UPIP supported (octet 4, bit 1) |  | 1 |  | EPS-UPIP supported |  |  |  |
| Dual connectivity with NR (DCNR) (octet 9, bit 5) |  | 1 |  | dual connectivity with NR supported |  |  |  |
| Last visited registered TAI |  | Not present or any allowed value |  |  |  |  |  |
| DRX parameter |  | Not present or any allowed value |  |  |  |  |  |
| UE radio capability information update needed |  | Not present or any allowed value |  |  |  |  |  |
| EPS bearer context status |  | Not present or (octet 3 = '00100000'B and octet 4 = '00000000'B) |  |  |  |  |  |
| MS network capability |  | Not present or any allowed value |  |  |  |  |  |
| Old location area identification |  | Not present or any allowed value |  |  |  |  |  |
| TMSI status |  | Not present or any allowed value |  |  |  |  |  |
| Mobile station classmark 2 |  | Not present or any allowed value |  |  |  |  |  |
| Mobile station classmark 3 |  | Not present or any allowed value |  |  |  |  |  |
| Supported Codecs |  | Not present or any allowed value |  |  |  |  |  |
| Additional update type |  | Not present |  |  |  | TA_only |  |
| Additional update type |  | Not present or any allowed value |  |  |  | combined_TA_LA |  |
| Additional update type |  | Any allowed value |  |  |  | TA_only_with_SMS |  |
| Additional update type |  | Not present or any allowed value |  |  |  | CIoT |  |
| Voice domain preference and UE's usage setting |  | Not present or any allowed value |  |  |  |  |  |
| Old GUTI type |  | Not present or any allowed value |  |  |  |  |  |
| Device properties |  | Not present or any allowed value |  |  |  |  |  |
| MS network feature support |  | Not present or any allowed value |  |  |  |  |  |
| TMSI based NRI container |  | Not present or any allowed value |  |  |  |  |  |
| T3324 value |  | Not present or any allowed value |  |  |  |  |  |
| T3412 extended value |  | Not present or any allowed value |  |  |  |  |  |
| Extended DRX parameters |  | Not present or any allowed value |  |  |  |  |  |
| UE additional security capability |  | Any allowed value (NOTE 1, NOTE 2) |  |  |  | EN-DC OR NR |  |
| UE status |  | Not present or any allowed value |  |  |  |  |  |
| Additional information requested |  | Not present or any allowed value |  |  |  |  |  |
| UE radio capability ID availability |  | Not present or any allowed value |  |  |  | pc_EPC_RACS |  |
| Requested IMSI offset |  | Not present or any allowed value |  |  |  |  |  |
| UE request type |  | Not present or any allowed value |  |  |  |  |  |
| Paging restriction |  | Not present or any allowed value |  |  |  |  |  |
| NOTE 1: For a UE supporting dual connectivity with NR, if the UE supports one of the encryption algorithms for 5GS in bits 8 to 5 of octet 3, it shall support the same algorithms for E-UTRAN as specified in 3GPP TS 33.401 [41]. NOTE 2: For a UE supporting dual connectivity with NR, if the UE supports one of the integrity protection algorithms for 5GS in bits 8 to 5 of octet 5, it shall support the same algorithms for E-UTRAN as specified in 3GPP TS 33.401 [41]. |  |  |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| TA_only | See the definition below table 4.7.2-24. |
| combined_TA_LA | See the definition below table 4.7.2-24. |
| EPS_only_with_SMS | If the UE wants to update for EPS attach with SMS only for CIoT optimization |
| CIoT | If UE supports CIoT Optimisation |
| EN-DC | Used in E-UTRA-NR dual connectivity test cases (TS 38.523-1 [71]) |
| NR | Used in NR test cases (TS 38.523-1 [71]) |

NOTE:	This message is always sent within SECURITY PROTECTED NAS MESSAGE message.

<End of modified section 2>