3GPP TSG-RAN5 Meeting #99	R5-233185

# Incheon, Korea, 22nd – 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-2 | CR | 0361 | rev | - | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to applicability of UAC TC11.3.1a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Bureau Veritas ADT, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | UAC TC11.3.1a currently is using TS36.523-2 PICS as part of test cases for data only requirement. However, it’s been identified that TS38.508-2 has more suitable PICS could representated for usage of data centric in 5GS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated test applicability and replaced “CS/PS mode 2 or PS mode 2” to “UE’s usage setting as data centric”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5G only device may not available to provide 4G related PICS for test cases selection. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1, 4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## {Start of changes}

<Unchanged Sections Skipped>

## 4.1	Protocol conformance test cases applicability

<Unchanged Sections Skipped>

Table 4.1-5a: Applicability of Protocol conformance Multi-layer test cases, ref. TS 38.523-1 [2]

| Clause | TC Title | Release | Applicability |  |
| --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |
| 11 | Multi-layer and Services |  |  |  |
| 11.1 | 5GS / EPS Fallback |  |  |  |
| 11.1.1 | MO MMTEL voice call setup from NR RRC_IDLE / EPS Fallback with redirection / Single registration mode with N26 interface / Success | Rel-15 | C54 | UEs supporting 5G Core and E-UTRA and EPS IMS Voice (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") and EPS fallback |
| 11.1.1a | MO MMTEL enhanced voice service call setup from NR RRC_IDLE / EPS Fallback with redirection / Single registration mode with N26 interface / Success | Rel-15 | C173 | UEs supporting 5G Core and E-UTRA and NG.114 v2.0 |
| 11.1.2 | MO MMTEL voice call setup from NR RRC_IDLE / EPS Fallback with redirection / Single registration mode without N26 interface / Success | Rel-15 | C54 | UEs supporting 5G Core and E-UTRA and EPS IMS Voice (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") and EPS fallback |
| 11.1.3 | MO MMTEL voice call setup from NR RRC_CONNECTED / EPS Fallback with handover / Single registration mode with N26 interface / Success | Rel-15 | C54 | UEs supporting 5G Core and E-UTRA and EPS IMS Voice (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") and EPS fallback |
| 11.1.3a | MO MMTEL enhanced voice service call setup from NR RRC_CONNECTED / EPS Fallback with handover / Single registration mode with N26 interface / Success | Rel-15 | C173 | UEs supporting 5G Core and E-UTRA and NG.114 v2.0 |
| 11.1.4 | MO MMTEL voice call setup from NR RRC_CONNECTED / EPS Fallback with redirection / Single registration mode with N26 interface / E-UTRAN cell selection using cell status barred / Success | Rel-15 | C54 | UEs supporting 5G Core and E-UTRA and EPS IMS Voice (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") and EPS fallback |
| 11.1.5 | MO MMTEL voice call setup from NR RRC_CONNECTED / EPS Fallback with redirection / Single registration mode without N26 interface / E-UTRAN cell selection using cell status reservation / Success | Rel-15 | C54 | UEs supporting 5G Core and E-UTRA and EPS IMS Voice (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") and EPS fallback |
| 11.1.6 | MT MMTEL voice call setup from NR RRC_IDLE / EPS Fallback with redirection / Single registration mode without N26 interface / Success | Rel-15 | C54 | UEs supporting 5G Core and E-UTRA and EPS IMS (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") Voice and EPS fallback |
| 11.1.7 | Emergency call setup from NR RRC_IDLE / Emergency Services Fallback to EPS with redirection / Single registration mode with N26 interface / Success | Rel-15 | C47 | UEs supporting 5G Core and E-UTRA and EPS IMS emergency call (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") and Emergency Services Fallback in NR connected to 5GCN |
| 11.1.8 | MO MMTEL voice call setup from NR RRC_CONNECTED / EPS Fallback with handover / Single registration mode with N26 interface / voiceFallbackIndication | Rel-16 | C95 | UEs supporting 5G Core and E-UTRA and EPS IMS (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") Voice and EPS fallback and voiceFallbackIndication |
| 11.1.9 | MO MMTEL voice call setup from NR RRC_IDLE / EPS Fallback with redirection / Single registration mode with N26 interface / voiceFallbackIndication | Rel-16 | C95 | UEs supporting 5G Core and E-UTRA and EPS IMS (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") Voice and EPS fallback and voiceFallbackIndication |
| 11.2 | 5G-SRVCC |  |  |  |
| 11.2.1 | 5G-SRVCC from NG-RAN to 3GPP UTRAN | Rel-16 | C127 | UEs supporting 5G Core and UTRA and NR to UTRA-FDD CELL_DCH CS handover |
| 11.3 | Unified Access Control (UAC) |  |  |  |
| 11.3.1 | UAC / Access Identity 0 / 0% access probability / MTSI MO speech call / SMSoIP | Rel-15 | C78 | UEs supporting 5G Core and Initiating session and MTSI speech and SMS over IP |
| 11.3.1a | UAC / Access Identity 0 / 0% access probability / Uplink user data transfer / RRC_INACTIVE | Rel-15 | C109A | UEs supporting 5G Core and RRC_INACTIVE and UE's usage setting as data centric |
| 11.3.2 | UAC / Access Identity 0 / 0% access probability / Paging for MT access/Emergency call | Rel-15 | C92 | UEs supporting 5G Core and emergency services in NR connected to 5GCN |
| 11.3.3 | UAC / Access Identity 0 / AC8 / RRC_INACTIVE / RNA update / RRC resume | Rel-15 | C109 | UEs supporting 5G Core and RRC_INACTIVE |
| 11.3.4 | UAC / Access Identity 0 / Registration procedure for mobility and periodic registration update / Barring per PLMN / Implicit AC barring list | Rel-15 | C21 | UEs supporting 5G Core |
| 11.3.5 | UAC / Access Identity 1 / New cell not in the country of its HPLMN/EHPLMN 0% access probability / MPS indicator / HPLMN/0%/100% accessibility AC5 / MMTEL-Video call | Rel-15 | C79 | UEs supporting 5G Core and Initiating session and MTSI video |
| 11.3.6 | UAC / Access Identity 2 / New cell not in the country of its HPLMN/EHPLMN 0% access probability / MCS indicator / HPLMN/0%/100% accessibility AC7 / RRC_INACTIVE | Rel-15 | C21 | UEs supporting 5G Core |
| 11.3.6a | UAC / Access Identity 2 / MCS indicator / SNPN / 0% / 100% accessibility AC7 / RRC_INACTIVE | Rel-16 | C231 | UEs supporting 5G Core and SNPN and configuration of access identities in the list of subscriber data |
| 11.3.7 | UAC / Access Identity 11..15 / High priority access / HPLMN/0% accessibility AC2 / Emergency call | Rel-15 | C92 | UEs supporting 5G Core and emergency services in NR connected to 5GCN |
| 11.3.8 | UAC / Access Identity 0 / NR RRC_IDLE / Cell re-selection while T390 is running | Rel-15 | C21 | UEs supporting 5G Core |
| 11.3.9 | UAC / Access Identity 0 / ODAC / PLMN / RPLMN / not EPLMN | Rel-15 | C21 | UEs supporting 5G Core |
| 11.3.9a | UAC / Access Identity 0 / ODAC / SNPN / RSNPN / new SNPN | Rel-16 | C131 | UEs supporting 5G Core and SNPN |
| 11.3.10 | UAC / Access Identity 0 / AC9 / 0% access probability / SIP Re-registration | Rel-16 | C198 | UEs supporting 5G Core and IMS security |
| 11.4 | Emergency Services |  |  |  |
| 11.4.1 | 5GMM-REGISTERED.NORMAL-SERVICE / 5GMM-IDLE / Emergency call / Utilising emergency number stored on the USIM / New emergency PDU session / Network failing the authentication check (5G AKA) | Rel-15 | C92 | UEs supporting 5G Core and emergency services in NR connected to 5GCN |
| 11.4.1a | 5GMM-REGISTERED.NORMAL-SERVICE / 5GMM-IDLE / Emergency call / Utilising emergency number stored on the USIM / New emergency PDU session / PEIPS assistance information | Rel-17 | C224 | UEs supporting 5G Core and PEI |
| 11.4.2 | 5GMM-DEREGISTERED.LIMITED-SERVICE / Emergency call / Utilisation of emergency numbers stored on the ME / Initial registration for emergency services / Handling of forbidden PLMNs | Rel-15 | C92 | UEs supporting 5G Core and emergency services in NR connected to 5GCN |
| 11.4.3 | 5GMM-DEREGISTERED.NO-SUPI / Emergency call / Utilisation of emergency numbers stored on the ME / Initial registration for emergency services | Rel-15 | C238 | UEs supporting 5G Core and emergency services in NR connected to 5GCN and test execution with No USIM |
| 11.4.4 | 5GMM-REGISTERED.ATTEMPTING-REGISTRATION-UPDATE T3346 running / Emergency call establishment / 5GMM-REGISTERED.NORMAL-SERVICE / Emergency call establishment before T3396 expiry | Rel-15 | C92 | UEs supporting 5G Core and emergency services in NR connected to 5GCN |
| 11.4.5 | 5GMM-REGISTERED.LIMITED-SERVICE / 5GMM-IDLE / Emergency call establishment and release / Handling of 5GS forbidden tracking areas for roaming | Rel-15 | C92 | UEs supporting 5G Core and emergency services in NR connected to 5GCN |
| 11.4.6 | 5GMM-REGISTERED.NON-ALLOWED-SERVICE / Emergency call establishment and release / Handling of non-allowed tracking areas | Rel-15 | C92 | UEs supporting 5G Core and emergency services in NR connected to 5GCN |
| 11.4.7 | Handling of Local and Extended emergency numbers / Mobility | Rel-15 | C92 | UEs supporting 5G Core and emergency services in NR connected to 5GCN |
| 11.4.8 | Handling of Local and extended emergency numbers / Switch-off and maximum local numbers storage | Rel-15 | C92 | UEs supporting 5G Core and emergency services in NR connected to 5GCN |
| 11.4.9 | 5GMM-DEREGISTERED.LIMITED-SERVICE No suitable cells in tracking area / Emergency call establishment and release | Rel-15 | C92 | UEs supporting 5G Core and emergency services in NR connected to 5GCN |
| 11.4.10 | Void |  |  |  |
| 11.4.10a | 5GMM-REGISTERED.NORMAL-SERVICE / N26 interface not supported / N1 mode to S1 mode transfer of an existing emergency PDU session | Rel-15 | C85B | UEs supporting 5G core and Emergency PDU session transfer from N1 mode to S1 mode when network does not support N26 interface, and, E-UTRA and EPS IMS emergency call (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") and IMS voice over NR |
| 11.4.11 | 5GMM-REGISTERED.NORMAL-SERVICE / N26 interface not supported / S1 mode to N1 mode transfer of an existing emergency PDN connection | Rel-15 | C85A | UEs supporting 5G core and Emergency PDN connection transfer from S1 mode to N1 mode when network does not support N26 interface, and, E-UTRA and EPS IMS emergency call (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") and emergency services in NR connected to 5GCN |
| 11.4.12 | 5GMM-REGISTERED.NORMAL-SERVICE / 5GMM-IDLE / Emergency call / Disabling N1 mode / Emergency call establishment over EPS / Success | Rel-15 | C176 | UEs supporting 5G Core and E-UTRA and EPS IMS emergency call (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") |
| 11.4.13 | 5GMM-REGISTERED.NORMAL-SERVICE / 5GMM-IDLE / Emergency call / obtaining new IP address different than the IP address | Rel-15 | C92 | UEs supporting 5G Core and emergency services in NR connected to 5GCN |
| 11.4.14 | 5GMM-REGISTERED.NORMAL-SERVICE / 5GMM-IDLE / Emergency call /Deregistration upon emergency registration expiration | Rel-15 | C92 | UEs supporting 5G Core and emergency services in NR connected to 5GCN |
| 11.5 | eCall over IMS |  |  |  |
| 11.5.1 | eCall Only mode / T3444 / eCall inactivity procedure / Removal of eCall only restriction after an eCall over IMS / 5GS to EPS | Rel-16 | C170 | UEs supporting 5G Core and E-UTRA and IMS eCall Only type of emergency services over 5GS and Manual type of eCall initiation |
| 11.5.2 | eCall Only mode / T3445 / eCall inactivity procedure / Removal of eCall only restriction after a call to URI for test service / 5GS to EPS | Rel-16 | C171 | UEs supporting 5G Core and E-UTRA and IMS eCall Only type of emergency services over 5GS and Manual type of eCall initiation and capable of triggering a Test eCall |
| 11.5.3 | eCall Only mode / 5GS supports IMS voice over PS session / 5GS does not support emergency service / eCall over EPS / eCall failure if EPS and CS domain are not available | Rel-16 | C197 | UEs supporting 5G Core and E-UTRA and IMS eCall Only type of emergency services over 5GS and Automatic type of eCall initiation |
| 11.5.4 | eCall Only mode / 5GS supports IMS voice over PS session / 5GS supports emergency service / eCall over IMS is supported on 5GS / RACH failure in NR cell / eCall over EPS | Rel-16 | C197 | UEs supporting 5G Core and E-UTRA and IMS eCall Only type of emergency services over 5GS and Automatic type of eCall initiation |
| 11.5.5 | eCall Only mode / Limited service state / Call to URI for test service should not be attempted / eCall over IMS should be attempted / 5GS | Rel-16 | C174 | UEs supporting 5G Core and IMS eCall Only type of emergency services over 5GS and Manual type of eCall initiation and capable of triggering a Test eCall |
| 11.5.6 | eCall capable / 5GS supports IMS voice over PS session / 5GS supports emergency service / eCall over IMS is not supported / eCall using the CS domain / emergency call over IMS if eCall using the CS domain is not available / UTRA | Rel-16 | C185 | UEs supporting 5G Core and IMS eCall type of emergency services over 5GS and Automatic type of eCall initiation and emergency services in NR connected to 5GCN |
| 11.5.7 | eCall Only mode / SRVCC Handover to CS domain / UTRAN / MSD Update / Success / 5GS | Rel-16 | C186 | UEs supporting 5G Core and UTRA and IMS eCall Only type of emergency services over 5GS and Manual type of eCall initiation and NR to UTRA-FDD CELL_DCH CS handover |
| 11.5.8 | eCall Only mode / 5GS supports IMS voice over PS session / 5GS supports emergency service / eCall over IMS is supported / RACH failure in NR cell / eCall using the CS domain | Rel-16 | C188 | UEs supporting 5G Core and (UTRA OR GERAN) and IMS eCall Only type of emergency services over 5GS and Automatic type of eCall initiation |
| 11.5.9 | eCall only mode / Manual initiation / Emergency registration / Abnormal case / IM CN sends a 486 (Busy Here) / UE performs eCall in CS domain / UTRAN or GERAN / 5GS | Rel-16 | C187 | UEs supporting 5G Core and (UTRA OR GERAN) and IMS eCall Only type of emergency services over 5GS and Manual type of eCall initiation |
| 11.5.10 | eCall only mode / Automatic initiation / Emergency registration / Abnormal case / IM CN sends a 600 (Busy Everywhere) / UE performs eCall in CS domain / UTRAN or GERAN / 5GS | Rel-16 | C188 | UEs supporting 5G Core and (UTRA OR GERAN) and IMS eCall Only type of emergency services over 5GS and Automatic type of eCall initiation |
| 11.5.11 | eCall only mode / Automatic initiation / Emergency registration / Abnormal case / IM CN sends a 603 (Decline) / UE performs eCall in CS domain / UTRAN or GERAN / 5GS | Rel-16 | C188 | UEs supporting 5G Core and (UTRA OR GERAN) and IMS eCall Only type of emergency services over 5GS and Automatic type of eCall initiation |
| 11.5.12 | eCall Only mode / 5GS supports IMS voice over PS session / 5GS supports emergency service / eCall over IMS is not supported on 5GS / eCall over EPS | Rel-16 | C197 | UEs supporting 5G Core and E-UTRA and IMS eCall Only type of emergency services over 5GS and Automatic type of eCall initiation |
| 11.5.13 | eCall over IMS / Manual initiation / MSD transfer Failure / UE performs eCall in CS domain after Timer expiry / UTRAN or GERAN / 5GS | Rel-16 | C189 | UEs supporting 5G Core and (UTRA OR GERAN) and eCall type of emergency services over 5GS and Manual type of eCall initiation |
| 11.5.14 | eCall Only mode / 5GS supports IMS voice over PS session / 5GS does not support emergency service / eCall using CS domain | Rel-16 | C188 | UEs supporting 5G Core and (UTRA OR GERAN) and IMS eCall Only type of emergency services over 5GS and Automatic type of eCall initiation |
| 11.6 | 3GPP PS Data Off |  |  |  |
| 11.6.1 | Data Off / MO Voice Call | Rel-15 | C162 | UEs supporting 5G Core and NG.114 v1.0 default configuration voice exempt and 3GPP PS data off and Initiating session and MTSI speech |
| 11.6.2 | Data Off / MO Video Call | Rel-15 | C172 | UEs supporting 5G Core and NG.114 v2.0 default configuration video exempt and 3GPP PS data off and Initiating session and MTSI video |
| 11.6.3 | Data Off / SMSoIP | Rel-15 | C162A | UEs supporting 5G Core and NG.114 v2.0 and 3GPP PS data off and Initiating session and SMS over IP |
| 11.7 | eDRX |  |  |  |
| 11.7.1 | eDRX / IDLE | Rel-17 | C210 | UEs supporting 5G Core and eDRX |
| 11.7.2 | eDRX / Inactive / RAN-initiated paging | Rel-17 | C210 | UEs supporting 5G Core and eDRX |
| 11.8 | Inter-system mobility between untrusted Non-3GPP and 3GPP system |  |  |  |
| 11.8.2 | Inter-system mobility between untrusted Non-3GPP and 3GPP system/Handover from N3IWF/5GC to NR / UE in 5GMM-DEREGISTERED states | Rel-15 | C248 | UEs supporting 5G Core and handover from 5G Core over non-3GPP Access Network to 5G Core Network |
| 11.8.4 | Inter-system mobility between untrusted Non-3GPP and 3GPP system/Handover from N3IWF/5GC to E-UTRAN/EPC | Rel-15 | C249 | UEs supporting 5G Core and handover from 5G Core over non-3GPP Access Network to EPC Network |
| 11.8.5 | Inter-system mobility between untrusted Non-3GPP and 3GPP system/Handover from 5GS to EPC/ePDG | Rel-15 | C208 | UEs supporting 5G Core and IMS and handover from 5G Core to EPC over non-3GPP Access Network and GSMA PRD IR.51: "IMS Profile for Voice, Video and SMS over Wi-Fi" and WLAN. |
| 11.8.6 | Inter-system mobility between untrusted Non-3GPP and 3GPP system/Handover from EPC/ePDG to 5GS/ UE in 5GMM-DEREGISTERED and EMM-DEREGISTERED states | Rel-15 | C237 | UEs supporting WLAN and GSMA PRD IR.51: "IMS Profile for Voice, Video and SMS over Wi-Fi" and handover from EPC over non-3GPP Access Network to 5G Core and IMS and 5G Core. |

<Unchanged Sections Skipped>

## 4.2	Protocol conformance test cases Applicability Condition

Table 4.2-1: Applicability of Protocol conformance test cases Conditions

| Condition |  | Test case Selection Expression |  | Comment |  |
| --- | --- | --- | --- | --- | --- |
| C01 |  | IF A.4.1-3/2 THEN R ELSE N/A |  | UEs supporting EN-DC |  |
| C02 |  | IF (A.4.3.4-1/2 OR A.4.3.4-1/3) THEN R ELSE N/A |  | UEs supporting 5GS and RLC UM Mode |  |
| C03 |  | IF A.4.3.5-1/1 THEN R ELSE N/A |  | UEs supporting 5GS and Long DRX Cycle |  |
| C04 |  | IF A.4.3.5-1/2 THEN R ELSE N/A |  | UEs supporting 5GS and short DRX cycle |  |
| C05 |  | IF A.4.3.4-1/3 THEN R ELSE N/A |  | UEs supporting 5GS and RLC UM with 6-bit length of RLC sequence number |  |
| C06 |  | IF A.4.3.4-1/2 THEN R ELSE N/A |  | UEs supporting 5GS and RLC UM with 12-bit length of RLC sequence number |  |
| C07 |  | IF A.4.3.4-1/1 THEN R ELSE N/A |  | UEs supporting 5GS and RLC AM with 12-bit length of RLC sequence number |  |
| C07A |  | IF A.4.3.4-1/1A THEN R ELSE N/A |  | UEs supporting 5GS and RLC AM with 18-bit length of RLC sequence number |  |
| C08 |  | IF A.4.3.3-1/1 THEN R ELSE N/A |  | UEs supporting 5GS and 12-bit length of PDCP sequence number |  |
| C08A |  | IF A.4.3.3-1/1A THEN R ELSE N/A |  | UEs supporting 5GS and 18-bit length of PDCP sequence number |  |
| C09 |  | IF [10] A.4.4-1/99 THEN R ELSE N/A |  | UEs supporting 5GS and ZUC Algorithm |  |
| C10 |  | IF A.4.1-3/2 AND A.4.3.7-1/2 THEN R ELSE N/A |  | UEs supporting EN-DC and UL transmission via both MCG path and SCG path for the split DRB |  |
| C11 |  | IF (A.4.3.2-1/2 OR A.4.3.2-1/3) THEN R ELSE N/A |  | UEs supporting 5GS and 256QAM for PDSCH for FR1/FR2 |  |
| C12 |  | IF (A.4.3.2-1/4) THEN R ELSE N/A |  | UEs supporting 5GS and 256QAM for PUSCH |  |
| C13 |  | IF A.4.1-3/2 AND A.4.3.6-1/1 THEN R ELSE N/A |  | UEs supporting EN-DC and NR measurements and Event A triggered reporting |  |
| C14 |  | IF A.4.1-3/2 AND A.4.3.6-1/1 AND A.4.3.6-1/3 THEN R ELSE N/A |  | UEs supporting EN-DC and NR measurements and Event A triggered reporting and (NR Intra-frequency and NR-Inter frequency measurements and at least periodical reporting) |  |
| C15 |  | IF A.4.1-3/2 AND A.4.3.6-1/1 AND A.4.3.6-1/3 AND (A.4.3.6-1/4 OR A.4.3.6-1/40) THEN R ELSE N/A |  | UEs supporting EN-DC and NR measurements and Event A triggered reporting and (NR Intra-frequency and Inter frequency measurements and at least periodical reporting) and CSI-RSRP and CSI-RSRQ measurement |  |
| C16 |  | IF A.4.1-3/2 AND [10] A.4.4-1/18 AND [10] A.4.4-1/19 THEN R ELSE N/A |  | UEs supporting EN-DC and UE requested bearer resource allocation and modification procedures |  |
| C17 |  | IF A.4.3.2-1/1 THEN R ELSE N/A |  | UEs supporting 5GS and PDSCH reception based on semi-persistent scheduling |  |
| C18 |  | IF A.4.3.2-1/10 THEN R ELSE N/A |  | UEs supporting 5GS and Type 1 PUSCH transmissions with configured grant |  |
| C19 |  | IF A.4.3.2-1/11 THEN R ELSE N/A |  | UEs supporting 5GS and Type 2 PUSCH transmissions with configured grant |  |
| C20 |  | IF A.4.3.2-1/12 THEN R ELSE N/A |  | UEs supporting 5GS and PDSCH aggregation |  |
| C21 |  | IF A.4.1-5/1 THEN R ELSE N/A |  | UEs supporting 5G Core |  |
| C21A |  | IF A.4.1-5/1 AND A.4.3.7-1/4 THEN R ELSE N/A |  | UEs supporting 5G Core and reflective QoS |  |
| C22 |  | IF A.4.1-3/2 AND A.4.3.7-1/3 THEN R ELSE N/A |  | UEs supporting EN-DC and SRB3 |  |
| C23 |  | IF A.4.1-3/2 AND A.4.3.7-1/3 AND A.4.3.7-1/1 THEN R ELSE N/A |  | UEs supporting EN-DC and SRB3 and (UL transmission via either MCG path or SCG path for the split SRB) |  |
| C24 |  | IF A.4.1-3/2 AND A.4.3.6-1/3 AND A.4.3.6-1/2 AND A.4.1-4/3 THEN R ELSE N/A |  | UEs supporting EN-DC and (NR intra-frequency and inter-frequency measurements and at least periodical reporting) and (two independent measurement gap configurations for FR1 and FR2) and Inter-Band EN-DC within FR1 |  |
| C25 |  | IF A.4.1-3/2 AND A.4.3.6-1/3 AND A.4.3.6-1/2 AND A.4.1-4/4 THEN R ELSE N/A |  | UEs supporting EN-DC and (NR intra-frequency and inter-frequency measurements and at least periodical reporting) and (two independent measurement gap configurations for FR1 and FR2) and Inter-Band EN-DC including FR2 |  |
| C26 |  | IF ([10] A.4.1-1/1 OR [10] A.4.1-1/2) THEN R ELSE N/A |  | UEs supporting 5GS and E-UTRA |  |
| C27 |  | IF A.4.1-5/1 AND A.4.3.6-1/1 THEN R ELSE N/A |  | UEs supporting 5G Core and NR measurements and Event A triggered reporting |  |
| C28 |  | IF A.4.3.2-1/13 THEN R ELSE N/A |  | UEs supporting 5GS and supplemental uplink with dynamic switch |  |
| C29 |  | IF A.4.1-5/2 AND [10] A.4.1-1/5 THEN R ELSE N/A |  | UEs supporting 5G core over non-3GPP Access Network and WLAN |  |
| C30 |  | IF A.4.1-5/2 AND A.4.3.7-1/6 AND [10] A.4.1-1/5 THEN R ELSE N/A |  | UEs supporting 5G core over non-3GPP Access Network and SMS over NAS and WLAN |  |
| C31 |  | IF A.4.1-5/1 AND A.4.3.6-1/5 THEN R ELSE N/A |  | UEs supporting 5G Core and Inter-RAT E-UTRA measurements and Event B triggered reporting |  |
| C32 |  | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) THEN R ELSE N/A |  | UEs supporting 5G Core and E-UTRA |  |
| C33 |  | IF A.4.1-5/1 AND A.4.3.7-1/6 AND NOT [10] A.4.4-2/32 THEN R ELSE N/A |  | UEs supporting 5G Core and SMS over NAS and UE configured to not use SMSoIP |  |
| C34 |  | IF A.4.1-5/1 AND [10] A.4.4-1/84 THEN R ELSE N/A |  | UEs supporting 5G Core and MinimumPeriodicSearchTimer |  |
| C35 |  | IF A.4.1-5/1 AND (A.4.3.7-1/8 OR A.4.3.7-1/7) THEN R ELSE N/A |  | UEs supporting 5G Core and (ETWS reception or CMAS reception) |  |
| C36 |  | IF A.4.1-5/1 AND [10] A.4.4-1/69 THEN R ELSE N/A |  | UEs supporting 5G Core and user initiated PLMN reselection in automatic mode on NR |  |
| C37 |  | IF A.4.1-5/1 AND (A.4.1-2/1 OR A.4.1-2/2) THEN R ELSE N/A |  | UEs supporting 5G Core and more than 1 FDD or TDD NR band |  |
| C38 |  | IF A.4.1-5/1 AND A.4.1-1/1 AND A.4.1-1/2 THEN R ELSE N/A |  | UEs supporting 5G Core and NR FDD and NR TDD |  |
| C39 |  | IF A.4.1-5/1 AND A.4.3.7-1/9 THEN R ELSE N/A |  | UEs supporting 5G Core and additional UE-requested PDU establishment |  |
| C40 |  | IF A.4.1-5/1 AND A.4.3.6-1/6 THEN R ELSE N/A |  | UEs supporting 5G Core and SS-SINR measurements |  |
| C41 |  | IF A.4.1-5/1 AND (A.4.1-4A/1 OR A.4.1-4A/3) THEN R ELSE N/A |  | UEs supporting 5G Core and intra-band contiguous CA |  |
| C42 |  | IF A.4.1-5/1 AND (A.4.1-4A/5 OR A.4.1-4A/6 OR A.4.1-4A/7) THEN R ELSE N/A |  | UEs supporting 5G Core and inter-band CA |  |
| C43 |  | IF A.4.1-5/1 AND (A.4.1-4A/2 OR A.4.1-4A/4) THEN R ELSE N/A |  | UEs supporting 5G Core and intra-band non-contiguous CA |  |
| C44 |  | IF (A.4.1-4A/1 OR A.4.1.4A/3) THEN R ELSE N/A |  | UEs supporting 5GS and intra-band contiguous CA |  |
| C45 |  | IF (A.4.1-4A/5 OR A.4.1-4A/6 OR A.4.1-4A/7) THEN R ELSE N/A |  | UEs supporting 5GS and inter-band CA |  |
| C46 |  | IF (A.4.1-4A/2 OR A.4.1.4A/4) THEN R ELSE N/A |  | UEs supporting 5GS and intra-band non-contiguous CA |  |
| C47 |  | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND [10] A.4.2.1.1-1/4 AND A.4.3.7-1/11 THEN R ELSE N/A |  | UEs supporting 5G Core and E-UTRA and EPS IMS emergency call (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") and Emergency Services Fallback in NR connected to 5GCN |  |
| C48 |  | Void |  |  |  |
| C49 |  | IF A.4.1-5/1 AND A.4.3.6-1/2 THEN R ELSE N/A |  | UE supporting 5G Core and two independent measurement gap configurations for FR1 and FR2 |  |
| C50 |  | IF A.4.1-5/1 AND A.4.3.6-1/5 AND A.4.3.6-1/42 THEN R ELSE N/A |  | UEs supporting 5G Core and Inter-RAT E-UTRA measurements and Event B triggered reporting and E-UTRA RS-SINR measurements |  |
| C51 |  | IF A.4.3.2-1/21 THEN R ELSE N/A |  | UEs supporting 5GS and PUSCH aggregation |  |
| C52 |  | IF A.4.1-5/1 AND A.4.3.6-1/1 AND A.4.3.6-1/3 AND (A.4.3.6-1/4 OR A.4.3.6-1/40) THEN R ELSE N/A |  | UEs supporting 5G Core and NR measurements and Event A triggered reporting and (NR Intra-frequency and Inter frequency measurements and at least periodical reporting) and CSI-RSRP and CSI-RSRQ measurement |  |
| C53 |  | IF A.4.3.5-1/4 THEN R ELSE N/A |  | UEs supporting 5GS and Logical Channel SR-Delay Timer |  |
| C54 |  | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND [10] A.4.4-1/33 AND A.4.3.7-1/12 THEN R ELSE N/A |  | UEs supporting 5G Core and E-UTRA and EPS IMS Voice (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") and EPS fallback |  |
| C55 |  | IF A.4.1-3/2 AND A.4.3.6-1/1 AND (A.4.1-4A/1 OR A.4.1-4A/3) THEN R ELSE N/A |  | UEs supporting EN-DC and NR measurements and Event A triggered reporting and intra-band contiguous CA |  |
| C56 |  | IF A.4.1-3/2 AND A.4.3.6-1/1 AND (A.4.1-4A/5 OR A.4.1-4A/6 OR A.4.1-4A/7) THEN R ELSE N/A |  | UEs supporting EN-DC and NR measurements and Event A triggered reporting and inter-band CA |  |
| C57 |  | IF A.4.1-3/2 AND A.4.3.6-1/1 AND (A.4.1-4A/2 OR A.4.1-4A/4) THEN R ELSE N/A |  | UEs supporting EN-DC and NR measurements and Event A triggered reporting and intra-band non-contiguous CA |  |
| C58 |  | IF A.4.1-5/2 AND [10] A.4.1-1/5.AND A.4.4-1/1 |  | UEs supporting 5G core over non-3GPP Access Network, WLAN and (ICMP or ICMP IPv6) |  |
| C59 |  | IF A.4.1-5/1 AND A.4.3.6-1/8 THEN R ELSE N/A |  | UEs supporting 5G Core and Support acquisition of relevant information from a neighbouring intra-frequency or inter-frequency NR cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when EN-DC is not configured |  |
| C60 |  | IF A.4.1-5/1 AND A.4.3.6-1/7 THEN R ELSE N/A |  | UEs supporting 5G Core and Support acquisition of relevant information from a neighbouring E-UTRA cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when the EN-DC is not configured |  |
| C61 |  | IF A.4.1-3/2 AND A.4.3.3-1/6 THEN R ELSE N/A |  | UEs supporting EN-DC and PDCP duplication over split SRB1/2 |  |
| C62 |  | IF A.4.1-3/2 AND A.4.3.3-1/4 THEN R ELSE N/A |  | UEs supporting EN-DC and PDCP duplication over split DRB |  |
| C63 |  | IF A.4.1-5/1 AND A.4.3.7-1/13 THEN R ELSE N/A |  | UEs supporting 5G Core and UE requested PDU session modification procedure |  |
| C64 |  | IF A.4.3.2-1/23 THEN R ELSE N/A |  | UEs supporting 5GS and The maximum number of spatial multiplexing layer(s) supported by the UE for DL reception is 8 Layers. For single CC standalone NR, it is mandatory with capability signalling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory for the given UE and at least 2 MIMO layers in FR2. If absent, the UE doesn’t support MIMO on this carrier |  |
| C65 |  | IF A.4.3.2-1/23 AND (A.4.3.2-1/4) THEN R ELSE N/A |  | UEs supporting 5GS and The maximum number of spatial multiplexing layer(s) supported by the UE for DL reception is 8 Layers. For single CC standalone NR, it is mandatory with capability signalling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory for the given UE and at least 2 MIMO layers in FR2. If absent, the UE doesn’t support MIMO on this carrier |  |
| C66 |  | IF (A.4.3.2-1/24 OR A.4.3.2-1/24A) AND ((A.4.3.2-1/42 OR A.4.3.2-1/42a OR A.4.3.2-1/42b) OR (A.4.3.2-1/43 OR A.4.3.2-1/43a OR A.4.3.2-1/43b)) THEN R ELSE N/A |  | UEs supporting 5GS and (DCI and timer based active BWP switching delay type1 or type2) and ((BWP adaptation up to 2 NR FR1 FDD or NR FR1 TDD or NR FR2) or (BWP adaptation up to 4 NR FR1 FDD or NR FR1 TDD or NR FR2)) |  |
| C67 |  | IF A.4.1-3/2 AND (A.4.1-4A/1 OR A.4.1-4A/3) THEN R ELSE N/A |  | UEs supporting EN-DC and Intra-Band Contiguous CA |  |
| C68 |  | IF A.4.1-3/2 AND (A.4.1-4A/2 OR A.4.1-4A/4) THEN R ELSE N/A |  | UEs supporting EN-DC and Intra-Band Non-Contiguous CA |  |
| C69 |  | IF A.4.1-3/2 AND (A.4.1-4A/5 OR A.4.1-4A/6 OR A.4.1-4A/7) THEN R ELSE N/A |  | UEs supporting EN-DC and Inter-Band CA |  |
| C70 |  | IF A.4.3.5-1/1 AND A.4.3.5-1/2 THEN R ELSE N/A |  | UEs supporting 5GS and Long DRX Cycle and Short DRX Cycle |  |
| C71 |  | IF A.4.1-3/2 AND A.4.3.7-1/3 AND A.4.3.6-1/3 THEN R ELSE N/A |  | UEs supporting EN-DC and SRB3 and NR intra-frequency and inter-frequency measurements and at least periodical reporting |  |
| C72 |  | IF A.4.1-5/1 AND (A.4.1-4A/1 OR A.4.1-4A/3) AND A.4.3.3-1/3 AND A.4.3.2A.1-2/1 THEN R ELSE N/A |  | UEs supporting 5G Core and intra-band contiguous CA and CA-based PDCP duplication over MCG or SCG DRB and UL NR CA with 2 carriers |  |
| C73 |  | IF A.4.1-5/1 AND (A.4.1-4A/5 OR A.4.1-4A/6 OR A.4.1-4A/7) AND A.4.3.3-1/3 AND A.4.3.2A.1-2/1 THEN R ELSE N/A |  | UEs supporting 5G Core and inter-band CA and CA-based PDCP duplication over MCG or SCG DRB and UL NR CA with 2 carriers |  |
| C74 |  | IF A.4.1-5/1 AND (A.4.1-4A/2 OR A.4.1-4A/4) AND A.4.3.3-1/3 AND A.4.3.2A.1-2/1 THEN R ELSE N/A |  | UEs supporting 5G Core and intra-band non-contiguous CA and CA-based PDCP duplication over MCG or SCG DRB and UL NR CA with 2 carriers |  |
| C75 |  | IF A.4.1-3/2 AND A.4.3.7-1/3 AND (A.4.1-4A/1 OR A.4.1-4A/3) AND A.4.3.3-1/3 AND A.4.3.2B.2.0-2A/2 THEN R ELSE N/A |  | UEs supporting EN-DC and SRB3 and intra-band contiguous CA and CA-based PDCP duplication over MCG or SCG DRB and EN-DC with 2 NR UL carriers |  |
| C76 |  | IF A.4.1-3/2 AND A.4.3.7-1/3 AND (A.4.1-4A/5 OR A.4.1-4A/6 OR A.4.1-4A/7) AND A.4.3.3-1/3 AND A.4.3.2B.2.0-2A/2 THEN R ELSE N/A |  | UEs supporting EN-DC and SRB3 and inter-band CA and CA-based PDCP duplication over MCG or SCG DRB and EN-DC with 2 NR UL carriers |  |
| C77 |  | IF A.4.1-3/2 AND A.4.3.7-1/3 AND (A.4.1-4A/2 OR A.4.1-4A/4) AND A.4.3.3-1/3 AND A.4.3.2B.2.0-2A/2 THEN R ELSE N/A |  | UEs supporting EN-DC and SRB3 and intra-band non-contiguous CA and CA-based PDCP duplication over MCG or SCG DRB and EN-DC with 2 NR UL carriers |  |
| C78 |  | IF A.4.1-5/1 AND [9] A.3A/50 AND [9] A.4/2B AND [9] A.15/1 AND [9] A.3A/61 THEN R ELSE N/A |  | UEs supporting 5G Core and Initiating session and MTSI speech and SMS over IP |  |
| C79 |  | IF A.4.1-5/1 AND [9] A.3A/50 AND [9] A.4/2B AND [9] A.15/3 THEN R ELSE N/A |  | UEs supporting 5G Core and Initiating session and MTSI video |  |
| C80 |  | IF A.4.1-4/6 THEN R ELSE N/A |  | UEs supporting NR-DC |  |
| C81 |  | IF A.4.1-5/1 AND (A.4.1-4A/1 OR A.4.1.4A/3) AND A.4.3.2A.1-2/1 THEN R ELSE N/A |  | UEs supporting 5G Core and intra-band contiguous CA and UL NR CA with 2 carriers |  |
| C81A |  | IF A.4.1-3/2 AND A.4.1-4/1 AND A.4.3.2B.2.0-2A/2 THEN R ELSE N/A |  | UEs supporting EN-DC and intra-band contiguous CA and EN-DC with 2 NR UL carriers |  |
| C82 |  | IF A.4.1-5/1 AND (A.4.1-4A/5 OR A.4.1-4A/6 OR A.4.1-4A/7) AND A.4.3.2A.1-2/1 THEN R ELSE N/A |  | UEs supporting 5G Core and inter-band CA and UL NR CA with 2 carriers |  |
| C82A |  | IF A.4.1-3/2 AND (A.4.1-4/3 OR A.4.1-4/4 OR A.4.1-4/5) AND A.4.3.2B.2.0-2A/2 THEN R ELSE N/A |  | UEs supporting EN-DC and inter-band CA and EN-DC with 2 NR UL carriers |  |
| C83 |  | IF A.4.1-5/1 AND (A.4.1-4A/2 OR A.4.1.4A/4) AND A.4.3.2A.1-2/1 THEN R ELSE N/A |  | UEs supporting 5G Core and intra-band non-contiguous CA and UL NR CA with 2 carriers |  |
| C83A |  | IF A.4.1-3/2 AND A.4.1-4/2 AND A.4.3.2B.2.0-2A/2 THEN R ELSE N/A |  | UEs supporting EN-DC and intra-band non-contiguous CA and EN-DC with 2 NR UL carriers |  |
| C84 |  | IF A.4.1-5/1 AND [10] A.4.4-1/99 THEN R ELSE N/A |  | UEs supporting 5G Core and ZUC algorithm |  |
| C85 |  | Void |  |  |  |
| C85A |  | IF (A.4.1-5/1 AND A.4.4-2/9) AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND [10] A.4.2.1.1-1/4 AND A.4.3.7-1/14 THEN R ELSE N/A |  | UEs supporting 5G core and Emergency PDN connection transfer from S1 mode to N1 mode when network does not support N26 interface, and, E-UTRA and EPS IMS emergency call (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") and emergency services in NR connected to 5GCN |  |
| C85B |  | IF (A.4.1-5/1 AND A.4.4-2/8) AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND [10] A.4.2.1.1-1/4 AND A.4.3.7-1/32 THEN R ELSE N/A |  | UEs supporting 5G core and Emergency PDU session transfer from N1 mode to S1 mode when network does not support N26 interface, and, E-UTRA and EPS IMS emergency call (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") and IMS voice over NR |  |
| C86 |  | IF A.4.1-4/6 AND A.4.3.7-1/3 THEN R ELSE N/A |  | UEs supporting NR-DC and SRB3 |  |
| C87 |  | IF A.4.1-4/6 AND A.4.3.7-1/3 AND A.4.3.6-1/3 THEN R ELSE N/A |  | UEs supporting NR-DC and SRB3 and NR intra-frequency and inter-frequency measurements and at least periodical reporting |  |
| C88 |  | IF A.4.1-4/6 AND A.4.3.7-1/3 AND (A.4.1-4A/1 OR A.4.1-4A/3) AND A.4.3.3-1/3 AND A.4.3.2A.1-2/1 THEN R ELSE N/A |  | UEs supporting NR-DC and SRB3 and intra-band contiguous CA and CA-based PDCP duplication over MCG or SCG DRB and UL NR CA with 2 carriers |  |
| C89 |  | IF A.4.1-4/6 AND A.4.3.7-1/3 AND (A.4.1-4A/5 OR A.4.1-4A/6 OR A.4.1-4A/7) AND A.4.3.3-1/3 AND A.4.3.2A.1-2/1 THEN R ELSE N/A |  | UEs supporting NR-DC and SRB3 and inter-band CA and CA-based PDCP duplication over MCG or SCG DRB and UL NR CA with 2 carriers |  |
| C90 |  | IF A.4.1-4/6 AND A.4.3.7-1/3 AND (A.4.1-4A/2 OR A.4.1-4A/4) AND A.4.3.3-1/3 AND A.4.3.2A.1-2/1 THEN R ELSE N/A |  | UEs supporting NR-DC and SRB3 and intra-band non-contiguous CA and CA-based PDCP duplication over MCG or SCG DRB and UL NR CA with 2 carriers |  |
| C91 |  | IF A.4.1-5/1 AND [10] A.4.4-1/98 THEN R ELSE N/A |  | UEs supporting 5G Core and ManualModeNetworkSelectionException |  |
| C92 |  | IF A.4.1-5/1 AND A.4.3.7-1/14 THEN R ELSE N/A |  | UEs supporting 5G Core and emergency services in NR connected to 5GCN |  |
| C93 |  | IF A.4.1-3/2 AND A.4.3.6-1/1 AND A.4.3.6-1/3 AND (A.4.1-2/1 OR A.4.1-2/2 OR (A.4.1-1/1 AND A.4.1-1/2)) THEN R ELSE N/A |  | UEs supporting EN-DC and NR measurements and Event A triggered reporting and (NR Intra-frequency and NR-Inter frequency measurements and at least periodical reporting) and multiple NR bands |  |
| C94 |  | IF A.4.1-5/1 AND (A.4.1-2/1 OR A.4.1-2/2 OR (A.4.1-1/1 AND A.4.1-1/2)) THEN R ELSE N/A |  | UEs supporting 5G Core and multiple NR bands |  |
| C95 |  | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND [10] A.4.4-1/33 AND A.4.3.7-1/12 AND A.4.3.7-1/15 THEN R ELSE N/A |  | UEs supporting 5G Core and E-UTRA and EPS IMS Voice (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") and EPS fallback and voiceFallbackIndication |  |
| C96 |  | IF A.4.1-5/1 AND A.4.1-3/2 AND A.4.3.8-1/10 THEN R ELSE N/A |  | UEs supporting 5G Core and EN-DC and inter-RAT Handover from NR to EN-DC |  |
| C97 |  | IF A.4.1-4/6 AND A.4.3.7-1/2 THEN R ELSE N/A |  | UEs supporting NR-DC and UL transmission via both MCG path and SCG path for the split DRB |  |
| C98 |  | IF A.4.1-4/6 AND A.4.3.3-1/4 THEN R ELSE N/A |  | UEs supporting NR-DC and PDCP duplication over split DRB |  |
| C99 |  | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND (A.4.3.8-1/6 OR A.4.3.8-1/7 OR A.4.3.8-1/8)THEN R ELSE N/A |  | UEs supporting 5G Core and E-UTRA and (inter-RAT Handover to NR FR1 TDD from EUTRA connected to EPC or inter-RAT Handover to NR FR1 FDD from EUTRA connected to EPC or inter-RAT Handover to NR FR2 TDD from EUTRA connected to EPC) |  |
| C100 |  | IF A.4.1-5/1 AND [9] A.15/1 AND A.4.3.5-1/9 THEN R ELSE N/A |  | UEs supporting 5G Core and MTSI speech and bit rate recommendation query message |  |
| C101 |  | IF A.4.1-5/1 AND A.4.3.8-1/9 THEN R ELSE N/A |  | UEs supporting 5G Core and intra-frequency DAPS handover |  |
| C102 |  | IF A.4.3.2-1/30 THEN R ELSE N/A |  | UEs supporting 5GS and cross slot scheduling |  |
| C103 |  | IF A.4.3.5-1/1 AND A.4.3.5-1/5 THEN R ELSE N/A |  | UEs supporting 5GS and Long DRX Cycle and DRX adaptation |  |
| C104 |  | IF (A.4.1-4A/1 OR A.4.1-4A/3) AND A.4.3.2A.1-1/2 AND A.4.3.2A.1-2/2 AND A.4.3.3-1/5 THEN R ELSE N/A |  | UEs supporting 5GC and Intra-band contiguous CA and DL and UL NR CA with 3 carriers and PDCP duplication with more than two RLC entities |  |
| C105 |  | IF (A.4.3.4-1/2 OR A.4.3.4-1/3) AND A.4.3.3-1/7 THEN R ELSE N/A |  | UEs supporting 5GS and RLC UM mode and PDCP ethernet header compression |  |
| C106 |  | IF A.4.1-5/1 AND A.4.3.10-1/1 THEN R ELSE N/A |  | UE supporting 5G core and NR sidelink mode 1 transmission |  |
| C107 |  | IF A.4.3.2-1/32 THEN R ELSE N/A |  | UE’s supporting multi-DCI based multi-TRP |  |
| C108 |  | IF A.4.1-5/1 AND A.4.3.7-1/17 THEN R ELSE N/A |  | UEs supporting 5G Core and RACS |  |
| C109 |  | IF A.4.1-5/1 AND A.4.3.7-1/19 THEN R ELSE N/A |  | UEs supporting 5G Core and RRC_INACTIVE |  |
| C109A |  | IF A.4.1-5/1 AND A.4.3.7-1/19 AND A.4.4-2/10 THEN R ELSE N/A |  | UEs supporting 5G Core and RRC_INACTIVE and UE’s usage setting as data centric |  |
| C110 |  | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND A.4.3.7-1/19 THEN R ELSE N/A |  | UEs supporting 5G Core and E-UTRA and RRC_INACTIVE |  |
| C111 |  | IF A.4.1-5/1 AND (A.4.3.7-1/8 OR A.4.3.7-1/7) AND A.4.3.7-1/19 THEN R ELSE N/A |  | UEs supporting 5G Core and (ETWS reception or CMAS reception) and RRC_INACTIVE |  |
| C112 |  | Void |  |  |  |

## {End of changes}