3GPP TSG-RAN5 Meeting #99	R5-232234r3

# Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2364 | rev | - | Current version: | 17.6.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to PRS based UE Rx-Tx measurement FR2 SA test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IIOT_URLLC_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2023-04-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | PRS based UE Rx-Tx measurement test case has several missing parts and need to be updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updated core requirements. - Completed the editor’s notes. - Added TTs to test case 7.6.13.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case implementation will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6.13.1, F.1.1.2, F.1.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR 38.903 CR 0524 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Corrected the AoA setup configuration in table 7.6.13.1.1.5-1 R2: Removed the reference to NOTE 4 in table 7.6.13.1.1.5-1 R3: Corrected the clause numbering |  |  |  |  |  |  |  |  |

## 7.6	Measurement procedures

-------- Start of first change --------

### 7.6.7 to 7.6.12

### 7.6.13	UE Rx-Tx time difference measurements for PDC

##### 7.6.13.0	Minimum conformance requirements

##### 7.6.13.0.1	PRS based UE Rx-Tx time difference for propagation delay compensation

When UE is configured with prs-Ref-r17 in MeasObjectRxTxDiff-r17 defined in TS 38.331 [2] and provided with PRS resource configuration, the UE shall be able to measure UE Rx-Tx time difference on PCell within the measurement period TUERx-Tx_PRS, where:

TUERxTx, PRS=NPRSslotN'*Nsample*Klayer3*LCM(TPRS, TDRX)

Where:

NPRSslot is the maximum number of DL PRS resources configured in a slot,

N’ is UE capability for number of DL PRS resources that it can process in a slot corresponding to maxNumberPRS-ResourceProcessedPerSlot-r17 as specified in TS38.331 [2],

Nsample is the number of UE Rx-Tx time difference measurement samples and Nsample= 4,

TPRS is the PRS resource periodicity specific for RTT-based propagation delay compensation,

TDRX is the DRX cycle length when DRX is in use, 1ms otherwise.

Klayer3 is

Ntotal / Navailable, when Navailable>0

Where,

-	For a window W of duration LCM(TPRS, MGRP_max, TSMTC), where TSMTC is the periodicity of SMTC for intra-frequency measurement without gap, and MGRP_max is the maximum MGRP across all configured per-UE measurement gaps and per-FR measurement gaps within the same FR as the PDC PRS for UE supporting concurrentMeasGap-r17, or is the MGRP of configured per-UE measurement gap or per-FR measurement gap within the same FR as the PDC PRS for UE not supporting concurrentMeasGap-r17,starting at the beginning of any PRS resource occasion:

-	Ntotal is the total number of PRS resource occasions within the window W, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and

-	Navailable is the number of PRS resource occasions that are not overlapped with:

-	For FR2: any measurement gap occasions after accounting for measurement gap collisions by applying the measurement gap collision rule in section 9.1.8.3 nor any SMTC occasions within the window W.

No requirements shall apply if Navailable=0.

UE is only required to perform UE Rx-Tx time difference on PRS within the active DL BWP.

When UE is configured to perform UE Rx-Tx time difference measurement based on PRS, the requirements apply provided that the SCS of the PRS is same as that of the active BWP on PCell.

For FR2, the requirements apply if QCL information is provided for all the PRS resources in the resource set and for each resource one RS has QCL-TypeD.

If PRS resources overlap with Type 1A/1B/2 PPW, the UE is allowed longer measurement period to measure UE Rx-Tx time difference on PCell.

The UE Rx-Tx time difference measurement values contained in measurement report shall be based on the

measurement report mapping requirements specified in clause 10.1.25.3, with k = 5.

The reporting range for the absolute UE Rx-Tx time difference measurement (TUE Rx-Tx) is defined from -985024×Tc to 985024×Tc with the resolution step of 2k×Tc.

Table 7.6.13.0.1-1: Absolute UE Rx-Tx time difference measurement report mapping for k=5

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFFERENCE_0000 | TUE Rx-Tx < -985024 | Tc |
| RX-TX_TIME_DIFFERENCE_0001 | -985024  TUE Rx-Tx < -984992 | Tc |
| RX-TX_TIME_DIFFERENCE_0002 | -984992  TUE Rx-Tx < -984960 | Tc |
|  |  | … |
| RX-TX_TIME_DIFFERENCE_30782 | -32  TUE Rx-Tx < 0 | Tc |
| RX-TX_TIME_DIFFERENCE_30783 | 0  TUE Rx-Tx < 32 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_61563 | 984960  TUE Rx-Tx < 984992 | Tc |
| RX-TX_TIME_DIFFERENCE_61564 | 984992  TUE Rx-Tx < 985024 | Tc |
| RX-TX_TIME_DIFFERENCE_61565 | 985024  TUE Rx-Tx | Tc |

The error in the reported value of UE Rx-Tx time difference measurement, including both the measurement error and the reporting quantization error, should be within the accuracy requirements specified in this clause.

The UE Rx-Tx time difference measurement accuracy requirements in this clause shall apply provided that:

-	The UE transmits SRS within [-160, 160] msec of at least one PDC DL PRS resource from the serving cell (PCell).

When a serving cell change occurs during the UE Rx-Tx measurement period, the UE Rx-Tx time difference measurement accuracy requirements in this clause shall apply provided that the serving cell change does not impact SRS configuration for the UE Rx-Tx measurement.

The accuracy requirements in Table 7.6.13.0.1-2 for FR2 are valid under the following conditions:

Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

PRP|dBm according to Annex [TBD] for a corresponding Band.

AWGN propagation condition.

Table 7.6.13.0.1-2: UE Rx-Tx time difference measurement accuracy in FR2 in AWGN

| Accuracy | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PRS Ês/Iot | Minimum PRS bandwidth | PRS SCS | PRS resource repetition(TrepPRS*LPRS/KcombPRSNote 3 | IoNote 4 range |  |
|  |  |  |  |  | Minimum IoNote 1 | Maximum Io |
| TcNote 5 | dB | RB | kHz |  | dBm / SCSPRS | dBm/BWChannel |
| ± [22+d+ℇ] | -3 | ≥[24] | 60 | ≥[1] | Same value as PRP in Table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| ± [15+d+ℇ] |  | ≥[64] |  | ≥[1] | NOTE 6 | NOTE 6 |
| ± [7+d+ℇ] |  | ≥[132] |  | ≥[1] | NOTE 6 | NOTE 6 |
| ± [12+d+ℇ] |  | ≥[32] | 120 | ≥[1] | Same value as PRP in Table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| ± [7+d+ℇ] |  | ≥[64] |  | ≥[1] | NOTE 6 | NOTE 6 |
| ± [4+d+ℇ] |  | ≥[128] |  | ≥[1] | NOTE 6 | NOTE 6 |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: NR operating band groups are as defined in Section 3.5. NOTE 3: TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeNdefined in TS 37.355 [34]. NOTE 4: The Io is defined in PRS slots. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same slot. NOTE 5: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 6: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 7: is the margin determined from Table 10.1.39.2-4. NOTE 8: ℇ is the margin for reporting quantitization error and ℇ=16 Tc. |  |  |  |  |  |  |

Table 7.6.13.0.1-3: Margin for UE Rx-Tx time difference measurement accuracy in FR2

| Min(PRS BW, SRS BW) (MHz) |  | Margin (Tc Note 1) |
| --- | --- | --- |
| SCS = 60 kHz | SCS = 120 kHz |  |
| ≥ 24 | N/A | [76] |
| ≥ 64 | ≥ 32 | [32] |
| ≥ 132 | ≥ 64 | [24] |
| N/A | ≥ 128 | [20] |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 2: If SRS and PRS have different SCS, the margin corresponding to the smallest RS BW in MHz applies. |  |  |

### 7.6.13.1	NR SA FR2 UE Rx-Tx time difference measurement for propagation delay compensation using PRS

Editor's Note: This test case has been completed for the following configurations:

- Test frequency f ≤ 40.8 GHz

- UE PC3

- The test is incomplete for UE power classes other than PC3

-

TUERxTx, PRS=NPRSslotN'*Nsample*Klayer3*LCM(TPRS, TDRX)

NPRSslot

N’

NsampleNsample

TPRS

TDRX

Klayer3

|  |  |  |  | (TrepPRS*LPRS/KcombPRS |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| TrepPRS, LPRS ,KcombPRS |  |  |  |  |  |  |

7.6.13.1.1	Test purpose

The purpose of the test is to verify that the UE Rx-Tx measurement for RTT-based PDC meets the requirements specified in TS 38.133[6] clause 9.12.4.1 in AWGN propagation condition in FR2 in standalone scenario.

7.6.13.1.2	Test applicability

This test applies to all types of NR UE release 17 onwards supporting rtt-BasedPDC-PRS-r17 or gNB-SideRTT-BasedPDC-r17.

7.6.13.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.6.13.1.0.

The normative reference for this requirement is TS 38.133 [6] clauses 9.12.4.1, 9.12.5, 10.1.39.2 and A.7.6.13.1.

7.6.13.1.4	Test description

7.6.13.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.6.13.1.4.1-1

Table 7.6.13.1.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 7.6.13.1-1 | 120 kHz SSB and PRS SCS, 100 MHz bandwidth, TDD duplex mode |

Configure the test requirement and the DUT according to the parameters in Table 7.6.13.1.4.1-2.

Table 7.6.13.1.4.1-2: Initial conditions for UE Tx-Rx time difference measurement for SA FR2

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.5-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.6.13.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.3.3.1-1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram |  |  |  |

1. The general test parameter settings are set up according to Table 7.6.13.1.4.1-3.

2. Message contents are defined in clause 7.6.13.1.4.3.

3. The AoA setup for this test is Setup 1 as defined in clause A.9. The UE RX Beam Peak direction has been obtained previously using one of the search procedures as described in Annex I.

Table 7.6.13.1.4.1-3: General test parameters

| Parameter | Unit | Test configuration | Value |
| --- | --- | --- | --- |
| Active cell |  | 1 | Cell 1 |
| RF Channel Number |  | 1 | 1 |
| BWchannel | MHz | 1 | 100: NRB,c = 66 |
| SSB configuration |  | 1 | SSB.2 FR2 |
| SMTC configuration |  | 1 | SMTC.1 |
| Measurement gap |  | 1 | GP#24 or GP#13 Note 1 |
| CP length |  | 1 | Normal |
| DRX |  | 1 | OFF |
| T1 | s | 1 | 5 |
| T2 | s | 1 | 20 |
| NOTE 1: GP#24 is configured if UE supports MG#24, otherwise GP#13 is configured. |  |  |  |

7.6.13.1.4.2	Test procedure

The test is considered with one cell (Cell 1) in FR2.

The test consists of two consecutive time intervals, with duration of T1 and T2. The Cell 1 mutes PRS transmission during T1 and transmits PRS during T2.

The MeasObjectRxTxDiff-r17 is configured with prs-Ref-r17, measObject  with measObjectRxTxDiff-r17, and NR-DL-PRS-PDC-Info is defined in as per 7.6.13.1.4.3 and shall be provided to the UE during T1.

The last TTI containing the RRC configuration shall be provided to the UE T ms before the start of T2, where T = [TBD] ms. The UE is configured to transmit SRS during T2.

The general and cell specific test parameters for PCell is given in Table 7.6.13.1.4.1-3 and Table 7.6.13.1.5-1, respectively. In the measurement control information, a measurement object is configured for the frequency of the PCell, and. The test consists of two successive time periods, with time duration of T1, and T2 respectively.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 7.6.13.1.5-1 as appropriate.

3. SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 7.6.13.1.5-1. T2 starts.

6. The UE shall transmit periodically MeasurementReport messages.

7. After 10s wait from Step 3, the SS shall check the result-k5-r17 reported values in the periodic MeasurementReport. The result-k5 value is outside the limits in Table 7.6.13.1.0-1 or the UE fails to report the measurement value for serving cell, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

8. The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

7.6.13.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table 4.6.3-76B in 38.508-1 [14] with condition PRS_REF |
| Specific message contents exceptions for Test Configuration 7.6.13.1-1 | Table 4.6.3-84AC in 38.508-1 [14] with condition SCS120 |

7.6.13.1.5	Test requirements

Table 7.6.13.1.4.1-3 and Table 7.6.13.1.5-1 define the primary level settings including test tolerances for UE Rx-Tx time difference measurement with PRS for RTT-based PDC in FR2 SA.

Table 7.6.13.1.5-1: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| AoA setup |  | 1 | Setup 1 as specified in clause A.9.1 |  |
| Beam Assumption |  | 1 | Rough |  |
| TDD configuration |  | 1 | TDDConf.3.1 |  |
| PDSCH RMC configuration |  | 1 | SR.3.1 TDD |  |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.1 TDD |  |
| OCNG Patterns |  | 1 | OP.1 |  |
| TRS Configuration |  | 1 | TRS.2.1 TDD |  |
| Initial BWP configuration |  | 1 | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1 | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1 | ULBWP.1.1 |  |
| PRS configuration |  | 1 | PRS.1.1 FR2 |  |
| PRS muting info |  | 1 | ‘10’ |  |
| SRS configuration |  | 1 | PDC-SRS.3 |  |
| Note 2 | dBm/SCS | 1 | -89+TT |  |
| Note 2 | dBm/15 kHz | 1 | -98+TT |  |
| PRS | dB | 1 | -Infinity | -2.41+TT |
| PRS | dB | 1 | -Infinity | -2+TT |
| PRS-RSRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -91+TT |
| Io | dBm/95.04 MHz | 1 | N/A | -57.63+TT |
| Propagation Condition |  | 1 | AWGN |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

Table 7.6.13.1.5-2: SRS Configuration for FR2 Propagation Delay Compensation

|  | PDC-SRS.3 |  |
| --- | --- | --- |
| Field |  | Comment |
| c-SRS | Same as NRB,c in the test case |  |
| b-SRS | 0 |  |
| b-hop | 0 | Frequency hopping is disabled |
| groupOrSequenceHopping | neither | No group or sequence hopping |
| freqDomainPosition | 0 | Frequency domain position of SRS |
| freqDomainShift | 0 |  |
| pathlossReferenceRS ssb-Index | 0 | SSB #0 is used for SRS path loss estimation |
| usage | usagePDC-r17 |  |
| startPosition | 5 | resourceMapping setting |
| nrofSymbols | 4 |  |
| repetitionFactor | n1 | without repetition. |
| transmissionComb | n4 |  |
| combOffset-n2 | 0 | transmissionComb setting |
| cyclicShift-n2 | 0 |  |
| nrofSRS-Ports | port1 | Number of antenna ports used for SRS transmission |
| resourceType | Periodic |  |
| periodicityAndOffset-p | Sl1280, 160 | SRS transmission periodicity |

The UE Rx-Tx time difference measurement time shall fulfils the requirements specified in clause 7.6.13.0.1. The UE shall perform and report the UE Rx-Tx time difference measurements for Cell 1 within the specified UE Rx-Tx time difference measurement time starting from the beginning of time interval T2.

The reported UE Rx-Tx measurement for each correct event shall be within the UE Rx-Tx reporting range specified in clause 10.1.25.3.1, for k=5.

The UE Rx-Tx time difference measurement time fulfils the UE Rx-Tx measurement accuracy requirements specified in clause 7.6.13.0.1 for Cell 1.

For the test to pass, the ratio of successful reported values shall be more than 90% with a confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

----- End of first change ------

----Unchanged Clauses Skipped -----

-------- Start of Second change --------

# F.1	Measurement uncertainties and test tolerances for FR1 and FR2

### F.1.1.2	Measurement of RRM requirements

----Unchanged Clauses Skipped -----

Table F.1.1.2-6: Maximum test system uncertainty for RRM requirements for SA FR2 test cases

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 7.6.6.1 NR SA FR2 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy | Average Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês0 / Noc ±0.3 dB Ês0 / Noc over meas PRBs ±0.3 dB Average Ês1 / Noc ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB | Ês0 / Noc is the SNR for the CSI-RS#0 Ês1 / Noc is the SNR for the CSI-RS#1 Meas PRB is the measurement PRB for CSI-RS-SINR #RBJ to RBJ+275 |
| 7.6.6.2 NR SA FR2 SSB based CMR and dedicated IMR L1-SINR measurement accuracy | Average Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês0 / Noc ±0.3 dB Ês0 / Noc over meas PRBs ±0.3 dB Average Ês1 / Noc ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB | Ês0 / Noc is the SNR for the SSB#0 Ês1 / Noc is the SNR for the SSB#1 Meas PRB is the measurement PRB for SS-SINR #RBJ to RBJ+19 |
| 7.6.6.3 NR SA FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy | Same as 7.6.6.1 | Same as 7.6.6.1 |
| 7.6.13.1 NR SA FR2 UE Rx-Tx time difference measurement for propagation delay compensation using PRS | Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês1 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB | Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 1 signal / AWGN Meas PRB is the measurement PRB for PRS-RSRP #RBJ to RBJ+19 |
| 7.7.1.1 NR SA FR2 SS-RSRP measurement accuracy | Test 1 Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB Test 2 Average Ês1 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês1 ±5.65 dB, f <= 40.8 GHz Average Ês2 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês2 ±5.65 dB, f <= 40.8 GHz | Test 1 Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 1 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 Test 2 Ês1 is NR cell 1 signal Ês2 is NR cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |

----- End of Second change ------

----Unchanged Clauses Skipped -----

------ Start of Third change ------

## F.1.3	Test Tolerance and Derivation of Test Requirements (informative)

### F.1.3.2	Measurement of RRM requirements

Table F.1.3.2-4: Derivation of test requirements for NR SA FR2 RRM tests

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 7.6.6.1 NR SA FR2 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy | Same as 5.6.6.1 | Same as 5.6.6.1 | Same as 5.6.6.1 |
| 7.6.6.2 NR SA FR2 SSB based CMR and dedicated IMR L1-SINR measurement accuracy | During T1: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: -infinity During T2: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB Reported SS-SINR values ± 4.5dB Reported Differential SS-SINR values ± 3.5dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB Via mapping | During T1: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: -infinity During T2: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB SSB#1: SINR_53 to SINR_72 SSB#0: DIFFSINR_5 to DIFFSINR_12 |
| 7.6.6.3 NR SA FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy | Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB Reported CSI-SINR values ± 4.0dB Reported Differential CSI-SINR values ± 3.0dB | 0dB 1.5dB 0dB Via mapping | Noc: -105dBm/15kHz Ês0 / Noc: +1.50dB Ês1 / Noc: +9.00dB CSI-RS#1: SINR_54 to SINR_71 CSI-RS#0: DIFFSINR_4 to DIFFSINR_10 |
| 7.6.13.1 NR SA FR2 UE Rx-Tx time difference measurement for propagation delay compensation using PRS | During T1: Noc : -98 dBm/15kHz Ês / Noc: infinity During T2: Noc: -98 dBm/15kHz Ês / Noc: -2 dB | During T1: 0 dB 0 dB During T2: 0 dB 0 dB | During T1: Noc : -98 dBm/15kHz Ês / Noc: infinity During T2: Noc: -98 dBm/15kHz Ês / Noc: -2 dB |
| 7.7.1.1 NR SA FR2 SS-RSRP measurement accuracy | Test 1: Noc: -91.60dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported absolute RSRP values: ±8dB Reported relative RSRP values: ±6dB For extreme conditions allow ±3dB + FFS MU additionally Test 2: n257 Ês1: -110.0dBm/120kHz n257 Ês2: -110.0dBm/120kHz n260 Ês1: -107.4dBm/120kHz n260 Ês2: -107.4dBm/120kHz Reported absolute RSRP values: ±8dB Reported relative RSRP values: ±6dB For extreme conditions allow ±3dB+ FFS MU additionally | Test 1: -5.80dB 0dB 0.4dB Via Mapping Test 2: 7.70dB 7.70dB 7.70dB 7.70dB Via Mapping | Test 1: Noc: -97.40dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.4dB Cell 1: RSRP_50 to RSRP_108 Cell 2: RSRP_46 to RSRP_103 Cell 2: RSRP_x-12 to RSRP_X+2 Test 2: n257 Ês1: -102.3dBm/120kHz n257 Ês2: -102.3dBm/120kHz n260 Ês1: -99.7dBm/120kHz n260 Ês2: -99.7dBm/120kHz Band dependent. See test case tables in clause 5.7.1.1.5 |

----- End of Third change ------