# Title:	Test Tolerances Methodology for Direct SCell activation at SCell addition of known SCell test cases

Source:			  Nokia, Nokia Shanghai Bell

# 1.	Introduction

The purpose of this document is to present the methodology and principles used for the development of Test Tolerances for RRM tests 4.5.3.5 and 6.5.3.4

The test cases in question, from TS 38.533, are:

- 4.5.3.5, Direct SCell activation at SCell addition of known SCell in FR1

- 6.5.3.4, Direct SCell activation at SCell addition of known SCell in FR1

This document describes the process to derive the Test Tolerances. The calculations are provided in the accompanying spreadsheet.

# 2. 	Prototype test case in Annex A of TS 38.133 for 6.5.3.4

The test conditions are defined in the following extract from TS 38.133 v16.16.0

#### A.6.5.3.4	Direct SCell activation at SCell addition of known SCell in FR1

##### A.6.5.3.4.1		Test Purpose and Environment

The purpose of this test is to verify fulfillment of direct SCell activation delay and interruption requirements at SCell addition as defined in clause 8.3.4 and 8.2.2, respectively. The supported test configurations for NR PCell are shown in Table A.6.5.3.4.1-1. The supported test configurations for NR SCell are shown in Table A.6.5.3.4.1-1A. Test configuration for NR PCell and test configuration for NR SCell are chosen independently.

The test scenario comprises one PCell (Cell 1) and one SCell (Cell 2) as outlined in Table A.6.5.3.4.1-2. Cell-specific parameters are provided in Table A.6.5.3.4.1-3 and Table A.6.5.3.4.1-4.

The test consists of two successive time periods with duration T1 and T2, respectively. There are two carriers, each with one cell. Cell 1 (PCell) is on RF channel 1 (PCC), and Cell 2 (SCell) is on RF channel 2 (SCC). Cell 1 and Cell 2 both operate according to one of the configurations in Table A.6.5.3.4.1-1.

Before the test starts the UE is connected to Cell 1 on RF channel 1. The UE is only monitoring RF channel 1 and is not aware of Cell 2 on RF channel 2.

The UE is continuously scheduled in PCell throughout the test.

At the beginning of T1 the UE is configured to measure RF channel 2 in measurement gaps. During T1, the UE detects and measures Cell 2 on RF channel 2, and sends a measurement report containing Cell 2 to the test equipment. After having received a measurement report containing Cell 2, the test equipment deconfigures the measurement gaps and thereafter sends a RRC connection reconfiguration message to the UE by which it configures the SCell (Cell 2) in activated state (sCellState is set to activated). The time between reception of the last measurement report carrying SCell and transmission of the RRC connection reconfiguration message directly activating SCell is kept short enough to allow the SCell to remain known to the UE.

Time period T2 starts when the UE receives the RRC connection reconfiguration message at the UE antenna connector. The corresponding slot at which the message is received at the UE antenna connector is denoted n. The UE shall complete activation of the SCell no later than in slot n + NdirectNR slot length, as specified in clause 8.3.4. From slot n+ NdirectNR slot length and onwards the UE shall report valid CSI both for PCell and SCell.

The test equipment verifies the activation time by counting the slots between the RRC connection reconfiguration message is sent and until CSI report with non-zero CQI for both PCell and SCell is received.

The test equipment verifies that interruptions on other serving cells are within the requirements by counting ACK/NACKs transmitted in PCell.

Table A.6.5.3.4.1-1: Supported test configurations for NR PCell

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |

Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration,

Table A.6.5.3.4.1-1A: Supported test configurations for NR SCell

| ConfigSCell | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |

Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration,

Table A.6.5.3.4.1-2: General test parameters

| Parameter | Unit | Test Configurations | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | 1, 2, 3 | 1, 2 | Two NR radio channels are used for this test |
| Active PCell |  | 1, 2, 3 | Cell 1 | Primary cell on NR RF channel number 1. |
| Inter-frequency neighbor cell (SCell to-be) |  | 1, 2, 3 | Cell 2 | Inter-frequency neighbor cell on NR RF channel number 2 |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX |  | 1, 2, 3 | OFF | Continuous monitoring of primary cell |
| Measurement gap pattern |  | 1, 2, 3 | gp0 | Measurement gap is used during parts of time period T1 for detection of Cell 2. |
| CSI reporting periodicity | ms | 1, 2, 3 | 2 | CSI reporting periodicity for periodic reporting of CQI for PCell and, when added, SCell. |
| SCell measurement cycle (measCycleSCell) | ms | 1, 2, 3 | 160 | Measurement cycle for SCell does not come into effect in direct activation at SCell addition. |
| Timing offset between Cell 1 and Cell 2 | s | 1, 2, 3 | MRTD | The value of maximum timing offset depends upon the carrier aggregation scenario. |
| T1 | s | 1, 2, 3 | 7 | During this time period the PCell shall be known and Cell 2 shall be detected as an inter-frequency neighbor cell. |
| T2 | s | 1, 2, 3 | 1 | During this time period Cell 2 shall be configured and directly activated as SCell. |
| A3-offset | dB | 1, 2, 3 | -15 |  |
| THARQ | ms | 1, 2, 3 | k1×NR slot length | k1 is a number of slots indicated by the PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format or provided by dl-DataToUL-ACK if the PDSCH-to-HARQ feedback timing field is not present in the DCI format, the value is defined in 38.213 [3] |
| TCSI_Reporting | ms | 1, 2, 3 | 2 | the delay uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2] |
| k | ms | 1, 2, 3 |  | As specified in clause 4.3 of TS 38.213 [3] |

Table A.6.5.3.4.1-3: NR Cell specific test parameters for NR Pcell

| Parameter |  |  | Unit | Cell 1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |
| Duplex mode |  | Config 1 |  | FDD |  |
|  |  | Config 2,3 |  | TDD |  |
| TDD configuration |  | Config 2 |  | TDDConf.1.1 |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |
| BWchannel |  | Config 1,2 | MHz | 10: NRB,c = 52 |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |
| BWP configuration |  | Initial DL |  | DLBWP.0.1 |  |
|  |  | Initial UL |  | ULBWP.0.1 |  |
|  |  | Dedicated DL |  | DLBWP.1.1 |  |
|  |  | Dedicated UL |  | ULBWP.1.1 |  |
| TCI state |  |  |  | TCI.State.0 |  |
| CSI-RS configuration for CSI reporting |  | Config 1 |  | CSI-RS.1.1 FDD |  |
|  |  | Config 2 |  | CSI-RS.1.1 TDD |  |
|  |  | Config 3 |  | CSI-RS.2.1 TDD |  |
| TRS Configuration |  | Config 1 |  | TRS.1.1 FDD |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |
|  |  | Config 3 |  | SR.2.1 TDD |  |
| Dedicated CORESET parameters |  | Config 1 |  | CCR.1.1 FDD |  |
|  |  | Config 2 |  | CCR.1.1 TDD |  |
|  |  | Config 3 |  | CCR.2.1 TDD |  |
| RMSI CORESET parameters |  | Config 1 |  | CR.1.1 FDD |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |
|  |  | Config 3 |  | CR.2.1 TDD |  |
| OCNG Pattern |  |  |  | OP.1 |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |
| SMTC configuration |  |  |  | SMTC.1 |  |
| reportConfigType |  |  |  | periodic |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |
| CSI reporting periodicity |  | Config 1,2 | slot | 5 |  |
|  |  | Config 3 |  | 10 |  |
| CSI reporting offset |  | Config 1,2 | slot | 3 |  |
|  |  | Config 3 |  | 5 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note1 |  |  |  |  |  |
| Noc Note2 | Config 1,2 |  | dBm/SCS | -104 |  |
|  | Config 3 |  |  | -101 |  |
| Ês/Iot |  |  | dB | 17 |  |
| Ês/Noc |  |  | dB | 17 |  |
| SS-RSRP Note3 | Config 1,2 |  | dBm/SCS | -87 |  |
|  | Config 3 |  |  | -84 |  |
| Io Note3 | Config 1,2 |  | dBm/9.36 MHz | -59.0 |  |
|  | Config 3 |  | dBm/38.16 MHz | -52.9 |  |
| Propagation condition |  |  |  | AWGN |  |
| Correlation Matrix and Antenna Configuration |  |  |  | 1x2 Low |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP, SCH_RP, and Io levels have been derived from other parameters for information purpose. They are not settable parameters themselves. |  |  |  |  |  |

Table A.6.5.3.4.1-4: NR Cell specific test parameters for NR Scell

| Parameter |  |  | Unit | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |
| Duplex mode |  | ConfigSCell 1 |  | FDD |  |
|  |  | ConfigSCell 2,3 |  | TDD |  |
| TDD configuration |  | ConfigSCell 2 |  | TDDConf.1.1 |  |
|  |  | ConfigSCell 3 |  | TDDConf.2.1 |  |
| BWchannel |  | ConfigSCell 1,2 | MHz | 10: NRB,c = 52 |  |
|  |  | ConfigSCell 3 |  | 40: NRB,c = 106 |  |
| BWP configuration |  | Initial DL |  | N/A | DLBWP.0.1 |
|  |  | Initial UL |  |  | N/A |
|  |  | Dedicated DL |  |  | DLBWP.1.1 |
|  |  | Dedicated UL |  |  | N/A |
| TCI state |  |  |  | N/A | TCI.State.0 |
| CSI-RS configuration for CSI reporting |  | ConfigSCell 1 |  | N/A | CSI-RS.1.1 FDD |
|  |  | ConfigSCell 2 |  |  | CSI-RS.1.1 TDD |
|  |  | ConfigSCell 3 |  |  | CSI-RS.2.1 TDD |
| TRS Configuration |  | ConfigSCell 1 |  | N/A | TRS.1.1 FDD |
|  |  | ConfigSCell 2 |  |  | TRS.1.1 TDD |
|  |  | ConfigSCell 3 |  |  | TRS.1.2 TDD |
| PDSCH Reference measurement channel |  | ConfigSCell 1 |  | N/A | SR.1.1 FDD |
|  |  | ConfigSCell 2 |  |  | SR.1.1 TDD |
|  |  | ConfigSCell 3 |  |  | SR.2.1 TDD |
| Dedicated CORESET parameters |  | ConfigSCell 1 |  | N/A | CCR.1.1 FDD |
|  |  | ConfigSCell 2 |  |  | CCR.1.1 TDD |
|  |  | ConfigSCell 3 |  |  | CCR.2.1 TDD |
| RMSI CORESET parameters |  | ConfigSCell 1 |  | N/A |  |
|  |  | ConfigSCell 2 |  |  |  |
|  |  | ConfigSCell 3 |  |  |  |
| OCNG Pattern |  |  |  | OP.1 |  |
| SSB Configuration |  | ConfigSCell 1,2 |  | SSB.1 FR1 |  |
|  |  | ConfigSCell 3 |  | SSB.2 FR1 |  |
| SMTC configuration |  |  |  | SMTC.1 |  |
| reportConfigType |  |  |  | N/A |  |
| reportQuantity |  |  |  | N/A |  |
| CSI reporting periodicity | ConfigSCell 1,2 |  | slot | N/A |  |
|  | ConfigSCell 3 |  |  | N/A |  |
| CSI reporting offset | ConfigSCell 1,2 |  | slot | N/A |  |
|  | ConfigSCell 3 |  |  | N/A |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note1 |  |  |  |  |  |
| Noc Note2 |  | ConfigSCell 1,2 | dBm/SCS | -104 |  |
|  |  | ConfigSCell 3 |  | -101 |  |
| Ês/Iot |  |  | dB | 17 |  |
| Ês/Noc |  |  | dB | 17 |  |
| SS-RSRP Note3 |  | ConfigSCell 1,2 | dBm/SCS | -87 |  |
|  |  | ConfigSCell 3 |  | -84 |  |
| Io Note3 |  | ConfigSCell 1,2 | dBm/9.36 MHz | -59.0 |  |
|  |  | ConfigSCell 3 | dBm/38.16 MHz | -52.9 |  |
| Propagation condition |  |  |  | AWGN |  |
| Correlation Matrix and Antenna Configuration |  |  |  | 1x2 Low |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP, SCH_RP, and Io levels have been derived from other parameters for information purpose. They are not settable parameters themselves. |  |  |  |  |  |

##### A.6.5.3.4.2	Test Requirements

The UE shall complete the direct activation of the SCell no later than at slot n + NdirectNR slot length.

The UE shall report non-zero CQI for SCell from slot n + NdirectNR slot length and onwards throughout time period T2.

The interruption on PCell during direct activation of the SCell shall occur within the interruption window specified in clause 8.3.4 and shall not exceed the length specified in clause 8.2.2.2.11.

The rate of correct events observed during repeated tests shall be at least 90%.

# 3. 	Discussion

The test case consists of 3 test configurations. Each test configuration has 2 time periods: T1, T2. Two cells have levels which are constant.

# 4. 	Calculation of Test Tolerances

General approach

The general approach is given in the steps below:

- Copy the originally specified key parameters from the core requirements

- Where relevant, calculate derived parameters from the core requirements

- Define uncertainties for a minimum set of parameters

- Define controlled parameters (critical to the test verdict), calculate sensitivity factors and uncertainty

- Determine which original or derived parameters to offset (apply Test Tolerances to) and by how much

- Recalculate original or derived parameters including Test Tolerances

- Check that the controlled parameters meet requirements to get the correct test verdict

Each step is explained below, and the calculations are given in the accompanying spreadsheet.

a) Original specified key parameters

The key parameters are copied from Table A.6.5.3.4.1-3 in TS 38.133. The key parameters are selected as the minimum set to define the cell power levels, and which are subject to a test system uncertainty which may affect the verdict of the test.

b) Derived parameters

A number of derived parameters are calculated, using the base information in a). The reason for deriving each additional parameter is given in the “Comment” column of section b) in the accompanying spreadsheet.

c) Uncertainties

The SS provides AWGN and 2 NR cells to the UE. We propose to control the following parameters:

- Frequency 1 AWGN Absolute power, Noc ±1.5 dB averaged over BWConfig

- Frequency 1 AWGN Absolute power, Noc ±1.5 dB for PRBs #RBJ-RBJ+19

- Ratio of NR Cell 1 signal / noise, Ês1 / Noc ±0.3 dB averaged over BWConfig

- Ratio of NR Cell 1 signal / noise, Ês1 / Noc ±0.3 dB for PRBs #RBJ-RBJ+19

- Frequency 2 AWGN Absolute power, Noc ±1.5 dB averaged over BWConfig

- Frequency 2 AWGN Absolute power, Noc ±1.5 dB for PRBs #RBJ-RBJ+19

- Ratio of NR Cell 2 signal / noise, Ês2 / Noc ±0.3 dB averaged over BWConfig

- Ratio of NR Cell 2 signal / noise, Ês1 / Noc ±0.3 dB for PRBs #RBJ-RBJ+19

In this test the UE measures the SS-RSRP over specific Physical Resource Block (PRB) numbers # RBJ to # RBJ+19. In addition, this test has separate constraints on the overall power in the configured bandwidth Io.

This choice forms a minimum set, so the superposition principle can be applied if necessary.

In addition the UE measurement accuracies need to be considered, it has some inaccuracy which could affect the test verdict.

For NR SS-RSRP measurement accuracy. The UE measurement accuracy is defined in the following extract from TS 38.133:

##### 10.1.4.1.2	Relative Accuracy of SS-RSRP in FR1

The relative accuracy of SS-RSRP in inter frequency case is defined as the RSRP measured from one cell on a frequency in FR1compared to the RSRP measured from another cell on a different frequency in FR1.

The accuracy requirements in Table 10.1.4.1.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] Clause 7.3 for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band for each relevant SSB.

-	|SSB_RP1dBm - SSB_RP2dBm| ≤ 27 dB

-	| Channel 1_Io -Channel 2_Io |  20 dB

Table 10.1.4.1.2-1: SS-RSRP Inter frequency relative accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 3 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
| 4.5 | 6 | -6 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

To ensure this relative SS-RSRP accuracy, the conditions of Io, SSB Es/Iot in Table 10.1.4.1.2-1, and conditions of SSB_RP and SSB Es/Iot in section Annex B.2.3 need to be fulfilled.

## B.2.3	Conditions for NR inter-frequency measurements

This clause defines the following conditions for NR inter-frequency measurements and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.3-1 for FR1 NR cells.

The conditions are defined in Table B.2.3-2 for FR2 NR cells.

Table B.2.3-1: Conditions for inter-frequency measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -125 | -122 | -4 |
|  | NR_FDD_FR1_B | -124.5 | -121.5 |  |
|  | NR_TDD_FR1_C | -124 | -121 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -124.5 | -120.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -123 | -120 |  |
|  | NR_FDD_FR1_F | -122.5 | -119.5 |  |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G | -122 | -119 |  |
|  | NR_FDD_FR1_H | -121.5 | -118.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

In addition, the NR Cell 2 needs to be detectable so that the requirement of inter-frequency measurement can apply. The condition that an inter-frequency cell is detectable is defined in TS 38.133 9.3.2.

### 9.3.2	Requirements applicability

The requirements in clause 9.3 apply, provided:

-	The cell being identified or measured is detectable.

An inter-frequency cell shall be considered detectable when for each relevant SSB:

-	SS-RSRP related side conditions given in clauses 10.1.4 and 10.1.5 for FR1 and FR2, respectively, for a corresponding Band,

-	SS-RSRQ related side conditions given in clauses 10.1.9 and 10.1.10 for FR1 and FR2, respectively, for a corresponding Band,

-	SS-SINR related side conditions given in clauses 10.1.14 and 10.1.15 for FR1 and FR2, respectively, for a corresponding Band,

-	SSB_RP and SSB Ês/Iot according to Annex B.2.3 for a corresponding Band.

In summary the SSB Es/Iot of both NR Cells shall be above -4 dB, the tightest requirement for Io of NR Cells shall be between -117.5dBm/15kHz (-89.5dBm/9.36MHz) or -114.5dBm/30kHz (-83.5dBm/38.16MHz) and -50dBm/BWchannel for band group NR_FDD_FR1_H, and the tightest requirement for SSB_RP shall be above -121.5dBm/15kHz or -118.5/30kHz for band groups NR_FDD_FR1_H.

d) Controlled parameters critical to verdict

The controlled parameters listed in the accompanying spreadsheet have been derived by study of the test case and the relevant clauses in TS 38.133.

Having identified the parameters which need to be controlled, we now need to consider how they are affected by the parameters which can be set by the test equipment. This is done by working out “sensitivity factors”. A sensitivity factor is just the ratio (effect on a critical parameter y / a test equipment uncertainty x), and is usually in dB/dB. Often it can be derived by inspection as one or zero. For example, an error of 1dB in the absolute AWGN level Noc would cause 1dB error in the Cell 1 SS-RSRP, so the sensitivity factor is 1.000 during T1, T2. However the same error of 1dB in the absolute AWGN level Noc would cause no change to Cell 1 Es/Iot, because all other powers are specified relative to Noc, so the sensitivity factor is zero.

In some cases, the sensitivity factor is an intermediate value. For example, the NR Cell 2 Es/Noc has an effect on NR Cell 1 Es/Iot which depends on ratios of the powers making up the total. In such cases a sensitivity factor value between 0 and 1 results. It is important to calculate these correctly to obtain the overall uncertainty.

Having filled in the matrix of sensitivity factors, the accompanying spreadsheet calculates the overall uncertainty for each controlled parameter, considering the uncertainties and sensitivity factors for each parameter that can be set by the test equipment and the UE measurement accuracy where relevant. This process follows the superposition principle. Uncertainties are calculated separately for T1, T2.

For the test system uncertainties, the normal procedure of combining uncorrelated uncertainties root-sum-square is followed.

When the test system uncertainties and the UE reporting accuracies are combined, the (root-sum square of test system uncertainties) is added arithmetically to the UE reporting accuracies. If all the uncertainties were combined root-sum square, the resulting smaller test limits could case a conformant test system to fail a conformant UE, which would be unacceptable.

e) Determine parameters to offset

During T1 the NR Cell 2 becomes detective. UE is expected to perform inter-frequency measurement. The nominal conditions in TS 38.133 Table A.6.5.3.4.1-3, A.6.5.3.4.1-4 give SS-RSRP values

The Event A3 measurement report trigger requirement in TS 38.331 clause 5.5.4.4 is “Event A3 (Neighbour becomes offset better than SpCell), inequality A3-1 (Entering condition) is: Mn + Ofn + Ocn – Hys > Mp + Ofp + Ocp + Off”.

The values in this equation are derived as follows:

- Mn is evaluated for NR Cell 2 at T1 and is nominally -87dBm/15kHz (-84dBm/30kHz)

- Ofn is the measurement object specific offset offsetMO of the reference signal of the neighbour cell. It’s default value is specified as 0dB in TS 38.508-1 Table 4.6.3-76.

- Ocn is the cell specific offset cellIndividualOffset of the neighbour cell and set to zero if not configured for the neighbour cell.

- Hys is the hysteresis parameter for this event, specified as 0dB in TS 38.133 Table A.4.6.2.1.2-2.

- Ms is evaluated for NR Cell 1 at T1 and is nominally -87dBm/15kHz (-84dBM/30kHz)

- Ofp is the measurement object specific offset offsetMO of the SpCell. It’s default value is specified as 0dB in TS 38.508-1 Table 4.6.3-76.

- Ocp is the cell specific offset cellIndividualOffset of the serving cell and set to zero if not configured for the neighbour cell.

- Off is the offset parameter for this event (i.e. A3-Offset), and is -15 dB for this test case.

The equation therefore reduces to:

Mn > Mp + Off. Putting in values and rearranging gives -87dBm - (-87)dBm > -15dB for configuration 1 and -84dBm - (-84)dBm > -15dB for configuration 3.

This inequality is satisfied by 15 dB under nominal conditions.

In addition, during T1, T2 the Es/Iot, SSB_RP of both NR Cells and the Io on NR RF channel 1 and channel 2 are fulfilling the required side conditions considering the uncertainties, therefore no offsets are required.

f) Parameters modified by Test Tolerances

Based on the decision in e), the set of parameters in a) and b) is reproduced in section f) of the accompanying spreadsheet, modified by the Test Tolerances (applied offsets).

Re-derived parameters are calculated using the same methods as were used in step b). In general, all the values are listed for each time interval T1, T2.

g) Check controlled parameters Min/Max

It can be seen that with the uncertainty values and Test Tolerances proposed, all the requirements are met.

# 5. Treatment of other configurations within the same test case

TS 38.533 test case 6.5.3.4 also contains configurations with different subcarrier spacings and different channel bandwidths. Furthermore, test configuration for NR PSCell and NR SCell can be chosen idependently. As the calculations use different values, a separate spreadsheet tab is provided. The following parameters are changed:

- Subcarrier spacing, section a)

- Channel bandwidth, section a)

- RBs in Channel bandwidth, section a) and associated comment

- Cell 1 and Cell 2 SSB_RP minimum requirement, section g) and associated Source/Reference

- Io minimum requirement, section g) and associated Source/Reference

# 6. Treatment of test case A.4.5.3.5

TS 38.133 also contains similar test case A.4.5.3.5. Since the test parameters are identical with that of A.6.5.3.4, the same level uncertainties and method of deciding Test Tolerances can be used.