3GPP TSG-RAN5 Meeting #99	R5-232548r1

# Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

Source:	MediaTek Inc.

Title:	Discussion paper of handling legacy test case for IoT NTN UE

# Document for:	Endorsement

Agenda Item:	6.3.25.6

1.	Introduction and Background

At RAN#96 the Rel-17 WI UE Conformance – NB-IoT/eMTC support for Non-Terrestrial Networks (NTN) including EPS aspects [1] was introduced.

Generally, NB-IoT/eMTC over NTN protocol is based on the legacy protocol defined for NB-IoT/eMTC over LTE as specified in TS 36.763 and TS 36.300.

Besides the new test cases specified in the Work Plan, to have a robust test scope the existing eMTC and NB-IoT test cases could be re-used for IoT NTN UE.

Meanwhile, depending on the additional RAT supported there are two types of NB-IoT/eMTC UEs:

Type 1 UE: UE supporting NB-IoT/eMTC over both LTE and NTN

Type 2 UE: UE supporting NB-IoT/eMTC over NTN only

As there were discussions and way-forwards for vertical UE in RAN5, such as SNPN and RedCap UEs [2] [3], similar approach could be adopted for IoT NTN test cases.

2.	Discussion

The legacy protocol conformance test cases defined to test the basic NB-IoT/eMTC protocol over LTE can be executed on UEs supporting NB-IoT/eMTC over both LTE and NTN without a change.

However, currently there is no test coverage for the basic NB-IoT/eMTC protocol for UEs supporting NB-IoT/eMTC over NTN only.

Below Figure-1 shows the test coverage and execution guidance for different UE types.

Figure-1: Test coverage analysis of different UE types

To avoid a case to be executed twice, for UEs that support NB-IoT/eMTC over LTE and NTN the preferred RAT for TC execution is LTE.

3.	Proposal/Way Forward

Proposal #1: For eMTC legacy test cases fully applicable to UEs supporting eMTC over NTN add a note to the Main behavior to mention the difference for NTN. See R5-232543 [5] for details.

Proposal #2: The prose of NB-IoT legacy test cases fully applicable to UEs supporting NB-IoT over NTN can be re-used as they are.

Proposal #3a: Add a new table for legacy eMTC test cases fully or partially applicable to UEs supporting eMTC over NTN. See R5-232546 [6] for details.

Proposal #3b: Add a new table for legacy NB-IoT test cases fully or partially applicable to UEs supporting NB-IoT over NTN. See R5-232547 [7] for details.

4.	References

[1] 	RP-221548: New WID on UE Conformance Test Aspects – NB-IoT/eMTC support for Non-Terrestrial

Networks (NTN) including EPS aspects, MediaTek Inc.

[2]	R5-226545: Discussion paper for handling legacy test case for Redcap Only UE, RAN5#97

[3]	R5-224322: Way Forward for Rel-15 NR Tests Applicability on SNPN Only UE, RAN5#96e

[4] 	R5-231915: Update of default configuration for IoT NTN, RAN5#98

[5] 	R5-232543: Correction to NAS Test Case to enable IoT NTN test, MediaTek Inc.

[6] 	R5-232546: Applicable eMTC cases for IoT NTN, MediaTek Inc.

[7] 	R5-232547: Applicable NB-IoT cases for IoT NTN, MediaTek Inc.