3GPP TSG-RAN5 Meeting #99	R5-232638r1

# Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2768 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to switch off test procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In 4.9.6.2.1, Service Request procedure is performed but there is no need for that to perform IMS deregistration when switched off in RRC_INACTIVE since the NAS signalling connection is already active. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed the steps for Service Request procedure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test cases requiring this procedure |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.9.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1->Updated the test sequence to always send the Deregistration Request and RRC Release (and the optional release of the PDUs) to simolify the test sequence and also the current procedure doesn’t reflect that in the non-IMS case (where there is no DRB traffic required) the DEREGISTRATION REQUEST will actually come inside the RRCResumeComplete itself, made changes to resolve that. |  |  |  |  |  |  |  |  |

### 4.9.6	Test procedures for Switch off / Power off UE

….

SKIPPED TEXT

#### 4.9.6.2	Switch off / Power off procedure in RRC_INACTIVE

4.9.6.2.1	Procedure

Table 4.9.6.2.1-1: Switch off procedure in RRC_INACTIVE

| Step | Procedure | Message Sequence |  |  |
| --- | --- | --- | --- | --- |
|  |  | U - S | Message |
| - | EXCEPTION: Steps 1a1 to 1b1 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if [36] pc_SwitchOnOff or [37] pc_USIM_Removal is supported | - | - |  |
| 1a1 | IF pc_SwitchOnOff THEN switch off UE, IF pc_USIM_Removal THEN remove the USIM (Note 1) | - | - |  |
| 1a2 | UE transmits an RRCResumeRequest message. | --> | NR RRC: RRCResumeRequest |  |
| 1a3 | SS transmit an RRCResume message. | <-- | NR RRC: RRCResume |  |
|  | EXCEPTION: Steps 1a3Aa1 to 1a3Ab1 describe behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place depending on the UE implementation. |  |  |  |
| 1a3Aa1 | The UE transmits an RRCResumeComplete message to confirm the successful resumption of the RRC connection not including any 5G NAS message | --> | NR RRC: RRCResumeComplete |  |
| 1a4Aa1-1a4Aa5 | Void | - | - |  |
| - | EXCEPTION: The generic test procedure in TS 34.229-5 [47] Annex A.11 may be performed depending on UE implementation. | - | - |  |
| - | EXCEPTION: Steps 1a3Aa2a1 - 1a3Aa3 may be performed depending on UE implementation | - | - |  |
| - | EXCEPTION: Steps 1a3Aa2a1 - 1a3Aa2a2 may be repeated for all active PDU sessions | - | - |  |
| 1a3Aaa2a1 | The UE transmits a PDU SESSION RELEASE REQUEST message. | --> | PDU SESSION RELEASE REQUEST |  |
| 1a3Aa2a2 | PDU session release procedure defined in clause 4.9.21 of TS 38.508-1 [4] is performed | - | - |  |
| 1a3Aa3 | The UE transmits a DEREGISTRATION REQUEST message | - | 5GMM: DEREGISTRATION REQUEST |  |
| 1a3Ab1 | The UE transmits an RRCResumeComplete message to confirm the successful resumption of the RRC connection, including the DEREGISTRATION REQUEST messgae | --> | NR RRC: RRCResumeComplete 5GMM : DEREGISTRATION REQUEST |  |
| 1a4 | Void | - | - |  |
| 1a5 | The SS transmits an RRCRelease message | <-- | NR RRC: RRCRelease |  |
| 1b1 | ELSE power off UE (Note 2) | - | - |  |
| Note 1: USIM removal is a feasible alternative to switch off UE. Note 2: Power off is used when UE don’t support switch off or USIM removal, in which case no UE originated deregistration procedure is expected. |  |  |  |  |

4.9.6.2.2	Specific Message contents

Table 4.9.6.2.2-1: RRCResumeRequest

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCResumeRequest ::= SEQUENCE { |  |  |  |  |
| rrcResumeRequest SEQUENCE { |  |  |  |  |
| resumeIdentity |  | Not checked |  |  |
| resumeMAC-I |  | Not checked |  |  |
| resumeCause |  | Not checked |  |  |
| Spare |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |