3GPP TSG-RAN5 Meeting #99	R5-232690r2

# Incheon, South Korea, 22nd May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2771 | rev | 1 | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to SIB19 for NR NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm CDMA Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To support NR NTN test cases, SIB19, EphemerisInfo and NTN-Config are needed to be defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | FFS in Table 4.6.2-18C, Table 4.6.3-56C and Table 4.6.3-84C are updated as required for NR NTN testing. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work plan will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.2, 4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-232690r1Ntn-config-r17 split in to two tables for GSO, NGSO; t-service-r17 changed to “Not Present” Added Condition GSO and NGSO to IE ephemerisinfo-r17 Changed the GSO Position Velocity Parameters Deleted Condition NGSO from t-Service-r17 as only one value is possible R5-232690r2Ta-info-r17 and Epochtime-r17 for NGSO is FFS Fixed formatting issues |  |  |  |  |  |  |  |  |

### 4.6.2	System information blocks

<<<Tables skipped>>>

<<< START OF CHANGES >>>

#### –	SIB19

Table 4.6.2-18C: SIB19

| Derivation Path: TS 38.331 [6], clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r17 | NTN-Config with condition GSO |  | GSO |
|  | NTN-Config with condition NGSO |  | NGSO |
| t-Service-r17 | Not Present |  |  |
| referenceLocation-r17 | Not present |  |  |
| distanceThresh-r17 | Not present |  |  |
| ntn-NeighCellConfigList-r17 | Not present |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| GSO | Geosynchronous Orbit scenario |
| NGSO | Non-geosynchronous Orbit scenario |

<<< END OF CHANGES >>>

### 4.6.3	Radio resource control information elements

<<<Tables skipped>>>

<<< START OF CHANGES >>>

#### –	EphemerisInfo

Table 4.6.3-56C: EphemerisInfo

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EphemerisInfo-r17 ::= CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  | GSO |
| positionX-r17 | -16976014 |  |  |
| positionY-r17 | 27636499 |  |  |
| positionZ-r17 | 0 |  |  |
| velocityVX-r17 | 0 |  |  |
| velocityVY-r17 | 0 |  |  |
| velocityVZ-r17 | 0 |  |  |
| } |  |  |  |
| orbital-r17 SEQUENCE { |  |  | NGSO |
| semiMajorAxis-r17 | FFS |  |  |
| eccentricity-r17 | FFS |  |  |
| periapsis-r17 | FFS |  |  |
| longitude-r17 | FFS |  |  |
| inclination-r17 | FFS |  |  |
| meanAnomaly-r17 | FFS |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| GSO | Geosynchronous Orbit scenario |
| NGSO | Non-geosynchronous Orbit scenario |

<<< END OF CHANGES >>>

<<<Tables skipped>>>

<<< START OF CHANGES >>>

#### –	NTN-Config

Table 4.6.3-84C: NTN-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NTN-Config-r17 ::= SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  | GSO |
| epochTime-r17 SEQUENCE { |  |  | NGSO |
| sfn-r17 | FFS |  |  |
| subFrameNR-r17 | FFS |  |  |
| } |  |  |  |
| ntn-UlSyncValidityDuration-r17 | s240 |  |  |
| cellSpecificKoffset-r17 | 550 |  |  |
| kmac-r17 | Not present |  |  |
| ta-Info-r17 | FFS |  |  |
| ntn-PolarizationDL-r17 | linear |  |  |
| ntn-PolarizationUL-r17 | linear |  |  |
| ephemerisInfo-r17 | EphemerisInfo with condition GSO |  | GSO |
|  | EphemerisInfo with condition NGSO |  | NGSO |
| ta-Report-r17 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| GSO | Geosynchronous Orbit scenario |
| NGSO | Non-geosynchronous Orbit scenario |

<<< END OF CHANGES >>>