3GPP TSG-RAN5 Meeting #99	R5-233044r1

# Incheon, Korea, May 22nd – 26th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0702 | rev | - | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of 5.2.2.2.21 2Rx TDD HST-SFN Scheme A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_feMIMO-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN4 specified the test case for HST-SFN scheme A. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding test case for HST-SFN scheme A |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case is not covered in RAN5 specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.2.21 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Removing placeholder for 5.2.2.2.16~5.2.2.2.18 to avoid conflict with R5-233010, R5-232457, R5-233224 and R5-232854. |  |  |  |  |  |  |  |  |

<Start of Changes>

5.2.2.2.15.3.4	Test requirement

Tables 5.2.2.2.15.0-2 define the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A 3.2.1 for each throughput test shall meet or exceed the specified value in Table 5.2.2.2.15.3.4-1 for the specified SNR including test tolerances for all throughput tests.

Table 5.2.2.2.15.3.4-1: Test requirements for Rank 2

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.2-18.1 TDD | 20 / 30 | 16QAM, 0.48 | FR1.30-7 | TDLA30-10 | 2x2, ULA Low | 70 | 13.8 + TT |
| 1-2 | R.PDSCH.2-18.2 TDD | 40 / 30 | 16QAM, 0.48 | FR1.30-7 | TDLA30-10 | 2x2, ULA Low | 70 | 14.1 + TT |
| 1-3 | R.PDSCH.2-18.3 TDD | 60 / 30 | 16QAM, 0.48 | FR1.30-7 | TDLA30-10 | 2x2, ULA Low | 70 | 14.2 + TT |
| 1-4 | R.PDSCH.2-18.4 TDD | 80 / 30 | 16QAM, 0.48 | FR1.30-7 | TDLA30-10 | 2x2, ULA Low | 70 | 14.5 + TT |

5.2.2.2.19 – 5.2.2.2.20	FFS

5.2.2.2.21	2Rx TDD FR1 PDSCH HST-SFN Scheme A performance - 2x2 MIMO for both SA and NSA

Editor's Note: This test cases is incomplete in following aspects:

- SNR requirement is within square brackets.

5.2.2.2.21.1	Test Purpose

To verify the UE performance in the HST-SFN Scheme A scenario.

5.2.2.2.21.2	Test applicability

This test case applies to all types of NR UE release 17 and forward that support SFN scheme A for PDCCH scheduling SFN Scheme A PDSCH.

5.2.2.2.21.3	Minimum conformance requirements

The performance requirements are specified in Table 5.2.2.2.21.3-3, with the addition of test parameters in Table 5.2.2.2.21.3-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.2.2.21.3-1.

Table 5.2.2.2.21.3-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify UE performance in the HST-SFN Scheme A scenario defined in B.3.5 | 1-1 |

Table 5.2.2.2.21.3-2: Test parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  |  | TDD |
| Active DL BWP index |  |  |  | 1 |
| PDCCH configuration | TCI state |  |  | Note 1 |
| PDSCH configuration | Mapping type |  |  | Type A |
|  | k0 |  |  | 0 |
|  | Starting symbol (S) |  |  | 2 |
|  | Length (L) |  |  | 12 |
|  | PDSCH aggregation factor |  |  | 1 |
|  | PRB bundling type |  |  | Static |
|  | PRB bundling size |  |  | 2 |
|  | Resource allocation type |  |  | Type 0 |
|  | RBG size |  |  | Config2 |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |
|  | TCI state |  |  | Note 1 |
| PDSCH DMRS configuration | DMRS Type |  |  | Type 1 |
|  | Number of additional DMRS |  |  | 2 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |
| CSI-RS for tracking | Resource set #1 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
|  |  | CSI-RS periodicity | Slots | 20 for CSI-RS resource 1,2,3,4. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 1 and 2 2 for CSI-RS resource 3 and 4 |
|  |  | QCL info |  | TCI state #3 |
|  | Resource set #2 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 6 for CSI-RS resource 5 and 6 l0 = 10 for CSI-RS resource 7 and 8 |
|  |  | CSI-RS periodicity | Slots | 20 for CSI-RS resource 5,6,7,8. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 5 and 6 2 for CSI-RS resource 7 and 8 |
|  |  | QCL info |  | TCI state #4 |
|  | Resource set #3 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 4 for CSI-RS resource 9 and 10 l0 = 8 for CSI-RS resource 11 and 12 |
|  |  | CSI-RS periodicity | Slots | 20 for CSI-RS resource 9,10,11,12. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 9 and 10 2 for CSI-RS resource 11 and 12 |
|  |  | QCL info |  | TCI state #5 |
| NZP CSI-RS for CSI acquisition | Resource set #4 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  |  | CSI-RS periodicity | Slots | 40 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #0 |
|  | Resource set #5 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 13 |
|  |  | CSI-RS periodicity | Slots | 40 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #1 |
|  | Resource set #6 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 7 |
|  |  | CSI-RS periodicity | Slots | 40 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #2 |
| TCI state #0 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking Resource set #1' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 5 from 'CSI-RS for tracking Resource set #2' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #2 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 9 from 'CSI-RS for tracking Resource set #3' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #3 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #4 | Type 1 QCL information | SSB index |  | SSB #1 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #5 | Type 1 QCL information | SSB index |  | SSB #2 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| Number of HARQ Processes |  |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |
| Note 1: SSB # (k mod 3), CSI-RS (for tracking) resource set # ((k mod 3) + 1) and CSI-RS (for CSI acquisition) resource set # ((k mod 3) + 4) are transmitted by kth RRH. Codepoint #0 is activated when UE receives PDCCH/PDSCH from RRH#3k and RRH#3k+1 with TCI States TCI state #0, TCI State #1. Codepoint #1 is activated when UE receives PDCCH/PDSCH from RRH#3k+1 and RRH#3k+2 with TCI States TCI state #1, TCI State #2. Codepoint #2 is activated when UE receives PDCCH/PDSCH from RRH#3k+2 and RRH#3k+3 with TCI States TCI state #2, TCI State #0. |  |  |  |  |

Table 5.2.2.2.21.3-3: Minimum performance for HST-SFN Scheme A

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.2-30.1 TDD | 40 / 30 | 16QAM, 0.48 | HST-SFN Scheme A | 2x2 | 70 | [12.9] |

The normative reference for this requirement is TS 38.101-4 [5], clause 5.2.2.2.21.

5.2.2.2.21.4	Test description

5.2.2.2.21.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 5.2.2.

For EN-DC within FR1 operation, setup the LTE link according to Annex D.

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.1 for TE diagram and clause A.3.2.2 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 5.2-1 and Table 5.2.2.2.21.3-2 as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions are set according to Annex B.3.5.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without Release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without release On, Test Mode On for NSA according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 5.2.2.2.21.4.3.

5.2.2.2.21.4.2	Test procedure

1.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix and the SNR according to Tables 5.2.2.2.21.5-1 as appropriate.

2.	SS is configured to transmit SSB and CSI-RS continuously and schedule PDSCH and PDCCH transmission according to Note 1 in 5.2.2.2.21.3-2. SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Tables 5.2.2.2.21.5-1. The SS sends downlink MAC padding bits on the DL RMC.

Note:	All TCI states are known to the UE through configuration inside RrcReconfiguration. There is no need to configure additional L1-RSRP measurements.

3. 	Send MAC CE command “Enhanced TCI States Indication for UE-specific PDCCH MAC CE” according to the timing described in Note 1 of table 5.2.2.2.21.5-1 to active TCI state codepoint 0, 1 or 2 for PDCCH periodically. PDSCH is automatically associated with TCI state codepoint 0, 1 or 2 as tci-PresentInDCI is not present. TCI states 3, 4 and 5 for SSBs are automatically activated through relation of QCL-Info in NZP CSI-RS.

4.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL during each subtest and decide pass or fail according to Table G.1.5-1 in Annex G clause G.1.5.

5.2.2.2.21.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clauses 4.6.1 and 5.4.2.

5.2.2.2.21.4.3.1	Message exceptions for SA

Table 5.2.2.2.21.4.3.1-1: PDSCH-Config

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| prb-BundlingType CHOICE { |  |  |  |
| staticBundling SEQUENCE { |  |  |  |
| bundleSize | Not present | n2 is used | Test 1-1 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.2.2.2.21.4.3.1-2: DMRS-DownlinkConfig

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DMRS-DownlinkConfig ::= SEQUENCE { |  |  |  |
| dmrs-AdditionalPosition | pos2 |  | Test 1-1 |
| } |  |  |  |

Table 5.2.2.2.21.4.3.1-3: PDSCH-ServingCellConfig

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ServingCellConfig ::= SEQUENCE { |  |  |  |
| nrofHARQ-ProcessesForPDSCH | n8 |  | Test 1-1 |
| } |  |  |  |

Table 5.2.2.2.21.4.3.1-4: NZP-CSI-RS-Resource for TRS

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-Resource ::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceId | i-1 for CSI-RS resource #i, i=1,2,3,4,5,6,7,8,9,10,11,12 | for test 1-1 |  |
| qcl-InfoPeriodicCSI-RS | 3 for CSI-RS resource #1, #2, #3, #4 4 for CSI-RS resource #5, #6, #7, #8 5 for CSI-RS resource #9, #10, #11, #12 | for test 1-1: TCI-StateId for TCI-State #3 for CSI-RS resource #1, #2, #3, #4 TCI-StateId for TCI-State #4 for CSI-RS resource #5, #6, #7, #8 TCI-StateId for TCI-State #5 for CSI-RS resource #9, #10, #11, #12 |  |
| } |  |  |  |

Table 5.2.2.2.21.4.3.1-5: CSI-RS-ResourceMapping for TRS (Table 5.2.2.2.21.4.3.1-4)

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-9 with condition TRS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| firstOFDMSymbolInTimeDomain | 5 for CSI-RS resource #1 and #3 9 for CSI-RS resource #2 and #4 6 for CSI-RS resource #5 and #6 10 for CSI-RS resource #7 and #8 4 for CSI-RS resource #9 and #10 8 for CSI-RS resource #11 and #12 | for test 1-1: l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 l0 = 6 for CSI-RS resource 5 and 6 l0 = 10 for CSI-RS resource 7 and 8 l0 = 4 for CSI-RS resource 9 and 10 l0 = 8 for CSI-RS resource 11 and 12 |  |
| } |  |  |  |

Table 5.2.2.2.21.4.3.1-6: CSI-ResourcePeriodicityAndOffset for CSI Tracking (Table 5.2.2.2.21.4.3.1-4)

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourcePeriodicityAndOffset ::= CHOICE { |  |  |  |
| slots20 | 1 for CSI-RS resource #1, #2, #5, #6, #9, #10 2 for CSI-RS resource #3 #4, #7, #8, #11, #12 | For test 1-1: periodicity: 20 slots. offset: 1 for CSI-RS resource 1 and 2 2 for CSI-RS resource 3 and 4 1 for CSI-RS resource 5 and 6 2 for CSI-RS resource 7 and 8 1 for CSI-RS resource 9 and 10 2 for CSI-RS resource 11 and 12 |  |
| } |  |  |  |

Table 5.2.2.2.21.4.3.1-7: NZP-CSI-RS-ResourceSet for TRS

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceSet ::= SEQUENCE { |  |  |  |
| nzp_CSI_ResourceSetId | 0 for Resource set #1 1 for Resource set #2 2 for Resource set #3 | For test 1-1 |  |
| nzp-CSI-RS-Resources SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId { | 4 entries | For test 1-1 | Resource set #1 |
| NZP-CSI-RS-ResourceId[1] | 0 | entry 1 CSI-RS resource #1 |  |
| NZP-CSI-RS-ResourceId[2] | 1 | entry 2 CSI-RS resource #2 |  |
| NZP-CSI-RS-ResourceId[3] | 2 | entry 3 CSI-RS resource #3 |  |
| NZP-CSI-RS-ResourceId[4] | 3 | entry 4 CSI-RS resource #4 |  |
| } |  |  |  |
| nzp-CSI-RS-Resources SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId { | 4 entries | For test 1-1 | Resource set #2 |
| NZP-CSI-RS-ResourceId[1] | 4 | entry 1 CSI-RS resource #5 |  |
| NZP-CSI-RS-ResourceId[2] | 5 | entry 2 CSI-RS resource #6 |  |
| NZP-CSI-RS-ResourceId[3] | 6 | entry 3 CSI-RS resource #7 |  |
| NZP-CSI-RS-ResourceId[4] | 7 | entry 4 CSI-RS resource #8 |  |
| } |  |  |  |
| nzp-CSI-RS-Resources SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId { | 4 entries | For test 1-1 | Resource set #3 |
| NZP-CSI-RS-ResourceId[1] | 8 | entry 1 CSI-RS resource #9 |  |
| NZP-CSI-RS-ResourceId[2] | 9 | entry 2 CSI-RS resource #10 |  |
| NZP-CSI-RS-ResourceId[3] | 10 | entry 3 CSI-RS resource #11 |  |
| NZP-CSI-RS-ResourceId[4] | 11 | entry 4 CSI-RS resource #12 |  |
| } |  |  |  |
| trs-Info | true |  |  |
| } |  |  |  |

Table 5.2.2.2.21.4.3.1-8: NZP-CSI-RS-Resource for CSI Acquisition

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-Resource ::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceId | 12 for CSI-RS resource #13 13 for CSI-RS resource #14 14 for CSI-RS resource #15 | for test 1-1 |  |
| qcl-InfoPeriodicCSI-RS | 0 for CSI-RS resource #13 1 for CSI-RS resource #14 2 for CSI-RS resource #15 | for test 1-1: TCI-State #0 for CSI-RS resource #13 TCI-State #1 for CSI-RS resource #14 TCI-State #2 for CSI-RS resource #15 |  |
| } |  |  |  |

Table 5.2.2.2.21.4.3.1-9: CSI-RS-ResourceMapping for CSI Acquisition (Table 5.2.2.2.21.4.3.1-8)

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| firstOFDMSymbolInTimeDomain | 12 for CSI-RS resource #13 13 for CSI-RS resource #14 7 for CSI-RS resource #15 | for test 1-1 l0=12 for CSI-RS resource #13 l0=13 for CSI-RS resource #14 l0=7 for CSI-RS resource #15 |  |
| } |  |  |  |

Table 5.2.2.2.21.4.3.1-10: CSI-ResourcePeriodicityAndOffset for CSI Acquisition (Table 5.2.2.2.21.4.3.1-8)

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourcePeriodicityAndOffset ::= CHOICE { |  |  |  |
| slots40 | 0 | For test 1-1 periodicity = 40 slots. offset = 0 slots |  |
| } |  |  |  |

Table 5.2.2.2.21.4.3.1-11: NZP-CSI-RS-ResourceSet for CSI Acquisition

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceSet ::= SEQUENCE { |  |  |  |
| nzp_CSI_ResourceSetId | 3 for Resource set #4 4 for Resource set #5 5 for Resource set #6 | For test 1-1 |  |
| nzp-CSI-RS-Resources SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId { | 1 entry | For test 1-1 | Resource set #4 |
| NZP-CSI-RS-ResourceId[1] | 12 | entry 1 CSI-RS resource #13 |  |
| } |  |  |  |
| nzp-CSI-RS-Resources SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId { | 1 entry | For test 1-1 | Resource set #5 |
| NZP-CSI-RS-ResourceId[1] | 13 | entry 1 CSI-RS resource #14 |  |
| } |  |  |  |
| nzp-CSI-RS-Resources SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId { | 1 entry | For test 1-1 | Resource set #5 |
| NZP-CSI-RS-ResourceId[1] | 14 | entry 1 CSI-RS resource #15 |  |
| } |  |  |  |
| } |  |  |  |

Table 5.2.2.2.21.4.3.1-12: TCI-State

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-190 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TCI-State ::= SEQUENCE { |  |  |  |
| tci-StateId | 0 for TCI state #0 1 for TCI state #1 2 for TCI state #2 3 for TCI state #3 4 for TCI state #4 5 for TCI state #5 | For test 1-1 |  |
| qcl-Type1 SEQUENCE { |  |  |  |
| bwp-Id | BWP-Id of active BWP |  | TCI state #0, TCI state #1, TCI state #2 |
|  | Not present |  | TCI state #3, TCI state #4, TCI state #5 |
| referenceSignal CHOICE { |  |  |  |
| csi-rs | 0 | CSI-RS resource #1 | TCI state #0 |
|  | 4 | CSI-RS resource #5 | TCI state #1 |
|  | 8 | CSI-RS resource #9 | TCI state #2 |
| ssb | 0 | SSB #0 | TCI state #3 |
|  | 1 | SSB #1 | TCI state #4 |
|  | 2 | SSB #2 | TCI state #5 |
| } |  |  |  |
| qcl-Type | typeA |  | TCI state #0, TCI state #1, TCI state #2 |
|  | typeC |  | TCI state #3, TCI state #4, TCI state #5 |
| } |  |  |  |
| } |  |  |  |

5.2.2.2.21.4.3.2	Message exceptions for NSA

Same as 5.2.2.2.21.4.3.1.

5.2.2.2.21.5	Test Requirement

Tables 5.2.2.2.21.3-3 defines the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A 3.2.1 for each throughput test shall meet or exceed the specified value in Table 5.2.2.2.21.5-1 for the specified SNR including test tolerances for all throughput tests.

Table 5.2.2.2.21.5-1: Test Requirements for HST-SFN Scheme A

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.2-30.1 TDD | 40 / 30 | 16QAM, 0.48 | HST-SFN Scheme A | 2x2 | 70 | [13.5] |

5.2.3	4RX requirements

5.2.3.1	FDD

5.2.3.1.1	4Rx FDD FR1 PDSCH mapping Type A performance

5.2.3.1.1.0	Minimum conformance requirements

The performance requirements are specified in Table 5.2.3.1.1.0-3, Table 5.2.3.1.1.0-4, Table 5.2.3.1.1.0-5, Table 5.2.3.1.1.0-6 and Table 5.2.3.1.1.0-7, with the addition of test parameters in Table 5.2.3.1.1.0-2 and the downlink physical channel setup according to Annex C.2.1.

The test purposes are specified in Table 5.2.3.1.1.0-1.

< End of Changes >