3GPP TSG-RAN5 Meeting #102	R5-241657

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 51.010-1 | CR | 5168 | rev | 1 | Current version: | 13.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removal of GEA1 and GEA2 algorithm verification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK, Bureau Veritas |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI_Test |  |  |  |  |  | Date: |  |  | 2024-02-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Support of ciphering algorithms GEA1 and GEA2 have been prohibited from all releases of 3GPP core specifications published in December 2023. RAN5 test specifications need to be updated to align with the latest version of 3GPP core specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Voided or updated, as appropriate, the test cases that need support of GEA1 or GEA2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification will not be aligned to 3GPP TSG decision |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 44.2.5.2.1, 44.2.5.2.4, 46.1.2.1.1, 46.1.2.7.3, 46.1.2.7.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-240475->R5-241657: Replaced execution counter with parameter and added PICS for GEA4 in the Specific PICS statements section in test case 44.2.5.2.4 |  |  |  |  |  |  |  |  |

### 20.22.29a	Void

SKIPPED TEXT

##### 44.2.5.2.1	Ciphering mode / start ciphering

44.2.5.2.1.1	Conformance requirements

1.	When the MS receives the AUTHENTICATION AND CIPHERING REQUEST message during the attach procedure, with Ciphering indicator information element set to 'ciphering mode off', the Mobile Station shall:

1.1.	responds with an AUTHENTICATION AND CIPHERING RESPONSE message;

1.2.	not start ciphering.

2.	When the MS receives the AUTHENTICATION AND CIPHERING REQUEST message during the routing area updating procedure, with Ciphering indicator information element set to 'ciphering mode on', the Mobile Station shall:

2.1.	responds with an AUTHENTICATION AND CIPHERING RESPONSE message;

2.2.	start ciphering and deciphering with the algorithm indicated by the Ciphering algorithm information element;

2.3.	the ciphering uses the cipher key determined during the authentication procedure.

Reference(s):

3GPP TS 04.08 / 3GPP TS 24.008 subclause 4.7.7.

44.2.5.2.1.2	Test purpose

To test the behaviour of the MS if the network accepts the authentication and ciphering procedure with ciphering.

44.2.5.2.1.3	Method of test

Initial conditions

System Simulator:

Two cells (not simultaneously activated), cell A in MCC1/MNC1/LAC1/RAC1, cell B in MCC1/MNC1/LAC1/RAC2.

Both cells are operating in network operation mode II.

Mobile Station:

For parameter K = 4 (GEA4) Test USIM has to be plugged into the MS 
The MS has a valid IMSI. MS is Idle Updated on cell A.

Specific PICS statements:

-	MS operation mode B (TSPC_operation_mode_B).

-	MS operation mode C (TSPC_operation_mode_C).

-	Switch off on button (TSPC_Feat_OnOff).

-	Automatic GPRS attach procedure at switch on or power on (TSPC_AddInfo_on_auto_GPRS_AP).

-	Supported encryption Algorithm: GEA3 (TSPC_Feat_GEA3)

-	Supported encryption Algorithm: GEA4 (TSPC_Feat_GEA4)

PIXIT statements:

-

Test procedure

A GPRS attach is performed. Authentication procedure without ciphering is performed.

The MS initiates a routing area updating procedure, and the SS initiates an authentication and ciphering procedure to start ciphering. GEA3 or GEA4 encryption is used depending on the parameter K.

The test is performed for all GEAx encryption algorithm supported by the MS.

Maximum duration of test

15 minutes.

Expected sequence

The sequence is performed for parameter K=3 when the MS supports GEA3 and for K=4 when the MS supports GEA4.

| Step | Direction | Message | Comments |
| --- | --- | --- | --- |
|  |  |  | The following messages are sent and shall be received on cell A. |
| 1 | SS |  | The SS activates cell A. |
| 2 | MS |  | The MS is set in MS operation mode C (see PICS). If MS operation mode C not supported, goto step 28. |
| 3 | MS |  | The MS is powered up or switched on and initiates an attach (see PICS). |
| 4 | MS -> SS | ATTACH REQUEST | Attach type = 'GPRS attach' Mobile identity = IMSI Message not ciphered |
| 5 | SS -> MS | AUTHENTICATION AND CIPHERING REQUEST | Request authentication. Ciphering off Set GPRS-CKSN-1 RANDFor K=4 AUTN Message not ciphered |
| 6 | MS -> SS | AUTHENTICATION AND CIPHERING RESPONSE | For K=3 SRES For K=4 RES "Auth. Response Parameter (extension)" IE might be included if the RES value is more than 4 octets long. Message not ciphered |
| 7 | SS -> MS | ATTACH ACCEPT | Attach result = 'GPRS only attached' Mobile identity = P-TMSI-2 P-TMSI-2 signature Routing area identity = RAI-1 Message not ciphered |
| 8 | MS -> SS | ATTACH COMPLETE | Message not ciphered |
| 9 | SS -> MS | PAGING REQUEST TYPE 1 | Mobile identity = P-TMSI-2 Paging order is for TBF establishment. Message not ciphered |
| 10 | MS -> SS | UPLINK RLC DATA BLOCK | LLC PDU implicitly indicating paging response. Message not ciphered |
|  |  |  | The following messages are sent and shall be received on cell B. |
| 11 | SS |  | Activate cell B with a lower signal strength than cell A The RF level of cell A is lowered until cell B is preferred by the MS. |
| 12 | MS -> SS | ROUTING AREA UPDATING REQUEST | Update type = 'RA updating' P-TMSI-2 signature Routing area identity = RAI-1 Message not ciphered |
| 13 | SS -> MS | AUTHENTICATION AND CIPHERING REQUEST | Request authentication. Ciphering on with encryption: GEA3 for K=3. GEA4 for K=4. Set GPRS-CKSN-2 RAND For K=4 AUTN Message not ciphered |
| 14 | MS -> SS | AUTHENTICATION AND CIPHERING RESPONSE | For K=3 SRES For K=4 RES "Auth. Response Parameter (extension)" IE might be included if the RES value is more than 4 octets long. Message not ciphered |
| 15 | SS -> MS | ROUTING AREA UPDATING ACCEPT | Update result = 'RA updated' Mobile identity = P-TMSI-1 P-TMSI-1 signature Routing area identity = RAI-4 Message ciphered |
| 16 | MS -> SS | ROUTING AREA UPDATING COMPLETE | Message ciphered |
| 17 | SS -> MS | PAGING REQUEST TYPE 1 | Mobile identity = P-TMSI-1 Paging order is for TBF establishment. Message not ciphered |
| 18 | MS -> SS | UPLINK RLC DATA BLOCK | LLC PDU implicitly indicating paging response. Message may be ciphered depending on the type of LLC PDU that are sent. The 'E' bit is therefore not checked. |
| 19 | SS -> MS | P-TMSI REALLOCATION COMMAND | Mobile identity = P-TMSI-2 P-TMSI-2 signature Routing area identity = RAI-4 Message ciphered |
| 20 | MS -> SS | P-TMSI REALLOCATION COMPLETE | Message ciphered |
| 21 | SS -> MS | IDENTITY REQUEST | Identity type = IMEI Message not ciphered |
| 22 | MS -> SS | IDENTITY RESPONSE | Mobile identity = IMEI Message not ciphered |
| 23 | SS -> MS | P-TMSI REALLOCATION COMMAND | Mobile identity = P-TMSI-1 P-TMSI-1 signature Routing area identity = RAI-4 Message ciphered |
| 24 | MS -> SS | P-TMSI REALLOCATION COMPLETE | Message ciphered |
| 25 | MS |  | The MS is switched off or power is removed (see PICS). |
| 26 | MS -> SS | DETACH REQUEST | Message not sent if power is removed. Detach type = 'power switched off, GPRS detach' Message ciphered |
| 27 | SS |  | Cell B is powered down and Cell A is restored to full power. |
| 28 | MS |  | The MS is set in MS operation mode B (see PICS) and the test is repeated from step 3 to step 26. |

Note that due to the test of ciphering, it is in this test case indicated whether each message is ciphered or not.

Specific message contents

AUTHENTICATION AND CIPHERING REQUEST in step 5:

Same as default content except:

| Information element | Value/remark |
| --- | --- |
| IE AUTN | Not present for K = 3 Present for K = 4, calculated as defined for Test USIM |
| Ciphering Algorithm |  |
| Type of Algorithm | No ciphering |

AUTHENTICATION AND CIPHERING REQUEST in step 13:

Same as default content except:

| Information element | Value/remark |
| --- | --- |
| IE AUTN | Not present for K = 3 Present for K = 4, calculated as defined for Test USIM |
| Ciphering Algorithm |  |
| Type of Algorithm | GEA/3 for K = 3 GEA/4 for K = 4 |

SKIPPED TEXT

##### 44.2.5.2.4	Ciphering mode/Cipher key Kc128 and algorithm changes

44.2.5.2.4.1	Conformance requirement

A Mobile Station shall correctly respond in an authentication and ciphering procedure by sending a response with the SRES information field set to the same value as the one produced by the authentication and ciphering algorithm in the network.

In a UMTS authentication challenge, if the AUTHENTICATION_AND_CIPHERING REQUEST message includes the UMTS authentication parameters GPRS CKSN, RAND and AUTN, then upon receipt of the message, the MS verifies the AUTN parameter and if this is accepted, the MS processes the challenge information and sends an AUTHENTICATION_AND_CIPHERING RESPONSE message to the network.

In a UMTS authentication challenge, the new UMTS ciphering key, the new GSM ciphering key and the new UMTS integrity key calculated from the challenge information shall overwrite the previous UMTS ciphering key, GSM ciphering key and UMTS integrity key. The new UMTS ciphering key, GSM ciphering key and UMTS integrity key are stored on the USIM together with the ciphering key sequence number. Furthermore, in A/Gb mode when after the authentication procedure an A5 ciphering algorithm that requires a 128-bit ciphering key is taken into use, then a new GSM Kc128 shall also be calculated as described in the subclause 4.3.2.3a

Reference(s):

3GPP TS 24.008 subclause 4.7.7.2.

3GPP TS 24.008 subclause 4.3.2.2.

44.2.5.2.4.2	Test purpose

To verify that the MS uses correctly Kc and Kc128 when the GPRS Encryption Algorithm is changed from GEA3 to GEA4 and from GEA4 to GEA3.

44.2.5.2.4.3	Method of test

Initial conditions

System Simulator:

Two cells (not simultaneously activated), cell A in MCC1/MNC1/LAC1/RAC1, cell B in MCC1/MNC1/LAC1/RAC2.

Both cells are operating in network operation mode II, SGSN is R99

Mobile Station:

Test USIM is plugged into the MS. The MS has a valid IMSI. MS is Idle Updated on cell A.

Specific PICS statements:

-	MS operation mode B (TSPC_operation_mode_B).

-	MS operation mode C (TSPC_operation_mode_C).

-	Switch off on button (TSPC_Feat_OnOff).

-	Automatic GPRS attach procedure at switch on or power on (TSPC_AddInfo_on_auto_GPRS_AP).

-	Supported encryption Algorithm: GEA3 (TSPC_Feat_GEA3)

-	Supported encryption Algorithm: GEA4 (TSPC_Feat_GEA4)

PIXIT statements:

-

Test procedure

A GPRS attach is performed, and the SS initiates an authentication and ciphering procedure with an UMTS challenge request; type of algorithm is GEA3.

The SS checks the value RES sent by the MS in the AUTHENTICATION AND CIPHERING RESPONSE message (calculated with UMTS AKA algorithm).

The cell A is deactivated and cell B activated .

The MS initiates a routing area updating procedure and the SS initiates an authentication and ciphering procedure with an UMTS challenge request. The SS sends ROUTING ARE UPDATING ACCEPT ciphered with GEA4 and the MS answer with ciphered ROUTING ARE UPDATING COMPLETE.

The cell B is deactivated and cell A activated

The MS initiates a routing area updating procedure and the SS initiates an authentication and ciphering procedure with an UMTS challenge request. The SS sends ROUTING ARE UPDATING ACCEPT ciphered with GEA3 and the MS answer with ciphered ROUTING ARE UPDATING COMPLETE.

Expected sequence

The sequence is executed with GEA3.

| Step | Direction | Message | Comments |
| --- | --- | --- | --- |
|  |  |  | The following messages are sent and shall be received on cell A. |
| 1 | SS |  | The SS activates cell A. |
| 2 | MS |  | The MS is set in MS operation mode C (see PICS). If MS operation mode C not supported, goto step 23. |
| 3 | MS |  | The MS is powered up or switched on and initiates an attach (see PICS). |
| 4 | MS -> SS | ATTACH REQUEST | Attach type = 'GPRS attach' Mobile identity = IMSI |
| 5 | SS -> MS | AUTHENTICATION AND CIPHERING REQUEST | Request UMTS authentication. Set GPRS-CKSN-1 RAND & AUTN included (see specific message content) Type of algorithm: GEA3 |
| 6 | MS -> SS | AUTHENTICATION AND CIPHERING RESPONSE | The SS checks the RES value. "Auth. Response Parameter (extension)" IE included if the RES value is more than 4 octets long. |
| 7 | SS -> MS | ATTACH ACCEPT | Message ciphered with GEA3 Attach result = 'GPRS only attached' Mobile identity = P-TMSI-2 P-TMSI-2 signature Routing area identity = RAI-1 |
| 8 | MS -> SS | ATTACH COMPLETE | Message ciphered |
| 9 | SS |  | The SS deactivates cell A and activates cell B. |
|  |  |  | The following messages are sent and shall be received on cell B. |
| 10 | MS -> SS | ROUTING AREA UPDATING REQUEST | Update type = 'RA updating' P-TMSI-2 signature Routing area identity = RAI-1 GPRS-CKSN-1 |
| 11 | SS -> MS | AUTHENTICATION AND CIPHERING REQUEST | Request UMTS authentication. Set GPRS-CKSN-1 RAND & AUTN included (see specific message content) Type of algorithm: GEA4 |
| 12 | MS -> SS | AUTHENTICATION AND CIPHERING RESPONSE | The SS checks the RES value. "Auth. Response Parameter (extension)" IE included if the RES value is more than 4 octets long. |
| 13 | SS -> MS | ROUTING AREA UPDATING ACCEPT | Message ciphered with GEA4 Update result = 'RA updated' Mobile identity = P-TMSI-1 P-TMSI-1 signature Routing area identity = RAI-4 |
| 14 | MS -> SS | ROUTING AREA UPDATING COMPLETE | Message ciphered |
| 15 | SS |  | The SS deactivates cell B and activates cell A. |
|  |  |  | The following messages are sent and shall be received on cell A. |
| 16 | MS -> SS | ROUTING AREA UPDATING REQUEST | Update type = 'RA updating' P-TMSI-1 signature Routing area identity = RAI-4 GPRS-CKSN-1 |
| 17 | SS -> MS | AUTHENTICATION AND CIPHERING REQUEST | Request UMTS authentication. Set GPRS-CKSN-1 RAND & AUTN included (see specific message content) Type of algorithm: GEA3 |
| 18 | MS -> SS | AUTHENTICATION AND CIPHERING RESPONSE | The SS checks the RES value. "Auth. Response Parameter (extension)" IE included if the RES value is more than 4 octets long. |
| 19 | SS -> MS | ROUTING AREA UPDATING ACCEPT | Message ciphered with GEA3 Update result = 'RA updated' Mobile identity = P-TMSI-2 P-TMSI-2 signature Routing area identity = RAI-1 |
| 20 | MS -> SS | ROUTING AREA UPDATING COMPLETE | Message ciphered |
| 21 | MS |  | The MS is switched off or power is removed (see PICS). |
| 22 | MS -> SS | DETACH REQUEST | Message not sent if power is removed. Detach type = 'power switched off, GPRS detach' |
| 23 | MS |  | The MS is set in MS operation mode B (see PICS) and the test is repeated from step 3 to step 22. |

Specific message contents

AUTHENTICATION AND CIPHERING REQUEST in step 5:

Same as default content except:

| Information element | Value/remark |
| --- | --- |
| IE AUTN | Calculated as defined for Test USIM |
| Ciphering Algorithm |  |
| Type of Algorithm | GPRS Encryption Algorithm - GEA3 |

AUTHENTICATION AND CIPHERING REQUEST in step 11:

Same as default content except:

| Information element | Value/remark |
| --- | --- |
| IE AUTN | Calculated as defined for Test USIM |
| Ciphering Algorithm |  |
| Type of Algorithm | GPRS Encryption Algorithm GEA4 |

AUTHENTICATION AND CIPHERING REQUEST in step 17:

Same as default content except:

| Information element | Value/remark |
| --- | --- |
| IE AUTN | Calculated as defined for Test USIM |
| Ciphering Algorithm |  |
| Type of Algorithm | GPRS Encryption Algorithm - GEA3 |

SKIPPED TEXT

##### 46.1.2.1.1	Data transmission in protected mode

46.1.2.1.1.1	Conformance requirement

LLC has two modes of operation - acknowledged and unacknowledged. In the unacknowledged mode of operation, layer3 information is transmitted in numbered Unconfirmed Information (UI) frames. The UI frames are not acknowledged at the LLC layer. Neither error recovery nor reordering mechanisms is defined, but transmission and format errors are detected. Duplicate UI frames are discarded.

In the protected mode of unacknowledged operation, the FCS field protects the frame header and the information field.

Unacknowledged mode of operation is defined for all SAPIs that are not reserved.

Reference

3GPP TS 04.64, subclause 4.3.

46.1.2.1.1.2	Test purpose

To verify that the MS performs unacknowledged data transfer for SAPIs 3, 5 and 11 in the protected mode to the network

46.1.2.1.1.3	Method of test

Initial conditions

For parameter K = 4 (GEA4) Test USIM has to be plugged into the MS 
The MS shall be GPRS attached with ciphering enabled. Encryption GEA3 or GEA4 is used depending on the parameter K.

Specific PICS statements:

-	Supported encryption Algorithm: GEA3 (TSPC_Feat_GEA3)

-	Supported encryption Algorithm: GEA4 (TSPC_Feat_GEA4)

PIXIT statements:

-

Test procedure

During GPRS attach ciphering GEA3 or GEA4 is activated depending on the parameter K.

During GPRS attach and PDP context activation, the GMM messages are sent and received by the LLC layer at the MS using UI frames in the unacknowledged mode, on SAPI 1. This implicitly verifies bi-directional unacknowledged data transmission on SAPI 1.

After the PDP context is activated, the MS is made to initiate unacknowledged data transfer on SAPI 3. MS shall transmit UI frames with the E and PM bits set to 1, indicating that encryption and protection are on. Transmit 5 000 octets from the MS.

Repeat the test case for SAPIs 5 and 11.

The test is performed for all GEAx encryption algorithm supported by the MS.

Maximum duration of the test

10 minutes.

Expected sequence

The sequence is performed for parameter K=3 when the MS supports GEA3 and for K=4 when the MS supports GEA4.

| Step | Direction | Message | Comments |
| --- | --- | --- | --- |
| 1 |  | {PDP Context Activation} | Macro. PDP context activation from the MS. The PDP context used here is PDP context 5. |
| 2 |  |  | Initiate data transfer of 5000 octets from the MS. |
| 3 | MS -> SS | UI frame | Verify that the number of octets in the UI frame does not exceed N201-U. Verify that E=1 and PM=1, PD=0, C/R = 0 and the FCS is correct. Check whether the SAPI is 3, 5 or 11 when data is sent from the MS on these SAPIs. Verify that the sequence numbers are correct and that there are no duplicate or missing frames |
| 4 |  |  | Repeat step 3 until 5000 octets are sent. |
| 5 |  |  | Repeat the test case for SAPIs 5 and 11.The PDP context used for SAPI 5 is PDP Context 8 and the one for SAPI 11 is PDP Context 9. |

SKIPPED TEXT

##### 46.1.2.7.3	Negotiation initiated by the SS (using XID, for IOV-UI)

46.1.2.7.3.1	Conformance requirement

The negotiation procedure is one-step, i.e., one side shall start the process by sending an XID command, offering a certain set of parameters from the applicable parameter repertoire the sending entity wants to negotiate, proposing values within the allowed range. In return, the other side shall send an XID response, either confirming these requested values, or offering higher or lower ones in their place.

XID frames shall always be used with the P/F bit set to 1.

LLC layer and layer-3 parameters may be negotiated with the exchange of XID frames or with the exchange of SABM and UA frames. After successful negotiations of SABM and UA frames, the LLE shall be in ABM mode of operation.

IOV-UI shall only be negotiated in ADM. IOV-UI and IOV-I shall only be transmitted in the downlink direction.

Reference

3GPP TS 04.64, subclauses 6.4.1.6 and 8.5.3.

46.1.2.7.3.2	Test purpose

To verify that when the SS sends IOV-UI to the MS in a XID, the MS shall cipher its output using this value of IOV-UI.

46.1.2.7.3.3	Method of test

Initial conditions

For parameter K = 4 (GEA4) Test USIM has to be plugged into the MS. The MS shall be GPRS attached with ciphering enabled. Encryption GEA3 or GEA4 is used depending on the parameter K.

Specific PICS statements:

-	Supported encryption Algorithm: GEA3 (TSPC_Feat_GEA3)

-	Supported encryption Algorithm: GEA4 (TSPC_Feat_GEA4)

PIXIT statements:

-

Test procedure

During GPRS attach ciphering GEA3 or GEA4 is activated depending on the parameter K.

Initiate link establishment from the SS. In the XID command, send a new value of IOV-UI, different from the default used. Send 1000 octets from the MS and verify that the frames have been ciphered as per the new value of IOV-UI.

The test is performed for all GEAx encryption algorithm supported by the MS.

Maximum duration of the test

5 minutes.

Expected sequence

The sequence is performed for parameter K=3 when the MS supports GEA3 and for K=4 when the MS supports GEA4.

| Step | Direction | Message | Comments |
| --- | --- | --- | --- |
| 1 |  | {PDP Context Activation} | Macro. Initiate PDP context activation from the MS using PDP Context 5. |
| 2 |  | {PDP Context Modification} | Macro. Initiate PDP context modification to PDP Context 3 from the SS. |
| 3 | SS -> MS | XID | With IOV-UI = 2 27 * 10. |
| 4 | MS -> SS | XID |  |
| 5 |  |  | Initiate data transfer from the MS. Send 1000 octets. |
| 6 | MS -> SS | UI | Verify that ciphering is as per the new value of IOV-UI. The SS shall check this by analyzing the FCS. |
| 7 |  |  | Repeat the steps 6 till the data transfer is complete. |

SKIPPED TEXT

##### 46.1.2.7.5	Negotiation initiated by the SS (during ADM, for IOV-UI)

46.1.2.7.5.1	Conformance requirement

The negotiation procedure is one-step, i.e., one side shall start the process by sending an XID command, offering a certain set of parameters from the applicable parameter repertoire the sending entity wants to negotiate, proposing values within the allowed range. In return, the other side shall send an XID response, either confirming these requested values, or offering higher or lower ones in their place.

XID frames shall always be used with the P/F bit set to 1.

IOV-UI shall only be negotiated in ADM. IOV-UI and IOV-I shall only be transmitted in the downlink direction.

IOV-UI is associated with a TLLI.

Reference

3GPP TS 04.64, subclauses 6.4.1.6, 8.5.3 and 8.9.

3GPP TS 04.08 subclause 4.7.12.

46.1.2.7.5.2	Test purpose

To verify that when the SS sends IOV-UI to the MS in an XID command:

-	The MS shall cipher its output using this value of IOV-UI.

-	This value of IOV-UI shall be applicable for all SAPIs using this TLLI.

-	Identity Response sent from the MS shall not be ciphered.

46.1.2.7.5.3	Method of test

Initial conditions

-	For parameter K = 4 (GEA4) Test USIM has to be plugged into the MS.

Specific PICS statements:

-	Supported encryption Algorithm : GEA3 (TSPC_Feat_GEA3)

-	Supported encryption Algorithm : GEA4 (TSPC_Feat_GEA4)

PIXIT statements:

-

Test procedure

Send a value of IOV-UI from the SS, different from the default used. Send 1 000 octets from the MS and verify that the frames have been ciphered as per the new value of IOV-UI.

Send 1 000 octets from the MS on SAPI 11. Verify that the frames have been ciphered as per the new value of IOV-UI.

The test is performed for all GEAx encryption algorithm supported by the MS.

Maximum duration of the test

3 minutes.

Expected sequence

The sequence is performed for parameter K=3 when the MS supports GEA3 and for K=4 when the MS supports GEA4.

| Step | Direction | Message | Comments |
| --- | --- | --- | --- |
| 1 |  | {PDP Context Activation} | Macro. Initiate a PDP context activation from the MS using PDP Context 10. |
| 2 | SS -> MS | UI [Authentication and Ciphering Request] | Send the Authentication and Ciphering Request from the SS to start ciphering using the following encryption algorithm: GEA3 for K=3, GEA4 for K=4. |
| 3 | MS -> SS | UI [Authentication and Ciphering Response] | For K=3 the SS checks the SRES For K=4, the SS check the RES and "Auth. Response Parameter (extension)" IE might be included if the RES value is more than 4 octets long. |
| 4 | SS -> MS | XID | XID command with IOV-UI = 5000. |
| 5 | MS -> SS | XID | Verify that the MS accepts this value of IOV-UI by sending an XID response. Verify the XID response received. If the MS has requested for a new set of XID parameters, verify that the values requested are within range. |
| 6 |  |  | Initiate unacknowledged data transfer of 1000 octets from the MS. |
| 7 | MS -> SS | UI | Verify that these frames have been ciphered as per the new value of IOV-UI. |
| 8 | SS -> MS | UI [Identity Request] | This UI frame must not be sent ciphered. |
| 9 | MS -> SS | UI [Identity Response] | This UI frame shall not be ciphered. The E bit shall not be set to 1. |
| 10 |  | {PDP Context Deactivation} | Macro. Deactivate PDP Context 10. |
| A11 (Optional step) |  |  | If the MS performs a GMM detach the SS completes the Detach procedure. |
| 11 |  | {PDP Context Activation} | Macro. Activate PDP Context 9. If the MS is detached in step A11, GPRS-re-attachment shall be performed. |
| A12 (Conditional step) |  |  | If the MS is re-attached in step 11, then step 2, 3, 4 and 5 shall be repeated to set IOV-UI parameter and restart ciphering. |
| 12 |  |  | Initiate a data transfer of 1000 octets from the MS on SAPI 11 for the same value of TLLI as before. |
| 13 | MS -> SS | UI | Verify that ciphering is as per the new value of IOV-UI. |

Specific message contents

AUTHENTICATION AND CIPHERING REQUEST in step 2:

Same as default content except :

| Information element | Value/remark |
| --- | --- |
| IE AUTN | Not present for K = 3 Present for K = 4 |
| Ciphering Algorithm |  |
| Type of Algorithm | GEA/3 for K = 3 GEA/4 for K = 4 |