3GPP TSG-RAN WG5 Meeting #102	R5-240111r1

Athens, Greece, 26 February – 1 March, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-1 | CR | 0458 | rev | - | Current version: | 17.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new NR RSTD accuracy test case 14.5.4 in RRC_INACTIVE state |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2024-01-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In RAN#97-e, WI of UE Conformance Test Aspects - NR positioning enhancements has been approved, related test cases should be introduced into TS 37.571-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added 14.5.4 UE Rx-Tx time difference measurement accuracy for single positioning frequency layer with reduced number of samples in FR2 SA |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test spec will not applicable to Release-17 NR positioning methods performance test |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.5.4 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 37.571-3 CR 0167 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | R1: Corrected the message contents table title |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

14.5.4	UE Rx-Tx time difference measurement accuracy for single positioning frequency layer with reduced number of samples in FR2 SA

Editor’s note: This test case is incomplete. The following aspect is either missing or TBD:

- Test tolerance are not added in and analysis is missing

14.5.4.1	Test purpose

The purpose of the test is to verify that the RSTD measurement in RRC_INACTIVE state with reduced number of samples meets the accuracy requirements specified in TS 38.133 [50] clause 10.1.23.2 in an environment with AWGN propagation conditions conditions in FR2 in standalone scenario when single positioning frequency layer is configured.

14.5.4.2	Test applicability

This test applies to all types of NR UE release 17 onwards that supports DL-TDOA positioning, DL-PRS measurement in RRC_INACTIVE and reduced number of samples for PRS measurement in RRC_INACTIVE state.

14.5.4.3	Minimum conformance requirements

The accuracy requirements for RSTD measurement shall be within ±(X+Y+Z+Δ) Tc.

…

X is defined in Table 10.1.23.2-8 for AWGN channel in FR2 provided that the following conditions are met.

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for RSTD measurements are fulfilled according to Annex B.2.14 for a corresponding Band for each relevant PRS resource configured for measurement.

-	UE supports positioning measurement with reduced number of sample and is indicated by LMF to perform positioning measurement with reduced number of samples.

…

If the UE doesn’t support Rx TEG reporting for RSTD measurement or when the measurements of reference cell and neighbour cell belong to different Rx TEGs, Y, Z and Δ are defined as follows:

-	When UE measures RSTD on PRS resources belonging to same PFL, Y=32 Tc, provided that the time offset between the two PRS resource instances from the reference cell and the neighbor cell, which are used for a single RSTD estimate, is no greater than 160 ms.

-	When UE measures RSTD on PRS resources belonging different PFLs, Y=256 Tc, provided that the time offset between the two PRS resource instances from the reference cell and the neighbor cell, which are used for a single RSTD estimate, is no greater than 1280 ms.

-	Z is defined in Table 10.1.23.2-5 for FR1 and Table 10.1.23.2-6 for FR2, respectively.

-	Δ is zero for single PFL, and is defined in Table 10.1.23.2-5a for FR1 and Table 10.1.23.2-6a for FR2, respectively, for dual PFL.

…

Table 10.1.23.2-8: RSTD absolute accuracy in FR2 for AWGN channel with reduced number of samples

| Accuracy | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PRS Ês/Iot | PRS SCS | PRS bandwidth Note 1 | PRS resource repetition (TrepPRS*LPRS/KcombPRS) Note 2 | Io Note 3 range |  |
|  |  |  |  |  | Minimum Io | Maximum Io |
| Tc Note 4 | dB | kHz | RB |  | dBm/SCS | dBm/BWChannel |
| 24 | (PRS Ês/Iot)ref ≥-3dB (PRS Ês/Iot)i ≥-6dB | 60 | ≥ 64 | ≥ 1 | Same value as PRS_RP in Table B.2.14-2, according to UE Power class, operating band and angle of arrival | Note 5 |
| 11 |  |  | ≥ 132 | ≥ 1 | Note 5 | Note 5 |
| 13 |  | 120 | ≥ 64 | ≥ 1 | Same value as PRS_RP in Table B.2.14-2, according to UE Power class, operating band and angle of arrival | Note 5 |
| 6 |  |  | ≥ 128 | ≥ 1 | Note 5 | Note 5 |
| NOTE 1: Minimum PRS bandwidth, which is minimum of the PRS bandwidths of the reference resource and the measured neighbour resource i. NOTE 2: Minimum number of PRS resource repetitions among the reference resource and the measured neighbour resource i. TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeNdefined in TS 37.355 [34], respectively. NOTE 3: Io is assumed to have constant EPRE across the bandwidth. NOTE 4: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 5: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 6: Void |  |  |  |  |  |  |

…

Table 10.1.23.2-6: Margin for RSTD measurement accuracy in FR2

| PRS BW (RB number) |  | Margin (Tc) |
| --- | --- | --- |
| SCS=60kHz | SCS=120kHz |  |
| ≥ 24 | N/A | 72 |
| ≥ 64 | ≥ 32 | 32 |
| ≥ 132 | ≥ 64 | 16 |
| N/A | ≥ 128 | 12 |

14.5.4.4	Test description

The supported test configurations in listed in Table 14.5.4.4-1

Table 14.5.4.4-1: Test Configurations

| Test Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB and PRS SCS, 100 MHz bandwidth, TDD duplex mode |

14.5.4.4.1	Initial conditions

Test Environment: Normal, as defined in TS 38.508-1 [45] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [45] clause 4.3.1.

Channel bandwidth to be tested: are specified in Table 14.5.4.4-1.

1.	Connect the SS and AWGN noise sources to the UE antenna connector or antenna connectors as shown in Annex A, Figure A.14.

2.	The general test parameter settings are set up according to Table 14.5.4.5-1 and Table 14.5.4.5-2.

3.	Propagation conditions are set according to clause 4.15.2.

4.	Message contents are defined in clause 14.5.4.4.3.

5.	In the test there are two synchronous cells: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell. Cell 2 is a neighbour cell. Both cells are on the same NR RF channel in FR2. The UE is configured with DRX cycle of 0.64s.

6.	The true RSTD (which is the receive time difference for frame 0 between cell 2 and cell 1 as seen at the UE antenna connector) is set to 5898 Tc (about 3 s)

14.5.4.4.2	Test procedure

Same as in clause 14.5.3.4.2.

14.5.4.4.3	Message contents

As defined in clause 14.4.3.4.3, with the following exceptions:

Table 14.5.4.4.3-1 replaces Table 14.4.3.4.3-7, Table 14.5.4.4.3-2 replaces Table 14.4.3.4.3-9 and Table 14.5.4.4.3-3 replaces Table 14.4.3.4.3-11.

Table 14.5.4.4.3-1: NR-DL-PRS-AssistanceData

| Derivation Path: 37.355 clause 6.4.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NR-DL-PRS-AssistanceData-r16 ::= SEQUENCE { |  |  |  |
| nr-DL-PRS-ReferenceInfo-r16 SEQUENCE { |  |  |  |
| dl-PRS-ID-r16 | 0 |  |  |
| nr-DL-PRS-ResourceID-List-r16 | Not present |  |  |
| nr-DL-PRS-ResourceSetID-r16 | Not present |  |  |
| } |  |  |  |
| nr-DL-PRS-AssistanceDataList-r16 SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF NR-DL-PRS-AssistanceDataPerFreq-r16 { | 1 entry |  |  |
| NR-DL-PRS-AssistanceDataPerFreq-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-PositioningFrequencyLayer-r16 SEQUENCE { |  |  |  |
| dl-PRS-SubcarrierSpacing-r16 | kHz120 |  |  |
| dl-PRS-ResourceBandwidth-r16 | 3 | 32 PRBs | Sub-test 1 |
|  | 27 | 128 PRBs | Sub-test 2 |
| dl-PRS-StartPRB-r16 | 0 |  |  |
| dl-PRS-PointA-r16 | absoluteFrequencyPointA as defined for the DL frequency of the Cell 1 |  |  |
| dl-PRS-CombSizeN-r16 | n2 |  | Sub-test 1 |
|  | n4 |  | Sub-test 2 |
| dl-PRS-CyclicPrefix-r16 | normal |  |  |
| } |  |  |  |
| nr-DL-PRS-AssistanceDataPerFreq-r16 SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF NR-DL-PRS-AssistanceDataPerTRP-r16{ | 2 entries |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[1] SEQUENCE { |  | entry 1 | Cell 1 |
| dl-PRS-ID-r16 | 0 |  |  |
| nr-PhysCellID-r16 | Cell 1 |  |  |
| nr-CellGlobalID-r16 | Cell 1 |  |  |
| nr-ARFCN-r16 | Cell 1 |  |  |
| nr-DL-PRS-SFN0-Offset-r16 SEQUENCE { |  |  |  |
| sfn-Offset-r16 | 0 |  |  |
| integerSubframeOffset-r16 | 0 |  |  |
| } |  |  |  |
| nr-DL-PRS-ExpectedRSTD-r16 | 0 |  |  |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | 0 |  |  |
| nr-DL-PRS-Info-r16 | As specified in Table 14.5.4.4.3-2 |  |  |
| } |  |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[2] SEQUENCE { |  | entry 2 | Cell 2 |
| dl-PRS-ID-r16 | 1 |  |  |
| nr-PhysCellID-r16 | Cell 2 |  |  |
| nr-CellGlobalID-r16 | Cell 2 |  |  |
| nr-ARFCN-r16 | Cell 2 |  |  |
| nr-DL-PRS-SFN0-Offset-r16 SEQUENCE { |  |  |  |
| sfn-Offset-r16 | 0 |  |  |
| integerSubframeOffset-r16 | 0 |  |  |
| } |  |  |  |
| nr-DL-PRS-ExpectedRSTD-r16 | 23 | About 3 s |  |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | 154 | About 5 s |  |
| nr-DL-PRS-Info-r16 | As specified in Table 14.5.4.4.3-2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.5.4.4.3-2: NR-DL-PRS-Info

| Derivation Path: 37.355 clause 6.4.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NR-DL-PRS-Info-r16 ::= SEQUENCE { |  |  |  |
| nr-DL-PRS-ResourceSetList-r16 SEQUENCE (SIZE (1..nrMaxSetsPerTrp-r16)) OF NR-DL-PRS-ResourceSet-r16 { | 1 entry |  |  |
| NR-DL-PRS-ResourceSet-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-ResourceSetID-r16 | 0 |  |  |
| dl-PRS-Periodicity-and-ResourceSetSlotOffset-r16 CHOICE { |  |  |  |
| scs120-r16 CHOICE { |  |  |  |
| n160-r16 | 80 |  |  |
| } |  |  |  |
| dl-PRS-ResourceRepetitionFactor-r16 | n2 |  | Sub-test 1 |
|  | Not present |  | Sub-test 2 |
| dl-PRS-ResourceTimeGap-r16 | s1 |  |  |
| dl-PRS-NumSymbols-r16 | n4 |  |  |
| dl-PRS-MutingOption1-r16 | Not present |  |  |
| dl-PRS-MutingOption2-r16 | Not present |  |  |
| dl-PRS-ResourcePower-r16 | 27 |  |  |
| dl-PRS-ResourceList-r16 SEQUENCE (SIZE (1..nrMaxResourcesPerSet-r16)) OF NR-DL-PRS-Resource-r16 { | 1 entry |  |  |
| NR-DL-PRS-Resource-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-ResourceID-r16 | 0 |  |  |
| dl-PRS-SequenceID-r16 | 0 |  |  |
| dl-PRS-CombSizeN-AndReOffset-r16 CHOICE { |  |  |  |
| n2-r16 | 0 |  | Sub-test 1 |
| n4-r16 | 0 |  | Sub-test 2 |
| } |  |  |  |
| dl-PRS-ResourceSlotOffset-r16 | 0 |  | Cell 1 |
|  | 4 |  | Cell 2 |
| dl-PRS-ResourceSymbolOffset-r16 | 0 |  |  |
| dl-PRS-QCL-Info-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.5.4.4.3-3: LPP ProvideLocation Information

| Derivation Path: TS 37.355 [49] clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |  |
| transactionID SEQUENCE { |  |  |  |  |
| initiator |  | locationServer |  |  |
| transactionNumber |  | 1 |  |  |
| } |  |  |  |  |
| endTransaction |  | TRUE |  |  |
| sequenceNumber |  | (0..255) |  |  |
| acknowledgement |  |  |  |  |
| lpp-MessageBody CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideLocationInformation SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideLocationInformation-r9 SEQUENCE { |  |  |  |  |
| commonIEsProvideLocationInformation |  | Not present. |  |  |
| a-gnss-ProvideLocationInformation |  | Not present |  |  |
| otdoa-ProvideLocationInformation |  | Not present |  |  |
| ecid-ProvideLocationInformation |  | Not present |  |  |
| epdu-ProvideLocationInformation |  | Not present |  |  |
| sensor-ProvideLocationInformation-r13 |  | Not present |  |  |
| tbs-ProvideLocationInformation-r13 |  | Not present |  |  |
| wlan-ProvideLocationInformation-r13 |  | Not present |  |  |
| bt-ProvideLocationInformation-r13 |  | Not present |  |  |
| nr-ECID-ProvideLocationInformation-r16 |  | Not present |  |  |
| nr-Multi-RTT-ProvideLocationInformation-r16 |  | Not present |  |  |
| nr-DL-AoD-ProvideLocationInformation-r16 |  | Not present |  |  |
| nr-DL-TDOA-ProvideLocationInformation-r16 SEQUENCE { |  |  |  |  |
| nr-DL-TDOA-SignalMeasurementInformation-r16 SEQUENCE { |  |  |  |  |
| dl-PRS-ReferenceInfo-r16 |  |  |  |  |
| nr-DL-TDOA-MeasList-r16 SEQUENCE (SIZE(1..nrMaxTRPs-r16)) OF NR-DL-TDOA-MeasElement-r16 { |  | 2 entries |  |  |
| NR-DL-TDOA-MeasElement-r16[1] SEQUENCE { |  |  | entry 1 |  |
| dl-PRS-ID-r16 |  |  |  |  |
| nr-PhysCellID-r16 |  | Cell 1 |  |  |
| nr-CellGlobalID-r16 |  |  |  |  |
| nr-ARFCN-r16 |  |  |  |  |
| nr-DL-PRS-ResourceID-r16 |  |  |  |  |
| nr-DL-PRS-ResourceSetID-r16 |  |  |  |  |
| nr-TimeStamp-r16 |  |  |  |  |
| nr-RSTD-r16 |  | Present |  |  |
| nr-AdditionalPathList-r16 |  |  |  |  |
| nr-TimingQuality-r16 |  |  |  |  |
| nr-DL-PRS-RSRP-Result-r16 |  |  |  |  |
| nr-DL-TDOA-AdditionalMeasurements-r16 |  |  |  |  |
| nr-UE-Rx-TEG-ID-r17 |  |  |  |  |
| nr-DL-PRS-FirstPathRSRP-Result-r17 |  |  |  |  |
| nr-los-nlos-Indicator-r17 |  |  |  |  |
| nr-AdditionalPathListExt-r17 |  |  |  |  |
| nr-DL-TDOA-AdditionalMeasurementsExt-r17 |  |  |  |  |
| } |  |  |  |  |
| NR-DL-TDOA-MeasElement-r16[2] SEQUENCE { |  |  | entry 2 |  |
| dl-PRS-ID-r16 |  |  |  |  |
| nr-PhysCellID-r16 |  | Cell 2 |  |  |
| nr-CellGlobalID-r16 |  |  |  |  |
| nr-ARFCN-r16 |  |  |  |  |
| nr-DL-PRS-ResourceID-r16 |  |  |  |  |
| nr-DL-PRS-ResourceSetID-r16 |  |  |  |  |
| nr-TimeStamp-r16 |  |  |  |  |
| nr-RSTD-r16 |  | Present |  |  |
| nr-AdditionalPathList-r16 |  |  |  |  |
| nr-TimingQuality-r16 |  |  |  |  |
| nr-DL-PRS-RSRP-Result-r16 |  |  |  |  |
| nr-DL-TDOA-AdditionalMeasurements-r16 |  |  |  |  |
| nr-UE-Rx-TEG-ID-r17 |  |  |  |  |
| nr-DL-PRS-FirstPathRSRP-Result-r17 |  |  |  |  |
| nr-los-nlos-Indicator-r17 |  |  |  |  |
| nr-AdditionalPathListExt-r17 |  |  |  |  |
| nr-DL-TDOA-AdditionalMeasurementsExt-r17 |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| nr-UE-RxTEG-TimingErrorMargin-r17 |  |  |  |  |
| } |  |  |  |  |
| nr-dl-tdoa-LocationInformation-r16 |  |  |  |  |
| nr-DL-TDOA-Error-r16 |  |  |  |  |
| nr-DL-TDOA-SignalMeasurementInstances-r17 |  |  |  |  |
| nr-DL-TDOA-LocationInformationInstances-r17 |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

14.5.4.5	Test requirement

FFS