3GPP TSG-RAN5 Meeting #102	R5-241435r1

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 1039 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to Annex E content and structure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Benelux B.V. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_cov_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Missing or incorrect Annex details in test specification |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added missing annex to align with core specification |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing annex detail which is necessary for testing within the test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex E |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1 corrects the Annex Structure by moving the EVM phase offset measurement procedure under Annex E from Annex F where it was incorrectly added. |  |  |  |  |  |  |  |  |

# <Unchanged Sections Skipped>

## E.6.10	Modified signal under test

Same as Annex E.5.10 and applies to EVM measurements on PRACH.

## E.6.11	Phase offset measurement for DMRS bundling

### E.6.11.1	Measurement point

The measurement point for phase offset measurement is defined in Figure F.8.1-1.

Figure F.8.1-1: Measurement point for phase offset for DMRS bundling

### E.6.11.2	Symbols used

Phase offset is determined based on DMRS REs (3 DMRS symbols per slot) with the option to use data symbols.

### E.6.11.3	Modified test signal

Same as described in Annex E.5.10 and Annex E.6.10.

### E.6.11.4	Phase offset measurement

The phase offset measurement is based on the phase response of the Tx chain  as derived based on Annex F.4.

The subcarrier at the carrier leakage frequency of the transmitted signal shall be excluded from the measured subcarriers.

The phase difference ∆φf for each measured subcarrier between a reference timeslot tref and the measurement timeslot tm is then calculated as defined below:

∆φf= φtm,f-φ(tref,f)

The phase offset between the reference and measurement timeslots are then calculated as the maximum over the results for all measured subcarriers as shown below:

PhaseOffset=maxf(∆φf)

## E.6.12	EVM for dual transmit polarizations

## E.6.12.1	General

A zero-forcing (ZF) MIMO receiver architecture is used so that transmissions by the UE, which are received by the test equipment on two polarizations, can be demodulated by the test equipment receiver.

Figure F.10.1-1: EVM calculation block diagram for 2-Layer UL MIMO

The TE receives signals from 2 different ports on two antenna polarizations in the test system.

For UL MIMO measurements a MIMO equalization step as described in section F.10.2 is performed to separate the layers.

For single layer transmissions received on two polarizations the MIMO equalization step as described in section F.10.2 is replaced by a maximum ratio combining step as described in section F.10.3.

Each layer is then processed as described in section F.10.4 to receive the measurement results for each individual layer.

## E.6.12.2	MIMO Equalization (UL MIMO transmission)

The MIMO equalization is based only on reference signals (DMRS) without using any data symbols. For the equalization process all available DMRS symbols shall be used.

The effective 2x2 channel matrix is estimated using reference signals of different subcarriers, e.g. in case of DMRS antenna ports 0 and 2. In case that same subcarriers are used, e.g. DMRS antenna ports 0 and 1, a channel decomposition is necessary taking advantage of the orthogonal codes wf and wt and assuming identical channel coefficients for adjacent subcarriers of same CDM group.

Effective channel including the precoding matrix P is:

H=HP= h0,0h0,1h1,0h1,1

with

hn,ν=ynrν*|rν|2

where y denotes the received symbol on port index n and r the reference signal for layer index ν.

Since reference signals of a specific layer are transmitted only on subcarriers of one CDM group channel, interpolation is needed in order to obtain channel coefficients for all subcarriers. Channel interpolation is done using the channel coefficients of active CDM group in all other CDM groups.

The channel coefficients used to calculate the equalizer coefficients are obtained after channel smoothing in frequency domain by computing the moving average of interpolated channel coefficients. The moving average window size is 7. For subcarriers at or near the edge of allocation the window size is reduced accordingly.

The ZF equalizer coefficients are calculated as the inverse of the effective channel matrix, in general:

GZF=H-1

## E.6.12.3	Maximum Ratio combining (Tx diversity transmission)

The maximum ratio combining is based only on reference signals (DMRS) without using any data symbols. For the equalization process all available DMRS symbols shall be used.

The effective 2x1 channel matrix is estimated using reference signals of different subcarriers. In case of transmit diversity, the effective channel includes the precoding matrix P:

H=HP= h0h1

with

hn=ynr*|r|2

where y denotes the received symbol on port index n and r the reference signal.

Since reference signals are transmitted only on subcarriers of one CDM group, channel interpolation is needed in order to obtain channel coefficients for all subcarriers. Channel interpolation is done using the channel coefficients of active CDM group in all other CDM groups.

The channel coefficients used to calculate the equalizer coefficients are obtained after channel smoothing in frequency domain by computing the moving average of interpolated channel coefficients. The moving average window size is 7. For subcarriers at or near the edge of allocation the window size is reduced accordingly.

The ZF equalizer coefficients for maximum ratio combining are calculated as pseudo inverse of effective channel, in general:

GZF=H+=(HHH)-1HH

## E.6.12.4	Layer processing

After performing either the MIMO equalization or maximum ratio combining as described in section F.10.2 or F.10.3 respectively, each layer is processed using the existing procedure as defined in Annex E of TS 38.521-2 [5].

Since the channel estimation is calculated only on the DMRS symbols, an averaging including all 14 symbols of one slot, i.e. data and reference signals, is needed in order to minimize EVM. The averaging is achieved by the least square (LS) equalization method described for single layer in Annex E.3. of TS 38.521-2 [5].

MS(f,t) and NS(f,t) are processed with a LS estimator, to derive one equalizer coefficient per time slot and per allocated subcarrier. EC(f) is defined for each layer as:

ECν(f)=t=013NSν(f,t)*NSν(f,t)t=013MSν(f,t)*NSν(f,t)

With * denoting complex conjugation. EC(f) are used to equalize layer data symbols.

EVM equalizer spectral flatness is derived from equalizer coefficients for each layer as follows:

cν=ECν(f) gν,02+gν,12

# <Unchanged Sections Skipped>

# F.8	FFS

∆φf

∆φf= φtm,f-φ(tref,f)

PhaseOffset=maxf(∆φf)

# F.9	FFS

# F.10	FFS

H=HP= h0,0h0,1h1,0h1,1

hn,ν=ynrν*|rν|2

GZF=H-1

H=HP= h0h1

hn=ynr*|r|2

GZF=H+=(HHH)-1HH

ECν(f)=t=013NSν(f,t)*NSν(f,t)t=013MSν(f,t)*NSν(f,t)

cν=ECν(f) gν,02+gν,12

# <End of changes>