# Title:	Test Tolerances for HD-FDD UE Transmit Timing Accuracy Test for Category NB1 UE in Standalone Mode for Satellite Access

Source:	Mediatek, Keysight

# 1.	Introduction

The purpose of this document is to present the methodology and principles used for the development of Test Tolerances for HD-FDD UE Transmit Timing Accuracy Test for Category NB1 UE in Standalone Mode for Satellite Access.

The test cases in question, from TS 36.521-3, are:

- 13.4.1.1	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under normal coverage for Satellite Access

- 13.4.1.2	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under enhanced coverage for Satellite Access

- 13.4.1.3	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under enhanced coverage with segment transmission in NGSO for Satellite Access

This document describes the process to derive the Test Tolerances.

# 2. 	Test case in TS 36.133

The test conditions are defined in the following extracts from TS 36.133 v18.5.0:

#### A.13.4.1.1	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under normal coverage for Satellite Access

##### A.13.4.1.1.1	Test Purpose and Environment

The purpose of this test is to verify that the Category NB1 UE under normal coverage is capable of following the frame timing change of the connected eNodeB and that the UE initial transmits timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits. This test will verify the requirements in clause 7.20A.

For this test a single NB-IoT cell is used. Test parameters are given in Table A.13.4.1.1.1-1, Table A.13.4.1.1.1-2 and A.13.4.1.1.1-3. The transmit timing is verified by the UE transmitting NPUSCH.

Table A.13.4.1.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, HD-FDD duplex mode |
| 2 | NGSO, HD-FDD duplex mode |
| Note: If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2. |  |

Table A.13.4.1.1.1-2: General Test Parameters for UE Transmit Timing Accuracy Tests for E-UTRAN HD-FDD Category NB1 UE in Standalone mode under normal coverage for Satellite Access

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| NB-IoT Operation mode |  |  | Standalone |
| DRX |  |  | OFF |
| Satellite information | Config 1 |  | SSC.1 |
|  | Config 2 |  | SSC.2 |
| NPRACH configuration |  |  | NPRACH.R-1 As specified in A.3.18 |
| NPDCCH repetition level |  |  | 1 |
| npdcch-StartSF-USS Note 1 |  |  | v8 |
| npdcch-NumRepetitions-r13 Note 1 |  |  | r1 |
| NPUSCH repetition level |  |  | 1 |
| Note 1: For further information see clause 6.7.3.2 in TS 36.331 [2]. |  |  |  |

Table A.13.4.1.1.1-3: nCell specific Test Parameters for UE Transmit Timing Accuracy Tests for E-UTRAN HD-FDD Category NB1 UE in Standalone mode under normal coverage for Satellite Access

| Parameter | Unit | Value |
| --- | --- | --- |
|  |  | Test 1 |
| RF Channel Number |  | 1 |
| BWchannel | kHz | 200 |
| NPDSCH parameter |  | R.18 HD-FDD |
| NPDCCH parameter |  | R.30 HD-FDD |
| NOCNG Patterns |  | NOP.3 FDD |
| NPBCH_RA | dB | 0 |
| NPBCH_RB |  |  |
| NPSS_RA |  |  |
| NSSS_RA |  |  |
| NPDCCH_RA |  |  |
| NPDCCH_RB |  |  |
| NPDSCH_RA |  |  |
| NPDSCH_RB |  |  |
| NOCNG_RA Note1 |  |  |
| NOCNG_RB Note1 |  |  |
|  | dBm/15 kHz | -88 |
|  | dB | 4 |
|  | dB | 4 |
| Antenna Configuration |  | 1x1 |
| Propagation condition | - | AWGN |
| Note 1 NOCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |

##### A.13.4.1.1.2	Test Requirements

For parameters specified in Tables A.13.4.1.1.1-1, A.13.4.1.1.1-2 and A.13.4.1.1.1-3. the initial transmit timing accuracy, the maximum amount of timing change in one adjustment, the minimum and the maximum adjustment rate shall be within the limits defined in clause 7.20A.2.

The following sequence of events shall be used to verify that the requirements are met.

The test sequence shall be carried out in RRC_CONNECTED:

a) 	After a connection is set up with the cell, the test system sends NPDCCH including uplink grant for NPUSCH transmission and the test system shall measure the UE transmit timing offset (nTs) and verify that it is within Te (nTs ≤ NTA_Ref+NTAoffset+NTA,common+NTA, UE-specific×TS   ± (97×TS – TGNSS_margin)) with respect to the first detected path (in time) of the corresponding downlink frame of NB-IoT cell 1.

TGNSS_margin is the margin for the GNSS position definition error considered in the core requirement, which needs to be substracted for the test requirement, due to the usuage of AT commands in the test. 
TGNSS_margin = 5.12TS

b) 	Using the value of n measured in a), the test system adjusts the downlink transmit timing for the cell:

-	if n < 0, by +(144 – |n|)TS compared to that in (a).

-	if n ≥ 0, by -(144 – |n|)TS compared to that in (a).

The timing adjustment is performed monotonically in multiple steps of |∆T| ≤ 9×TS per 256 ms (∆T is to be defined in the test procedure) until the above required total timing change is achieved, during which no grant is transmitted for the UE.

c) 	Immediately after (b), the test system sends NPDCCH including uplink grant for NPUSCH transmission and immediately after receiving NPUSCH the test system repeatedly sends NPDCCH including uplink grant for NPUSCH transmission until the UE transmit timing offset is within NTA_Ref+NTAoffset+NTA,common+NTA, UE-specific×TS   ± (97×TS – TGNSS_margin) with respect to the first detected path (in time) of the corresponding downlink frame of cell 1. The test system shall verify that the difference in timing between the first NPUSCH transmission in step c) and the NPUSCH transmission in step a) shall be not greater than the maximum amount of the magnitude of the timing change in one adjustment requirement in clause 7.20.2. Using the first NPUSCH transmission in step c) and subsequent NPUSCH transmissions. The test system shall verify that the adjustment step size and the adjustment rate shall be according to the requirements in clause 7.20A.2 until the UE transmit timing offset is within NTA_Ref+NTAoffset+NTA,common+NTA, UE-specific×TS   ± (97×TS – TGNSS_margin) with respect to the first detected path (in time) of the corresponding downlink frame of cell 1.

d) 	The test system the test system sends NPDCCH including uplink grant for NPUSCH transmission and shall verify that the UE transmit timing offset stays within NTA_Ref+NTAoffset+NTA,common+NTA, UE-specific×TS   ± (97×TS– TGNSS_margin) with respect to the first detected path  (in time) of the corresponding downlink frame of NB-IoT cell 1.

#### A.13.4.1.2	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under enhanced coverage for Satellite Access

##### A.13.4.1.2.1	Test Purpose and Environment

The purpose of this test is to verify that the Category NB1 UE under enhanced coverage is capable of following the frame timing change of the connected eNode B, that the UE initial transmit timing accuracy is within the specified limits and that the UE shall not adjust the uplink transmission timing autonomously during an ongoing repetition period other than at initial transmission. This test will verify the requirements in clause 7.20A.

For this test a single NB-IoT cell is used. Test parameters are given in Table A.13.4.1.2.1-1 and Table A.13.4.1.2.1-2, Table A.13.4.1.2.1-3 and Table A.13.4.1.2.1-4. The transmit timing is verified by the UE transmitting NPUSCH.

Table A.13.4.1.2.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, HD-FDD duplex mode |
| 2 | NGSO, HD-FDD duplex mode |
| Note: If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2. |  |

Table A.13.4.1.2.1-2: General Test Parameters for UE Transmit Timing Accuracy Tests for E-UTRAN HD-FDD Category NB1 UE in Standalone mode under enhanced coverage for Satellite Access

| Parameter |  | Unit | Value |  |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |
| NB-IoT Operation mode |  |  | Standalone | Standalone |
| Satellite information | Config 1 |  | SSC.1 | SSC.1 |
|  | Config 2 |  | SSC.2 | SSC.2 |
| DRX |  |  | OFF | ON |
| NPRACH configuration |  |  | NPRACH.R-1 As specified in A.3.18 |  |
| NPDCCH repetition level |  |  | 32 | 32 |
| npdcch-StartSF-USS Note 2 |  |  | v2 | v2 |
| npdcch-NumRepetitions-r13 Note 2 |  |  | r32 | r32 |
| NPUSCH resource units |  |  | 1 | 1 |
| NPUSCH repetition level |  |  | 128 | 128 |
| NPUSCH subcarrier spacing |  | kHz | 15 | 15 |
| NPUSCH number of subcarriers |  |  | 1 | 1 |
| NPUSCH modulation |  |  |  /4QPSK |  /4QPSK |
| NPUSCH Transport block size |  | Bits | 40 | 40 |
| Note 1: DRX related parameters are defined in Table A.13.4.1.2.1-4. Note 2: For further information see clause 6.7.3.2 in TS 36.331 [2]. |  |  |  |  |

Table A.13.4.1.2.1-3: nCell specific Test Parameters for UE Transmit Timing Accuracy Tests for E-UTRAN HD-FDD Category NB1 UE in Standalone mode under enhanced coverage for Satellite Access

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |
| RF Channel Number |  | 1 | 1 |
| BWchannel | kHz | 200 | 200 |
| NPDSCH parameter |  | R.18 HD-FDD | R.18 HD-FDD |
| NPDCCH parameter |  | R.30 HD-FDD | R.30 HD-FDD |
| NOCNG Patterns |  | NOP.3 FDD | NOP.3 FDD |
| NPBCH_RA | dB | 0 | 0 |
| NPBCH_RB |  |  |  |
| NPSS_RA |  |  |  |
| NSSS_RA |  |  |  |
| NPDCCH_RA |  |  |  |
| NPDCCH_RB |  |  |  |
| NPDSCH_RA |  |  |  |
| NPDSCH_RB |  |  |  |
| NOCNG_RA Note1 |  |  |  |
| NOCNG_RB Note1 |  |  |  |
|  | dBm/15 kHz | -88 | -88 |
|  | dB | -11 | -11 |
|  | dB | -11 | -11 |
| Antenna Configuration |  | 1x1 | 1x1 |
| Propagation condition | - | AWGN | AWGN |
| Note 1 NOCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table A.13.4.1.2.1-4: drx-Configuration to be used in UE Transmit Timing Accuracy Test 2 for E-UTRAN HD-FDD Category NB1 UE Standalone mode under enhanced coverage for Satellite Access

| Field | Value | Comment |
| --- | --- | --- |
|  | Test 2 |  |
| onDurationTimer | pp1 |  |
| drx-InactivityTimer | pp0 |  |
| drx-RetransmissionTimer | pp0 |  |
| longDRX-CycleStartOffset | sf2048 |  |
| shortDRX | disable |  |
| Note 1: For further information see clause 6.7.3 in TS 36.331 [2]. |  |  |

##### A.13.4.1.2.2	Test Requirements

For parameters specified in Tables A.13.4.1.2.1-1, Tables A.13.4.1.2.1-2, Tables A.13.4.1.2.1-3 and Tables A.13.4.1.2.1-4, the initial transmit timing accuracy shall be within the limits defined in clause 7.20A.2 and the UE shall not adjust the uplink transmission timing autonomously during an ongoing repetition period other than at initial transmission.

The following sequence of events shall be used to verify that the requirements are met.

The test sequence shall be carried out in RRC_CONNECTED for both non-DRX (for Test1) and DRX with a cycle length of 2048 ms (Tests 2):

a) 	After a connection is set up with the cell, the test system sends NPDCCH including uplink grant for NPUSCH transmission and the test system shall measure the UE transmit timing offset (nTs) and verify that it is within Te (nTs ≤NTA_Ref+NTAoffset+NTA,common+NTA, UE-specific×TS   ± (97×TS – TGNSS_margin)) with respect to the first detected path (in time) of the corresponding downlink frame of NB-IoT cell 1.

TGNSS_margin is the margin for the GNSS position definition error considered in the core requirement, which needs to be substracted for the test requirement, due to the usuage of AT commands in the test. 
TGNSS_margin = 5.12TS

b) 	The test system sends NPDCCH including uplink grant for NPUSCH transmission. After 16ms from the initial NPUSCH transmission, the test system adjusts the downlink transmit timing for the cell, using the value of n measured in a),

- if n < 0, by +(144 – |n|)TS compared to that in (a).

- if n ≥ 0, by -(144 – |n|)TS compared to that in (a).

The timing adjustment is performed monotonically in multiple steps of |∆T| ≤ 9×TS per 256 ms (∆T is to be defined in the test procedure) until the above required total timing change is achieved, during which no grant is transmitted for the UE.

c)	For test 2, the test system sends NPDCCH including uplink grant for NPUSCH transmission and shall verify that the UE transmit timing offset stays within NTA_Ref+NTAoffset+NTA,common+NTA, UE-specific×TS   ± (97×TS– TGNSS_margin) with respect to the first detected path  (in time) of the corresponding downlink frame of NB-IoT cell 1. The UE transmit timing offset shall be verified for the first transmission in the DRX cycle immediately after DL timing adjustment.

#### A.13.4.1.3	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under enhanced coverage with segment transmission in NGSO for Satellite Access

##### A.13.4.1.3.1	Test Purpose and Environment

The purpose of this test is to verify that the Category NB1 UE, which is not supporting the capability of ntn-SegmentedPrecompensationGaps-r17, under enhanced coverage is capable of following the frame timing change of the connected eNode B, that the UE initial transmit timing accuracy is within the specified limits and that the UE shall not adjust the uplink transmission timing autonomously during an ongoing repetition period other than at initial transmission or at the start of a transmission segment boundary. This test will verify the requirements in clause 7.20A.

For this test a single NB-IoT cell is used. Test parameters are given in Table A.13.4.1.3.1-1 and Table A.13.4.1.3.1-2. The transmit timing is verified by the UE transmitting NPUSCH.

Table A.13.4.1.3.1-1: General Test Parameters for UE Transmit Timing Accuracy Tests for E-UTRAN HD-FDD Category NB1 UE in Standalone mode under enhanced coverage for Satellite Access

| Parameter | Unit | Value | Commnet |
| --- | --- | --- | --- |
|  |  | Test 1 |  |
| NB-IoT Operation mode |  | Standalone |  |
| Satellite information |  | SSC.2 | For NGSO |
| DRX |  | OFF |  |
| NPRACH configuration |  | NPRACH.R-1 As specified in A.3.18 |  |
| NPDCCH repetition level |  | 32 |  |
| npdcch-StartSF-USS Note 2 |  | v2 |  |
| npdcch-NumRepetitions-r13 Note 2 |  | r32 |  |
| NPUSCH resource units |  | 1 |  |
| NPUSCH repetition level |  | 128 |  |
| NPUSCH subcarrier spacing | kHz | 15 |  |
| NPUSCH number of subcarriers |  | 1 |  |
| NPUSCH modulation |  |  /4QPSK |  |
| NPUSCH Transport block size | Bits | 40 |  |
| npusch-TxDuration | ms | 64 |  |
| Note 1: void Note 2: For further information see clause 6.7.3.2 in TS 36.331 [2]. |  |  |  |

Table A.13.4.1.3.1-2: nCell specific Test Parameters for UE Transmit Timing Accuracy Tests for E-UTRAN HD-FDD Category NB1 UE in Standalone mode under enhanced coverage for Satellite Access

| Parameter | Unit | Value | Commnet |
| --- | --- | --- | --- |
|  |  | Test 1 |  |
| RF Channel Number |  | 1 |  |
| BWchannel | kHz | 200 |  |
| NPDSCH parameter |  | R.18 HD-FDD |  |
| NPDCCH parameter |  | R.30 HD-FDD |  |
| NOCNG Patterns |  | NOP.3 FDD |  |
| NPBCH_RA | dB | 0 |  |
| NPBCH_RB |  |  |  |
| NPSS_RA |  |  |  |
| NSSS_RA |  |  |  |
| NPDCCH_RA |  |  |  |
| NPDCCH_RB |  |  |  |
| NPDSCH_RA |  |  |  |
| NPDSCH_RB |  |  |  |
| NOCNG_RA Note1 |  |  |  |
| NOCNG_RB Note1 |  |  |  |
|  | dBm/15 kHz | -88 |  |
|  | dB | -11 |  |
|  | dB | -11 |  |
| Antenna Configuration |  | 1x1 |  |
| Propagation condition | - | AWGN |  |
| Note 1 NOCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

##### A.13.4.1.3.2	Test Requirements

For parameters specified in Tables A.13.4.1.3.1-1, and Tables A.13.4.1.3.1-2, the initial transmit timing accuracy shall be within the limits defined in clause 7.20A.2 and the UE shall not adjust the uplink transmission timing autonomously during an ongoing repetition period other than at initial transmission or at the start of a transmission segment boundary.

The following sequence of events shall be used to verify that the requirements are met.

The test sequence shall be carried out in RRC_CONNECTED for both non-DRX (for Test1):

a) 	After a connection is set up with the cell, the test system sends NPDCCH including uplink grant for NPUSCH transmission and the test system shall measure the UE transmit timing offset (nTs) of the first transmission in each segment and verify that it is within Te (nTs ≤NTA_Ref+NTAoffset+NTA,common+NTA, UE-specific×TS   ± (97×TS – TGNSS_margin)) with respect to the first detected path (in time) of the corresponding downlink frame of NB-IoT cell 1.

TGNSS_margin is the margin for the GNSS position definition error considered in the core requirement, which needs to be substracted for the test requirement, due to the usuage of AT commands in the test. 
TGNSS_margin = 5.12TS

b) 	The test system sends NPDCCH including uplink grant for NPUSCH transmission. After 16ms from the initial NPUSCH transmission, the test system adjusts the downlink transmit timing for the cell, using the value of n measured in a),

- if n < 0, by +(144 – |n|)TS compared to that in (a).

- if n ≥ 0, by -(144 – |n|)TS compared to that in (a).

The timing adjustment is performed monotonically in multiple steps of |∆T| ≤ 9×TS per 256 ms (∆T is to be defined in the test procedure) until the above required total timing change is achieved, during which no grant is transmitted for the UE.

The test requirements in TS 36.133 refer to the core requirements in clause 7.20A.2:

### 7.20A.2	Requirements

The UE initial transmission timing error shall be less than or equal to Te where the timing error limit value Te is specified in Table 7.20A.2-1. This requirement applies when it is the first transmission in a DRX cycle or the first transmission in a repetition period (R>1) for NPUSCH and NPRACH, the first transmission after an uplink transmission gap in a repetition period (R>1) for NPUSCH and NPRACH transmission, or it is the transmission on PUR. The reference point for the UE initial transmit timing control requirement shall be the downlink timing of the serving NB-IoT cell minus NTA_Ref+NTAoffset+NTA,common+NTA, UE-specific×Ts. The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the serving NB-IoT cell. NTA_Ref for NPRACH is defined as 0. NTA_Ref+NTAoffset+NTA,common+NTA, UE-specific (in Ts units) for other channels is the difference between UE transmission timing and the Downlink timing immediately after when the last timing advance in clause 7.22A was applied. NTA_Ref for other channels is not changed until next timing advance is received.

Table 7.20A.2-1: Te Timing Error Limit

| Downlink Bandwidth (MHz) | Te_ |
| --- | --- |
| 0.18 | 97*TS |
| Note 1: TS is the basic timing unit defined in TS 36.211 |  |

When it is not the first transmission in a DRX cycle or there is no DRX cycle, and when it is the transmission for NPUSCH the UE shall, when no repetitions are configured on the uplink or the repetition period is R=1, be capable of changing the transmission timing according to the received downlink frame of the serving NB-IoT cell except when the timing advance in clause 7.22A is applied such that the UE transmission timing error shall be less than or equal to ±Te, where the timing error limit value Te is specified in Table 7.20A.2-1.

When no repetition period is configured, or the configured repetition period is R=1, all adjustments made to the UE uplink timing shall follow these rules:

1)	The maximum amount of the magnitude of the timing change, apart from a change of NTA, UE-specific due to satellite position update and NTA,common between the previous transmission and the current transmission, in one adjustment shall be 58.33*TS seconds.

2)	The minimum aggregate adjustment rate, apart from a change of NTA, UE-specific due to satellite position update and NTA,common between the previous transmission and the current transmission, shall be 7*TS per 1second.

3)	The maximum aggregate adjustment rate, apart from a change of NTA, UE-specific due to satellite position update and NTA,common between the previous transmission and the current transmission, shall be 58.33*TS per 200ms.

when a repetition is configured on the uplink for which R>1, the UE shall not adjust the uplink transmission timing autonomously during an ongoing repetition period other than at initial transmission or at the start of a transmission segment boundary, as defined above.

# 3. 	Calculation of Test Tolerances

The test case 13.4.1.1 and 13.4.1.3 both consist of one subtest with a single NB-IoT cell, and the test case 13.4.1.2 consists of two similar subtests. The difference between the subtests is that DRX is OFF in Test 1 and DRX is ON in Test 2. Each subtest consists of one time period. The downlink timing of Cell 1 is changed and the changes in UE transmit timing are observed. The transmit timing is verified by the UE transmitting NPUSCH used as a measurement reference facilitating the SS timing estimation.

3.1	Calculations for Transmit Timing

The UE initial transmission timing error shall be less than or equal to  Te. The timing error limit value Te is specified both in core requirement and in test requirement.

In core requirement, the timing error limit value Te is specified in TS 36.133 Table 7.20A.2-1:

Table 7.20A.2-1: Te Timing Error Limit

| Downlink Bandwidth (MHz) | Te_ |
| --- | --- |
| 0.18 | 97*TS |
| Note 1: TS is the basic timing unit defined in TS 36.211 |  |

In test requirement, the timing error limit value Te is specified in TS 36.133 clause A.13.4.1.1.2, A.13.4.1.2.2 and A.13.4.1.3.2. TGNSS_margin is the margin for the GNSS position definition error considered in the core requirement, which needs to be substracted for the test requirement, due to the usage of AT commands in the test.

The measurement uncertainty of the test system is the only additional effect to be considered. The Test tolerance for uplink signal transmit timing relative to downlink is 3*Ts, as already specified for related timing test cases in TS 36.521-3 and which is also defined in TS 36.903 clause B.6.

For 13.4.1.1, 13.4.1.2 and 13.4.1.3, the test requirements are:

Formula: Upper limit + TT, Lower limit – TT:

Normal conditions (±(97*TS-TGNSS_margin) ±3*TS) = ± (100*TS-TGNSS_margin)

This value is therefore used in TS 36.521-3 Table 13.4.1.1.5-2, 13.4.1.2.5-3 and 13.4.1.3.5-2: Test requirements for Te Timing Error Limit.

For 13.4.1.2 Test 2, DRX is ON. The first transmission in the DRX cycle immediately after DL timing adjustment shares the same requirement and can use the same test tolerance as initial transmission.

For 13.4.1.3, transmission segment is configured. The first transmission in each segment shares the same requirement and can use the same test tolerance as initial transmission.

For 13.4.1.2 and 13.4.1.3, the uplink transmission repetition is configured. The test system shall also verify that the UE shall not adjust the uplink transmission timing autonomously during an ongoing repetition period. Taking the tolerance of 3*Ts into consideration, UE uplink transmission timing adjustment shall not exceed ±3*TS。

In 13.4.1.1, it is also verified that the UE is capable to follow the frame timing change of the connected eNodeB. The UE shall be capable of changing the transmission timing according to the received downlink frame of the serving NB-IoT cell. When the transmission timing error between the UE and the reference timing exceeds ±Te, the UE is required to adjust its timing to within ±Te. The reference timing shall be  NTA_Ref+NTAoffset+NTA,common+NTA, UE-specific×Ts before the downlink timing.

The timing adjustment rules are specified in TS 36.133 clause 7.20A.2:

When no repetition period is configured, or the configured repetition period is R=1, all adjustments made to the UE uplink timing shall follow these rules:

1)	The maximum amount of the magnitude of the timing change, apart from a change of NTA, UE-specific due to satellite position update and NTA,common between the previous transmission and the current transmission, in one adjustment shall be 58.33*TS seconds.

2)	The minimum aggregate adjustment rate, apart from a change of NTA, UE-specific due to satellite position update and NTA,common between the previous transmission and the current transmission, shall be 7*TS per 1second.

3)	The maximum aggregate adjustment rate, apart from a change of NTA, UE-specific due to satellite position update and NTA,common between the previous transmission and the current transmission, shall be 58.33*TS per 200ms.

when a repetition is configured on the uplink for which R>1, the UE shall not adjust the uplink transmission timing autonomously during an ongoing repetition period other than at initial transmission or at the start of a transmission segment boundary, as defined above.

For the time adjustment test, in addition to the measurement uncertainty of the test system, two additional effects need to be considered. Both the allowed UE frequency error and the UE time adjustment quantisation can affect the result of the test, and it is not possible to test the UE Transmit Timing Accuracy in isolation from these core requirements. They are therefore taken into account when setting the Test Tolerances and the Test Requirements.

Test system uncertainty

The maximum test system uncertainty for relative transmit timing during adjustment is 0.5*Ts, as already specified for related timing test cases in TS 36.521-3 and which is also defined in TS 36.903 clause B.6.

Allowed UE frequency error

In addition to the effect of test system uncertainty for relative transmit timing, the effect of allowed UE frequency error specified in TS 36.102 clause 6.4B.1 should be taken into account. According to TS 36.102 Clause 5.2, band 255 and band 256 are used for E-UTRA satellite access. The carrier frequencies of band 255 and band 256 are both over 1GHz, so frequency error requirement is ±0.1 ppm.

## 5.2	Operating bands

E-UTRA satellite access is designed to operate in the operating bands defined in Table 5.2-1.

Table 5.2-1 E-UTRA operating bands for satellite access

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 256 | 1980 MHz | – | 2010 MHz | 2170 MHz | – | 2200 MHz | FDD |
| 255 | 1626.5 MHz | – | 1660.5 MHz | 1525 MHz | – | 1559 MHz | FDD |
| NOTE: Satellite bands are numbered in descending order from 256 |  |  |  |  |  |  |  |

## 5.2A	Operating bands for UE category M1

UE category M1 is designed to operate in the E-UTRA satellite access operating bands defined in Table 5.2-1 in both half duplex FDD mode and full-duplex FDD mode.

## 5.2B	Operating bands for category NB1 and NB2

Category NB1 and NB2 UE are designed to operate in the E-UTRA satellite access operating bands defined in Table 5.2-1.

Category NB1 and NB2 UE operate in HD-FDD duplex mode.

For operation in Band 255 in USA and Canada when NS_02N is signalled, only channels positions which guarantee at least 90 kHz guard band from RF channel edge to the lower and upper limit of the band shall be used.

<< some clauses skipped >>

### 6.4B.1	Frequency error for UE category NB1 and NB2

For UE category NB1 and NB2, the UE pre-compensates the uplink modulated carrier frequency by the estimated Doppler shift based on received ephemeris information of the SAN in IE EphemerisInfo (TS 36.331 [6]), its own location and UL carrier frequency signalled to the UE by the SAN (according to TS36.300 [8] clause 23.21.2.2).

The UE pre-compensated modulated carrier frequency shall be accurate to within the limits in Table 6.4B.1-1, observed over a period of one time slot (0.5 ms for 15 kHz sub-carrier spacing and 2 ms excluding the 2304Ts gap for 3.75 kHz sub-carrier spacing) and averaged over 72/LCtone slots (where LCtone = {1, 3, 6, 12} is the number of sub-carriers used for the transmission), compared to the ideally pre-compensated reference uplink carrier frequency.

When a repetition period is configured on the uplink for which repetition period (R ) >1, the UE shall not change Doppler pre-compensation during an ongoing repetition period, except in the transmission gaps as defined in clause 10.1.3.6 of TS 36.211[3]. When segmentation is applied, then the UE shall update pre-compensation at the beginning of each segment prior to segment transmission.

[NOTE:	The ideally pre-compensated reference uplink carrier frequency consists of the UL carrier frequency signalled to the UE by SAN and UL pre-compensated Doppler frequency shift corresponding to the estimated Doppler frequency at the beginning of the transmission. For the test case, the location of the UE is explicitly provided to the UE from the test equipment.]

Requirement will be verified for at least two cases of which one has zero Doppler conditions.

Table 6.4B.1-1: Frequency error requirement for UE category NB1 and NB2

| Carrier frequency [GHz] | Frequency error [ppm] |
| --- | --- |
| ≤1 | ±0.2 |
| >1 | ±0.1 |

The effect of allowed UE frequency error on the timing adjustment rules is calculated as follows:

Rule 1: The maximum amount of the magnitude of the timing change in one adjustment shall be 58.33*TS seconds.

For 13.4.1.1, the NPDCCH and NPUSCH repetition level is 1. In NB-IoT NTN, k-Offset is non-zero. Especially for GSO, k-Offset is 258 ms specified in TS 36.508 Table 8.4.2.1.3-2. So it’s important to calculate the time interval between the two NPUSCH transmission. The time interval between the two NPUSCH transmission consists of four parts.

The 1st part is NPUSCH transmission duration. NPUSCH default setting is specified in TS 36.508 Table 8.1.3.3.4-1:

Table 8.1.3.3.4-1: NB-IoT Default Uplink setting

|  | Unit | Channel bandwidth |
| --- | --- | --- |
|  |  | 200 kHz |
| Number of Tones |  | 1 |
| Sub-carrier spacing | kHz | 15 |

According to TS 36.211 Table 10.1.2.3-1, one NPUSCH RU is 8ms in time domain. There is one NPUSCH RU, specified clearly in general test parameters tables of TC 13.4.1.2 and TC 13.4.1.3.

Table 10.1.2.3-1: Supported combinations of , , and  for frame structure type 1.

| NPUSCH format |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | 3.75 kHz | 1 | 16 | 7 |
|  | 15 kHz | 1 | 16 |  |
|  |  | 3 | 8 |  |
|  |  | 6 | 4 |  |
|  |  | 12 | 2 |  |
| 2 | 3.75 kHz | 1 | 4 |  |
|  | 15 kHz | 1 | 4 |  |

The 2nd part is the time interval between NPUSCH and the next NPDCCH. According to TS 36.213 clause 16.6, if the NB-IoT UE has a NPUSCH transmission ending in subframe n, the UE is not required to monitor NPDCCH in any downlink subframe that overlaps with uplink subframe n+1 to subframe n+Kmac+3. Kmac is specified in TS 36.508 Table 8.4.2.1.3-2 and default value is 0. The time interval between NPUSCH and the next NPDCCH is 3ms.

The 3rd part is the NPDCCH transmission duration. According to TS 36.211 clause 10.2.5.1, NPDCCH has 2 NCCEs at most. Both NCCEs belong to the same subframe. NPDCCH repetition level is 1. NPDCCH transmission duration is 1ms.

The 4th part is the time interval between NPDCCH and NPUSCH. According to TS 36.213 clause 16.5.1, UE shall upon detection a NPDCCH with DCI format N0 ending in NB-IoT DL subframe n scheduling NPUSCH after the end of n+k0+Koffset DL subframe for FDD. The time interval NPDCCH and NPUSCH is k0+Koffset DL subframes. Scheduling delay in DCI format N0 is 0 specified TS 36.508 Table 8.1.3.6.1.1-1. According to TS 36.213 Table 16.5.1-1, k0 is 8. Koffset is specified in TS 36.508 Table 8.4.2.1.3-2. For GSO, Koffset is 258ms. For NGSO, Koffset is 14ms.

For GSO, the time interval between NPDCCH and NPUSCH is 266ms. For NGSO, the time interval between NPDCCH and NPUSCH is 22ms.

In summary, the total time interval between the two NPUSCH transmission, for GSO is 278ms, and for NGSO is 34ms. In addition, according to TS 36.213 clause 16.6, NPDCCH starts in a subframe satisfying the condition 10nf+ns2modT=αoffset⋅T, where T=Rmax⋅G, T≥4. For NPDCCH UE-specific search space, G is 8 given by the higher layer parameter npdcch-StartSF-USS, αoffset is 0 given by the higher layer parameter npdcch-Offset-USS. The condition is simplified to 10nf+ns2mod8=0. It means, if don’t consider other factors, NPDCCH can be transmitted in every 8ms.

Considering NPDCCH subframe condition, for GSO, the NPUSCH initial transmission can be adjusted to transmit every 280ms which is multiple of 8, and  the UE timing change from frequency error could be (0.28s x (0.1 x 10-6)) = 28ns. As Ts = 1/(15000 x 2048) s or 32.6ns, the UE timing change from frequency error could be ± 0.9* Ts. For NGSO, the NPUSCH initial transmission can be adjusted to transmit every 40ms which is multiple of 8, and the UE timing change from frequency error could be (0.04s x (0.1 x 10-6)) = 4ns. As Ts = 1/(15000 x 2048) s or 32.6ns, the UE timing change from frequency error could be ± 0.2* Ts.

Rule 2: The minimum aggregate adjustment rate shall be 7*Ts per second.

During 1s, the UE timing change from frequency error could be (1s x (0.1 x 10-6)) = 100ns. As Ts = 1/(15000 x 2048) s or 32.6ns, the UE timing change from frequency error could be ± 3.1* Ts.

Rule 3: The maximum aggregate adjustment rate shall be 58.33*TS per 200ms.

During 200ms, the UE timing change from frequency error could be (0.2s x (0.1 x 10-6)) = 20ns. As Ts = 1/(15000 x 2048) s or 32.6ns, the UE timing change from frequency error could be ± 0.6* Ts.

UE time adjustment quantisation

Although not explicitly stated in TS 36.133, the UE timing adjustment is assumed to be quantised, and a quantisation step size up to 58.33*Ts should be taken into account.

The effect of quantised UE timing adjustment on the timing adjustment rules is calculated as follows:

Rule 1: The maximum amount of the magnitude of the timing change in one adjustment shall be 58.33*Ts seconds.

As the magnitude of the timing change in one adjustment is being directly measured, no additional allowance is needed for the rule 1 requirement.

Rule 2: The minimum aggregate adjustment rate shall be 7* Ts per second.

If a time window of exactly 1s is used, and the UE makes timing adjustment with the longest allowed time between two changes which satisfying the minimum aggregate adjustment requirement as shown in the Figure 1. The SS could catch zero, one or two UE timing changes depending on the synchronisation between the SS measurement period of 1s and the UE timing change. For 13.4.1.1, as this part of the test aims to check the UE minimum adjustment rate, the sliding time window is increased by the minimum amount of one sample, which is 8ms, to 1.008s. This ensures that the SS will not miss any timing changes occurring during the 1s time period specified in the core requirements.

Rule 3: The maximum aggregate adjustment rate shall be 58.33*TS per 200ms.

If a time window of exactly 200ms is used, and the UE makes timing adjustment with the longest allowed time between two changes as shown in the Figure 2. The SS could catch zero, one or two UE timing changes depending on the synchronisation between the SS measurement period of 200ms and the UE timing change.

For 13.4.1.1, as this part of the test aims to check the UE maximum adjustment rate, the sliding time window is decreased by the minimum amount of one sample, which is 8ms, to 192ms.

The case shown in the Figure 2 is that a UE making the largest allowed timing adjustment of 58.33* Ts with the longest allowed time between changes (1 change every 200ms) to meet the minimum aggregate adjustment rate of 58.33* Ts per 200ms.

The Figure 1 and Figure 2 below shows the combined effect of allowed UE frequency error, UE time adjustment quantisation and the use of a sliding measurement window (for simplicity the test system timing measurement uncertainty is not included in the below figures).

Figure 1: Combined effect of allowed UE frequency error and UE time adjustment quantisation for Rule 2

Figure 2: Combined effect of allowed UE frequency error and UE time adjustment quantisation for Rule 3

The effect of frequency error can make the UE Tx timing earlier or later than expected. The combined effect is shown by the lines with sloping steps.

In Figure 1, the bule dashed lines show the specified minimum aggregate adjustment rate. The red stepped lines show UE possible behaviour when considering -3.1*Ts per second effect of frequency error. The 1.008s window “Position A” shows a typical measurement, where the test system would expect to see two timing quantisation steps, together with a frequency error which may decrease the timing change. In window “Position B” the test system could see three timing quantisation steps, but as the test requirement is a one-sided limit, so this is unimportant.

In Figure 2, the green dashed lines show the specified maximum aggregate adjustment rates. The red stepped lines show UE possible behaviour if the UE uses the largest possible time quantisation step, and changes the timing infrequently, which gives the worst case for measurement. Smaller or more frequent steps are also allowed. The 192ms window “Position C” shows a typical measurement, where the test system would expect to see one timing quantisation step, together with a frequency error which may increase the timing change. In window “Position D” the test system would see no timing quantisation steps, but as the test requirement is a one-sided limit, so this is unimportant.

When calculating the test requirements, the test system uncertainty, the effect of allowed UE frequency error where relevant, and the effect of UE time adjustment quantisation where relevant, are all included.

For 13.4.1.1, the test requirements are:

- The maximum timing change in one adjustment calculated as shown:

|  | Lower limit | GSO Upper limit | NGSO Upper limit |
| --- | --- | --- | --- |
| Core requirement | Not applicable | +58.33 * Ts | +58.33 * Ts |
| Test system uncertainty | Not applicable | +0.5* Ts | +0.5* Ts |
| UE frequency error allowance | Not applicable | +0.9* Ts | +0.2* Ts |
| Test requirement | Not applicable | +59.73 * Ts | +59.03 * Ts |

- The minimum aggregate adjustment rate per second:

|  | Lower limit | Upper limit |
| --- | --- | --- |
| Core requirement | +7.0 * Ts | Not applicable |
| Test system uncertainty | -0.5 * Ts | Not applicable |
| UE frequency error allowance | -3.1 * Ts | Not applicable |
| Test requirement | 3.4 * Ts | Not applicable |

- The maximum aggregate adjustment rate per 200ms calculated as shown:

|  | Lower limit | Upper limit |
| --- | --- | --- |
| Core requirement | Not applicable | +58.33 * Ts |
| Test system uncertainty | Not applicable | +0.5 * Ts |
| UE frequency error allowance | Not applicable | +0.6 * Ts |
| Test requirement | Not applicable | +59.43 * Ts |

3.2	Downlink Signal Level Uncertainties

The SS provides 1 cell with AWGN to the UE. We propose to control the following parameters:

- AWGN absolute power, Noc ±3 dB averaged over BWConfig

- Ratio of cell 1 signal / AWGN, Ês/Noc ±0.3 dB averaged over BWConfig

The downlink uncertainty values are chosen to be the same as the performance tests in section 8 of TS 36.521-1.

3.3	Key Signal level parameters in test case

To ensure that the SNR does not fall below the core requirement specified in Annex A of TS 36.133, the Ês/Noc values are increased by the amount of the SNR uncertainty 0.3dB. Therefore, the test requirements are:

For test case 13.4.1.1 NCell:

- = -88 dBm/15 kHz

- = (4dB+0.3dB) = 4.3 dB

For test case 13.4.1.2 and 13.4.1.3 NCell:

- = -88 dBm/15 kHz

- = (-11dB+0.3dB) = -10.7 dB