# Title:	Test tolerance methodology for NR SA FR1 event triggered reporting tests for PRS and SSB measurement in FR1 without SSB time index detection when DRX is not used

# Source:	MediaTek Inc.

# 1.	Introduction

The purpose of this document is to present the methodology and principles used for the development of Test Tolerances for RRM tests 6.6.18.4.

The test cases in question, from TS 38.533, are:

- 6.6.18.4 NR SA FR1 event triggered reporting tests for PRS and SSB measurement in FR1 without SSB time index detection when DRX is not used.

This document describes the process to derive the test tolerances. The calculations are provided in the accompanying spreadsheet.

# 2. 	Test Tolerances Methodology for 6.6.18.4 in TS 38.133

The test conditions are defined in the following extract from TS 38.133 v17.13.0.

#### A.6.6.18.4	SA event triggered reporting tests for PRS and SSB measurement in FR1 without SSB time index detection when DRX is not used

##### A.6.6.18.4.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA NR measurements with concurrent gaps requirements in clause 9.2.6(when one of concurrent gaps in same frequency layer of serving cells), 9.3.6(when one of concurrent gaps in the different frequency layer of serving cells) and 9.9.2(when one of concurrent gaps used for PRS measurement).

In this test, there are three cells: NR cell 1 as PCell in FR1 on NR RF channel 1, NR cell 2 as neighbour cell in FR1 on NR RF channel 2 and NR cell 3 as neighbor cell in FR1 on NR RF channel 1.  The test parameters are given in Tables A.6.6.18.4.1-1, A.6.6.18.4.1-2 and A.6.6.18.4.1-3.

Two measurement gap patterns (MeasGapId #1 and MeasGapId #2) are configured with the gap pattern ID #0 and #24 as defined in Table A.6.6.18.4.1-2. MeasGapId #2 is configured with a higher priority than MeasGapId #1. MeasGapId #1 and MeasGapId #2 are associated with the MOs for RF channel numbers #1 and #2, respectively.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2 and NR cell 3. Cell 1 and cell 3 transmit PRS during T2.

The NR-DL-AoD-RequestLocationInformation message and NR-DL-AoD-ProvideAssistanceData message as defined in TS 37.355 shall be provided to the UE during T1. The last slot containing the two messages for the assistance data and location information request is denoted as #n.

The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance of MeasGapId #2 containing the PRS resources that is T after slot #n, where T = 50 ms is the maximum processing time of the assistance data and location information request.

Table A.6.6.18.4.1-1: SA event triggered reporting tests without SSB index reading for FR1-FR1

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table A.6.6.18.4.1-2: General test parameters for SA inter-frequency event triggered reporting for concurrent gaps with partially partial overlapping scenario for SSB-based measurements and PRS measurement

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | 1,2,3 | 1: Cell 1 and Cell 3 2: Cell 2 | Two TDD carrier frequencies are used for the NR cells. |
| Active cell |  | 1,2,3 | NR cell 1 (Pcell) | Cell 1 is the PCell and the DL-AoD reference cell in the positioning assistance data. |
| Neighbour cell |  | 1,2,3 | NR cell 2, NR cell 3 | Cell 2 is an inter-frequency cell neighbor cell Cell 3 is a neighbour cell in the positioning assistance data. |
| Gap Pattern Id |  | 1,2,3 | 0 for MeasGapId #1 24 for MeasGapId #2 |  |
| Measurement gap offset | ms | 1,2,3 | 7 for MeasGapId #1 11 for MeasGapId #2 |  |
| DRX |  | 1, 2, 3 | NA | OFF |
| Time offset between serving and neighbour cells | s | 1, 2, 3 | 3 | Synchronous cells |
| Expected RSTD | s | 1, 2, 3 | 3 |  |
| Expected RSTD uncertainty | s | 1, 2, 3 | 5 |  |
| T1 | s | 1, 2, 3 | 2 |  |
| T2 | s | 1, 2, 3 | 5 |  |
| NOTE 1: GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. |  |  |  |  |

Table A.6.6.18.4.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 concurrent gap with partially-partial overalpping scenario for SSB-based measurements and PRS measurement

|  |  |  |  | T1 | T2 | T1 | T2 |  | T1 | T2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TDD configuration |  |  | 1 | N/A |  | N/A |  |  | N/A |  |
|  |  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |  | TDDConf.1.1 |  |
|  |  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |  | TDDConf.2.1 |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  | 10: NRB,c = 52 |  |  | 10: NRB,c = 52 |  |
|  |  |  | 3 | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  |  | 40: NRB,c = 106 |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  | 10: NRB,c = 52 |  |  | 10: NRB,c = 52 |  |
|  |  |  | 3 | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  |  | 40: NRB,c = 106 |  |
| BWP configuration | Initial DL BWP |  | Config 1,2,3 | DLBWP.0.1 |  | N/A |  |  | N/A |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | N/A |  |  | N/A |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | N/A |  |  | N/A |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | N/A |  |  | N/A |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |  | OP.1 |  |
| PDSCH RMC configuration |  |  | 1 | SR.1.1 FDD |  | N/A |  |  | N/A |  |
|  |  |  | 2 | SR.1.1 TDD |  |  |  |  |  |  |
|  |  |  | 3 | SR.2.1 TDD |  |  |  |  |  |  |
| RMSI CORESET RMC configuration |  |  | 1 | CR.1.1 FDD |  | N/A |  |  | N/A |  |
|  |  |  | 2 | CR.1.1 TDD |  | N/A |  |  | N/A |  |
|  |  |  | 3 | CR.2.1 TDD |  | N/A |  |  | N/A |  |
| Dedicated CORESET RMC configuration |  |  | 1 | CCR.1.1 FDD |  | N/A |  |  | N/A |  |
|  |  |  | 2 | CCR.1.1 TDD |  | N/A |  |  | N/A |  |
|  |  |  | 3 | CCR.2.1 TDD |  | N/A |  |  | N/A |  |
| SSB parameters |  |  | 1 | SSB.1 FR1 |  | SSB.1 FR1 |  |  | SSB.1 FR1 |  |
|  |  |  | 2 | SSB.1 FR1 |  | SSB.1 FR1 |  |  | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.2 FR1 |  | SSB.2 FR1 |  |  | SSB.2 FR1 |  |
| SMTC configuration defined in A.3.11 |  |  | 1 | SMTC.2 |  | SMTC.2 |  |  | SMTC.2 |  |
|  |  |  | 2 | SMTC.1 |  | SMTC.1 |  |  | SMTC.1 |  |
|  |  |  | 3 | SMTC.1 |  | SMTC.1 |  |  | SMTC.1 |  |
| TRS Configuration |  |  | 1 | TRS.1.1 FDD |  | N/A |  |  | N/A |  |
|  |  |  | 2 | TRS.1.1 TDD |  | N/A |  |  | N/A |  |
|  |  |  | 3 | TRS.1.2 TDD |  | N/A |  |  | N/A |  |
| PRS configuration |  |  | 1 | PRS.1.4 FR1 |  | N/A |  |  | PRS.1.4 FR1 |  |
|  |  |  | 2 | PRS.1.4 FR1 |  | N/A |  |  | PRS.1.4 FR1 |  |
|  |  |  | 3 | PRS.2.4 FR1 |  | N/A |  |  | PRS.2.4 FR1 |  |
| PRS muting configuation |  |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  |  | ‘01’ |  |
| PDSCH/PDCCH |  | KHz | 1, 2 | 15 |  |  |  |  |  |  |
| subcarrier spacing |  |  | 3 | 30 |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz Note5 | 1 | -98 |  | -98 |  |  | -98 |  |
|  |  |  | 2 | -98 |  | -98 |  |  | -98 |  |
|  |  |  | 3 | -98 |  | -98 |  |  | -98 |  |
| Note2 |  | dBm/SCS Note4 | 1 | -98 |  | -98 |  |  | -98 |  |
|  |  |  | 2 | -98 |  | -98 |  |  | -98 |  |
|  |  |  | 3 | -95 |  | -95 |  |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS Note5 | 1 | -94 | -94 | -Infinity |  | -94 | N/A | N/A |
|  |  |  | 2 | -94 | -94 | -Infinity |  | -94 | N/A | N/A |
|  |  |  | 3 | -91 |  |  |  |  |  |  |
| PRS-RSRP Note 3 |  | dBm/SCS Note5 | 1 | -Infinity | -101 | N/A |  | N/A | -Infinity | -108 |
|  |  |  | 2 | -Infinity | -101 | N/A |  | N/A | -Infinity | -108 |
|  |  |  | 3 | -Infinity | -98 | N/A |  | N/A | -Infinity | -105 |
| PRS |  | dB | 1,2,3 | -Infinity | -3 | N/A |  | N/A | -Infinity | -10 |
| PRS |  | dB | 1,2,3 | -Infinity | -3 | N/A |  | N/A | -Infinity | -10 |
| IoNote3 |  | dBm/9.36 MHz | 1 | -62.25 |  | -64.60 |  | -62.25 | -62.25 |  |
|  |  | dBm/9.36 MHz | 2 | -62.25 |  | -64.60 |  | -62.25 | -62.25 |  |
|  |  | dBm/38.16 MHz | 3 | -56.16 |  | -58.50 |  | -56.16 | -56.16 |  |
| Propagation Condition |  |  | 1,2,3 | AWGN |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP/PRS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: PRS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Void Note 6: Void Note 7: Void |  |  |  |  |  |  |  |  |  |  |

##### A.6.6.18.4.2	Test Requirements

The UE shall send one Event A3 triggered measurement report for cell 2, with a measurement reporting delay less than 1840ms from the beginning of time period T2.

The PRS RSRP measurement time fulfils the requirements specified in Clause 9.9.3.5. The UE shall perform and report the PRS RSRP measurements for Cell 3 with respect to the reference cell in the DL-AoD assistance data, Cell 1, within the time duration specified in section 9.9.3.5 starting from the beginning of time interval T2.

The rate of the correct events for the neighbour cell observed during repeated tests shall be at least 90%, where the reported PRS RSRP measurement for each correct event shall be within the PRS RSRP reporting range specified in Clause 10.1.24.3, i.e., between PRS RSRP_0 and PRS RSRP_126.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

IUE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

# 3. 	Discussion

The test case consists of 3 test configurations and each test configuration has 2 time periods: T1 and T2.

In the accompanying spreadsheet only configurations 1 and 3 are considered. For other configurations, critical test parameters are identical with test configuration 1 and 3, thus the same level uncertainties and method of deciding test tolerances can be used.

Observations

During T1 only NR Cell 1 is present and the “Event A3 (Neighbour becomes offset(-15dB) better than SpCell) condition is not met since the NR inter-frequency neighbour Cell 2 and intra-frequency neighbour Cell 3 are off.

During T2, NR Cell 2 and Cell 3 are switched on and NR Cell 1 and Cell 3 transmit PRS. For configurations 1, 2 and 3, the SS-RSRP of NR Cell 2 is 0 dB, which is higher than the a3-Offset, better than the SS-RSRP of NR Cell 1. So “Event A3 (Neighbour becomes offset(-15 dB) better than SpCell)” entry condition is met. The effect of uncertainties on the test verdict needs to be checked. Then a decision can be made on whether Test Tolerances need to be applied.

A detailed analysis of the test case is given in section 5 of this document.

# 4. 	Calculation of Test Tolerances

General approach

The general approach is given in the steps below:

- Copy the originally specified key parameters from the core requirements

- Where relevant, calculate derived parameters from the core requirements

- Define uncertainties for a minimum set of parameters

- Define controlled parameters (critical to the test verdict), calculate sensitivity factors and uncertainty

- Determine which original or derived parameters to offset (apply Test Tolerances to) and by how much

- Recalculate original or derived parameters including Test Tolerances

- Check that the controlled parameters meet requirements to get the correct test verdict

Each step is explained below, and the calculations are given in the accompanying spreadsheet.

a) Original specified key parameters

The key parameters are copied from Tables A.6.6.18.4.1-3 in TS 38.133. The key parameters are selected as the minimum set to define the cell power levels, and which are subject to a test system uncertainty which may affect the verdict of the test.

b) Derived parameters

Several derived parameters are calculated, using the base information in a). The reason for deriving each additional parameter is given in the “Comment” column of section b) in the accompanying spreadsheet.

In this test case at the start of T2, NR Cell 2 becomes detectable, and the UE is expected to detect and send an event triggered measurement report. The criterions are:

- Event A3: Neighbour becomes offset better than SpCell. The UE measures the SS-RSRP of NR Cell 1 and Cell 2, then compares the offset with the signalled a3-Offset. Further step is done to calculate SS-RSRP power difference with a3-Offset:

- (NR Cell 2 SS-RSRP – NR Cell 1 SS-RSRP)

c) Uncertainties

The SS provides AWGN, 2 NR inter-frequency cells to the UE. We propose to control the following parameters:

- AWGN absolute power on carrier frequency, Noc uncertainty: ±1.5 dB averaged over BWConfig

- AWGN absolute power, Noc ±1.5 dB for allocated PRBs

- Frequency 1 AWGN Absolute power, Noc ±1.5 dB

- Frequency 2 AWGN Absolute power, Noc ±1.5 dB

- Ratio of NR Cell 1 signal / AWGN, Ês1 / Noc ±0.3 dB

- Ratio of NR Cell 2 signal / AWGN, Ês2 / Noc ±0.3 dB

This choice forms a minimum set, so the superposition principle can be applied if necessary.

In addition, the UE measurement accuracies need to be considered, as such inaccuracy could affect the test verdict.

Firstly, for NR SS-RSRP measurement accuracy, the UE measurement accuracy of inter-freq Pcell (NR Cell 1) is defined in the following extract from TS 38.133:

10.1.4	Inter-frequency RSRP accuracy requirements for FR1

10.1.4.1	Inter-frequency SS-RSRP accuracy requirements

10.1.4.1.1	Absolute Accuracy of SS-RSRP in FR1

The requirements for absolute accuracy of SS-RSRP in this clause apply to a cell on a frequency in FR1 that has different carrier frequency from the serving cell. The accuracy requirements in this clause are also applicable when highSpeedMeasInterFreq-r17 is configured.

The accuracy requirements in Table 10.1.4.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band for each relevant SSB.

Table 10.1.4.1.1-1: SS-RSRP Inter frequency Absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 3 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -70 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -70 |
| 4.5 | 9 | -6 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -70 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -70 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | N/A | -70 |
| 8 | 11 | -6 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H, NR_FDD_FR1_N | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: Void NOTE 3: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

To ensure this absolute SS-RSRP accuracy, the conditions of Io, SSB Es/Iot in Table 10.1.4.1.1-1, and conditions of SSB_RP and SSB Es/Iot in section Annex B.2.3 need to be fulfilled.

B.2.3	Conditions for NR inter-frequency measurements

This clause defines the following conditions for NR inter-frequency measurements and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.3-1 for FR1 NR cells.

The conditions are defined in Table B.2.3-2 for FR2 NR cells.

Table B.2.3-1: Conditions for inter-frequency measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -125 | -122 | -4 |
|  | NR_FDD_FR1_B | -124.5 | -121.5 |  |
|  | NR_TDD_FR1_C | -124 | -121 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -124.5 | -120.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -123 | -120 |  |
|  | NR_FDD_FR1_F | -122.5 | -119.5 |  |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G | -122 | -119 |  |
|  | NR_FDD_FR1_H | -121.5 | -118.5 |  |
|  | NR_FDD_ FR1_N | -118.5 | -115.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

All the side conditions mentioned above refer to Annex B.2.3, which is already considered.

In summary the SSB Es/Iot of both NR Cell 1 shall be above -4dB, the tightest requirement for Io of NR Cell 1 shall be between -117.5dBm/15kHz (-89.5dBm/9.36MHz) or -114.5dBm/30kHz (-83.5dBm/38.16MHz) and -50dBm/BWchannel for band group NR_FDD_FR1_H, and the tightest requirement for SSB_RP shall be above -121.5dBm/15kHz or -118.5/30kHz for band groups NR_FDD_FR1_H.

For the analysis therefore we take the tightest requirement to be 1dB higher than Band FDD_N, giving E-UTRAN Cell 2 RSRP side condition of (-118.5dBm/15kHz + 1dB). A similar approach is taken in the spreadsheet when applying the minimum Io requirement.

d) Controlled parameters critical to verdict

The controlled parameters listed in the accompanying spreadsheet have been derived by study of the test case and the relevant clauses in TS 38.133.

In general, all the values are listed for each time interval T1 and T2 but as NR Cell 2 and Cell 3 are not present during T1 those entries are greyed out.

Having identified the parameters which need to be controlled, we now need to consider how they are affected by the parameters which can be set by the test equipment. This is done by working out “sensitivity factors”. A sensitivity factor is just the ratio (effect on a critical parameter y / a test equipment uncertainty x) and is usually in dB/dB. Often it can be derived by inspection as one or zero. For example, an error of 1dB in the absolute AWGN level Noc would cause 1dB error in the Cell 1 SS_RSRP, so the sensitivity factor is 1.000 during T1 and T2. However, the same error of 1dB in the absolute AWGN level Noc would cause no change to NR Cell 2 Es/Iot, because all other powers are specified relative to Noc, so the sensitivity factor is zero.

Having filled in the matrix of sensitivity factors, the accompanying spreadsheet calculates the overall uncertainty for each controlled parameter, taking into account the uncertainties and sensitivity factors for each parameter that can be set by the test equipment, and the UE measurement accuracies where relevant. This process follows the superposition principle. Uncertainties are calculated separately for T1 and T2.

For the test system uncertainties, the normal procedure of combining uncorrelated uncertainties root-sum-square is followed.

When the test system uncertainties and the UE reporting accuracies are combined, the (root-sum square of test system uncertainties) is added arithmetically to the UE reporting accuracies. If all the uncertainties were combined root-sum square, the resulting smaller test limits could cause a conformant test system to fail a conformant UE, which would be unacceptable.

e) Determine parameters to offset

From TS 38.533 clause 6.6.18.4, NR Cell 2 and Cell 3 with PRS are undetectable during T1.

During T1 and T2, NR cell 1 shall meet the Ês/Iot, SS-RSRP and Io side conditions.

During T2, the UE shall send Event A3 triggered measurement report for NR Cell 2 to the PCell. The UE measures NR Cell 1 and Cell 2 SS-RSRP, compares the difference to the corresponding a3-Offset, and sends a measurement report. We need to consider the power level conditions and uncertainties to make sure a good UE will send the measurement report.

The Event A3 measurement report trigger requirement in TS 38.331 clause 5.5.4.4 is Event A3 (Neighbour becomes offset better than SpCell). Inequality A3-1 (Entering condition) are:

Inequality A3-1 (Entering condition):

Mn + Ofn + Ocn – Hys > Mp + Ofp + Ocp + Off

Event A3 during T1

During T1, as NR Cell 2 is off, the Inequality A3-1 is not satisfied. So no measurement report shall be triggered by Event A3.

Event A3 during T2,

During T2, the NR neighbour cell (NR Cell 2) should be a3-Offset better than the SpCell (NR Cell 1). This is verified by satisfying inequalities A3-1 accordingly, where the values are derived as follows:

- Mn is the measurement result of NR Cell 2, not taking into account any offsets and is nominally -94dBm/15kHz or -91dBm/30kHz.

- Ofn is the measurement object specific offset of the neighbour cell (i.e. offsetMO as defined within measObjectNR corresponding to the neighbour cell). We set the default value to 0 dB.

- Ocn is the measurement object specific offset of the neighbour cell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the neighbour cell), and set to zero if not configured for the neighbour cell.

- Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event), and set to zero if not configured for the neighbour cell.

- Mn is the measurement result of NR Cell 1, not taking into account any offsets and is nominally -94dBm/15kHz or -91dBm/30kHz.

- Ofp is the measurement object specific offset of the SpCell (i.e. offsetMO as defined within measObjectNR corresponding to the SpCell). We set the default value to 0 dB.

- Ocp is the measurement object specific offset of the SpCell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the SpCell), and set to zero if not configured for the neighbour cell.

- Off is the offset parameter for this event (i.e. a3-Offset as defined within reportConfigNR for this event) and is configured as -15 dB.

The inequalities are therefore reduced to:

Mn > Mp +  Off, Putting in values and rearranging gives -94dBm - (-94)dBm > -15 dB for SCS 15 kHz or -91dBm - (-91)dBm > -15 dB for 30 kHz.

For NR Cell 2, the uncertainties of UE Measured ((NR Cell 2 SS-RSRP – NR Cell 1 SS-RSRP) > a3-Offset) meet the requirement of the UE Measured (NR Cell 2 SS-RSRP – NR Cell 1 SS-RSRP) > -15dB. The Es/Noc of NR Cell 2 doesn`t need to be decreased to meet this requirement.

f) Parameters modified by Test Tolerances

Based on the decision in e), the set of parameters in a) and b) is reproduced in section f) of the accompanying spreadsheet, but this time modified by the Test Tolerances (applied offsets).

Re-derived parameters are calculated using the same methods as were used in step b).

g) Check controlled parameters Min/Max

It can be seen that with the uncertainty values and Test Tolerances proposed, all the requirements are met.

# 5. Treatment of other configurations within the same test case

TS 38.533 test case 6.6.18.4 also contains configurations with different subcarrier spacings and different channel bandwidths. As the calculations use different values, a separate spreadsheet tab is provided. The following parameters are changed:

- Subcarrier spacing, section a)

- Channel bandwidth, section a)

- RBs in Channel bandwidth, section a) and associated comment

- PRS-RSRP for NR Cell 1 and Cell 3

- Io minimum requirement, section g) and associated Source/Reference.