3GPP TSG-RAN5 Meeting #103	R5-242692r1

# Fukuoka City, Fukuoka, Japan, 20th May 2024 - 24th May 2024

Title:	Discussion on minimum test time simulation results for 8.3.1.1.1

Source:	MediaTek Inc

Agenda item:	5.5.1.13

Document for:	Discussion and endorsement

1.	Introduction

As discussed in prerequisite discussion R5-242427.

This paper is aiming to provide the simulation results for TC 8.3.1.1.1.

2. 	Discussion

The simulation assumption and steps are shown below,

1.	Run the PDSCH simulations for an extended period of time at the SNR of interest for multiple seeds.

2.	The minimum test time is determined by the time it took the throughput curve to settle within +/- 2% of the target throughput value across all seeds.

3. 	If the throughput does not settle within the +/- 2% throughput limit within a reasonable amount of time for any seed, expand the threshold to +/- 3.5% of the target throughput value.

4.	To account for the additional 1.5% throughput variation, determine from the Throughput vs SNR curve, what is the SNR delta corresponding to this 1.5 % throughput increase.

5.	That SNR value is the added SNR uncertainty due to finite test time.

30 random seeds are used in this simulation, Fig.1 and Fig.2 shows the results. Several pair of Tput corridor were added in these Fig. Red, green, purple and grey line are represented 2%, 3.5%, 5% and 10% limit around 70% max Tput.

Additionally, considering the long test time of current NB IoT NTN test cases. The simulation of 8.3.1.1.2 are rerun to get a reasonable test time.

Fig.1 Simulation results for 8.3.1.1.1 test 1

Fig.2 Simulation results for 8.3.1.1.1 test 2

Fig.3 Simulation results for 8.3.1.1.2 test 1

Then the convergence time within different corridor is summarised in Table.1.

Table.1 Convergence time for test cases

| Convergence time(s) | Tput 70%: +/-2% | Tput 70%: +/-3.5% | Tput 70%: +/-5% | Tput 70%: +/-10% |
| --- | --- | --- | --- | --- |
| 8.3.1.1.1 Case1 TDLC5-200 NGSO | N/A | N/A | 107 | 31 |
| 8.3.1.1.1 Case2 TDLA100-10 NGSO | N/A | N/A | 388 | 200 |
| 8.3.1.1.2 Case1 EPA5 NGSO | N/A | 99 | 61 | 24 |

The simulation is run under 0.113ppm doppler shift (0.013 doppler shift for NGSO according to the ephemeris in 36.508 and 0.1 ppm max residual doppler according to RAN4 requirement)

The MNAS is calculated based on the configuration of NPDSCH,

Table.2 MNAS for test cases under NGSO condition

| MNAS | Tput 70%: +/-2% | Tput 70%: +/-3.5% | Tput 70%: +/-5% | Tput 70%: +/-10% |
| --- | --- | --- | --- | --- |
| 8.3.1.1.1 Case1 TDLC5-200 | N/A | N/A | 669 (107000/32/4) | 243 |
| 8.3.1.1.1 Case2 TDLA100-10 | N/A | N/A | 758 (388000/128/4) | 391 |
| 8.3.1.1.2 Case1 EPA5 | N/A | 774 (99000/32/4) | 477 | 188 |

Considering the large test time in current NTN IODT, it’s better to use less MNAS and add SNR uncertainty for these test cases due to finite test time.

Proposal 1: Use 669 as the MNAS for TC 8.3.1.1.1 Test 1 and 758 for Test 2 in 36.521-4. Add SNR uncertainty due to finite test time.

3. 	Conclusion

This paper is aiming to provide the simulation results for TC 8.3.1.1.1.

The following observations and proposals are made:

Proposal 1: Use 669 as the MNAS for TC 8.3.1.1.1 Test 1 and 477 for Test 2 in 36.521-4. Add SNR uncertainty due to finite test time.

The associated CRs is R5-242422.

4. 	References

- TS 36.521-4 v18.1.0

- TS 36.521-1 v18.4.0

- TS 38.521-1 v18.2.0