3GPP TSG-RAN5 Meeting #107	R5-252122

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-3 | CR | 3087 | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to IoT NTN enh multi cell TCs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek India Technology Pvt. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_enh_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Inplement proposal 1-4 of R5-252120. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update location related parameters to location based TCs. - Remove note “Ephemeris in SIB33 for GSO/NGSO is missing in 36.508” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test Cases can’t be finalised. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.1.1.1_1, 13.1.1.2_1, 13.1.1.3_1, 13.1.1.4, 13.1.1.5, 13.1.1.6, 13.3.1.1_1, 13.3.1.2_1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Depend on R5-252120. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES 1>>>

#### 13.1.1.1_1	HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage for GSO and NGSO

<<< END OF CHANGES 1>>>

<<< START OF CHANGES 2>>>

#### 13.1.1.2_1	HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with serving cell RRM measurement relaxation for GSO and NGSO

<<< END OF CHANGES 2>>>

<<< START OF CHANGES 3>>>

#### 13.1.1.3_1	HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with UE specific DRX for GSO and NGSO

<<< END OF CHANGES 3>>>

<<< START OF CHANGES 4>>>

#### 13.1.1.4	HD – FDD Inter frequency case for UE Category NB1 Standalone mode in normal coverage

Editor’s Note: This test is incomplete. The following aspects are not yet determined:

- MU/TT analysis is missing

- test applicability is missing

- cell mapping information is missing

<<< END OF CHANGES 4>>>

<<< START OF CHANGES 5>>>

#### 13.1.1.5	HD – FDD Intra frequency case for UE Category NB1 Standalone mode in enhanced coverage, location-based cell reselection for NGSO

Editor’s Note: This test is incomplete. The following aspects are not yet determined:

- MU/TT analysis is missing

- test applicability is missing

- cell mapping information is missing

<<< Skip unchanged part>>>

13.1.1.5.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 8.1.4.3, 8.1.5B and 8.1.6 using condition "standalone", "RRM_NTN_Enh", "NTN_enh" and "NTN" with the following exception:

Table 13.1.1.5.4.3-1: SystemInformationBlockType3-NB

| Derivation Path: 3GPP TS 36.508 [7] clause 8.1.4.3.3, Table 8.1.4.3.3-2: SystemInformationBlockType3-NB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3-NB-r13 ::= SEQUENCE { |  |  |  |
| intraFreqCellReselectionInfo-r13 SEQUENCE { |  |  |  |
| q-RxLevMin-r13 | -78 (-156 dBm) |  |  |
| } |  |  |  |
| } |  |  |  |

<<< END OF CHANGES 5>>>

<<< START OF CHANGES 6>>>

#### 13.1.1.6	HD – FDD Inter frequency case for UE Category NB1 Standalone mode in enhanced coverage, time-based cell reselection for NGSO

Editor’s Note: This test is incomplete. The following aspects are not yet determined:

- MU/TT analysis is missing

- test applicability is missing

- cell mapping information is missing

<<< END OF CHANGES 6>>>

<<< START OF CHANGES 7>>>

#### 13.3.1.1_1		HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under normal coverage for GSO and NGSO

<<< END OF CHANGES 7>>>

<<< START OF CHANGES 8>>>

#### 13.3.1.2_1	HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under enhanced coverage for GSO and NGSO

<<< END OF CHANGES 8>>>