3GPP TSG-RAN5 Meeting #107	R5-252147

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 1929 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UL harmonic for DC_8A_n77A and DC_8A_n78A not correctly tested - band 8 CBW correction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Minimum requirements related to UL harmonic for DC_8A_n77A and DC_8A_n78A are defined for 5MHz CBW in band 8. However, corresponding test IDs are defined setting 10MHz instead. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For DC_8A_n77A and DC_8A_n78A, in test IDs 1 and 2, set band 8 CBW to 5MHz. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification will remain inconsistent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3B.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

7.3B.2.3.4	Test description

7.3B.2.3.4.1	Void

7.3B.2.3.4.2	Test description

7.3B.2.3.4.2.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths based on EN-DC operating bands specified in clause 5.5B, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each EN-DC configuration, and are shown in Table 7.3B.2.3.4.2.1-0 to Table 7.3B.2.3.4.2.1-6.

The details of the uplink and downlink reference measurement channels (RMCs) are specified in TS 36.521-1 [10] Annex A.2 and A.3 for E-UTRA RMC, and in TS 38.521-1 [8] Annex A.2 and A.3 for NR RMC respectively. The details of the OCNG patterns used are specified in TS 36.521-1 [10] Annex A.5 and in TS 38.521-1 [8] Annex A.5 for E-UTRA CG and NR CG, respectively. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG, respectively.

For configurations without any reference sensitivity exception, in a FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. For configurations with reference sensitivity exception, in an E-UTRA band or FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected.

Table 7.3B.2.3.4.2.1-0: Test configurations table for all EN-DC configurations for FR1 non-exception requirements

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause4.3.1, E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  |  | Mid range for E-UTRA and Mid range for NR |  |  |  |
| Test EN-DC channel bandwidth as specified in TS 36.508 [11] clause 4.3.1 and TS 38.508-1 [6] clause 4.3.1 |  |  |  |  |  | Refer to "NR NRB "and "E-UTRA NRB " columns |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  |  |  | Lowest supported SCS |  |  |  |
| Network signalling value |  |  |  |  |  | NS_01 by default Unless given by Table 7.3.3-3 in TS 36.521-1 [10] for the E-UTRA band and Table 7.3.2.3-4 in TS 38.521-1 [8] for the NR band. |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |  |
| ID | PCC – E-UTRA |  |  |  | SCG -NR |  |  |  |  |
|  | Band | Range | NRB |  | Band |  | Range | NRB |  |
|  | UL MOD | DL MOD | CH BW | DLalloc / UL alloc | UL MOD |  | DL MOD | UL/DL Ch BW | DLalloc / UL alloc |
| Test Settings for a DC_XA_nYA Configuration (Inter-band EN-DC) – Note 2 |  |  |  |  |  |  |  |  |  |
| 1 | X | default |  |  | nY |  | default |  |  |
|  | N/A | N/A | 5 MHz | N/A | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | Highest NRB | All RBs / REFSENS_NR |
| NOTE 1: REFSENS_NR refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3 of TS 38.521-1 [8]. NOTE 2: LTE anchor agnostic configuration NOTE 3: In a FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. NOTE 4: The test configuration table only applies to EN-DC only capable devices and devices not supporting NR/5GC mode in the tested band. NOTE 5: Only one EN-DC combination per FR1 band is tested for each EN-DC configuration as defined in clause 5.5B. |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-1: Void

Table 7.3B.2.3.4.2.1-2_1: Void

Table 7.3B.2.3.4.2.1-2_2: Void

Table 7.3B.2.3.4.2.1-2_3: Void

Table 7.3B.2.3.4.2.1-2_4: Void

Table 7.3B.2.3.4.2.1-2_5: Void

Table 7.3B.2.3.4.2.1-2_6: Void

Table 7.3B.2.3.4.2.1-2_7: Void

Table 7.3B.2.3.4.2.1-2_8: Void

Table 7.3B.2.3.4.2.1-2_9: Void

Table 7.3B.2.3.4.2.1-2_11: Void

Table 7.3B.2.3.4.2.1-2_12: Void

Table 7.3B.2.3.4.2.1-2_13: Void

Table 7.3B.2.3.4.2.1-2_14: Void

Table 7.3B.2.3.4.2.1-2_15: Void

Table 7.3B.2.3.4.2.1-2_16: Void

Table 7.3B.2.3.4.2.1-2_17: Void

Table 7.3B.2.3.4.2.1-2_18: Void

Table 7.3B.2.3.4.2.1-2_19: Void

Table 7.3B.2.3.4.2.1-2_20: Void

Table 7.3B.2.3.4.2.1-2_21: Void

Table 7.3B.2.3.4.2.1-2_22: Void

Table 7.3B.2.3.4.2.1-2_23: Void

Table 7.3B.2.3.4.2.1-4: Void

Table 7.3B.2.3.4.2.1-4a: Void

Table 7.3B.2.3.4.2.1-4b: Void

Table 7.3B.2.3.4.2.1-4c: Void

Table 7.3B.2.3.4.2.1-4d: Void

Table 7.3B.2.3.4.2.1-4e: Void

Table 7.3B.2.3.4.2.1-4f: Void

Table 7.3B.2.3.4.2.1-4g: Void

Table 7.3B.2.3.4.2.1-4h: Void

Table 7.3B.2.3.4.2.1-4i: Void

Table 7.3B.2.3.4.2.1-4j: Void

Table 7.3B.2.3.4.2.1-4k: Void

Table 7.3B.2.3.4.2.1-4l: Void

Table 7.3B.2.3.4.2.1-4m: Void

Table 7.3B.2.3.4.2.1-4n: Void

Table 7.3B.2.3.4.2.1-5: Void

Table 7.3B.2.3.4.2.1-6: Test Configuration Table for EN-DC configurations affected by Reference sensitivity exceptions (two bands)

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1, E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  |  | For test frequencies refer to “Range” columns. |  |  |  |
| Test EN-DC channel bandwidth as specified in TS 36.508 [11] clause 4.3.1 and TS 38.508-1 [6] clause 4.3.1. |  |  |  |  |  | Refer to "NR NRB"and "E-UTRA NRB " columns |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  |  |  | Lowest supported SCS |  |  |  |
| Network signalling value |  |  |  |  |  | NS_01 by default Unless given by Table 7.3.3-3 in TS 36.521-1 [10] for the E-UTRA band and Table 7.3.2.3-4 in TS 38.521-1 [8] for the NR band. |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |  |
| ID | PCC – E-UTRA |  |  |  | SCG -NR |  |  |  |  |
|  | Band | Range | NRB |  | Band |  | Range | NRB |  |
|  | UL MOD | DL MOD | CH BW | DLalloc / UL alloc | UL MOD |  | DL MOD | UL/DL Ch BW | DLalloc / UL alloc |
| Test Settings for a DC_1A_n3A Configuration |  |  |  |  |  |  |  |  |  |
| 12,11 | 1 | Mid |  |  | n3 |  | Mid |  |  |
|  | QPSK | QPSK | 20 MHz | All RBs / REFSENS_ENDC_3 | N/A |  | CP-OFDM QPSK | 40 MHz | All RBs / 0 |
| 24 | 1 | UL 1950 / DL 2140 |  |  | n3 |  | UL 1760 / DL 1855 |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |
| 36 | 1 | Low |  |  | n3 |  | High |  |  |
|  | QPSK | QPSK | 20 MHz | All RBs / REFSENS_ENDC_3 | N/A |  | CP-OFDM QPSK | 40 MHz | All RBs / 0 |
| Test Settings for a DC_1A_n8A Configuration |  |  |  |  |  |  |  |  |  |
| 14 | 1 | UL 1965 / DL 2155 |  |  | n8 |  | UL 887.5 / DL 932.5 |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_1A_n28A Configuration |  |  |  |  |  |  |  |  |  |
| 13 | 1 | UL 1950 / DL 2140 |  |  | n28 |  | UL 713.3 / DL 768.3 |  |  |
|  | N/A | QPSK | 20 MHz | All RBs / 0 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_1 |
| 28 | 1 | UL 1970 / DL 2160 |  |  | n28 |  | UL 708 / DL 763 |  |  |
|  | N/A | QPSK | 20 MHz | All RBs / 0 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_1 |
| Test Settings for a DC_1A_n41A Configuration (PC2 and PC3) |  |  |  |  |  |  |  |  |  |
| 16 | 1 | High |  |  | n41 |  | Low |  |  |
|  | QPSK | QPSK | 20 MHz | All RBs / REFSENS_ENDC_3 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 100 MHz | All RBs / 0 |
| 26 | 1 | High |  |  | n41 |  | Low |  |  |
|  | QPSK | QPSK | 20 MHz | All RBs / 0 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 50 MHz | All RBs / REFSENS_ENDC_3 |
| Test Settings for a DC_1A_n77A Configuration (PC2 and PC3) |  |  |  |  |  |  |  |  |  |
| 13 | 1 | UL 1950/ DL 2140 |  |  | n77 |  | UL/DL 3900 |  |  |
|  | QPSK | QPSK | 20 MHz | All RBs / REFSENS_ENDC_1 | N/A |  | CP-OFDM QPSK | 100 MHz | All RBs / 0 |
| 23 | 1 | UL 1950/ DL 2140 |  |  | n77 |  | UL/DL 3870 |  |  |
|  | QPSK | QPSK | 20 MHz | All RBs / REFSENS_ENDC_1 | N/A |  | CP-OFDM QPSK | 20 MHz | All RBs / 0 |
| 34 | 1 | UL 1950/ DL 2140 |  |  | n77 |  | UL/DL 4090.005 |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |
| 44 | 1 | UL 1950/ DL 2140 |  |  | n77 |  | UL/DL 3709.995 |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_1A_n78A Configuration (PC2 and PC3) |  |  |  |  |  |  |  |  |  |
| 14 | 1 | UL 1950 / DL 2140 |  |  | n78 |  | UL/DL 3709.995 |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_2A_n41A Configuration (PC2 and PC3) |  |  |  |  |  |  |  |  |  |
| 16 | 2 | DL 1987.5 |  |  | n41 |  | UL/DL 2546 |  |  |
|  | N/A | QPSK | 5 MHz | All RBs / 0 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 100 MHz | All RBs / REFSENS_ENDC_3 |
| Test Settings for a DC_2A_n66A Configuration |  |  |  |  |  |  |  |  |  |
| 14 | 2 | UL 1855 / DL 1935 |  |  | n66 |  | DL 2175 |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |
| 24 | 2 | UL 1883.3 /DL1963.3 |  |  | n66 |  | DL 2150 |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_2A_n71A Configuration |  |  |  |  |  |  |  |  |  |
| 13 | 2 | DL 1980 MHZ |  |  | n71 |  | UL 665.5 MHz |  |  |
|  | QPSK | QPSK | 20 MHz | All RBs / 0 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 5 MHz | All RBs / REFSENS_ENDC_1 |
| 23 | 2 | DL1980 MHZ |  |  | n71 |  | UL 673 MHz |  |  |
|  | QPSK | QPSK | 20 MHz | All RBs / 0 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_1 |
| 35 | 2 | UL 1881 MHZ |  |  | n71 |  | Low |  |  |
|  | QPSK | QPSK | 20 MHz | All RBs / REFSENS_ENDC_2 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 20 MHz | All RBs / 0 |
| Test Settings for a DC_2A_n77A Configuration |  |  |  |  |  |  |  |  |  |
| 13 | 2 | UL 1860 |  |  | n77 |  | UL/DL 3720 |  |  |
|  | QPSK | QPSK | 20 MHz | All RBs / REFSENS_ENDC_1 | N/A |  | CP-OFDM QPSK | 100 MHz | All RBs / 0 |
| 23 | 2 | UL 1860 |  |  | n77 |  | UL/DL 3690.005 |  |  |
|  | QPSK | QPSK | 20 MHz | All RBs / REFSENS_ENDC_1 | N/A |  | CP-OFDM QPSK | 20 MHz | All RBs / 0 |
| 38 | 2 | UL 1860 |  |  | n77 |  | UL/DL 3850.005 |  |  |
|  | QPSK | QPSK | 20 MHz | All RBs / REFSENS_ENDC_1 | N/A |  | CP-OFDM QPSK | 10 MHz | All RBs / 0 |
| 45 | 2 | DL Mid |  |  | n77 |  | UL/DL 3920.01 |  |  |
|  | N/A | QPSK | 20 MHz | All RBs / 0 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 100 MHz | All RBs / REFSENS_ENDC_2 |
| 4a9 | 2 | DL Mid |  |  | n77 |  | UL/DL Mid |  |  |
|  | N/A | QPSK | 20 MHz | All RBs / 0 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 100 MHz | All RBs / REFSENS_ENDC_2 |
| 54 | 2 | UL 1855/DL 1935 |  |  | n77 |  | UL/DL 3790.005 |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |
| 64 | 2 | UL 1900/DL 1980 |  |  | n77 |  | UL/DL 3720 |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |
| 74 | 2 | UL 1885/DL 1965 |  |  | n77 |  | UL/DL 3810 |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |
|  | 2 | DL 1987.5 |  |  | n77 |  | UL/DL 3350 |  |  |
| 86,16 | QPSK | QPSK | 5 MHz | All RBs / 0 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 100 MHz | All RBs / REFSENS_ENDC_3 |
| Test Settings for a DC_2A_n78A Configuration |  |  |  |  |  |  |  |  |  |
| 13 | 2 | UL 1870 |  |  | n78 |  | UL/DL 3740.01 |  |  |
|  | QPSK | QPSK | 20 MHz | All RBs / REFSENS_ENDC_1 | N/A |  | CP-OFDM QPSK | 100 MHz | All RBs / 0 |
| 23 | 2 | UL 1885 |  |  | n78 |  | UL/DL 3739.995 |  |  |
|  | QPSK | QPSK | 20 MHz | All RBs / REFSENS_ENDC_1 | N/A |  | CP-OFDM QPSK | 20 MHz | All RBs / 0 |
| 34 | 2 | UL 1855/ DL 1935 |  |  | n78 |  | UL/DL 3790.005 |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |
| 44 | 2 | UL 1885/ DL 1965 |  |  | n78 |  | UL/DL 3690 |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_3A_n1A Configuration |  |  |  |  |  |  |  |  |  |
| 111 | 3 | Mid |  |  | n1 |  | Mid |  |  |
|  | N/A | QPSK | 20 MHz | All RBs / 0 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 50 MHz | All RBs / REFSENS_ENDC_3 |
| 26 | 3 | High |  |  | n1 |  | Low |  |  |
|  | N/A | QPSK | 20 MHz | All RBs / 0 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 50 MHz | All RBs / REFSENS_ENDC_3 |
| 34 | 3 | UL 1760 / DL 1855 |  |  | n1 |  | UL 1950 / DL 2140 |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_3A_n5A Configuration |  |  |  |  |  |  |  |  |  |
| 14 | 3 | UL 1771 / DL 1866 |  |  | n5 |  | UL 838 / DL 883 |  |  |
|  | QPSK | QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |
| 24 | 3 | UL 1721 / DL 1816 |  |  | n5 |  | UL 838 / DL 883 |  |  |
|  | QPSK | QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_3A_n7A Configuration |  |  |  |  |  |  |  |  |  |
| 14 | 3 | UL 1730 / DL 1825 |  |  | n7 |  | UL 2535 / DL 2655 |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_3A_n8A Configuration |  |  |  |  |  |  |  |  |  |
| 14 | 3 | UL 1755 / DL 1850 |  |  | n8 |  | UL 900 / DL 945 |  |  |
|  | QPSK | QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |
| 24 | 3 | UL 1747.5 / DL 1842.5 |  |  | n8 |  | UL 897.5 / DL 942.5 |  |  |
|  | QPSK | QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_3A_n41A Configuration |  |  |  |  |  |  |  |  |  |
| 16 | 3 | High |  |  | n41 |  | Low |  |  |
|  | QPSK | QPSK | 20 MHz | All RBs / REFSENS_ENDC_3 | N/A |  | CP-OFDM QPSK | 100 MHz | All RBs / 0 |
| 26 | 3 | High |  |  | n41 |  | Low |  |  |
|  | N/A | QPSK | 20 MHz | All RBs / 0 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 50 MHz | All RBs / REFSENS_ENDC_3 |
| 34 | 3 | UL 1740 / DL 1835 |  |  | n41 |  | UL/DL 2657.5 |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |
| Test Settings for DC_3A_n77A (PC2 and PC3) and DC_3A_n78A Configurations |  |  |  |  |  |  |  |  |  |
| 13 | 3 | Mid |  |  | n77/n78 |  | UL/DL 3495 |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_1 | N/A |  | CP-OFDM QPSK | 10 MHz | All RBs / 0 |
| 1a8 | 3 | Low |  |  | n77/n78 |  | Low |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_1 | N/A |  | CP-OFDM QPSK | 10 MHz | All RBs / 0 |
| 23 | 3 | Mid |  |  | n77/n78 |  | UL/DL 3525 |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_1 | N/A |  | CP-OFDM QPSK | 100 MHz | All RBs / 0 |
| 2a8 | 3 | Low |  |  | n77/n78 |  | Low |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_1 | N/A |  | CP-OFDM QPSK | 100 MHz | All RBs / 0 |
| 35 | 3 | Mid |  |  | n77/n78 |  | UL/DL 3685.005 |  |  |
|  | N/A | QPSK | 10 MHz | All RBs / 0 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 5 MHz | All RBs / REFSENS_ENDC_2 |
| 3a5 | 3 | Mid |  |  | n77/n78 |  | UL/DL 3685.005 |  |  |
|  | N/A | QPSK | 10 MHz | All RBs / 0 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 20 MHz | All RBs / REFSENS_ENDC_2 |
| 49 | 3 | Low |  |  | n77/n78 |  | High |  |  |
|  | QPSK | QPSK | 10 MHz | All RBs/0 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 5 MHz | All RBs / REFSENS_ENDC_2 |
| 4a9 | 3 | Low |  |  | n77/n78 |  | High |  |  |
|  | QPSK | QPSK | 10 MHz | All RBs/0 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 20 MHz | All RBs / REFSENS_ENDC_2 |
| 54 | 3 | UL 1740 / DL 1835 |  |  | n77/n78 |  | UL/DL 3574.995 |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |
| 64 | 3 | UL 1765 / DL 1860 |  |  | n77/n78 |  | UL/DL 3435 |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_5A_n66A Configuration |  |  |  |  |  |  |  |  |  |
| 14 | 5 | UL 838/ DL 883 |  |  | n66 |  | UL 1721/ DL 2121 |  |  |
|  | QPSK | N/A | 5 MHz | All RBs / REFSENS_ENDC_4 | N/A |  | CP-OFDM QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_5A_n77A Configuration |  |  |  |  |  |  |  |  |  |
| 13 | 5 | UL 837.5 |  |  | n77 |  | UL/DL 3350.01 |  |  |
|  | QPSK | QPSK | 10 MHz | All RBs / REFSENS_ENDC_1 | N/A |  | CP-OFDM QPSK | 100 MHz | All RBs / 0 |
| 23 | 5 | UL 829 |  |  | n77 |  | UL/DL 4145.01 |  |  |
|  | QPSK | QPSK | 10 MHz | All RBs / REFSENS_ENDC_1 | N/A |  | CP-OFDM QPSK | 100 MHz | All RBs / 0 |
| 38 | 5 | UL Mid (836.5 MHz) |  |  | n77 |  | UL/DL Mid |  |  |
|  | QPSK | QPSK | 10 MHz | All RBs / REFSENS_ENDC_1 | N/A |  | CP-OFDM QPSK | 100 MHz | All RBs / 0 |
| 44,13 | 5 | UL 844/DL 889 |  |  | n77 |  | UL/DL 3421.005 |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |
| 54,13 | 5 | UL 826.5/DL 871.5 |  |  | n77 |  | UL/DL 4177.5 |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |
| Test Settings for DC_5A_n78A Configuration |  |  |  |  |  |  |  |  |  |
| 13 | 5 | UL 840 |  |  | n78 |  | UL/DL 3360 |  |  |
|  | QPSK | QPSK | 10 MHz | All RBs / REFSENS_ENDC_1 | N/A |  | CP-OFDM QPSK | 100 MHz | All RBs / 0 |
| 1a3 | 5 | UL 840 |  |  | n78 |  | UL/DL 3360 |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_1 | N/A |  | CP-OFDM QPSK | 10 MHz | All RBs / 0 |
| 28 | 5 | UL 840 |  |  | n78 |  | UL/DL 3560.01 |  |  |
|  | QPSK | QPSK | 10 MHz | All RBs / REFSENS_ENDC_1 | N/A |  | CP-OFDM QPSK | 100 MHz | All RBs / 0 |
| 34 | 5 | UL 844/DL 889 |  |  | n78 |  | UL/DL 3421.005 |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |
| 45 | 5 | MID |  |  | n78 |  | UL/DL 3525.99 |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / 0 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_2 |
| 59 | 5 | High |  |  | n78 |  | MID |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / 0 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_2 |
| Test Settings for a DC_7A_n3A Configuration |  |  |  |  |  |  |  |  |  |
| 14 | 7 | DL 2655 |  |  | n3 |  | DL 1825 |  |  |
|  | QPSK | N/A | 10 MHz | All RBs / REFSENS_ENDC_4 | N/A |  | CP-OFDM QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_7A_n5A Configuration |  |  |  |  |  |  |  |  |  |
| 14 | 7 | DL 2667 |  |  | n5 |  | DL 879 |  |  |
|  | QPSK | N/A | 10 MHz | All RBs / REFSENS_ENDC_4 | N/A |  | CP-OFDM QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_7A_n66A Configuration |  |  |  |  |  |  |  |  |  |
| 14 | 7 | DL 2655 |  |  | n66 |  | DL 2130 |  |  |
|  | QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 | N/A |  | CP-OFDM QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_7A_n71A Configuration |  |  |  |  |  |  |  |  |  |
| 13 | 7 | DL 2672 |  |  | n71 |  | UL/ 668 |  |  |
|  | QPSK | QPSK | 20 MHz | All RBs / 0 | N/A |  | CP-OFDM QPSK | 5 MHz | All RBs / REFSENS_ENDC_1 |
| 28 | 7 | DL 2630 |  |  | n71 |  | UL 668 |  |  |
|  | QPSK | QPSK | 20 MHz | All RBs / 0 | N/A |  | CP-OFDM QPSK | 5 MHz | All RBs / REFSENS_ENDC_1 |
| Test Settings for a DC_7A_n77A Configuration |  |  |  |  |  |  |  |  |  |
| 14 | 7 | DL 2660 |  |  | N77 |  | DL 3870 |  |  |
|  | QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 | N/A |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_7A_n78A (PC2 and PC3) Configuration |  |  |  |  |  |  |  |  |  |
| 16 | 7 | High |  |  | n78 |  | Low |  |  |
|  | N/A | QPSK | 5 MHz | All RBs / 0 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 100 MHz | All RBs / REFSENS_ENDC_3 |
| Test Settings for a DC_8A_n1A Configuration |  |  |  |  |  |  |  |  |  |
| 14 | 8 | DL 932.5 |  |  | n1 |  | DL 2155 |  |  |
|  | QPSK | N/A | 5 MHz | All RBs / REFSENS_ENDC_4 | N/A |  | CP-OFDM QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_8A_n3A Configuration |  |  |  |  |  |  |  |  |  |
| 14 | 8 | DL 945 |  |  | n3 |  | DL 1850 |  |  |
|  | QPSK | N/A | 5 MHz | All RBs / REFSENS_ENDC_4 | N/A |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |
| 24 | 8 | DL 942.5 |  |  | n3 |  | DL 1842.5 |  |  |
|  | QPSK | N/A | 5 MHz | All RBs / REFSENS_ENDC_4 | N/A |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_8A_n20A Configuration |  |  |  |  |  |  |  |  |  |
| 14 | 8 | UL 890.5 / DL 935.5 |  |  | n20 |  | UL 849.5 / DL 808.5 |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |
| 24 | 8 | UL 892.5 / DL 937.5 |  |  | n20 |  | UL 847.5 / DL 806.5 |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_8A_n41A Configuration |  |  |  |  |  |  |  |  |  |
| 13 | 8 | UL 885 / DL 942.5 MHz |  |  | n41 |  | UL / DL 2655 MHz |  |  |
|  | QPSK | QPSK | 10 MHz | All RBs / REFSENS_ENDC_1 | N/A |  | CP-OFDM QPSK | 60 MHz | All RBs / 0 |
| 28 | 8 | UL 885 / DL 942.5 MHz |  |  | n41 |  | UL / DL 2546 MHz |  |  |
|  | QPSK | QPSK | 10 MHz | All RBs / REFSENS_ENDC_1 | N/A |  | CP-OFDM QPSK | 60 MHz | All RBs / 0 |
| 34 | 8 | UL 882.5 / DL 927.5 MHz |  |  | n41 |  | UL / DL 2685MHz |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |
| Test Settings for DC_8A_n77A and DC_8A_n78A Configurations |  |  |  |  |  |  |  |  |  |
| 13 | 8 | Mid |  |  | n77 |  | UL/DL 3590.01 |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_1 | N/A |  | CP-OFDM QPSK | 100 MHz | All RBs / 0 |
| 28 | 8 | Mid |  |  | n77 |  | UL/DL 3520.005 |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_1 | N/A |  | CP-OFDM QPSK | 100 MHz | All RBs / 0 |
| 34 | 8 | UL 897.5 |  |  | n77 |  | UL/DL 3634.995 |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_11A_n79A Configuration |  |  |  |  |  |  |  |  |  |
| 15 | 11 | Mid |  |  | n79 |  | UL/DL 4457.7 |  |  |
|  | N/A | QPSK | 10 MHz | All RBs / 0 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 100 MHz | All RBs / REFSENS_ENDC_2 |
| 29 | 11 | Mid |  |  | n79 |  | UL/DL 4512.69 |  |  |
|  | N/A | QPSK | 10 MHz | All RBs / 0 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 100 MHz | All RBs / REFSENS_ENDC_2 |

## <<< END OF CHANGES >>>