3GPP TSG-RAN5 Meeting #107	R5-252351

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4987 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Idle Mode NR NTN test cases 6.7.1.1, 6.7.1.2, 6.7.1.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Please refer to Prose CR R5-252347 |  |  |  |  |  |  |  |  |
| Summary of change: |  | NR-28 sys info combination is replaced with NR-29 sys info combination. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Prose will refer to a voided sys info combination |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.1.1, 6.7.1.2 and 6.7.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.508-1 (R5-252347) |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 6.7.1.1	NR NTN / Cell Selection / GNSS location

6.7.1.1.1	Test Purpose (TP)

(1)

with { UE in 5GMM-DEREGISTERED/RRC-IDLE and UE finds a cell which provides access by NR NTN RAT }

ensure that {

when { UE has no valid GNSS position }

then { UE shall not establish RRC connection to perform a Registration procedure }

}

(2)

with { UE in 5GMM-DEREGISTERED/RRC-IDLE and UE finds a cell which provides access by NR NTN }

ensure that {

when { Cell broadcasts SIB19 and UE has a valid GNSS position }

then { UE establishes RRC connection and successfully performs Registration procedure}

}

6.7.1.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS38.306 clauses 16.14.1, 16.14.2.1 and 16.14.5, TS 38.331 clauses 5.2.2.3.1, 5.2.2.4.1, 5.2.2.4.21, 5.2.2.5 and 5.2.2.6.

Unless otherwise stated these are Rel-17 requirements.

[TS 38.300, clause 16.14.1]

Figure 16.14.1-1 below illustrates an example of a Non-Terrestrial Network (NTN) providing non-terrestrial NR access to the UE by means of an NTN payload and an NTN Gateway, depicting a service link between the NTN payload and a UE, and a feeder link between the NTN Gateway and the NTN payload.

Figure 16.14.1-1: Overall illustration of an NTN

NOTE 1:	Figure 16.14.1-1 illustrates an NTN; RAN4 aspects are out of scope.

The NTN payload transparently forwards the radio protocol received from the UE (via the service link) to the NTN Gateway (via the feeder link) and vice-versa. The following connectivity is supported by the NTN payload:

-	An NTN gateway may serve multiple NTN payloads;

-	An NTN payload may be served by multiple NTN gateways.

NOTE 2:	In this release, the NTN-payload may change the carrier frequency, before re-transmitting it on the service link, and vice versa (respectively on the feeder link).

For NTN, the following applies in addition to Network Identities as described in clause 8.2:

-	A Tracking Area corresponds to a fixed geographical area. Any respective mapping is configured in the RAN;

-	A Mapped Cell ID as specified in clause 16.14.5.

Three types of service links are supported:

-	Earth-fixed: provisioned by beam(s) continuously covering the same geographical areas all the time (e.g., the case of GSO satellites);

-	Quasi-Earth-fixed: provisioned by beam(s) covering one geographic area for a limited period and a different geographic area during another period (e.g., the case of NGSO satellites generating steerable beams);

-	Earth-moving: provisioned by beam(s) whose coverage area slides over the Earth surface (e.g., the case of NGSO satellites generating fixed or non-steerable beams).

With NGSO satellites, the gNB can provide either quasi-Earth-fixed service link or Earth-moving service link, while gNB operating with GSO satellite can provide Earth fixed service link.

In this release, the UE supporting NTN is GNSS-capable.

[TS 38.306, clause 16.14.2.1]

DL and UL are frame aligned at the uplink time synchronization reference point (RP) with an offset given by NTA,offset (see clause 4.2 of TS 38.213 [38]).

To accommodate the propagation delay in NTNs, several timing relationships are enhanced by a Common Timing Advance (Common TA) and two offsets Koffset and kmac:

-	Common TA is a configured timing offset that is equal to the RTT between the RP and the NTN payload.

-	Koffset is a configured scheduling offset that needs to be larger or equal to the sum of the service link RTT and the Common TA.

-	kmac is a configured offset that is approximately equal to the RTT between the RP and the gNB.

The scheduling offset Koffset is used to allow the UE sufficient processing time between a downlink reception and an uplink transmission, see TS 38.213 [38].

The offset kmac is used to delay the application of a downlink configuration indicated by a MAC CE command on PDSCH, see TS 38.213 [38], and in estimation of UE-gNB RTT, see TS 38.321 [6]. It may be provided by the network when downlink and uplink frame timing are not aligned at gNB. The kmac is also used in the random access procedure, to determine the start time of RAR window/MsgB window after a Msg1/MsgA transmission (see TS 38.213 [38]).

The Service link RTT, Feeder link RTT, RP, Common TA, kmac and TTA (see clause 16.14.2.2) are illustrated in Figure 16.14.2.1-1.

Figure 16.14.2.1-1: Illustration of timing relationship (for collocated gNB and NTN Gateway)

The network may configure the HARQ operation as follows:

-	For downlink, HARQ feedback can be enabled or disabled per HARQ process (as specified in clause 5.3.2.2 and clause 5.7 of TS 38.321 [6]). Disabling HARQ feedback allows scheduling a HARQ process before one HARQ RTT has elapsed since last scheduled.

-	For uplink, HARQ mode (i.e. HARQ mode A or HARQ mode B) can be configured per HARQ process (as specified in clause 5.4.3.1 and clause 5.7 of TS 38.321 [6]). HARQ mode B allows scheduling a HARQ process before one HARQ RTT has elapsed since last scheduled.

NOTE:	For the HARQ processes configured with HARQ feedback enabled/disabled, it is up to network implementation to ensure a proper configuration of HARQ feedback (e.g., either all enabled or all disabled) for HARQ processes used by an SPS configuration. For the HARQ processes configured with HARQ mode, it is up to network implementation to ensure a proper configuration of HARQ mode (e.g., either all HARQ mode A or all HARQ mode B) for HARQ processes used by a CG configuration.

[TS 38.331, 5.2.2.1]

Figure 5.2.2.1-1: System information acquisition

The UE applies the SI acquisition procedure to acquire the AS, NAS- and positioning assistance data information. The procedure applies to UEs in RRC_IDLE, in RRC_INACTIVE and in RRC_CONNECTED.

The UE in RRC_IDLE and RRC_INACTIVE shall ensure having a valid version of (at least) the MIB, SIB1 through SIB4, SIB5 (if the UE supports E-UTRA), SIB11 (if the UE is configured for idle/inactive measurements), SIB12 (if UE is capable of NR sidelink communication/discovery and is configured by upper layers to receive or transmit NR sidelink communication/discovery), and SIB13, SIB14 (if UE is capable of V2X sidelink communication and is configured by upper layers to receive or transmit V2X sidelink communication), SIB15 (if UE is configured by upper layers to report disaster roaming related information), SIB16 (if the UE is capable of slice-based cell reselection and the UE receives NSAG information for cell reselection from upper layer), SIB17 (if the UE is using TRS resources for power saving in RRC_IDLE and RRC_INACTIVE) and SIB19 (if UE is accessing NR via NTN access).

[TS 38.331, clause 5.2.2.3.1]

The UE shall:

1>	apply the specified BCCH configuration defined in 9.1.1.1;

1>	if the UE is in RRC_IDLE or in RRC_INACTIVE; or

1>	if the UE is in RRC_CONNECTED while T311 is running:

2>	acquire the MIB, which is scheduled as specified in TS 38.213 [13];

2>	if the UE is unable to acquire the MIB;

3>	perform the actions as specified in clause 5.2.2.5;

2>	else:

3>	perform the actions specified in clause 5.2.2.4.1.

…

1>	if the UE is in RRC_IDLE or in RRC_INACTIVE; or

…

2>	else if SIB1 acquisition is required for the UE and ssb-SubcarrierOffset indicates that SIB1 is not scheduled in the cell:

3>	perform the actions as specified in clause 5.2.2.5.

NOTE 1:	The UE in RRC_CONNECTED is only required to acquire broadcasted SIB1 and MBS broadcast if the UE can acquire it without disrupting unicast or MBS multicast data reception, i.e., the broadcast and unicast/MBS multicast beams are quasi co-located. The UE in RRC_INACTIVE state while SDT procedure is ongoing, is only required to acquire broadcasted SIB1 and MIB if the UE can acquire them without disrupting unicast data reception, i.e. the broadcast and unicast beams are quasi co-located.

NOTE 2:	UE in RRC_INACTIVE that does not support inactiveStateNTN-r17 enters RRC_IDLE upon cell reselection between TN cell and NTN cell, and initiates the NAS signalling connection recovery (see TS 24.501 [23]).

[TS 38.331, clause 5.2.2.4.1]

Upon receiving the MIB the UE shall:

1>	store the acquired MIB;

1>	if the UE is in RRC_IDLE or in RRC_INACTIVE, or if the UE is in RRC_CONNECTED while T311 is running:

...

2>	else:

3>	apply the received systemFrameNumber, pdcch-ConfigSIB1, subCarrierSpacingCommon, ssb-SubcarrierOffset and dmrs-TypeA-Position.

NOTE:	A UE capable of NTN access should acquire SIB1 to determine whether the cell is an NTN cell.

[TS 38.331, clause 5.2.2.5]

The UE shall:

1>	if in RRC_IDLE or in RRC_INACTIVE or in RRC_CONNECTED while T311 is running:

2>	if the UE is unable to acquire the MIB:

3>	consider the cell as barred in accordance with TS 38.304 [20];

3>	perform barring as if intraFreqReselection, or intraFreqReselectionRedCap for RedCap UEs, is set to allowed;

2>	else if the UE is unable to acquire the SIB1:

3>	consider the cell as barred in accordance with TS 38.304 [20];

….

NOTE:	The SIB19 is essential for NTN access. If UE is unable to acquire the SIB19 for NTN access, the action is up to UE implementation (e.g., cell re-selection to other cells).

6.7.1.1.3	Test description

6.7.1.1.3.1	Pre-test conditions

System Simulator:

NR Cell 1 as specified in TS 38.508-1 [4] Table 4.4.2.1.

System information combination NR-29 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

UE:

-	The UE's positioning engine (e.g., standalone GNSS receiver) shall be disabled or no UE location is pre-configured.

Preamble:

-	The UE is in Switched OFF (State 0N-B) as defined in TS 38.508-1 [4] Table 4.4A.2-0.

6.7.1.1.3.2	Test procedure sequence

Table 6.7.1.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Power on the UE. | - | - | - | - |
| 2 | Check: Does the UE send an RRCSetupRequest on NR Cell 1 within 6 minutes? | - | - | 1 | F |
| 3 | Power off the UE. | - | - | - | - |
| 4 | Power on UE and pre-configure the UE location with UE`s positioning engine enabled. (Note 1) | - | - | - | - |
| 5 | Check: Does the UE send an RRCSetupRequest on NR Cell 1 within 6 minutes? | --> | NR RRC: RRCSetupRequest | 2 | P |
| 6-11 | Steps 3 to 8 of the registration procedure described in TS 38.508-1 [4] Table 4.5.4.2-3 are performed on NR Cell 1. | - | - | - | - |
| 12 | The SS transmits an RRCRelease message to move to RRC_IDLE state. | <-- | NR RRC: RRCRelease | - | - |
| Note 1: This could be done by use of AT command and the detailed UE location is defined in TS 38.508-1 [4] Clause 4.5C. |  |  |  |  |  |

6.7.1.1.3.3	Specific message contents

None

#### 6.7.1.2	NR NTN / Cell Selection / MultiTAC / trackingAreaList-r17

6.7.1.2.1	Test Purpose (TP)

(1)

with { UE in NR RRC_IDLE and UE finds a suitable cell which provides access by NR NTN RAT }

ensure that {

when { SIB1 broadcast more than one TAC per PLMN }

then { UE forwards the trackingAreaList to upper layers for mobility management procedure and reselects the cell}

}

6.7.1.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS38.306 clauses 5.2.2.4.1, and 5.2.2.4.2.

Unless otherwise stated these are Rel-17 requirements.

[TS 38.331, clause 5.2.2.4.1]

Upon receiving the MIB the UE shall:

1>	store the acquired MIB;

1>	if the UE is in RRC_IDLE or in RRC_INACTIVE, or if the UE is in RRC_CONNECTED while T311 is running:

2>	if the access is not for NTN or the UE is not capable of NTN; and

2>	if the cellBarred in the acquired MIB is set to barred:

…

3>	consider the cell as barred in accordance with TS 38.304 [20];

3>	perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20];

2>	else:

3>	apply the received systemFrameNumber, pdcch-ConfigSIB1, subCarrierSpacingCommon, ssb-SubcarrierOffset and dmrs-TypeA-Position.

NOTE:	A UE capable of NTN access should acquire SIB1 to determine whether the cell is an NTN cell.

[TS 38.331, clause 5.2.2.4.2]

Upon receiving the SIB1 the UE shall:

1>	store the acquired SIB1;

1>	if the access is for NTN, and the cellBarredNTN in the acquired SIB1 is set to barred or the cellBarredNTN is not included in the acquired SIB1:

2>	consider the cell as barred in accordance with TS 38.304 [20];

2>	perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20];

….

1>	if the cellAccessRelatedInfo contains an entry of a selected SNPN or PLMN and in case of PLMN the UE is either allowed or instructed to access the PLMN via a cell for which at least one CAG ID is broadcast:

2>	in the remainder of the procedures use npn-IdentityList, trackingAreaCode, and cellIdentity for the cell as received in the corresponding entry of npn-IdentityInfoList containing the selected PLMN or SNPN;

1>	else if the cellAccessRelatedInfo contains an entry with the PLMN-Identity of the selected PLMN:

2>	in the remainder of the procedures use plmn-IdentityList, trackingAreaCode, trackingAreaList, and cellIdentity for the cell as received in the corresponding PLMN-IdentityInfo containing the selected PLMN;

…

1>	else:

…

…

3>	else:

…

4>	forward the cellIdentity to upper layers;

4>	forward the trackingAreaCode to upper layers;

4>	forward the trackingAreaList to upper layers, if included;

6.7.1.2.3	Test description

6.7.1.2.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 and NR Cell 11 as specified in TS 38.508-1 [4] Table 4.4.2.1.

-	System information combination NR-29 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

UE:

-	The pre-configured UE location is defined in TS 38.508-1 [4] Clause 4.5C.

Preamble:

-	The UE is in test state 1N-A on NR Cell 1 according to TS 38.508-1 [4].

6.7.1.2.3.2	Test procedure sequence

Table 6.7.1.2.3.2-1 for FR1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions for preamble, while columns marked "T1" to be applied subsequently in the Main behaviour. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 6.7.1.2.3.2-1: Cell configuration changes over time for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 11 | Remarks |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | "Off" | Power level “Off” is defined in TS 38.508-1 Table 6.2.2.1-3 |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | "Off" | -88 | Power level “Off” is defined in TS 38.508-1 Table 6.2.2.1-3 |

Table 6.7.1.2.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS adjusts cell levels according to row T1 of table 6.7.1.2.3.2-1. | - | - | - | - |
| 2 | Check: Does the UE send an RRCSetupRequest on NR Cell 11 within 3 minutes? | -> | NR RRC: RRCSetupRequest | 1 | F |
| 3 | The SS transmits a Paging message on NR Cell 11. | <-- | NR RRC: Paging | - | - |
| 4 | Check: Does the UE transmit an RRCSetupRequest message on NR Cell 11? |  | NR RRC: RRCSetupRequest | 1 | P |
| 5-10 | Steps 3 to 8 of the generic test procedure in TS 38.508-1 [4] subclause 4.9.4.2.2-1 are performed on NR Cell 11. | - | - | - | - |
| 11 | The SS transmits an RRCRelease message to move to RRC_IDLE state. | <-- | NR RRC: RRCRelease | - | - |

6.7.1.2.3.3	Specific message contents

Table 6.7.1.2.3.3-1: SIB1 for Ncell 1 (preamble and all steps)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-Identity { | 1 entry |  |  |
| plmn-Identity[1] |  |  |  |
| trackingAreaList-r17SEQUENCE { |  |  |  |
| trackingAreaCode[1] | 1 | Entry 1 |  |
| trackingAreaCode[2] | 2 | Entry 2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.7.1.2.3.3-2: SIB1 for Ncell 11 (preamble and all steps)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-Identity { | 1 entry |  |  |
| plmn-Identity[1] |  |  |  |
| trackingAreaList-r17SEQUENCE { |  |  |  |
| trackingAreaCode[1] | 1 | Entry 1 |  |
| trackingAreaCode[2] | 2 | Entry 2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.7.1.2.3.3-3: SIB19 for Ncell 11 (preamble and all steps)

| Derivation Path: TS 38.508-1 [6], Table 4.6.2-18C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| NTN-Config-r17 ::= SEQUENCE { |  |  |  |
| ephemerisInfo-r17 | Refer to the Second cell in TS 38.508-1[4] Table 6.3.4.1-1 |  | GSO |
| ephemerisInfo-r17 | Refer to the Second cell in TS 38.508-1[4] Table 6.3.4.2-1 |  | NGSO |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| GSO | Geosynchronous Orbit scenario |
| NGSO | Non-geosynchronous Orbit scenario |

#### 6.7.1.3	NR NTN / Cell Selection / Serving cell becomes non-suitable (CellBarredNTN)

6.7.1.3.1	Test Purpose (TP)

(1)

with { UE in NR RRC_IDLE state }

ensure that {

when { UE finds a suitable cell that provides access by NR NTN RAT and cellBarredNTN in SIB1 is not present }

then { the UE considers the cell as barred and no camping on this cell can take place }

}

(2)

with { UE in NR RRC_IDLE state }

ensure that {

when { UE finds a suitable cell that provides access by NR NTN RAT  and cellBarredNTN in SIB1 is “not barred” }

then { the UE considers the cell suitable and camps on it }

}

(3)

with { UE in NR RRC_IDLE state }

ensure that {

when { UE finds a suitable cell that provides access by NR NTN RAT and cellBarredNTN in SIB1 changes to “barred” }

then { the UE considers the cell as barred and no camping on this cell can take place }

}

6.7.1.3.2	Conformance requirements

[TS 38.304, clause 5.3.1]

Cell status and cell reservations are indicated in the MIB or SIB1 message as specified in TS 38.331 [3] by means of following fields:

-	cellBarred (IE type: "barred" or "not barred") 
Indicated in MIB message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is common for all PLMNs and NPNs. This field is ignored by UEs supporting NTN while cellBarredNTN is included in SIB1.

-	cellBarredNTN (IE type: "barred" or "not barred")
Indicated in SIB1 message. In case of multiple PLMNs indicated in SIB1, this field is common for all PLMNs. This field is ignored if the UE does not support NTN connectivity.

…

NOTE 0:	IAB-MT ignores the cellBarred, cellReservedForOperatorUse, cellReservedForFutureUse, and intraFreqReselection (i.e. treats intraFreqReselection as if it was set to allowed) as defined in TS 38.331 [3]. IAB-MT also ignores cellReservedForOtherUse for cell barring determination (i.e. NPN capable IAB-MT considers cellReservedForOtherUse for determination of an NPN-only cell) as defined in TS 38.331 [3].

…

When cell status is indicated as "not barred" and "not reserved" for operator use and not "true" for other use and not "true" for future use,

-	UEs shall treat this cell as candidate during the cell selection and cell reselection procedures.

…

When cellBarredNTN is not broadcast in this cell,

-	For NTN access, the UE shall treat this cell as if cell status is "barred".

…

When cell status "barred" is indicated or to be treated as if the cell status is "barred",

-	The UE is not permitted to select/reselect this cell, not even for emergency calls.

-	The UE shall select another cell according to the following rule:

-	If the cell is to be treated as if the cell status is "barred" due to being unable to acquire the MIB:

-	the UE may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds.

-	the UE may select another cell on the same frequency if the selection criteria are fulfilled.

-	else:

…

-	If the UE is not a RedCap UE, or if the UE is a RedCap UE and intraFreqReselectionRedCap in SIB1 is available:

-	If the field intraFreqReselection in MIB message is set to "allowed":

-	the UE may select another cell on the same frequency if re-selection criteria are fulfilled;

-	If the cell is to be treated as if the cell status is "barred" due to being unable to acquire the SIB1:

-	the UE may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds;

-	else:

-	the UE shall exclude the barred cell as a candidate for cell selection/reselection for 300 seconds.

-	If the field intraFreqReselection in MIB message is set to "not allowed":

-	If the cell is to be treated as if the cell status is "barred" due to being unable to acquire the SIB1:

-	the UE may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds;

-	If the cell operates in licensed spectrum:

-	the UE shall not re-select to another cell on the same frequency as the barred cell and exclude such cell(s) as candidate(s) for cell selection/reselection for 300 seconds;

-	else:

-	the UE may select to another cell on the same frequency if the reselection criteria are fulfilled.

-	else:

-	If the cell operates in licensed spectrum, or if this cell belongs to a PLMN which is indicated as being equivalent to the registered PLMN or the selected PLMN of the UE, or if this cell belongs to the registered SNPN or the selected SNPN of the UE:

-	the UE shall not re-select to another cell on the same frequency as the barred cell and exclude such cell(s) as candidate(s) for cell selection/reselection for 300 seconds;

-	else:

-	the UE may select to another cell on the same frequency if the reselection criteria are fulfilled.

-	the UE shall exclude the barred cell as a candidate for cell selection/reselection for 300 seconds.

The cell selection of another cell may also include a change of RAT.

NOTE 2:	If barring of a cell is triggered by the condition of trackingAreaCode and trackingAreaList not being provided, as specified in TS 38.331 [3], the barring only applies to this PLMN and the UE can re-evaluate the barring condition again due to selection of another PLMN.

6.7.1.3.3	Test description

6.7.1.3.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 according to TS 38.508-1 [4] Table 4.4.2-3.

-	System information combination NR-29 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

UE:

The UE is in Automatic PLMN selection mode.

-	The pre-configured UE location is defined in TS 38.508-1 [4] Clause 4.5C.

Preamble:

-	The UE is in state Switched OFF (state 0N-B) according to TS 38.508-1 [4].

6.7.1.3.3.2	Test procedure sequence

Table 6.1.7.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Power on the UE | - | - | - | - |
| 2 | Check: Does the UE send an RRCSetupRequest on NR Cell 1 within the next 60 s? | --> | NR RRC: RRCSetupRequest | 1 | F |
| 3 | Power Off the UE | - | - | - | - |
| 4 | SS change NR Cell 1 SIB1 to include cellBarredNTN and set to "not barred" | - | - | - | - |
| 5 | Power on the UE | - | - | - | - |
| 6 | Check: Does the UE send an RRCSetupRequest on NR Cell 1 within the next 60 s? | --> | NR RRC: RRCSetupRequest | 2 | P |
| 7-19 | Steps 3 to 15 of the registration procedure described in TS 38.508-1 [4] Table 4.5.2.2-2 are performed on NR Cell 1. | - | - | - | - |
| 20 | The SS transmits an RRCRelease message to move to RRC_IDLE state. | <-- | RRCRelease | - | - |
| 21 | SS change NR Cell 1 SIB1 to include cellBarredNTN and set to "barred" | - | - | - | - |
| 22 | The SS transmits a Short message on PDCCHusing P-RNTI indicating a systemInfoModification. | - | PDCCH (DCI 1_0): ShortMessage | - | - |
| 23 | Wait for 2.1* modification period to allow the new system information to take effect. | - | - | - | - |
| 24 | The SS transmits a Paging message within a paging occasion | <-- | NR RRC: Paging | - | . |
| 25 | Check: Does the UE send an RRCSetupRequest on NR Cell 1 within the next 60 s? | --> | NR RRC: RRCSetupRequest | 3 | F |

6.7.1.3.3.3	Specific message contents

Table 6.7.1.3.3.3-1: SIB1 for NR cell 1 (Preamble)

| Derivation Path: 38.508-1 [4] Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| cellBarredNTN-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.7.1.3.3.3-2: SIB1 for NR Cell 1(Step 4 of Table 6.1.7.3.3.2-1)

| Derivation Path: 38.508-1 [4] Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| cellBarredNTN-r17 | notBarred |  | NTN |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.7.1.3.3.3-3: SIB1 for NR Cell 1 (Step 22 of Table 6.1.7.3.3.2-1)

| Derivation Path: 38.508-1 [4] Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| cellBarredNTN-r17 | Barred |  | NTN |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |