3GPP TSG-RAN5 Meeting #107	R5-252445

# St Julian’s, Malta, May 19-23, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3905 | rev | - | Current version: | 18.6.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case 7.5.13.4 joint TCI for sDCI mTRP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_evo_DL_UL-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The test case 7.5.13.4 joint TCI for sDCI mTRP is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding new test case 7.5.13.4 joint TCI for sDCI mTRP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The joint TCI for sDCI mTRP can’t be tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5.13.0.4(new), 7.5.13.4.1(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

7.5.13.0.3	Minimum conformance requirements for MAC-CE based uplink TCI state switch delay for unified TCI for 2 known TCI states

The requirements in this clause shall apply for UL TCI state switch using separate UL TCI state or joint TCI state of unified TCI state switch framework and having two indicated TCI-States.

In case of joint TCI state based dual TCI state switch, UE is not expected to transmit on UL before UE completes the DL and UL TCI state switch of both TRPs.

For separate UL TCI state switch or joint TCI state switch for PUCCH or PUSCH, or semi-persistent/aperiodic/periodic SRS, when beamCorrespondenceWithoutUL-BeamSweeping is set to 1, upon receiving PDSCH carrying MAC-CE activation command in slot n on serving cell,

If both target TCI states are known,

-	The UE shall be able to transmit uplink signal with the target TCI state(s) in the slot n+THARQ + 3Nslotsubframe,µ + max{NM1* (Tfirst_target-PL-RS1 + 4*Ttarget_PL-RS1 + 2ms), NM2* (Tfirst_target-PL-RS2 + 4*Ttarget_PL-RS 2+ 2ms) }/ NR slot length.

If both target TCI states are unknown,

-	The UE shall be able to transmit uplink signal with the target TCI state(s) in the slot n+THARQ + 3Nslotsubframe,µ + max{TL1-RSRP1 + Tfirst_target-PL-RS1 + 4*Ttarget_PL-RS1 + 2ms, TL1-RSRP2 + Tfirst_target-PL-RS2 + 4*Ttarget_PL-RS 2+ 2ms }/ NR slot length.

If one of the target TCI states is unknown and the other TCI state is known,

-	The UE shall be able to transmit uplink signal with the target TCI state(s) in the slot n+THARQ + 3Nslotsubframe,µ + max{ TL1-RSRP1 + Tfirst_target-PL-RS1 + 4*Ttarget_PL-RS1 + 2ms, NM2* (Tfirst_target-PL-RS2 + 4*Ttarget_PL-RS 2+ 2ms) }/ NR slot length.

Where,

-	THARQ (in slot) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [8].

-	NM1 = 1, if the target PL-RS of the first TCI state is not maintained by the UE, 0 otherwise.

-	NM2 = 1, if the target PL-RS of the second first TCI is not maintained by the UE, 0 otherwise.

In FR2, in case the target PL-RS associated with or included in the target UL or joint TCI state is SSB, the requirements in this clause shall apply when this target PL-RS is maintained by the UE.

-	PL-RS is maintained provided:

-	the target PL-RS is associated with or included in the UL or joint TCI states in the active TCI list for PUSCH/PUCCH/SRS transmissions

-	There are no more than 4 different RS activated as PL-RS per serving cell among all active UL TCI states (UL or joint TCI state) for PUSCH/PUCCH/SRS transmissions

-	The target pathloss reference signal remains detectable during TCI state switching period

-	SNR of the target pathloss reference signal≥-3dB

-	The associated SSBs with the target pathloss reference signal remain detectable during the TCI state switching period.

-	SNR of the associated SSB ≥-3dB

-	Tfirst_target-PL-RS1 is time to first pathloss RS transmission of the first TCI state after L1-RSRP measurement when first target TCI state is unknown.

-	Tfirst_target-PL-RS2 is time to first pathloss RS transmission of the second TCI state after L1-RSRP measurement when second target TCI state is unknown.

-	Tfirst_target-PL-RS1 is time to first pathloss RS transmission of the first TCI state after MAC CE command is decoded by the UE when the first target TCI state is known.

-	Tfirst_target-PL-RS2 is time to first pathloss RS transmission of the second TCI state after MAC CE command is decoded by the UE when the first seond TCI state is known.

-	Ttarget_PL-RS1 is the periodicity of the target pathloss reference signal of the first TCI state which would SSB or NZP CSI-RS when PL-RS is associated with serving cell.

-	Ttarget_PL-RS2 is the periodicity of the target pathloss reference signal of the second TCI state which would SSB or NZP CSI-RS when PL-RS is associated with serving cell.

-	T L1-RSRP1 and T L1-RSRP2 are the time for Rx beam refinement in FR2 for the first and second TCI state, defined as

-	TL1-RSPR_Measurement_Period_SSB for SSB as specified in TS 38.133 [6] clause 9.5.4.1,

-	with the assumption of M=1

-	with TReport = 0

-	TL1-RSRP_Measurement_Period_CSI-RS for CSI-RS as specified in TS 38.133 [6] clause 9.5.4.2

-	CSI-RS based L1-RSRP measurement only apply for TCI state switch when source RS is associated with serving cell

-	configured with higher layer parameter repetition set to ON

-	with the assumption of M=1 for periodic CSI-RS

-	for aperiodic CSI-RS if number of resources in resource set at least equal to MaxNumberRxBeam

-	with TReport = 0

The requirements specified in this clause are applicable if no more than 4 different RSs are activated as PL-RS per serving cell among all active UL (or joint) TCI states.

Note: In FR2, the requirements when the target PL-RS is not maintained in this clause apply only when PL-RS are not overlapped or adjacent to the PL-RS of the other TRP.

The normative reference for this requirement is TS 38.133 [6] clause 8.23.3.

7.5.13.0.4	Minimum conformance requirements for active downlink TCI state switching delay for unified TCI for single-DCI mTRP

The requirements in this clause shall apply for DL TCI state switch using separate DL TCI state or joint TCI state of unified TCI state switch framework and having two indicated TCI states.

In case of joint TCI state based dual TCI state switch, if the target PL-RS is not maintained, UE is not expected to receive on DL based on the target TCI state before UE completes the DL and UL TCI state switch of both TRPs. In case of single TCI state switch requirements specified in TS 38.133 [6] clause 8.15.3 apply.

If both the target TCI states are known, upon receiving PDSCH carrying MAC-CE activation command in slot n, UE shall be able to receive UE-dedicated PDCCH/PDSCH with target TCI state of the serving cell on which TCI state switch occurs at the first slot that is after slot n+ THARQ + 3Nslotsubframe,µ+ max{TOk1*(Tfirst-SSB1 + AD1*TSSB1  + TSSB-proc), TOk2*(Tfirst-SSB2 + AD2*TSSB2 + TSSB-proc)} / NR slot length.

Among the dual target TCI states indicated for switch, if the one of the  target TCI state is unknown, upon receiving PDSCH carrying MAC-CE activation command in slot n, UE shall be able to receive UE-dedicated PDCCH/PDSCH with target TCI state of the serving cell on which TCI state switch occurs at the first slot that is after slot THARQ + 3Nslotsubframe,µ + TL1-RSRP1 + max {TOuk1*(Tfirst-SSB1+ AD1*TSSB1 + TSSB-proc), TOk2*(Tfirst-SSB2 + TSSB-proc)} / NR slot length.

If the both of the dual target TCI state are unknown, upon receiving PDSCH carrying MAC-CE activation command in slot n, UE shall be able to receive UE-dedicated PDCCH/PDSCH with target TCI state of the serving cell on which TCI state switch occurs at the first slot that is after slot n+ THARQ +3Nslotsubframe,µ + max{TL1-RSRP1 +TOuk1*(Tfirst-SSB1+ TSSB-proc), TL1-RSRP2 +TOk2*(Tfirst-SSB2+ TSSB-proc)} / NR slot length. The UE shall be able to receive UE-dedicated PDCCH/PDSCH with the old TCI state until slot n+ THARQ + 3Nslotsubframe,µ .

Where

-	THARQ (in slot) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3];

-	Tfirst-SSB1 is time to first SSB transmission of first TCI states of the pair of TCI states after MAC CE command is decoded by the UE; The SSB shall be the QCL-TypeA or QCL-TypeC to target TCI state; Tfirst-SSB2 is time to first SSB transmission of second TCI states of the pair of TCI states after MAC CE command is decoded by the UE; The SSB shall be the QCL-TypeA or QCL-TypeC to target TCI state

-	TSSB-proc = 2 ms;

-	TOk1 = 1 if first target TCI state is not in the active TCI state list for PDSCH/PDCCH, 0 otherwise.

-	TOk2 = 1 if second target TCI state is not in the active TCI state list for PDSCH/PDCCH, 0 otherwise.

-	AD1 = 1 if SSBs are adjacent in FR2 and TSSB1 =TSSB2 ; 0 otherwise

-	AD2 = 1 if SSBs are adjacent in FR2 and TSSB2 = TSSB1 ; 0 otherwise

-	T L1-RSRP1 = 0 for the first TCI state and T L1-RSRP2 = 0 for the second TCI state in FR1 or when the TCI state switching not involving QCL-TypeD in FR2. Otherwise,

-	T L1-RSRP1 for the first TCI state and T L1-RSRP2 for the second TCI state are the time for Rx beam refinement in FR2, defined as

-	TL1-RSRP_Measurement_Period_SSB for SSB as specified in clause 9.5.4.1,

-	with the assumption of M=1

-	with TReport = 0

-	TL1-RSRP_Measurement_Period_CSI-RS for CSI-RS as specified in clause 9.5.4.2

-	CSI-RS based L1-RSRP measurement only apply for TCI state switch when source RS is associated with serving cell

-	configured with higher layer parameter repetition set to ON

-	with the assumption of M=1 for periodic CSI-RS

-	for aperiodic CSI-RS if number of resources in resource set at least equal to MaxNumberRxBeam

-	with TReport = 0

-	TOuk1 for the first TCI state and TOuk2 for the second TCI state  TOuk1 = 1 and TOuk2 = 1 for CSI-RS based L1-RSRP measurement, and 0 for SSB based L1-RSRP measurement when TCI state switching involves QCL-TypeD

-	TOuk1 = 1 and TOuk2 = 1when TCI state switching involves other QCL types only

-	Tfirst-SSB1 is time to first SSB transmission of first TCI states of the pair of TCI states after L1-RSRP measurement when TCI state switching involves QCL-TypeD; Tfirst-SSB2 is time to second SSB transmission of first TCI states of the pair of TCI states after L1-RSRP measurement when TCI state switching involves QCL-TypeD;

-	Tfirst-SSB1 is time to first SSB transmission of first TCI states of the pair of TCI states after MAC CE command is decoded by the UE for other QCL types; Tfirst-SSB2 is time to second SSB transmission of first TCI states of the pair of TCI states after MAC CE command is decoded by the UE for other QCL types;

-	The SSB shall be the QCL-TypeA or QCL-TypeC to target TCI state

For FR2 both of the dual target TCI state are unknown when SSB are adjacent, longer delay is expected.

The normative reference for this requirement is TS 38.133 [6] clause 8.21.3.

<Unchanged Text Skipped>

During T2, UE shall send L1-RSRP report with results for both SSB0 and SSB1.

After receiving MAC-CE command in slot n, UE shall:

-	be able to continue to receive on DL TCI state 0 till slot n+ THARQ +3Nslotsubframe,µ

-	be able to start receiving on DL TCI state 1 after n+ THARQ + (5ms + Tfirst-SSB) / NR slot length

The rate of correct events observed during repeated tests shall be at least 90%.

7.5.13.4	sDCI MAC-CE based joint TCI state switching

7.5.13.4.1	NR SA FR2 PCell dual downlink and uplink TCI state switch in sDCI for known case

Editor's Note: This test case is incomplete. Following aspects are either missing or TBD:

- MU/TT analysis is missing.

- Message content is imcomplete.

7.5.13.4.1.1	Test purpose

To verify both active downlink and uplink TCI state switch delay requirement defined in defined in 38.133 [6] clause 8.21 and 8.23, respectively, by using joint TCI state of unified TCI state switch framework when tci-JointTCI-UpdateSingleActiveTCI-PerCC-r18 is supported and when tci-SeparateTCI-UpdateSingleActiveTCI-PerCC-r18 is not supported.

7.5.13.4.1.2	Test applicability

This test applies to all types of NR UEs from Release 18 onwards supporting unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP.

7.5.13.4.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.5.13.0.3 and 7.5.13.0.4.

The normative reference for this requirement is TS 38.133 [6] clause A.7.5.13.4.1.

7.5.13.4.1.4	Test description

7.5.13.4.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.5.13.4.1.4.1-1.

Table 7.5.13.4.1.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Configure the test equipment and the DUT according to the parameters in Table 7.5.13.4.1.4.1-2 and Table 7.5.13.4.1.4.1-3.

Table 7.5.13.4.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.5-1 and TS 38.508-1 [14] clause 7.2.3 for NR. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.5.13.4.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.3.1.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 7.5.13.4.1.4.1-3: General test parameters for TCI state switch

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| L1-RSRP reporting period | slot | 320 | Periodic L1-RSRP reporting configured |
| L1-RSRP measured RS |  | SSB0 and SSB2 of TRP0, SSB1 and SSB3 of TRP1 | L1-RSRP measurements of SSB0, SSB1 SSB2 and SSB3. |
| Number of RS for L1-RSRP reporting |  | 4 | SSB0, SSB1, SSB2 and SSB3 in Joint TCI state 0, 1, 2, and 3. |
| Cell 2 timing offset to cell1 | us | <CP |  |
| T1 | s | 0.2 |  |
| T2 | s | 1 | UE is required to activate dual joint TCI states (TCI state 2 and TCI state 3). |

1.	The general test parameter settings are set up according to Table 7.5.13.4.1.4.1-3.

2.	Message contents are defined in clause 7.5.13.4.1.4.3.

2. Single Cell is used, which is NR FR2 PCell containing two TRPs (i.e., TRP 0 and TRP 1). The connection setup is done according to the settings in Annex C.1.2 and C.1.3.

7.5.13.4.1.4.2	Test procedure

Before the test starts,

-	UE is connected to Cell 1 (PCell) on radio channel 1 (PCC).

-	PDCCHs indicating new transmissions shall be sent continuously on PCell to ensure that the UE would have ACK/NACK sending.

-	UE is provided with dl-OrJoint-TCIStateList-r17 and UE’s higher layer signalling unifiedTCI-StateType-r17 in IE MIMOParam-r17 is set to joint.

-	applyIndicatedTCI-State-r18 is set as {first} for TRP0 (CORESET index p associated with Joint TCI state 0 and Joint TCI state 2) and as {second} for TRP1 (CORESET index q associated with Joint TCI state 1 and Joint TCI state 3).

-	tci-SelectionPresentInDCI-r18 is configured in the BWP configuration, i.e. TCI state for the PDSCH is indicated by DCI format 1_1 and PDSCHs on two TRPs are scheduled in TDM manner.

-	UE is configured with two joint TCI states (TCI state 0 and TCI state 2) for TRP0 and two joint TCI states (TCI state 1 and TCI state 3) for TRP1. QCL info to Joint TCI state 0,1,2 and 3 are provided by SSB0, SSB1, SSB2 and SSB3, respectively.

-	UE is indicated in TCI state 0 and TCI state 1 as the active TCI state for TRP0 and TRP1.

The test consists of two time periods, T1 and T2. During T1, SSB0 in joint TCI state 0 and SSB1 in joint TCI state 1 are transmitted. At the beginning of T2, SSB2 in joint TCI state 2 and SSB3 in joint TCI state 3 start transmitting. The UE is configured to provide periodic L1-RSRP reports. In slot n which is within 1280 ms after the slot in which UE provides L1-RSRP report with results for SSB2 and SSB3 in joint TCI state 2 and joint TCI state 3, UE receives a MAC-CE command indicating a switch to dual joint TCI state 2 and TCI state 3 for two TRPs.

The test equipment verifies that UE can be scheduled by two TRPs on joint TCI state 0 and joint TCI state 1 till slot n+ THARQ +3Nslotsubframe,µ. The test equipment also verifies the TCI state switch time for two TRPs by scheduling the UE on joint TCI state 2 and joint TCI state 3 after slot n+THARQ + 3Nslotsubframe,µ + max{NM1* (Tfirst_target-PL-RS1 + 4*Ttarget_PL-RS1 + 2 ms), NM2* (Tfirst_target-PL-RS2 + 4*Ttarget_PL-RS 2+ 2 ms) }/ NR slot length.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.4.

2.	Set the parameters according to T1 in Table 7.5.13.4.1.5-1. Propagation conditions are set according to clause C.2.2. T1 starts. During T1 only SSB0 and SSB1 are transmitted.

3.	When T1 expires the SS starts transmitting SSB2 and SSB3. T2 starts.

4.	The SS transmits an RRCReconfiguration message to configure periodic L1-RSRP reporting.

5	The UE transmits an RRCReconfigurationComplete message

6.	The SS sends a MAC-CE to indicate a switch to dual joint TCI state 2 and TCI state 3 for two TRPs in slot n which is within 1280ms of UE providing L1-RSRP report with results for both SSB2 and SSB3.

7.	If the SS:

a)	receives ACK/NACK on each UL transmission occasion scheduled on joint TCI state 2 and joint TCI state 3 until slot n+THARQ+3ms, and

b)	receives ACK/NACK on each UL transmission occasion scheduled on joint TCI state 2 and joint TCI state 3 after slot n+THARQ + 3Nslotsubframe,µ + max{NM1* (Tfirst_target-PL-RS1 + 4*Ttarget_PL-RS1 + 2 ms), NM2* (Tfirst_target-PL-RS2 + 4*Ttarget_PL-RS 2+ 2 ms) }/ NR slot length, where NM1=1, NM2=1.

the number of successful tests is increased by one, otherwise the number of failed tests is increased by one.

8.	When T2 expires the SS shall sends a MAC-CE to indicate switch to joint TCI state 0 and TCI state 1.

9.	Wait 1s for the UE to switch to joint TCI state 0 and TCI state 1. If the SS receives ACK/NACK on each UL transmission occasion scheduled on joint TCI state 0 and TCI state 1 continue to step 11. Otherwise continue to step 10.

10.	Switch the UE on and off. Ensure the UE is in RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.4.

11.	Repeat steps 2-10 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

7.5.13.4.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 7.5.13.4.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.6-2 with conditions PERIODIC and SS-RSRP Table H.3.6-3 with condition SSB |

7.5.13.4.1.5	Test requirement

Tables 7.5.13.4.1.5-1 and 7.5.13.4.1.5-2 defines the primary level settings including test tolerances.

Table 7.5.13.4.1.5-1: NR Cell specific test parameters for dual TCI state switch

| Parameter | Unit | Cell 1 |
| --- | --- | --- |
| Frequency Range |  | FR2 |
| Duplex mode |  | TDD |
| TDD configuration |  | TDDConf.3.1 |
| BWchannel |  | 100 MHz: NPRB,c = 66 |
| Data PRBs allocated |  | 66 |
| Initial BWP Configuration |  | DLBWP.0.2 ULBWP.0.2 |
| Dedicated BWP Configuration |  | DLBWP.1.1 ULBWP.1.1 |
| PDSCH Reference measurement channel |  | SR.3. 2 TDD |
| RMSI CORESET parameters |  | CR.3.1 TDD |
| Dedicated CORESET parameters (CORESET index p) |  | CCR.3.1 TDD |
| Dedicated CORESET parameters (CORESET index q) |  | CCR.3.2 TDD |
| OCNG Patterns |  | OP.5 defined in A.3.2.1 |
| SSB Configuration |  | SSB.17 FR2 |
| SMTC Configuration |  | SMTC.1 |
| Joint TCI State 0 |  | DLorJoint TCI.State.2 |
| Joint TCI State 1 |  | DLorJoint TCI.State.6 |
| Joint TCI State 2 |  | DLorJoint TCI.State.7 |
| Joint TCI State 3 |  | DLorJoint TCI.State.8 |
| TRS Configuration |  | TRS.2.1 TDD for DLorJoint TCI.State.2 TRS.2.4 TDD for DLorJoint TCI.State.6 TRS.2.5 TDD for DLorJoint TCI.State.7 TRS.2.6 TDD for DLorJoint TCI.State.8 |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |
| EPRE ratio of PSS to SSS | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |
| Propagation Condition |  | AWGN |
| NOTE 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |

Table 7.5.13.4.1.5-2: OTA related test parameters for dual TCI state switch

| Parameter | Units | TRP0 |  |  |  | TRP1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SSB0 |  | SSB2 |  | SSB1 |  | SSB3 |  |
|  |  | T1 | T2 | T1 | T2 | T1 | T2 | T1 | T2 |
| Angle of arrival configuration |  | Setup [TBD] according to clause A.3.XX |  |  |  |  |  |  |  |
|  |  | AoA1 |  | AoA2 |  | AoA2 |  | AoA3 |  |
| Assumption for UE beams Note 6 |  | Rough |  |  |  |  |  |  |  |
| Ês | dBm/SCS | -80.6+TT | -80.6+TT | -Infinity | -80.6+TT | -80.6+TT | -80.6+TT | -Infinity | -80.6+TT |
| SS B_RP Note 2 | dBm/ SCS | -80.6+TT | -80.6+TT | -Infinity | -80.6+TT | -80.6+TT | -80.6+TT | -Infinity | -80.6+TT |
| BB Note 7 | dB | 8.3+TT | 8.3+TT | -Infinity | 8.3+TT | 8.3+TT | 8.3+TT | -Infinity | 8.3+TT |
| IoNote2 | dBm/95.04 MHz Note4 | -56.0+TT | -56.0+TT | - Infinity | -56.0+TT | -56.0+TT | -56.0+TT | - Infinity | -56.0+TT |
| NOTE 1: SS B_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 2: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone NOTE 3: As observed with 0 dBi gain antenna at the center of the quiet zone. NOTE 4: Information about types of UE beam is given in clause B.2.1.3 and does not limit UE implementation or test system implementation. NOTE 5: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1 dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |  |  |  |  |

During T2, the test verifies that UE can be scheduled by two TRPs on joint TCI state 2 and joint TCI state 3.

After receiving MAC-CE command in slot n, UE shall:

-	be able to continue to receive and transmit with joint TCI state 0 and joint TCI state 1 till slot n+ THARQ +3Nslotsubframe,µ

-	be able to start receiving and transmitting with joint TCI state 2 and joint TCI state 3 after slot n+THARQ + 3Nslotsubframe,µ + max{NM1* (Tfirst_target-PL-RS1 + 4*Ttarget_PL-RS1 + 2 ms), NM2* (Tfirst_target-PL-RS2 + 4*Ttarget_PL-RS 2+ 2 ms) }/ NR slot length.

-	where NM1=1, NM2=1.

The rate of correct events observed during repeated tests shall be at least 90 %.

<End of Changes>