3GPP TSG-RAN5 Meeting #107	R5-252460

# St Julian’s, Malta, 19th May 2025 – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1606 | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new LTE IMS data channel test case 12.34 - MT Voice call without precondition and BDC success |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_IMS_dataCH-UEConTest |  |  |  |  |  | Date: |  |  | 2025-04-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This new IMS data channel (IMS DC) test case is added to test: SS send SDP offer with IMS DC and MT UE reply SDP answer with IMS DC. The UE can establish voice call without precondition + IMS DC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new LTE IMS data channel test case 12.34 IMS Data Channel / DCMTSI MT Voice call / Bootstrap Data Channel success / without preconditions at both originating UE and terminating UE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NG_RTC_IMS_dataCH-UEConTest is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.34 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 34.229-2 CR 0369 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

12.34	IMS Data Channel / DCMTSI MT Voice call / Bootstrap Data Channel success / without preconditions at both originating UE and terminating UE

12.34.1	Definition

Test to verify that the UE correctly performs IMS mobile terminated voice call setup without preconditions and IMS Bootstrap data channel setup. This process is described in 3GPP TS 24.186 [159].

12.34.2	Conformance requirement

[TS 24.186 clause 9.3.3.1.2]

If the terminating UE determines that the UE and the network supports the IMS data channel, on the reception of SIP initial INVITE request, the terminating UE shall include the media feature tags defined in IETF RFC 5688 [5] for supported streaming media type with +sip.app-subtype="webrtc-datachannel" as specified in 3GPP TS 26.114 [4] in the Contact header field of SIP 18x and 2xx responses to the SIP INVITE request.

If the terminating UE receives the initial INVITE request with an SDP offer which includes the data channel media descriptions, i.e. the "m=" line containing the media set to "application", the UDP port number, the proto value set to "UDP/DTLS/SCTP" and the fmt value set to "webrtc-datachannel" and with associated "dcmap" attribute lines containing a subprotocol parameter set to "http" and any "stream-id" parameter set to values 0, 10, 100 or 110, and the terminating UE:

…

2)	is configured with IMS_DC_configuration node as specified in 3GPP TS 24.275 [11] and the DC_allowed leaf indicates that IMS data channel:

a)	is allowed and if the terminating UE accepts the offered bootstrap data channel(s), it shall generate the SDP answer based on the 3GPP TS 26.114 [4] and IETF RFC 8864 [14]; or

…

and the terminating UE shall return a 18x or 2xx response to the INVITE request with the above generated the SDP answer.

Reference(s)

3GPP TS 24.186 [159], clauses 9.3.3.1.2.

12.34.3	Test purpose

1)	To verify that UE setups IMS Bootstrap data channel successfully during MT Voice call establishment; and

2)	To verify that UE includes media feature tag +sip.app-subtype="webrtc-datachannel" when it receive INVITE message.

3)	To verify that UE generates the correct SDP answer for IMS data channel media negotiation when it receives the INVITE request with an SDP offer including the IMS data channel media descriptions

12.34.4	Method of test

Initial conditions

UE contains either SIM application (GIBA), ISIM and USIM applications or only USIM application on UICC. UE has discovered P-CSCF and registered to IMS services, by executing the generic test procedure in Annex C.2 or C.2a (GIBA only) up to the last step.

SS is configured with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipped into the UE. SS has performed AKAv1-MD5 authentication with the UE and accepted the registration (IMS security). UE is configured to not use the precondition mechanism. UE is configured to use IMS data channel.

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1-11)	UE executes the procedures described in TS 36.508 [94] table 4.5A.7.3-1 steps 1 to26. The qos flow for IMS bootstrap data channel (specified in TS 36.508[94] table 4.5A.32.3-1) is established after 200OK for INVITE.

Expected sequence

NOTE:	Only the IMS procedure relevant to the test purpose is described below.

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1-6 |  |  | Steps 1-6 defined in annex C.11 | MTSI MT speech call. Referred from 36.508 [94] table 4.5A.7.3-1 for a UE with E-UTRA support. The qos flow for IMS bootstrap data channel is established after 200OK for INVITE message, and the test procedure is referred from 36.508 [94] table 4.5A.32.3-1. |
| 7 |  |  | Step 9 defined in annex C.11 | The UE responds to INVITE with 180 Ringing |
| 8-9 |  |  | Step 12-13 defined in annex C.11 | SS responds to INVITE with a 200 OK final response and SS acknowledges the receipt of 200 OK for INVITE. |
| 10-11 |  |  | Step 14-15 defined in annex C.11 | The SS sends BYE to release the call and UE sends 200 OK for the BYE request and ends the call. |

Specific Message Contents

INVITE (Step 1)

Use the default message “INVITE for MT Call” in annex A.2.9 with the following exceptions:

| Header/param | Cond | Value/remark |
| --- | --- | --- |
| Contact feature-param |  | Feature-param is same as Step 2 of C.21f except adding one tag as follow: +sip.app-subtype="webrtc-datachannel" |
| Message-body |  | Session description: - v=0 - o=- 1111111111 1111111111 IN (addrtype) (unicast-address for SS) - s=- - c=IN (addrtype) (connection-address for SS) - b=AS:37 Time description: - t=0 0 Media description for Voice: - m=audio (transport port) RTP/AVP 97 98 99 100 - b=AS:37 - b=RS:0 - b=RR:2000 Attributes for media for Voice: - a=rtpmap:97 AMR-WB/16000/1 - a=fmtp:97 mode-change-capability=2; max-red=220 - a=rtpmap: 98 telephone-event/16000 - a=fmtp: 98 0-15 - a=rtpmap:99 AMR/8000/1 - a=fmtp:99 mode-change-capability=2; max-red=220 - a=rtpmap: 100 telephone-event/8000 - a=fmtp: 100 0-15 - a=ptime:20 - a=maxptime:240 Media description for IMS data channel: - m=application (transport port) UDP/DTLS/SCTP webrtc-datachannel - b=AS:500 Attributes for media for IMS data channel: - a=max-message-size:64000 - a=sctp-port:(sctp-port-value) [Note 2] - a=fingerprint:SHA-256 (fingerprint) [Note 3] - a=setup:actpass - a=tls-id:(tls-id-value) - a=dcmap:0 subprotocol="http” Note 1: Transport port is the port number of the SS. Note 2: The sctp-port-value is the SCTP port of the SS. Note 3: Fingerprint is calculated by SS. Note 4: The tls-id-value is random number generated by SS. |

183 Session Progress (Step 4)

Use the default message “183 Session Progress” in annex A.2.3 with the following exceptions:

| Header/param | Cond | Value/remark |
| --- | --- | --- |
| Contact feature-param |  | Feature-param is same as annex A.2.3 except adding one tag as follow: +sip.app-subtype="webrtc-datachannel" |
| Message-body |  | The following SDP types and values shall be present. Session description: - v=0 - o=(user-name) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) - s=(session name) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) Time description: - t=0 0 Media description for voice: - m=audio (transport port) RTP/AVP (fmt) [Note 3] - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) - b=RS: (bandwidth-value) - b=RR: (bandwidth-value) Attributes for media for voice: - a=rtpmap:(payload type) AMR-WB/16000 [Note 2] - a=fmtp:(format) [Note 2, 3] Media description for IMS data channel: [Note 4] - m=application (transport port) UDP/DTLS/SCTP webrtc-datachannel Attributes for media for IMS data channel: [Note 4] - a=max-message-size: (max-message-size-value) [Note 5] - a=sctp-port:(sctp-port-value) [Note 6] - a=setup: active - a=fingerprint: (hash-func) (fingerprint) [Note 7] - a=tls-id: (tls-id-value) - a=dcmap:0 subprotocol="http" Note 1: At least one "c=" field shall be present. Note 2: The values for fmt, payload type and format are not checked Note 3: Parameters for the AMR codec are not checked Note4: The media description and attributes for media for IMS data channel shall present in 183 Session Progress or 200 OK for INVITE. Note 5: Optional attribute. If the SDP”max-message-size” attribute is not present the default value is 64K. Note 6: SCTP port range is between 0 and 65535(both included). Leading zeroes must not be used. Note7: SHA-1, SHA-224, SHA-256, SHA-384, or SHA-512 is used as hash-func. |

200OK for INVITE (Step 8)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1 with the following exceptions:

| Header/param | Cond | Value/remark |
| --- | --- | --- |
| Contact feature-param |  | Feature-param is same as annex A.3.1 except adding one tag as follow: +sip.app-subtype="webrtc-datachannel" |
| Message-body | A2 A1 | SDP body is as follow: Session description: Same as 183 Session Progress (Step 4) Time description: Same as 183 Session Progress (Step 4) Media description for voice: Same as 183 Session Progress (Step 4) Attributes for media for voice: Same as 183 Session Progress (Step 4) Media description for IMS data channel: - m=application (transport port) UDP/DTLS/SCTP webrtc-datachannel Attributes for media for IMS data channel: - a=max-message-size: (max-message-size-value) [Note 1] - a=sctp-port:(sctp-port-value) [Note 2] - a=setup: active - a=fingerprint: (hash-func) (fingerprint) [Note 3] - a=tls-id: (tls-id-value) - a=dcmap:0 subprotocol="http" Note 1: Optional attribute. If the SDP”max-message-size” attribute is not present the default value is 64K. Note 2: SCTP port range is between 0 and 65535(both included). Leading zeroes must not be used. Note 3: SHA-1, SHA-224, SHA-256, SHA-384, or SHA-512 is used as hash-func. SDP body is empty. |

| Condition | Explanation |
| --- | --- |
| A1 | 183 Session Progress includes SDP offer containing media lines both for voice and IMS data channel in Step4. |
| A2 | 183 Session Progress does not includes SDP offer containing media lines for IMS data channel in Step4. |

<End of modified section 1>