3GPP TSG-RAN5 Meeting #107 	R5-252505

# St Julian's, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 1938 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to FR1 inter-band EN-DC REFSENS for Rel-15 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In 7.3B.2.3.4.2.1-6, - Information for power class is missing. - Modulation is incorrect at some test points. - Some test points are defined for both DC_XA_n77A and DC_XA_n78A, which makes confusion for applicability of each test point. - Test points for DC_18A_n77A and DC_19A_n77A have undefined notes X and Y. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In Table 7.3B.2.3.4.2.1-6: - Power class was clarified as below. - (PC3) or (PC2 and PC3) was added to title rows of each EN-DC configuration. - Notes 19 (PC3) was added to each test ID. - Modulation was corrected as below. - Missing UL/DL modulation was added. - Unnecessary UL/DL modulation was removed. - UL modulation was corrected from CP-OFDM to DFT-s-OFDM. - Test points for both DC_XA_n77A and DC_XA_n78A were separated to each EN-DC configuration. - Notes in DC_18A_n77A and DC_18A_n78A were corrected as below according to R5-245834 which has been agreed but not implemented correctly. - Note X was corrected to Note 17. - Note Y was corrected to Note 18. Editorial correction: - Test points for DC_3A_n77A, DC_5A_n78A, DC_26A_n41A were renumbered. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 7.3B.2.3 wil be tested under incorrect modulation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3B.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

7.3B.2.3	Reference sensitivity for Inter-band EN-DC within FR1 (2 CCs)

Editor's note:	The minimum conformance requirements for the following test points are missing in 7.3B.2.0.3.

- DC_40A_n78A (REFSENS_ENDC_2) in Table 7.3B.2.3.4.2.1-6

- DC_48A_n66A (REFSENS_ENDC_1) in Table 7.3B.2.3.4.2.1-6

7.3B.2.3.1	Test purpose

Same as in clause 7.3B.2.1.1.

7.3B.2.3.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting inter-band EN-DC within FR1 with 2 DL CCs.

7.3B.2.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.3B.2.0.

LTE anchor agnostic approach is not applied.

7.3B.2.3.4	Test description

7.3B.2.3.4.1	Void

7.3B.2.3.4.2	Test description

7.3B.2.3.4.2.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths based on EN-DC operating bands specified in clause 5.5B, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each EN-DC configuration, and are shown in Table 7.3B.2.3.4.2.1-0 to Table 7.3B.2.3.4.2.1-6.

The details of the uplink and downlink reference measurement channels (RMCs) are specified in TS 36.521-1 [10] Annex A.2 and A.3 for E-UTRA RMC, and in TS 38.521-1 [8] Annex A.2 and A.3 for NR RMC respectively. The details of the OCNG patterns used are specified in TS 36.521-1 [10] Annex A.5 and in TS 38.521-1 [8] Annex A.5 for E-UTRA CG and NR CG, respectively. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG, respectively.

For configurations without any reference sensitivity exception, in a FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. For configurations with reference sensitivity exception, in an E-UTRA band or FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected.

<<Unchanged tables skipped>>

Table 7.3B.2.3.4.2.1-6: Test Configuration Table for EN-DC configurations affected by Reference sensitivity exceptions (two bands)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  |  |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1, E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. |  |  |  |  |  |  |  |
| Test EN-DC channel bandwidth as specified in TS 36.508 [11] clause 4.3.1 and TS 38.508-1 [6] clause 4.3.1. |  |  |  |  |  |  |  |  |  |  |  |  | Refer to "NR NRB"and "E-UTRA NRB " columns |  |  |  |  |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  |  |  |  |  |  |  |  |  |  | Lowest supported SCS |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  |  |  |  |  | NS_01 by default Unless given by Table 7.3.3-3 in TS 36.521-1 [10] for the E-UTRA band and Table 7.3.2.3-4 in TS 38.521-1 [8] for the NR band. |  |  |  |  |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | PCC – E-UTRA |  |  |  |  |  |  |  |  |  |  | SCG -NR |  |  |  |  |  |  |  |  |
|  | Band | Range |  |  |  |  | NRB |  |  |  |  | Band |  |  | Range |  | NRB |  |  |  |
|  | UL MOD | DL MOD |  |  |  |  | CH BW |  |  | DLalloc / UL alloc |  | UL MOD |  |  | DL MOD |  | UL/DL Ch BW | DLalloc / UL alloc |  |  |
| Test Settings for a DC_1A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12,11 | 1 | Mid |  |  |  |  |  |  |  |  |  | n3 |  |  | Mid |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_3 |  | N/A |  |  | CP-OFDM QPSK |  | 40 MHz | All RBs / 0 |  |  |
| 24 | 1 | UL 1950 / DL 2140 |  |  |  |  |  |  |  |  |  | n3 |  |  | UL 1760 / DL 1855 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 36 | 1 | Low |  |  |  |  |  |  |  |  |  | n3 |  |  | High |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_3 |  | N/A |  |  | CP-OFDM QPSK |  | 40 MHz | All RBs / 0 |  |  |
| Test Settings for a DC_1A_n8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 1 | UL 1965 / DL 2155 |  |  |  |  |  |  |  |  |  | n8 |  |  | UL 887.5 / DL 932.5 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_1A_n28A Configuration (PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 1 | UL 1950 / DL 2140 |  |  |  |  |  |  |  |  |  | n28 |  |  | UL 713.3 / DL 768.3 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_1 |  |  |
| 28 | 1 | UL 1970 / DL 2160 |  |  |  |  |  |  |  |  |  | n28 |  |  | UL 708 / DL 763 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_1 |  |  |
| Test Settings for a DC_1A_n41A Configuration (PC2 and PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 1 | High |  |  |  |  |  |  |  |  |  | n41 |  |  | Low |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_3 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 26 | 1 | High |  |  |  |  |  |  |  |  |  | n41 |  |  | Low |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 50 MHz | All RBs / REFSENS_ENDC_3 |  |  |
| Test Settings for a DC_1A_n77A Configuration (PC2 and PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 1 | UL 1950/ DL 2140 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3900 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 23 | 1 | UL 1950/ DL 2140 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3870 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 20 MHz | All RBs / 0 |  |  |
| 34 | 1 | UL 1950/ DL 2140 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 4090.005 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 44 | 1 | UL 1950/ DL 2140 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3709.995 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_1A_n78A Configuration (PC2 and PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 1 | UL 1950 / DL 2140 |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL 3709.995 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_2A_n41A Configuration (PC2 and PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 2 | DL 1987.5 |  |  |  |  |  |  |  |  |  | n41 |  |  | UL/DL 2546 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_3 |  |  |
| Test Settings for a DC_2A_n66A Configuration (PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 2 | UL 1855 / DL 1935 |  |  |  |  |  |  |  |  |  | n66 |  |  | DL 2175 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 24 | 2 | UL 1883.3 /DL1963.3 |  |  |  |  |  |  |  |  |  | n66 |  |  | DL 2150 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_2A_n71A Configuration (PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 2 | DL 1980 MHZ |  |  |  |  |  |  |  |  |  | n71 |  |  | UL 665.5 MHz |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_1 |  |  |
| 23 | 2 | DL1980 MHZ |  |  |  |  |  |  |  |  |  | n71 |  |  | UL 673 MHz |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_1 |  |  |
| 35 | 2 | UL 1881 MHZ |  |  |  |  |  |  |  |  |  | n71 |  |  | Low |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_2 |  | N/A |  |  | CP-OFDM QPSK |  | 20 MHz | All RBs / 0 |  |  |
| Test Settings for a DC_2A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 2 | UL 1860 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3720 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 23 | 2 | UL 1860 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3690.005 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 20 MHz | All RBs / 0 |  |  |
| 38 | 2 | UL 1860 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3850.005 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / 0 |  |  |
| 45 | 2 | DL Mid |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3920.01 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 4a9 | 2 | DL Mid |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL Mid |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 54 | 2 | UL 1855/DL 1935 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3790.005 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 64 | 2 | UL 1900/DL 1980 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3720 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 74 | 2 | UL 1885/DL 1965 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3810 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
|  | 2 | DL 1987.5 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3350 |  |  |  |  |  |
| 86,16 | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_3 |  |  |
| Test Settings for a DC_2A_n78A Configuration (PC2 and PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 2 | UL 1870 |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL 3740.01 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 23 | 2 | UL 1885 |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL 3739.995 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 20 MHz | All RBs / 0 |  |  |
| 34 | 2 | UL 1855/ DL 1935 |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL 3790.005 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 44 | 2 | UL 1885/ DL 1965 |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL 3690 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_3A_n1A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 111 | 3 | Mid |  |  |  |  |  |  |  |  |  | n1 |  |  | Mid |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 50 MHz | All RBs / REFSENS_ENDC_3 |  |  |
| 26 | 3 | High |  |  |  |  |  |  |  |  |  | n1 |  |  | Low |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 50 MHz | All RBs / REFSENS_ENDC_3 |  |  |
| 34 | 3 | UL 1760 / DL 1855 |  |  |  |  |  |  |  |  |  | n1 |  |  | UL 1950 / DL 2140 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_3A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 3 | UL 1771 / DL 1866 |  |  |  |  |  |  |  |  |  | n5 |  |  | UL 838 / DL 883 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 24 | 3 | UL 1721 / DL 1816 |  |  |  |  |  |  |  |  |  | n5 |  |  | UL 838 / DL 883 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_3A_n7A Configuration (PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 3 | UL 1730 / DL 1825 |  |  |  |  |  |  |  |  |  | n7 |  |  | UL 2535 / DL 2655 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_3A_n8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 3 | UL 1755 / DL 1850 |  |  |  |  |  |  |  |  |  | n8 |  |  | UL 900 / DL 945 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 24 | 3 | UL 1747.5 / DL 1842.5 |  |  |  |  |  |  |  |  |  | n8 |  |  | UL 897.5 / DL 942.5 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_3A_n41A Configuration (PC2 and PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 3 | High |  |  |  |  |  |  |  |  |  | n41 |  |  | Low |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_3 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 26 | 3 | High |  |  |  |  |  |  |  |  |  | n41 |  |  | Low |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 50 MHz | All RBs / REFSENS_ENDC_3 |  |  |
| 34 | 3 | UL 1740 / DL 1835 |  |  |  |  |  |  |  |  |  | n41 |  |  | UL/DL 2657.5 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for DC_3A_n77A and DC_3A_n78A Configurations (PC2 and PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 3 | Mid |  |  |  |  |  |  |  |  |  | n77/n78 |  |  | UL/DL 3495 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / 0 |  |  |
| 28 | 3 | Low |  |  |  |  |  |  |  |  |  | n77/n78 |  |  | Low |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / 0 |  |  |
| 33 | 3 | Mid |  |  |  |  |  |  |  |  |  | n77/n78 |  |  | UL/DL 3525 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 48 | 3 | Low |  |  |  |  |  |  |  |  |  | n77/n78 |  |  | Low |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 55 | 3 | Mid |  |  |  |  |  |  |  |  |  | n77/n78 |  |  | UL/DL 3685.005 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 65 | 3 | Mid |  |  |  |  |  |  |  |  |  | n77/n78 |  |  | UL/DL 3685.005 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 20 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 79 | 3 | Low |  |  |  |  |  |  |  |  |  | n77/n78 |  |  | High |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 10 MHz |  |  | All RBs/0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 89 | 3 | Low |  |  |  |  |  |  |  |  |  | n77/n78 |  |  | High |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 10 MHz |  |  | All RBs/0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 20 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 94 | 3 | UL 1740 / DL 1835 |  |  |  |  |  |  |  |  |  | n77/n78 |  |  | UL/DL 3574.995 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 104 | 3 | UL 1765 / DL 1860 |  |  |  |  |  |  |  |  |  | n77/n78 |  |  | UL/DL 3435 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_5A_n66A Configuration (PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 5 | UL 838/ DL 883 |  |  |  |  |  |  |  |  |  | n66 |  |  | UL 1721/ DL 2121 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_5A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 5 | UL 837.5 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3350.01 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 23 | 5 | UL 829 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 4145.01 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 38 | 5 | UL Mid (836.5 MHz) |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL Mid |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 44,13 | 5 | UL 844/DL 889 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3421.005 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 54,13 | 5 | UL 826.5/DL 871.5 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 4177.5 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for DC_5A_n78A Configuration (PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 5 | UL 840 |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL 3360 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 23 | 5 | UL 840 |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL 3360 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / 0 |  |  |
| 38 | 5 | UL 840 |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL 3560.01 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 44 | 5 | UL 844/DL 889 |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL 3421.005 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 55 | 5 | MID |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL 3525.99 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 69 | 5 | High |  |  |  |  |  |  |  |  |  | n78 |  |  | MID |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| Test Settings for a DC_7A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 7 | DL 2655 |  |  |  |  |  |  |  |  |  | n3 |  |  | DL 1825 |  |  |  |  |  |
|  | QPSK | N/A |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | N/A |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_7A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 7 | DL 2667 |  |  |  |  |  |  |  |  |  | n5 |  |  | DL 879 |  |  |  |  |  |
|  | QPSK | N/A |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | N/A |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_7A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 7 | DL 2655 |  |  |  |  |  |  |  |  |  | n66 |  |  | DL 2130 |  |  |  |  |  |
|  | QPSK |  |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | N/A |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_7A_n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 7 | DL 2672 |  |  |  |  |  |  |  |  |  | n71 |  |  | UL/ 668 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / 0 |  | N/A |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_1 |  |  |
| 28 | 7 | DL 2630 |  |  |  |  |  |  |  |  |  | n71 |  |  | UL 668 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / 0 |  | N/A |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_1 |  |  |
| Test Settings for a DC_7A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 7 | DL 2660 |  |  |  |  |  |  |  |  |  | N77 |  |  | DL 3870 |  |  |  |  |  |
|  | QPSK |  |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | N/A |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_7A_n78A (PC2 and PC3) Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 7 | High |  |  |  |  |  |  |  |  |  | n78 |  |  | Low |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_3 |  |  |
| Test Settings for a DC_8A_n1A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 8 | DL 932.5 |  |  |  |  |  |  |  |  |  | n1 |  |  | DL 2155 |  |  |  |  |  |
|  | QPSK | N/A |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | N/A |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_8A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 8 | DL 945 |  |  |  |  |  |  |  |  |  | n3 |  |  | DL 1850 |  |  |  |  |  |
|  | QPSK | N/A |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | N/A |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 24 | 8 | DL 942.5 |  |  |  |  |  |  |  |  |  | n3 |  |  | DL 1842.5 |  |  |  |  |  |
|  | QPSK | N/A |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | N/A |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_8A_n20A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 8 | UL 890.5 / DL 935.5 |  |  |  |  |  |  |  |  |  | n20 |  |  | UL 849.5 / DL 808.5 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 24 | 8 | UL 892.5 / DL 937.5 |  |  |  |  |  |  |  |  |  | n20 |  |  | UL 847.5 / DL 806.5 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_8A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 8 | UL 885 / DL 942.5 MHz |  |  |  |  |  |  |  |  |  | n41 |  |  | UL / DL 2655 MHz |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 60 MHz | All RBs / 0 |  |  |
| 28 | 8 | UL 885 / DL 942.5 MHz |  |  |  |  |  |  |  |  |  | n41 |  |  | UL / DL 2546 MHz |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 60 MHz | All RBs / 0 |  |  |
| 34 | 8 | UL 882.5 / DL 927.5 MHz |  |  |  |  |  |  |  |  |  | n41 |  |  | UL / DL 2685MHz |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for DC_8A_n77A Configurations (PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 8 | Mid |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3590.01 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 28 | 8 | Mid |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3520.005 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 34 | 8 | UL 897.5 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3634.995 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for DC_8A_n78A Configurations (PC2 and PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 8 | Mid |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL 3590.01 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 28 | 8 | Mid |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL 3520.005 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 34 | 8 | UL 897.5 |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL 3634.995 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_11A_n79A Configuration (PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 11 | Mid |  |  |  |  |  |  |  |  |  | n79 |  |  | UL/DL 4457.7 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 29 | 11 | Mid |  |  |  |  |  |  |  |  |  | n79 |  |  | UL/DL 4512.69 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| Test Settings for a DC_12A_n66A Configuration (PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 12 | UL 710 |  |  |  |  |  |  |  |  |  | n66 |  |  | UL 1730 /DL 2130 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / 0 |  |  |
| 23 | 12 | UL 710 |  |  |  |  |  |  |  |  |  | n66 |  |  | UL 1730 /DL 2130 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 40 MHz | All RBs / 0 |  |  |
| 38 | 12 | UL 710 |  |  |  |  |  |  |  |  |  | n66 |  |  | UL 1760 /DL 2160 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 40 MHz | All RBs / 0 |  |  |
| Test Settings for a DC_12A_n77A Configuration (PC2 and PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 12 | Mid |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3537.495 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 28 | 12 | Mid |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3351 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 35 | 12 | Mid |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3687.495 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 49 | 12 | Mid |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3750 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 54 | 12 | UL 702/DL 732 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3540 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_12A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 12 | UL 707/DL 737 MHz |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL 3535 MHz |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 28 | 12 | UL 707/DL 737 MHz |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL 3750 MHz |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 34 | 12 | UL 710/DL 740 MHz |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL 3580 MHz |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_13A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 13 | UL Mid |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3909.99 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 28 | 13 | UL Mid |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL Mid |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 35 | 13 | DL Mid |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3755 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 49 | 13 | DL Mid |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3850 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 54 | 13 | UL 784.5/DL 753.5 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3891.495 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_14A_n77A Configuration (PC2 and PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 14 | Mid |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3964.995 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 18 | 14 | Mid |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3350.1 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 25 | 14 | Mid |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3814.995 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 29 | 14 | Mid |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3350.01 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 34 | 14 | UL 795.5/DL 765.5 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3947.5 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_18A_n77A Configuration (PC2 and PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 18 | UL 820 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 4100 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 40 MHz | All RBs / 0 |  |  |
| 28 | 18 | UL 820 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 4000 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 40 MHz | All RBs / 0 |  |  |
| 34,17 | 18 | UL 827.5 / DL 872.5 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3355 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 44,17 | 18 | UL 817.5 / DL 862.5 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 4130 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_18A_n78A Configuration (PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14,18 | 18 | UL 827.5 / DL 872.5 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3355 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_19A_n77A Configuration (PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 19 | UL 835 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 4175 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 40 MHz | All RBs / 0 |  |  |
| 28 | 19 | UL 835 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 4100 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 40 MHz | All RBs / 0 |  |  |
| 33 | 19 | UL 840 |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL3360 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| Test Settings for a DC_19A_n78A Configuration (PC2 and PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 19 | UL 840 |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL3360 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / 0 |  |  |
| 24 | 19 | UL 836.5/ DL 881.5 |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL3391 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 35 | 19 | DL 880 |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL3520 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| Test Settings for a DC_19A_n79A Configuration (PC2 and PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 19 | DL 884 |  |  |  |  |  |  |  |  |  | n79 |  |  | UL/DL 4420 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 40 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 29 | 19 | DL 884 |  |  |  |  |  |  |  |  |  | n79 |  |  | UL/DL 4445.025 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 10 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 40 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| Test Settings for a DC_20A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 20 | UL 840 / DL 799 |  |  |  |  |  |  |  |  |  | n3 |  |  | UL 1775 / DL 1870 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 24 | 20 | UL 847 / DL 806 |  |  |  |  |  |  |  |  |  | n3 |  |  | UL 1735 / DL 1830 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_20A_n7A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 20 | UL 851 / DL 810 |  |  |  |  |  |  |  |  |  | n7 |  |  | UL 2512 / DL 2632 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_20A_n8A Configuration (PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 20 | UL 849.5 / DL 808.5 |  |  |  |  |  |  |  |  |  | n8 |  |  | UL 892.5 / DL 937.5 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_20A_n78A Configuration (PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 18 | 20 | Mid |  |  |  |  |  |  |  |  |  | n78 |  |  | Mid |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 23 | 20 | Mid |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL 3387.99 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |  |
| 34 | 20 | UL 850 / DL 809 |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL 3358.995 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_21A_n28A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 21 | UL 1450.4/ DL 1498.4 |  |  |  |  |  |  |  |  |  | n28 |  |  | UL 735.5 / DL 790.5 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_21A_n79A Configuration (PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 21 | DL 1503.4 |  |  |  |  |  |  |  |  |  | n79 |  |  | UL/DL 4510.2 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 15 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 29 | 21 | DL 1503.4 |  |  |  |  |  |  |  |  |  | n79 |  |  | UL/DL 4800 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 15 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 34 | 21 | UL 1457.5/ DL 1505.5 |  |  |  |  |  |  |  |  |  | n79 |  |  | UL/DL 4420.5 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 40 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_21A_n79A Configuration (PC2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 21 | DL 1503.4 |  |  |  |  |  |  |  |  |  | n79 |  |  | UL/DL 4510.2 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 15 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 29 | 21 | DL 1503.4 |  |  |  |  |  |  |  |  |  | n79 |  |  | UL/DL 4800 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  |  | 15 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 34 | 21 | UL 1457.5/ DL 1505.5 |  |  |  |  |  |  |  |  |  | n79 |  |  | UL/DL 4420.5 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_25A_n41A Configuration (PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 25 | High |  |  |  |  |  |  |  |  | n41 |  |  |  | Low |  |  |  |  |  |
|  | N/A | QPSK |  |  |  | 10 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / REFSENS_ENDC_3 |
| 26 | 25 | High |  |  |  |  |  |  |  |  | n41 |  |  |  | Low |  |  |  |  |  |
|  | N/A | QPSK |  |  |  | 15 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  | QPSK |  | 100 MHz |  |  | All RBs / REFSENS_ENDC_3 |
| 36 | 25 | High |  |  |  |  |  |  |  |  | n41 |  |  |  | Mid |  |  |  |  |  |
|  | N/A | QPSK |  |  |  | 20 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / REFSENS_ENDC_3 |
| Test Settings for a DC_26A_n41A Configuration (PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 26 | High |  |  |  |  |  |  |  |  | n41 |  |  |  | Low |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / 0 |
| 23 | 26 | High |  |  |  |  |  |  |  |  | n41 |  |  |  | UL/DL 2537 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  |  | CP-OFDM QPSK |  | 10 MHz |  |  | All RBs / 0 |
| 38 | 26 | High |  |  |  |  |  |  |  |  | n41 |  |  |  | High |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  |  | QPSK |  | 10 MHz |  |  | All RBs / 0 |
| 44 | 26 | UL 839 |  |  |  |  |  |  |  |  | n41 |  |  |  | UL/DL 2562 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_4 |
| 55 | 26 | DL 864 |  |  |  |  |  |  |  |  | n41 |  |  |  | Mid |  |  |  |  |  |
|  | N/A | QPSK |  |  |  | 5 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_2 |
| 65 | 26 | Low |  |  |  |  |  |  |  |  | n41 |  |  |  | Mid |  |  |  |  |  |
|  | N/A | QPSK |  |  |  | 15 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 15 MHz |  |  | All RBs / REFSENS_ENDC_2 |
| 78 | 26 | High |  |  |  |  |  |  |  |  | n41 |  |  |  | Mid |  |  |  |  |  |
|  | N/A | QPSK |  |  |  | 15 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 15 MHz |  |  | All RBs / REFSENS_ENDC_2 |
| Test Settings for a DC_26A_n77A Configuration (PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 26 | High |  |  |  |  |  |  |  |  | n77 |  |  |  | UL/DL 3366 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  | 15 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / 0 |
| 28 | 26 | High |  |  |  |  |  |  |  |  | n77 |  |  |  | UL/DL 3446.01 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  | 15 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / 0 |
| 34 | 26 | UL 836.5 |  |  |  |  |  |  |  |  | n77 |  |  |  | UL/DL 3391.005 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_26A_n78A Configuration (PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 26 | High |  |  |  |  |  |  |  |  | n78 |  |  |  | UL/DL 3366 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  | 15 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / 0 |
| 28 | 26 | High |  |  |  |  |  |  |  |  | n78 |  |  |  | UL/DL 3446.01 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  | 15 MHz |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / 0 |
| 34 | 26 | UL 836.5 |  |  |  |  |  |  |  |  | n78 |  |  |  | UL/DL 3391.005 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_26A_n79A Configuration (PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 26 | High |  |  |  |  |  |  |  |  | n79 |  |  |  | UL/DL 4432.5 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  | 15 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 60 MHz |  |  | All RBs / REFSENS_ENDC_2 |
| 22,9 | 26 | High |  |  |  |  |  |  |  |  | n79 |  |  |  | UL/DL 4470.51 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  | 15 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 60 MHz |  |  | All RBs / REFSENS_ENDC_2 |
| Test Settings for a DC_28A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 28 | High |  |  |  |  |  |  |  |  | n5 |  |  |  | Low |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  | 20MHz |  |  | All RBs / 0 |  | N/A |  |  |  | CP-OFDM QPSK |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_3 |
| Test Settings for a DC_28A_n77A Configuration (PC2 and PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 28 | Low |  |  |  |  |  |  |  |  |  | n77 |  | UL/DL 3540 |  |  |  |  |  |  |
|  | QPSK | QPSK |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / 0 |  |
| 28 | 28 | Low |  |  |  |  |  |  |  |  |  | n77 |  | Low |  |  |  |  |  |  |
|  | QPSK | QPSK |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / 0 |  |
| 34 | 28 | UL 705.5 / DL 760.5 |  |  |  |  |  |  |  |  |  | n77 |  | UL/DL 3582.495 |  |  |  |  |  |  |
|  | QPSK | QPSK |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  | CP-OFDM QPSK |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_4 |  |
| 45 | 28 | Low |  |  |  |  |  |  |  |  |  | n77 |  | UL/DL 3815.01 |  |  |  |  |  |  |
|  | N/A | QPSK |  |  | 10 MHz |  |  | All RBs / 0 |  |  |  | DFT-s-OFDM QPSK |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / REFSENS_ENDC_2 |  |
| Test Settings for a DC_28A_n78A Configuration (PC2 and PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 28 | Low |  |  |  |  |  |  |  |  |  | n78 |  | UL/DL 3540 |  |  |  |  |  |  |
|  | QPSK | QPSK |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / 0 |  |
| 28 | 28 | Low |  |  |  |  |  |  |  |  |  | n78 |  | Low |  |  |  |  |  |  |
|  | QPSK | QPSK |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / 0 |  |
| 34 | 28 | UL 705.5 / DL 760.5 |  |  |  |  |  |  |  |  |  | n78 |  | UL/DL 3582.495 |  |  |  |  |  |  |
|  | QPSK | QPSK |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  | CP-OFDM QPSK |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_30A_n66A Configuration (PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 30 | Low |  |  |  |  |  |  |  |  | n66 |  |  |  | High |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_3 |  | N/A |  |  |  | CP-OFDM QPSK |  | 40 MHz |  |  | All RBs / 0 |
| Test Settings for a DC_30A_n77A Configuration (PC2 and PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 30 | UL 2310 |  |  |  |  |  |  |  |  | n77 |  |  |  | UL/DL 3487.5 |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  | CP-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_38A_n78A Configuration (PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 38 | DL High |  |  |  |  |  |  |  |  | n78 |  |  |  | UL Low |  |  |  |  |  |
|  | N/A | QPSK |  |  |  | 10 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_3 |
| Test Settings for a DC_39A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 39 | UL 1910 |  |  |  |  |  |  |  |  | n41 |  |  |  | DL 2501 |  |  |  |  |  |
|  | QPSK | N/A |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_3 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 10 MHz |  |  | All RBs / 0 |
| 26 | 39 | DL 1917.5 |  |  |  |  |  |  |  |  | n41 |  |  |  | UL 2546 |  |  |  |  |  |
|  | QPSK | N/A |  |  |  | 5 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / REFSENS_ENDC_3 |
| Test Settings for a DC_40A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 40 | DL 2397.5 |  |  |  |  |  |  |  |  | n41 |  |  |  | UL 2546 |  |  |  |  |  |
|  | QPSK | N/A |  |  |  | 5 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / REFSENS_ENDC_3 |
| Test Settings for a DC_40A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 40 | Mid |  |  |  |  |  |  |  |  | n78 |  |  |  | UL/DL 3525 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  | 20 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_2 |
| 29 | 40 | Low |  |  |  |  |  |  |  |  | n78 |  |  |  | Mid |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  | 20 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / REFSENS_ENDC_2 |
| Test Settings for a DC_41A_n77A Configuration (PC2 and PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 41 | Low |  |  |  |  |  |  |  |  | n77 |  |  |  | UL/DL 3750 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  | 20 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / REFSENS_ENDC_2 |
| 26 | 41 | High |  |  |  |  |  |  |  |  | n77 |  |  |  | Low |  |  |  |  |  |
|  | N/A | QPSK |  |  |  | 20 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / REFSENS_ENDC_3 |
| 36,19 | 41 | High |  |  |  |  |  |  |  |  | n77 |  |  |  | Low |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  | 20MHz |  |  | All RBs / REFSENS_ENDC_3 |  | N/A |  |  |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / 0 |
| Test Settings for a DC_41A_n78A Configuration (PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 41 | Low |  |  |  |  |  |  |  |  | n78 |  |  |  | UL/DL 3750 |  |  |  |  |  |
|  | N/A | QPSK |  |  |  | 20 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / REFSENS_ENDC_2 |
| 26 | 41 | High |  |  |  |  |  |  |  |  | n78 |  |  |  | Low |  |  |  |  |  |
|  | N/A | QPSK |  |  |  | 20 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / REFSENS_ENDC_3 |
| 36 | 41 | High |  |  |  |  |  |  |  |  | n78 |  |  |  | Low |  |  |  |  |  |
|  | QPSK | QPSK |  |  |  | 20MHz |  |  | All RBs / REFSENS_ENDC_3 |  | N/A |  |  |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / 0 |
| Test Settings for a DC_48A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 48 |  |  | UL/DL 3555 |  |  |  |  |  |  | n66 |  |  |  | UL 1760 MHZ |  |  |  |  |  |
|  | N/A |  |  | QPSK |  |  | 10 MHz |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 40 MHz |  |  | All RBs / REFSENS_ENDC_1 |
| 23 | 48 |  |  | UL/DL 3565 |  |  |  |  |  |  | n66 |  |  |  | UL 1770 |  |  |  |  |  |
|  | N/A |  |  | QPSK |  |  | 10 MHz |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_1 |
| 38 | 48 |  |  | UL/DL 3600 |  |  |  |  |  |  | n66 |  |  |  | UL1760 |  |  |  |  |  |
|  | N/A |  |  | QPSK |  |  | 10 MHz |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 40 MHz |  |  | All RBs / REFSENS_ENDC_1 |
| 44 | 48 |  |  | UL/DL 3630 |  |  |  |  |  |  | n66 |  |  |  | UL 1715 / DL 2115 |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  | 20 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_66A_n2A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 66 |  |  | UL1775/ DL2175 |  |  |  |  |  |  | n2 |  |  |  | UL 1855 / DL 1935 |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |
| 24 | 66 |  |  | UL 1750/ DL 2150 |  |  |  |  |  |  | n2 |  |  |  | UL 1883.3 / DL 1963.3 |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_66A_n5A Configuration (PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 66 |  |  | UL 1721/ DL 2121 |  |  |  |  |  |  | n5 |  |  |  | UL 838/ DL 883 |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_66A_n25A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 66 |  |  | UL 1775/ DL 2175 |  |  |  |  |  |  | n25 |  |  |  | UL 1855/ DL 1935 |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |
| 24 | 66 |  |  | UL 1712.5/ DL 2112.5 |  |  |  |  |  |  | n25 |  |  |  | UL 1912.5/ DL 1992.5 |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |
| 34 | 66 |  |  | UL 1750/ DL 2150 |  |  |  |  |  |  | n25 |  |  |  | UL 1883.3/ DL 1963.3 |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_66A_n41A Configuration (PC2 and PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 66 |  |  | DL 2177.5 |  |  |  |  |  |  | n41 |  |  |  | UL/DL 2546 |  |  |  |  |  |
|  | N/A |  |  | QPSK |  |  | 20 MHz |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / REFSENS_ENDC_3 |
| Test Settings for a DC_66A_n71A Configuration (PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 66 |  | UL 1750/ DL 2150 |  |  |  |  |  |  |  | n71 |  |  |  | UL 675/ DL 629 |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_66A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 66 |  | UL 1720 |  |  |  |  |  |  |  | n77 |  |  |  | UL/DL 3440.01 |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 20 |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / 0 |
| 23 | 66 |  | UL 1720 |  |  |  |  |  |  |  | n77 |  |  |  | UL/DL 3410 |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 20 |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  |  | CP-OFDM QPSK |  | 20 MHz |  |  | All RBs / 0 |
| 38 | 66 |  | UL Mid |  |  |  |  |  |  |  | n77 |  |  |  | UL/DL Mid |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 20 |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / 0 |
| 44 | 66 |  | UL 1775/ DL 2175 |  |  |  |  |  |  |  | n77 |  |  |  | UL/DL 3949.995 |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_4 |
| 54 | 66 |  | UL 1760 / DL 2160 |  |  |  |  |  |  |  | n77 |  |  |  | UL/DL 3720 |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_4 |
|  | 66 |  | DL 2177.5 |  |  |  |  |  |  |  | n77 |  |  |  | UL/DL 3350 |  |  |  |  |  |
| 66,16 | QPSK |  | QPSK |  |  |  | 5 MHz |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / REFSENS_ENDC_3 |
| Test Settings for a DC_66A_n78A Configuration (PC2 and PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 66 |  | UL 1755 |  |  |  |  |  |  |  | n78 |  |  |  | UL/DL 3510 |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 20 MHz |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / 0 |
| 23 | 66 |  | UL 1755 |  |  |  |  |  |  |  | n78 |  |  |  | UL/DL 3480 |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 20 MHz |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  |  | CP-OFDM QPSK |  | 20 MHz |  |  | All RBs / 0 |
| 34 | 66 |  | UL 1730/ DL2130 |  |  |  |  |  |  |  | n78 |  |  |  | UL/DL 3660 |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_4 |
| Test Settings for DC_71A_n2A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 71 |  | UL 665.5 |  |  |  |  |  |  |  | n2 |  |  |  | DL 1987.5 |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5MHz |  | All RBs / REFSENS_ENDC_1 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 5 MHz |  |  | All RBs / 0 |
| 23 | 71 |  | UL 673 |  |  |  |  |  |  |  | n2 |  |  |  | DL1980 |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 20 MHz |  | All RBs / REFSENS_ENDC_1 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 20 MHz |  |  | All RBs / 0 |
| 35 | 71 |  | Low |  |  |  |  |  |  |  | n2 |  |  |  | UL 1881 |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 20 MHz |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_2 |
| 49 | 71 |  | High |  |  |  |  |  |  |  | n2 |  |  |  | UL 1881 |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 20 MHz |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_2 |
| Test Settings for DC_71A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 71 |  | UL 675/ DL 629 |  |  |  |  |  |  |  | n66 |  |  |  | UL1750/ DL 2150 |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |
| NOTE 1: REFSENS_LTE refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.3-2 of TS 36.521-1 [10]. REFSENS_NR refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3 of TS 38.521-1 [8]. REFSENS_ENDC_1 refers to the Uplink RB allocation for reference sensitivity exceptions due to UL harmonic interference according to table 7.3B.2.0.3.1-1. REFSENS_ENDC_2 refers to the Uplink RB allocation for reference sensitivity exceptions due to receiver harmonic mixing according to table 7.3B.2.0.3.2-1. REFSENS_ENDC_3 refers to the Uplink RB allocation for reference sensitivity exceptions due to cross band isolation according to table 7.3B.2.0.3.4-1 for PC3 and Table 7.3B.2.0.3.4-1a for PC2 REFSENS_ENDC_4 refers to the Uplink RB allocation for reference sensitivity exceptions due to dual uplink operation for ENDC according to table 7.3B.2.0.3.5.1-1 for PC3 and table 7.3B.2.0.3.5.1-1a for PC2. NOTE 2: Void NOTE 3: Test ID with UL harmonic exception NOTE 4: Test ID with 2UL intermodulation exception. The test point is only applicable to UEs not supporting UE capability singleUL-Transmission. NOTE 5: Test ID with UL receiver harmonic mixing NOTE 6: Test ID with UL cross band isolation NOTE 7: Void NOTE 8: Test ID with UL harmonic exception avoided NOTE 9: Test ID with UL receiving harmonic mixing exception avoided NOTE 10: Void NOTE 11: Test ID with Cross band isolation exception avoided, which is only applicable to DC_1_n3, DC_3_n1 and DC_3_n84. NOTE 12: In an E-UTRA band or FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. NOTE 13: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 14: If the NR frequency does not match to a valid NR-ARFCN, apply the closest NR frequency with a valid NR-ARFCN. NOTE 15: The test points are executed only when UEs support simultaneous Rx/Tx capability as indicated in Table A.4.3.2B.2.3.1-2 of TS 38.508-2. NOTE 16: This test ID is for UL PC2 only. NOTE 17: For a UE which supports this band combination only when the Band n77 frequency range restriction of 3400 - 4100 MHz in Japan applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 18: For a UE which supports this band combination only when the Band n78 frequency range restriction of 3400 - 3800 MHz in Japan applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 19: This test ID is for UL PC3 only. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

7.3B.2.3.5	Test requirement

For inter-band EN-DC configurations affected by reference sensitivity exceptions, when test points without NOTE 8, 9 and 11 in Table 7.3B.2.3.4.2.1-6 are tested, the throughput of each CG shall be ≥ 95% of the maximum throughput for the reference receive power level specified in Table 7.3B.2.3.5-0, 7.3B.2.3.5-0a, Table 7.3B.2.3.5-0b, Table 7.3B.2.3.5-1, Table 7.3B.2.3.5-2, Table 7.3B.2.3.5-3, Table 7.3B.2.3.5-3a, Table 7.3B.2.3.5-4 and Table 7.3B.2.3.5-5 for MSDs due to uplink harmonic, harmonic mixing, cross band isolation for PC3 EN-DC, cross band isolation for PC2 EN-DC, dual uplinks for PC3 EN-DC and dual uplink operation for PC2 EN-DC, respectively. For a given EN-DC combo, if more than one category of MSD applies, UE shall pass all requirement.

For test points with NOTE 8, 9 or 11 in Table 7.3B.2.3.4.2.1-6, reference sensitivity test requirements are specified in Table 7.3.2.5-1 in TS 38.521-1 [8] for the NR CC, and Table 7.3.5-1 in TS 36.521-1 [10] for E-UTRA CC.

Reference sensitivity test requirements for test points in Table 7.3B.2.3.4.2.1-0 are specified in Table 7.3.2.5-1 in TS 38.521-1 [8].

For the UE which supports inter-band EN-DC, the minimum requirement for reference sensitivity in Table 7.3.2.5-1 of TS 38.521-1 [8] for NR band and Table 7.3.5-1 of TS 36.521-1 [10] for EUTRA band, shall be increased by the amount given in ΔRIB,c defined in clause 7.3B.3.3 for the applicable two, three, four and five bands operation.

Table 7.3B.2.3.5-0: Reference sensitivity test requirement for PC3 EN-DC

| EN-DC configuration | Test ID | E-UTRA/NR band | UL/DL CBW (MHz) | REFSENS test requirement (dBm) | Exception type |
| --- | --- | --- | --- | --- | --- |
| DC_1A_n78A | 1 | 1 | 5 | -91.3 | dual uplink operation |
|  |  | n78 | 10 | REFSENS |  |
| DC_2A_n41A | 1 | 2 | 5 | -96.7 | Cross band isolation |
|  |  | n41 | 100 | REFSENS |  |
| DC_3A_n77A | 1 | 3 | 5 | REFSENS | UL harmonic |
| DC_3A_n78A |  | n78 | 10 | -71.9+TT | interference |
|  | 2 | 3 | 5 | REFSENS | UL harmonic exception |
|  |  | n78 | 10 | REFSENS | avoided |
|  | 3 | 3 | 10 | REFSENS | UL harmonic |
|  |  | n78 | 100 | -71.7+TT | interference |
|  | 4 | 3 | 10 | REFSENS | UL harmonic exception |
|  |  | n78 | 100 | REFSENS | avoided |
|  | 5 | 3 | 5 | -90.6 | Harmonic mixing |
|  |  | n78 | 10 | REFSENS |  |
|  | 6 | 3 | 20 | -88.0 | Harmonic mixing |
|  |  | n78 | 10 | REFSENS |  |
|  | 7 | 3 | 5 | REFSENS | Harmonic mixing |
|  |  | n78 | 10 | REFSENS | avoided |
|  | 8 | 3 | 20 | REFSENS | Harmonic mixing |
|  |  | n78 | 10 | REFSENS | avoided |
|  | 9 | 3 | 5 | -70.3 | Dual uplink operation |
|  |  | n78 | 10 | REFSENS |  |
|  | 10 | 3 | 5 | -88.8 | Dual uplink operation |
|  |  | n78 | 10 | REFSENS |  |
| DC_5A_n78A | 1 | 5 | 5 | REFSENS | UL harmonic |
|  |  | n78 | 100 | -84.1+TT | Interference |
|  | 2 | 5 | 5 | REFSENS |  |
|  |  | n78 | 10 | -85.3+TT |  |
|  | 3 | 5 | 5 | REFSENS | UL harmonic exception |
|  |  | n78 | 100 | REFSENS | avoided |
|  | 4 | 5 | 5 | -89.0 | Dual uplink operation |
|  |  | n78 | 10 | REFSENS |  |
|  | 5 | 5 | 5 | -91.6 | Harmonic mixing |
|  |  | n78 | 10 | REFSENS |  |
|  | 6 | 5 | 5 | REFSENS | Harmonic mixing |
|  |  | n78 | 10 | REFSENS | avoiding |
| DC_26A_n41A | 1 | 26 | 5 | REFSENS | UL harmonic |
|  |  | n41 | 100 | -81.8+TT |  |
|  | 2 | 26 | 5 | REFSENS |  |
|  |  | n41 | 10 | -81.1+TT |  |
|  | 3 | 26 | 5 | REFSENS | UL harmonic exception |
|  |  | n41 | 10 | REFSENS | avoided |
|  | 4 | 26 | 5 | -81.2 | dual uplink operation |
|  |  | n41 | 10 | REFSENS |  |
|  | 5 | 26 | 5 | -73 | harmonic mixing |
|  |  | n41 | 10 | REFSENS |  |
|  | 6 | 26 | 15 | -73 |  |
|  |  | n41 | 15 | REFSENS |  |
|  | 7 | 26 | 15 | REFSENS | harmonic mixing |
|  |  | n41 | 15 | REFSENS | avoiding |
| DC_39A_n41A | 1 | 39 | 20 | REFSENS | Cross band isolation |
|  |  | n41 | 10 | -91.5+TT |  |
|  | 2 | 39 | 5 | -97.7 | Cross band isolation |
|  |  | n41 | 100 | REFSENS |  |
| DC_40A_n41A | 1 | 40 | 5 | -67.9 | Cross band isolation |
|  |  | n41 | 100 | REFSENS |  |
| NOTE 1: The symbol "REFSENS" in this table refers to the reference sensitivity values for single carrier specified in Table 7.3.5-1 of TS 36.521-1 [10] for 2 antenna port E-UTRA band, Table 7.3_1.5-1 of TS 36.521-1 [10] for 4 antenna port E-UTRA band, Table 7.3.2.5-1a and Table 7.3.2.5-1b of TS 38.521-1 [8] for 2 antenna port NR band, Table 7.3.2.5-2a and Table 7.3.2.5-2b of TS 38.521-1 [8] for 4 antenna port NR band. |  |  |  |  |  |

<<End of change>>