3GPP TSG-RAN5 Meeting #107	R5-252525

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3484 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of SL-ConfigDedicatedNR and SL-RLC-ChannelConfig for SL relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - The IE sl-PHY-MAC-RLC-Config-v1700 of Table 4.6.6-7 needs to set for SL relay. - There is FFS in Table 4.6.6-27AA SL-RLC-ChannelConfig. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updating Table 4.6.6-7. - Updating Table 4.6.6-27AA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification remains incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changed Section>

–	SL-ConfigDedicatedNR

Table 4.6.6-7: SL-ConfigDedicatedNR

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-ConfigDedicatedNR-r16 ::= SEQUENCE { |  |  |  |  |
| sl-PHY-MAC-RLC-Config-r16 |  | Not present |  |  |
| sl-PHY-MAC-RLC-Config-r16 SEQUENCE { |  |  |  | SCHEDULING, SELECTED |
| sl-ScheduledConfig-r16 |  | Not present |  |  |
| sl-ScheduledConfig-r16 CHOICE { |  |  |  | SCHEDULING |
| setup |  | SL-ScheduledConfig |  |  |
| } |  |  |  |  |
| sl-UE-SelectedConfig-r16 |  | Not present |  |  |
| sl-UE-SelectedConfig-r16 CHOICE { |  |  |  | SELECTED |
| setup |  | SL-UE-SelectedConfig |  |  |
| } |  |  |  |  |
| sl-FreqInfoToReleaseList-r16 |  | Not present |  |  |
| sl-FreqInfoToAddModList-r16 |  | Not present |  |  |
| sl-FreqInfoToAddModList-r16 SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-FreqConfig-r16 { |  | 1 entry |  | SCHEDULING, SELECTED |
| SL-FreqConfig-r16[1] |  | SL-FreqConfig | entry 1 |  |
| } |  |  |  |  |
| sl-RLC-BearerToReleaseList-r16 |  | Not present |  |  |
| sl-RLC-BearerToAddModList-r16 |  | Not present |  |  |
| sl-RLC-BearerToAddModList-r16 SEQUENCE (SIZE (1..maxSL-LCID-r16)) OF SL-RLC-BearerConfig-r16 { |  | 1 entry |  | SL_DRB |
| SL-RLC-BearerConfig-r16[1] |  | SL-RLC-BearerConfig | entry 1 |  |
| } |  |  |  |  |
| sl-MaxNumConsecutiveDTX-r16 |  | Not present |  |  |
| sl-CSI-Acquisition-r16 |  | Not present |  |  |
|  |  | enabled |  | SL_CSI_REPORT |
| sl-CSI-SchedulingRequestId-r16 |  | Not present |  |  |
| sl-SSB-PriorityNR-r16 |  | 1 |  |  |
| networkControlledSyncTx-r16 |  | off | UE doesn't send SL SSB by default |  |
| } |  |  |  |  |
| sl-RadioBearerToReleaseList-r16 |  | Not present |  |  |
| sl-RadioBearerToAddModList-r16 |  | Not present |  |  |
| sl-RadioBearerToAddModList-r16 SEQUENCE (SIZE (1..maxNrofSLRB-r16)) OF SL-RadioBearerConfig-r16 { |  | 1 entry |  | SL_DRB |
| SL-RadioBearerConfig-r16[1] |  | SL-RadioBearerConfig |  |  |
| } |  |  |  |  |
| sl-MeasConfigInfoToReleaseList-r16 |  | Not present |  |  |
| sl-MeasConfigInfoToAddModList-r16 |  | Not present |  |  |
| sl-MeasConfigInfoToAddModList-r16 SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-MeasConfigInfo-r16 { |  | 1 entry |  | SL_MEAS |
| SL-MeasConfigInfo-r16[1] |  | SL-MeasConfigInfo |  |  |
| } |  |  |  |  |
| t400-r16 |  | ms2000 |  |  |
| sl-PHY-MAC-RLC-Config-v1700 |  | Not present |  |  |
| sl-PHY-MAC-RLC-Config-v1700 SEQUENCE { |  |  |  | L2RelayUE, L2RemoteUE |
| setup SEQUENCE { |  |  |  |  |
| sl-DRX-Config-r17 |  | Not present |  |  |
| sl-RLC-ChannelToReleaseList-r17 |  | Not present |  |  |
| sl-RLC-ChannelToAddModList-r17 SEQUENCE (SIZE (1..maxSL-LCID-r16)) OF SL-RLC-ChannelConfig-r17 { |  | 1 entry |  | PC5RelaySRB1 |
| SL-RLC-ChannelConfig-r17[1] |  | SL-RLC-ChannelConfig-r17 with condition PC5RelaySRB1 | entry 1 |  |
| } |  |  |  |  |
| sl-RLC-ChannelToAddModList-r17 SEQUENCE (SIZE (1..maxSL-LCID-r16)) OF SL-RLC-ChannelConfig-r17 { |  | 2 entries |  | PC5RelaySRB2_DRB1 |
| SL-RLC-ChannelConfig-r17[1] |  | SL-RLC-ChannelConfig-r17 with condition PC5RelaySRB2 | entry 1 |  |
| SL-RLC-ChannelConfig-r17[2] |  | SL-RLC-ChannelConfig-r17 with condition PC5RelayDRB1 | entry 2 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| sl-DiscConfig-r17 |  | Not present |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| SCHEDULING | To configure UE performing mode 1 SL transmission |
| SELECTED | To configure UE performing mode 2 SL transmission |
| SL_DRB | To establish SL DRB |
| SL_MEAS | To configure UE performing measurements based on RS sent by peer UE |
| SL_CSI_REPORT | To configure UE performing SL CSI reporting |
| L2RelayUE | Setup L2 U2N Relay UE configuration |
| L2RemoteUE | Setup L2 U2N Remote UE configuration |
| PC5RelaySRB1 | Establish PC5 RelayRLC Channel for SRB1 |
| PC5RelaySRB2_DRB1 | Establish PC5 RelayRLC Channel for SRB2 and DRB1 |

< Unchanged Section Skipped >

–	SL-RLC-ChannelConfig

Table 4.6.6-27AA: SL-RLC-ChannelConfig

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-RLC-ChannelConfig-r17 ::= SEQUENCE { |  |  |  |  |
| sl-RLC-ChannelID-r17 |  | SL-RLC-ChannelID-r17 with condition PC5RelaySRBn |  | PC5RelaySRBn |
|  |  | SL-RLC-ChannelID-r17 with condition PC5RelayDRBn |  | PC5RelaySRBn |
| sl-RLC-Config-r17 CHOICE { |  | SL-RLC-Config with condition AM |  |  |
| sl-MAC-LogicalChannelConfig-r17 |  | SL-LogicalChannelConfig |  |  |
| sl-PacketDelayBudget-r17 |  | Not present |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| PC5RelaySRBn | Establishment of PC5 RelayRLC Channel for SRBn; n=1..3 |
| PC5RelayDRBn | Establishment of PC5 RelayRLC Channel for DRBn; n=1..16 |

<End of Changed Section>