3GPP TSG-RAN5 Meeting #107	R5-252673

# St Julian’s, Malta, 19 – 23 May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3371 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case general ON/OFF time mask for Tx Diversity for UE supporting 4Tx |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_RF_FR1_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | General ON/OFF time mask for Tx Diversity for UE supporting 4Tx test case is absent in 6.3G.3.1 and need to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | General ON/OFF time mask for Tx Diversity for UE supporting 4Tx test case is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | General ON/OFF time mask for Tx Diversity for UE supporting 4Tx test case can not be tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3G.3.1_1(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.3G.3.1_1	General ON/OFF time mask for Tx Diversity for UE supporting 4Tx

6.3G.3.1_1.1	Test purpose

Same test purpose as in 6.3.3.2.1.

6.3G.3.1_1.2	Test applicability

This test case applies to all types of NR Power Class 1.5 UE release 18 and forward that support 4Tx Tx diversity.

6.3G.3.1_1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3G.3.1.3

6.3G.3.1_1.4	Test description

Same test description as specified in clause 6.3G.3.1.4 with following exceptions:

-	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2a for TE diagram and section A.3.2 for UE diagram.

-	The ON power is measured as sum of all tansmit antenna connectors.

6.3G.3.1_1.5	Test requirement

The measured ON power and OFF power shall not exceed the values specified in Table 6.3G.3.1_1.5-1.

Table 6.3G.3.1_1.5-1: General ON/OFF time mask

|  |  | Channel bandwidth / minimum output power / measurement bandwidth(MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| Transmit OFF power |  | ≤ -50+TT dBm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Transmission OFF Measurement bandwidth |  | 4.515 | 9.375 | 14.235 | 19.095 | 23.955 | 28.815 | 33.855 | 38.895 | 43.575 | 48.615 | 58.35 | 68.07 | 78.15 | 88.23 | 98.31 |
| Transmit ON power |  | Same as Test ID 9 of Tables 6.2G.2_1.5-1 to 6.2G.2_1.5-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NOTE 1: TT of OFF power for each frequency and channel bandwidth is specified in Table 6.3G.3.1_1.5-2 NOTE 2: TT of ON power for each frequency and channel bandwidth is specified in the referred ON power test requirement tables. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.3G.3.1_1.5-2: Test Tolerance for OFF power

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 1.5 dB | 1.8 dB |
| 40MHz < BW ≤ 100MHz | 1.7 dB | 1.8 dB |

< End of Changes >