3GPP TSG-RAN5 Meeting #107                                                 	                    R5-252976

# St Julian's, Malta, 19th May 2025 – 23rd May 2025

Agenda item:	5.3.5.9

Source: 	Qualcomm Korea

Title: 	Discussion on Measurement Uncertainties for FR1-NTN Timing Tests

Document for:	Discussion and Endorsement

## 1.	Introduction

This paper presents our views on measurement uncertainty derivation for FR1-NTN as well as the values we propose to be used in the elaboration of related TT analysis.

## 2.	Discussion

During the early stages of 5G RRM test case implementation, MUs for RRM timing test cases were decided to be implemented in TS 38.533 [2] specification using a different methodology than the one used in LTE. Discussion paper R5-190058 [3] introduced the timing error MU derivation methodology that is still applicable today. The main reasoning for this differentiation was that, effectively, 5G had different core requirements than LTE [1]. The proposed timing MU values were implemented in Annex F.1.1.2 in R5-190448 [4] and have been in use ever since.

Briefly, the methodology used to derivate the timing MU was decided as 25% of the smallest FR1 requirement:

# [ . . . ] In LTE, absolute timing measurement MU was 25% of the Timing error limit. Hence, we propose to set the FR1 MU to 25% of smallest FR1 requirement, and apply it for all SCS. # FR1 absolute timing measurement MU : 7*64*0.25*Tc = 112*Tc # FR1 relative timing measurement MU : 5.5*64*0.25*Tc = 88*Tc # Proposal 1: ±112 Tc for FR1 Absolute timing measurement MU # Proposal 2: ±88 Tc for FR1 Relative timing measurement MU

Source: R5-190058

Observation1: Transmit timing measurement uncertainty is derived based on 25% of the tightest applicable core requirement.

Similarly, for FR2, the same methodology has been applied. In the FR2 case, the tightest core requirement for FR2 is 3*64*Tc (reference: Table 7.1.2-1 of TS 38.133 [1]), with a resulting MU of 25% of that value (= 48*Tc). Both FR1 and FR2 MU values have been added to Annex F.1.1.2 of TS 38.533 [2].

In other words, FR1 MU has not been applied to FR2, but instead a new value for FR2 has been derived using the same methodology.

Observation2: Different measurement uncertainty values have been derived from different core requirements, for different technologies.

Given the adoption and success of the current timing MU derivation methodology, we propose to extend it to the new NR-NTN transmit timing requirements. From TS 38.133 [1] Table 7.1C.2-1 we observe that the tightest requirement for FR1-NTN is 22*64*Tc:

Table 7.1C.2-1: Te_NTN Timing Error Limit

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| FR1-NTN | 15 | 15 | 29*64*Tc |
|  |  | 30 | 24*64*Tc |
|  |  | 60 | N/A |
|  | 30 | 15 | 24*64*Tc |
|  |  | 30 | 22*64*Tc |
|  |  | 60 | N/A |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |  |

Similarly, from TS 38.133 [1] Table 7.1C.2.1-1, we observe that the tightest autonomous timing adjustment requirements for FR1-NTN are:

Table 7.1C.2.1-1: Tq_NTN Maximum Autonomous Time Adjustment Step and Tp_NTN Minimum Aggregate Adjustment rate

| Frequency Range | SCS of uplink signals (kHz) | Tq_NTN | Tp_NTN |
| --- | --- | --- | --- |
| FR1-NTN | 15 | 5.5*64*Tc | 5.5*64*Tc |
|  | 30 | 5.5*64*Tc | 5.5*64*Tc |
|  | 60 | N/A | N/A |
| FR2-NTN | 60 | 2.5*64*Tc | 2.5*64*Tc |
|  | 120 | 2.5*64*Tc | 2.5*64*Tc |
| NOTE: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

Therefore, applying the existing methodology to these requirements we have:

- FR1-NTN absolute timing measurement MU = 22*64*0.25*Tc = 352*Tc

- FR1-NTN relative timing measurement MU = 5.5*64*0.25*Tc = 88*Tc

We propose to use these values as MU for FR1-NTN timing test cases. They have been introduced into TS 38.533 [2] via R5-252956 [5] this meeting.

Proposal1: Use ±352*Tc for FR1-NTN absolute timing measurement MU.

Proposal2: Use ±88 Tc for FR1-NTN relative timing measurement MU.

## 3.	Conclusion

Following observations and proposals have been raised from this discussion:

Observation1: Transmit timing measurement uncertainty is derived based on 25% of the tightest applicable core requirement.

Observation2: Different measurement uncertainty values have been derived from different core requirements, for different technologies.

Proposal1: Use ±352*Tc for FR1-NTN absolute timing measurement MU.

Proposal2: Use ±88 Tc for FR1-NTN relative timing measurement MU.

## 4.		References

[1] 3GPP TS 38.133: "NR; Requirements for support of radio resource management".

[2] 3GPP TS 38.533: "NR; User Equipment (UE) conformance specification; Radio Resource Management (RRM)".

[3] R5-190058: “FR1 MU for RRM timing measurements”, Anritsu

[4] R5-190448: “Updating FR1 MU for timing measurements”, Anritsu

[5] R5-252956: “Addition of FR1-NTN timing measurement MU”, Qualcomm