3GPP TSG-RAN5 Meeting #107                             		    R5-252995

Malta, Malta, 19th May 2025 - 23rd May 2025

Agenda item:	5.3.34.2

Source: 	Apple Inc

Title:       	Update to FR1 TRP TRS TT for Rel-18

Document for:	DISCUSSION AND ENDORSEMENT

# 1	Introduction

The purpose of this paper is to propose finalization of the Test Tolerance for Release 18 FR1 TRP TRS scope tobased on agreements already reached during Rel-17 for browsing mode and additional guidance for talk mode (tested with head and hand phantoms) received from RAN4 in [ ] for Rel-18. This is in follow-up to the finalization of MU that was achieved for Talk Mode TRP TRS measurements at RAN5#105.

# 2	Discussion

## 2.1 RAN5 work on Rel-18 NR FR1 TRP TRS

RAN5 initiated a Rel-18 WID on NR FR1 TRP TRS at the Feb 2024 RAN5#103 meeting. The new scope in this release of the FR1 OTA specification was the addition of requirements for new bands such as n1 and n28 and in addition the testing in talk mode. Talk mode involves TRP TRS testing with the head and hand phantom. The objective has been to create test requirements for the above scenarios to be added to the conformance test specification TS 38.561. THis is to incorporate the core requirements defined in RAN4 in Rel-18 version of TS 38.161 along with other test methodology and test parameter aspects relevant to NR FR1 TRP TRS as collected in TR 38.870.

It should be noted that a preliminary assessment of the MU for the new scope (Talk Mode) has also been provided by RAN4 and this process has been facilitated by a coordinator across the two working groups. As done during Rel-17, an LS [1] was sent to RAN4 to consider providing the core requirements along with TT recommendations as a package for the Talk Mode requirements.

In response to an LS from RAN5, RAN4 discussed and provided core requirements and TT recommendation as a package via the LS response in [11] to be used in the Rel-18 work by RAN5.

Observation 1: RAN4 responded with a recommended TT, included as a package with the Rel-18 core requirements in the LS response provided in [11].

The below snippet highlights the key Rel-18 TT recommendation from RAN4 in

Observation 2: RAN5 can utilize the recommended TT provided by RAN4 (as ratio of MU) for Talk mode FR1 TRP TRS testing. This has already been done for Browsing Mode in Rel-17 which remains as-is.

Of course, RAN5 shall utilize the finalized MU values for Talk Mode testing in [12] and finalized at RAN5#106 as indicated below.

## 2.2 Finalize Rel-18 NR FR1 TRP TRS

Annex D of the FR1 OTA test specification already contains the framework for addition of MU and TT. There we can update the same with the missing TT values for Rel-18 (Talk Mode).

Proposal: Agree and update the Test Tolerance for FR1 TRP TRS testing in Talk Mode.

# Summary

Observation 1: RAN4 responded with a recommended TT, included as a package with the Rel-18 core requirements in the LS response provided in [11].

Observation 2: RAN5 can utilize the recommended TT provided by RAN4 (as ratio of MU) for Talk mode FR1 TRP TRS testing. This has already been done for Browsing Mode in Rel-17 which remains as-is.

Proposal: Agree and update the Test Tolerance for FR1 TRP TRS testing in Talk Mode.

# References

R5-223638, LS to RAN4 on TT work for NR FR1 TRP TS, RAN95e, May9th – May 20th, 2022

R4-2214797/R5-225919, Reply LS on TT and requirements, Vivo, RAN4#104e, August 15th- August 25th, 2022

R5-221623, TP to TR 38.834 for Annex B MU tables, Rohde and Schwarz, RAN4#102e, February 21st – March 3rd, 2022

R4-2212817 – Outcome of 3GPP Lab Alignment Activity. Vivo, RAN4#104e, August 16th – August 25th, 2022

R5-227314 – MU contributors and workplan for TRP-TRS, Rohde and Schwarz, RAN5#97, Toulouse, France; November 14th- 18th, 2022,

TS 38.161, User Equipment (UE) TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements; Range 1 Standalone and Range 1 Interworking operation with other radios (Release 17)

TR 38.834, Measurements of User Equipment (UE) Over-the-Air (OTA) performance for NR FR1; Total Radiated Power (TRP) and Total Radiated Sensitivity (TRS) test methodology (Release 17)

R5-227822, Views on MU TT for NR FR1 TRP TRS, RAN5#97, Toulouse, France; November 14th- 18th, 2022,

R5-237843, Discussion on MU TT milestones for FR1 TRP TRS, Apple, RAN5#101, November 13th-16th, 2023

R5-241423, "Views on TRP TRS MU TT", 3GPP TSG RAN5 Meeting #102, Athens, Greece, 26th Feb – Mar 1st, 2024

- [11] R5-242118, " Reply LS on TT work for Rel-18 NR FR1 TRP TRS, 3GPP TSG RAN5 Meeting #103, Fukuoka, Japan, May17th - May 21, 2024

- [12] R5-251722, " MU contents for BHH and WR", RAN5#106, Athens, Greece, Feb 17th - Feb 21st, 2025