3GPP TSG-RAN5 Meeting #107	R5-253043

# Malta, Malta, 19th – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.579-7 | CR | 0004 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to test case 6.7.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, FirstNet, Element, Samsung R&D Institute UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc17_enh3MCPTT_eMCData3_MCOver5GS-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | When this test case was added, in Rel-16, IP connectivity in was besed on GRE (Generic Routing Encapsulation) for the IP tunnel between two MCData clients. For Rel-17 the core requirements were updated to use GRE-in-UDP Encapsulation. As this test case was never implemented in TTCN for Rel-16, it can be updated to Rel-17. See discussion paper R5-246421. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test case updated to Rel-17 requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 37.579-4 CR 002 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the final version of R5-252756 |  |  |  |  |  |  |  |  |

### 6.7.1	On-network / IP Connectivity / Client Originated (CO)

6.7.1.1	Test Purpose (TP)

(1)

with { UE (MCDATA Client) registered and authorised for MCDATA Service }

ensure that {

when { the MCDATA User requests to initiate a IP Connectivity session }

then { UE (MCDATA Client) sends a request to establish a IP Connectivity session via a SIP INVITE message and then responds to the SIP 200 (OK) message with a SIP ACK message and sends a payload using GRE-in-UDP Encapsulation }

}

(2)

with { UE (MCDATA Client) having established a IP Connectivity session }

ensure that {

when { the MCDATA User requests to release the IP Connectivity session }

then { UE (MCDATA Client) sends a SIP BYE message }

}

6.7.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.282, clause 20.2.1 and 20.2.0a, TS 24.582, clause 13.1.2. Unless otherwise stated, these are Rel-17 requirements.

[TS 24.282, clause 20.2.1]

When a MCData client receives the request by a user or user application to establish a IP Connectivity session with another MCData client the MCData client shall generate a SIP INVITE request in accordance with 3GPP TS 24.229 [5] with the clarifications given below. The MCData ID or the functional alias of the target MCData client may be explicitly included in the request from the user or user application. If the target MCData ID or functional alias is not included in the request, the MCData client may implicitly determine the target MCData ID by using the target IP Information included in the request to find a match in the One-to-One communication list of the MCData user profile document as specified in 3GPP TS 24.484 [12]. If the MCData ID of the target MCData client is determined implicitly by the target IP Information included in the request, the client searches in leaves below /<x>/<x>/Common/OnetoOne/UserList/<x>/Entry/IPInformation/<x>/Entry/ for a match in the IP Information. The MCData ID is given by matching the user entry.

The MCData client:

1)	shall include the g.3gpp.mcdata.ipconn  media feature tag and the g.3gpp.icsi-ref media feature tag with the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.ipconn " in the Contact header field of the SIP INVITE request according to IETF RFC 3840 [16];

2)	shall include an Accept-Contact header field containing the g.3gpp.mcdata.ipconn media feature tag along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [8];

3)	shall include an Accept-Contact header field with the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.ipconn" along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [8];

4)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcdata.ipconn" (coded as specified in 3GPP TS 24.229 [5]), in a P-Preferred-Service header field according to IETF RFC 6050 [7] in the SIP INVITE request;

5)	should include the "timer" option tag in the Supported header field;

6)	should include the Session-Expires header field according to IETF RFC 4028 [38]. It is recommended that the "refresher" header field parameter is omitted. If included, the "refresher" header field parameter shall be set to "uac";

7)	shall insert in the SIP INVITE request a MIME resource-lists body with the MCData ID of the invited MCData user or the functional alias, according to rules and procedures of IETF RFC 5366 [18];

8)	shall contain an application/vnd.3gpp.mcdata-info+xml MIME body with the <mcdatainfo> element containing the <mcdata-Params> element with:

a)	the <request-type> element set to a value of "one-to-one-ipconn";

b)	the <call-to-functional-alias-ind> set to "true" if the functional alias is used as a target of the communication request;

c)	if the MCData client is aware of active functional aliases and if an active functional alias is to be included in the SIP INVITE request, the <functional-alias-URI> element set to the URI of the used functional alias; and

d)	if the MCData user has requested an application priority, the <anyExt> element with the <user-requested-priority> element set to the user provided value.

9)	shall set the Request-URI of the SIP INVITE request to the public service identity identifying the participating MCData function serving the MCData user;

NOTE 1:	The MCData client is configured with public service identity identifying the participating MCData function serving the MCData user.

10)	may include a P-Preferred-Identity header field in the SIP INVITE request containing a public user identity as specified in 3GPP TS 24.229 [5];

11)	shall include an SDP offer according to 3GPP TS 24.229 [5] with the clarifications given in clause 20.2.0a; and

12)	shall send the SIP INVITE request towards the MCData server according to 3GPP TS 24.229 [5].

On receipt of a SIP 2xx response to the SIP INVITE request, the MCData client:

1)	shall send a SIP ACK request as specified in 3GPP TS 24.229 [5];

2)	shall start the SIP Session timer according to rules and procedures of IETF RFC 4028 [38]; and

3)	shall interact with the media plane as specified in 3GPP TS 24.582 [15] clause 13.1.2.

Upon receiving a SIP 300 (Multiple Choices) response to the SIP INVITE request the MCData client shall use the MCData ID of MCData user contained in the <mcdata-request-uri> element of the received application/vnd.3gpp.mcdata-info MIME body as the MCData ID of the invited MCData user and shall generate an initial SIP INVITE request by following the UE originating session procedures specified in 3GPP TS 24.229 [5], with the clarifications given in this clause and with the following additional clarifications:

1)	shall insert in the newly generated SIP INVITE request a MIME resource-lists body with the MCData ID of the invited MCData user in the <mcdata-request-uri> element of the application/vnd.3gpp.mcdata-info MIME body  in the received SIP 300 (Multiple Choices) response;

2)	shall not include a <call-to-functional-alias-ind> element into the <mcdata-Params> element of the <mcdatainfo> element of the application/vnd.3gpp.mcdata-info+xml MIME body; and

3)	shall include a <called-functional-alias-URI> element into the <mcdata-Params> element of the <mcdatainfo> element of the application/vnd.3gpp.mcdata-info+xml MIME body with the target functional alias used in the initial SIP INVITE request for the IP connectivity session establishment.

On receipt of a SIP 4xx response, a SIP 5xx response or a SIP 6xx response to the SIP INVITE request, the MCData client:

1)	shall indicate to the MCData user or user application that the IP Connectivity session could not be established; and

2)	shall send a SIP ACK request as specified in 3GPP TS 24.229 [5].

On receipt of an indication from the media plane indicating that the IP Connectivity session could not be established, the MCData client:

1)	shall generate a SIP BYE request according to 3GPP TS 24.229 [5] with:

a)	Reason code set to "FAILURE_CAUSE";

b)	cause set to "1"; and

c)	text set to "Media bearer or QoS lost";

2)	shall set the Request-URI to the MCData session identity to release; and

3)	shall send a SIP BYE request towards MCData server according to 3GPP TS 24.229 [5].

[TS 24.282, clause 20.2.0a]

The SDP offer shall contain one SDP media-level section for MCData including an attribute for IP Connectivity according to 3GPP TS 24.582 [15]. When composing an SDP offer the MCData client shall:

1)	set the IP address of the MCData client for the offered MCData IP Connectivity session; and

NOTE:	The MC service operator policy determines if the MCData client can use an already assigned IP address or can request a new IP address following the procedures defined in 3GPP TS 24.301 [43].

2)	shall include an "m=application" media-level section as specified in 3GPP TS 24.582 [15] consisting of:

a)	the port number selected for the media plane as specified in 3GPP TS 24.582 [15] clause 13.5; and

b)	the 'fmtp' attribute as specified in 3GPP TS 24.582 [15] clause 13.6.

[TS 24.582, clause 13.1.2]

Upon receiving a request by an MCData user, or an IP packet from an IP application, the MCData client shall follow the procedure in 20.2.1 in 3GPP TS 24.282 [8]. The IP address and port number received in the SDP payload of the 200 OK response in this procedure shall be used to establish an IP tunnel. The IP tunnel shall be based on GRE-in-UDP Encapsulation as specified in IETF RFC 8086 [23], and as specified in clause 13.4. Generic Routing Encapsulation (GRE) as specified in IETF RFC 2784 [19] is used as a basis of GRE-in-UDP Encapsulation.

The MCData client shall perform encapsulation and decapsulation according to clause 13.4.

The MCData client acts as an IP relay for IP traffic between the IP application and the IP tunnel to the far endpoint. Once the IP tunnel is established, the IP applications can exchange IP data. The MCData client that receives the IP packets from the IP application shall perform encapsulation to the tunnelling protocol adding a GRE header and a UDP header to the IP packet, and send the outgoing UDP traffic to the IP address and port present in the SDP answer. When the originating MCData client receives IP packets from the IP tunnel it shall perform de-capsulation from the tunnelling protocol, removing the UDP header and the GRE header from the received packet, before passing the IP data to the IP application.

6.7.1.3	Test description

6.7.1.3.1	Pre-test conditions

Test configuration:

-	Single cell configuration according to TS 37.579-1 [2] clause 5.2.2.2.1.

Preamble:

-	The UE has performed procedure 'Initial registration' as described in TS 37.579-1 [2] clause 5.4.2.

-	UE States at the end of the preamble:

-	The UE is in RRC_IDLE state.

-	The client is registered for MCData.

6.7.1.3.2	Test procedure sequence

Table 6.7.1.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the UE (MCData client) establish a IP Connectivity session with another MCData client. (NOTE 1) | - | - | - | - |
| 2-6 | Check: Does the UE (MCData client) correctly perform steps 1 through 5 of procedure 'CO MCData Call Establishment' as described in TS 37.579-1 [2] Table 5.3C.2.3-1 to establish an IP Connectivity session with another MCData client? | - | - | 1 | P |
| 6A | Make the application at the UE send data through the GRE-in-UDP tunnel. (NOTE 1) | - | - | - | - |
| - | EXCEPTION: The communication described in in step 7 is transmitted over an IP connection that has been established based on GRE-in-UDP Encapsulation. | - | - | - | - |
| 7 | Check: Does the UE (MCData client) correctly send a payload packet via the GRE-in-UDP tunnel? | - | - | 1 | P |
| 8 | Make the UE (MCData client) release the IP Connectivity session. (NOTE 1) | - | - | - | - |
| 9 | Check: Does the UE (MCData client) correctly perform procedure 'CO MCData call release' as described in TS 37.579-1 [2] Table 5.3C.6.3-1? (NOTE 2) | - | - | 2 | P |
| 10 | The procedure 'MCX communication release' as described in TS 37.579-1 [2] clause 5.4.14 is performed to release the RRC connection. | - | - | - | - |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. NOTE 2: The procedure does not release the RRC connection. |  |  |  |  |  |

6.7.1.3.3	Specific message contents

Table 6.7.1.3.3-1: SIP INVITE from the UE (step 3, Table 6.7.1.3.2-1;
step 2, TS 37.579-1 [2] Table 5.3C.2.3-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.2.5.1-1, condition MCDATA_IPCONN, MCD_1to1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | SDP message |  |  |
| MIME-part-body | As described in Table 6.7.1.3.3-2 |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.7.1.3.3-3 |  |  |  |

Table 6.7.1.3.3-2: SDP for SIP INVITE (Table 6.7.1.3.3-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.3.1.1-3, condition MCDATA_SDS, SDP_OFFER, SDS_SESSION, MCD_1to1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Media description[1] |  | Media description for data |  |  |
| media description |  | m= line media = application |  |  |
| media | “application” |  | TS 24.582 [32] clause 13.5 |  |
| port | any allowed value |  | TS 24.582 [32] clause 13.5 |  |
| proto | “udp” |  | TS 24.582 [32] clause 13.5 |  |
| fmt | “MCDATA” |  | TS 24.582 [32] clause 13.5 |  |
| media attribute |  | a= line attribute = fmtp |  |  |
| fmtp | "fmtp" |  | TS 24.582 [32] clause 13.5 |  |
| fmtp-attr-ipconn | “MCDATA” |  | TS 24.582 [32] clause 13.5 |  |
| attr-param | "mcdata-ipconn-s-port=<any allowed value>” | UDP source port of the GRE-in-UDP tunnel | TS 24.582 [32] clause 13.5 |  |

Table 6.7.1.3.3-3: MCData-Info (Table 6.7.1.3.3-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.3.2.1-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| request-type | "one-to-one-ipconn" |  |  |  |

Table 6.7.1.3.3-4: SIP 200 (OK) from the SS (step 5, Table 6.7.1.3.2-1;
step 4, TS 37.579-1 [2] Table 5.3C.2.3-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.2.17.1.2-1, condition INVITE-RSP, MCDATA_IPCONN |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| SDP message | As described in Table 6.1.5.9.3-5 |  |  |  |

Table 6.7.1.3.3-5: SDP for SIP 200 (OK) (Table 6.7.1.3.3-4)

| Derivation Path: TS 37.579-1 [2], Table 5.5.3.1.2-3, condition MCDATA_SDS, SDP_ANSWER, SDS_SESSION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Media description[1] |  | Media description for data |  |  |
| media description |  | m= line media = application |  |  |
| media | “application” |  | TS 24.582 [32] clause 13.5 |  |
| port | same number as indicated by the <attr-param> parameter in Table 6.7.1.3.3-2 |  |  |  |
| proto | “udp” |  | TS 24.582 [32] clause 13.5 |  |
| fmt | “MCDATA” |  | TS 24.582 [32] clause 13.5 |  |
| media attribute |  | a= line attribute = fmtp |  |  |
| fmtp | "fmtp" |  | TS 24.582 [32] clause 13.5 |  |
| fmtp-attr-ipconn | “MCDATA” |  | TS 24.582 [32] clause 13.5 |  |
| attr-param | "mcdata-ipconn-s-port=<port number assigned by the SS>” | UDP source port of the GRE-in-UDP tunnel | TS 24.582 [32] clause 13.5 |  |

Table 6.7.1.3.3-6: GRE-in-UDP encapsulated user data packet (step 7, Table 6.7.1.3.2-1)

| Derivation Path: 24.582 [32], Clause 13.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| GRE header |  |  |  |
| C | 0 |  |  |
| Reserved | 0 |  |  |
| K | 0 |  |  |
| S | 0 |  |  |
| Reserved | 0000 |  |  |
| Reserved | 00000 |  |  |
| Ver | 000 |  |  |
| Protocol Type | ‘0800’H |  | IPv4 |
| Protocol Type | ‘86DD’H |  | IPv6 |
| Payload packet |  |  |  |
| Payload packet | valid IP packet | Editor's note: It needs to be clarified what "valid IP packet" means and how this shall be checked (e.g. destination address may be checked to be px_MCData_IPConnectivityEndpointAddress_B) |  |