3GPP TSG-RAN5 Meeting #107	R5-253075

# Malta, Malta, May.19th – 23rd 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 5028 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Cov_Enh SIG test case 7.1.1.3.14.4 - PUSCH repetition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_test, NR_cov_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - To correct Cov_Enh SIG test case 7.1.1.3.14.4 based on comments received: - availableSlotCounting-r17 in Table 7.1.1.3.14.4.3.3-3 should be configured as enabled, rather than true |  |  |  |  |  |  |  |  |
| Summary of change: |  | - availableSlotCounting-r17 is configured as enabled in Table 7.1.1.3.14.4.3.3-3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work plan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.3.14.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from: R5-252739 |  |  |  |  |  |  |  |  |

<Start of Change 1>

7.1.1.3.14.4	Correct Handling of UL HARQ process / PUSCH Repetition Type A enhancement / repetition based on available slots / configured grant

7.1.1.3.14.4.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state and is configured with availableSlotCounting-r17.}

ensure that {

when { UE has data available for transmission, UE is provided a UL configured grant with repK }

then { UE transmits repetitions of PUSCH on available UL slots and selects the redundancy version correctly }

}

7.1.1.3.14.4.2	Conformance requirements

<Unchanged contents are omitted>

7.1.1.3.14.4.3	Test description

7.1.1.3.14.4.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except that DRB is configured in RLC AM mode according to Table 7.1.1.3.14.4.3.1-1.

Table 7.1.1.3.14.4.3.1-1: RLC parameters

| t-PollRetransmit | ms80 |
| --- | --- |

7.1.1.3.14.4.3.2	Test procedure sequence

Table 7.1.1.3.14.4.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits an RRCReconfiguration message including ConfiguredGrantConfig and availableSlotCounting-r17. NOTE: For EN-DC. the RRCReconfiguration message is embedded in RRCConnectionReconfiguration specified in 36.508 [7] Table 4.6.1-8 with condition EN-DC_EmbedNR_RRCRecon. | <--- | NR RRC: RRCReconfiguration | - | - |
| 2 | UE transmits an RRCReconfigurationComplete message. NOTE: For EN-DC, the RRCReconfigurationComplete message is embedded in RRCConnectionReconfigurationComplete specified in 36.508 [7] Table 4.6.1-9 with condition MCG_and_SCG | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 3 | IF pc_type1_PUSCH_RepetitionMultiSlots = FALSE, the SS sends a DCI to activate the configured grant which is sufficient to transmit the MAC SDU to loop back in a slot. |  |  |  |  |
| 4 | The SS transmits a valid MAC PDU containing one RLC PDU. | <--- | MAC PDU | - | - |
| 5 | Check: Does the UE transmit a MAC PDU including one RLC SDU and repeat transmitting this MAC PDU in the next 3 slots which are suitable for UL transmission with same resource allocation but different redundancy version? NOTE: The redundancy version for the first transmission and all possible repetitions are set in the following order {0, 2, 3, 1} according to repK-RV. Usage of correct redundancy version is implicitly checked upon correct decoding by the SS of the UE UL repetitions | --> | MAC PDU | 1 | P |
| 6 | SS transmits a MAC PDU containing an RLC STATUS PDU acknowledging the reception of the AMD PDU in step 4. | <-- | MAC PDU (RLC STATUS PDU) | - |  |

7.1.1.3.14.4.3.3	Specific message contents

Table 7.1.1.3.14.4.3.3-1: RRCReconfiguration (Step 1, Table 7.1.1.3.14.4.3.2-1)

| Derivation Path: 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| secondaryCellGroup | CellGroupConfig | Table 7.1.1.3.14.4.3.3-2 | EN-DC |
|  | Not present |  | NR |
| nonCriticalExtension | Not present |  | EN-DC |
| nonCriticalExtension SEQUENCE { |  |  | NR |
| masterCellGroup | CellGroupConfig | Table 7.1.1.3.14.4.3.3-2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.3.14.4.3.3-2: CellGroupConfig (Table 7.1.1.3.14.4.3.3-1)

| Derivation Path: 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| cellGroupConfig::= SEQUENCE { |  |  |  |
| cellGroupId | 0 |  | NR |
|  | 1 |  | EN-DC |
| spCellConfig SEQUENCE { |  |  |  |
| servCellIndex | Not present |  | NR |
|  | 1 |  | EN-DC |
| spCellConfigDedicated SEQUENCE { |  |  |  |
| servingCellConfig | ServingCellConfig | Table 7.1.1.3.14.4.3.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.3.14.4.3.3-3: ServingCellConfig (Table 7.1.1.3.14.4.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP SEQUENCE { |  |  |  |
| pusch-Config CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| availableSlotCounting-r17 | enabled |  |  |
| } |  |  |  |
| } |  |  |  |
| configuredGrantConfig CHOICE { |  |  |  |
| setup | ConfiguredGrantConfig | Table 7.1.1.3.14.4.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.3.14.4.3.3-5: ConfiguredGrantConfig (Table 7.1.1.3.14.4.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-26 with condition CG_Config_Type1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ConfiguredGrantConfig ::= SEQUENCE { |  |  |  |
| repK | n4 |  |  |
| repK-RV | s1-0231 |  |  |
| rrc-ConfiguredUplinkGrant | Not present |  | NOT pc_type1_PUSCH_RepetitionMultiSlots |
| } |  |  |  |

<End of Change 1>