3GPP TSG-RAN5 Meeting #107	R5-253411

# Malta, May 19 - 23 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3502 | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of test frequencies for n104 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_lic_bands_BW_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2025-04-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Addition of test frequencies for n104. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of test frequencies for n104. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No available test frequencies for n104. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.1.1.104 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Final version, originates from R5-252585r1 |  |  |  |  |  |  |  |  |

##### << Beginning of changes>>

Table 4.3.1.1.1.101-2: Test frequencies for NR operating band n101 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink & Uplink | Low | 1905 | 381000 | 1900,68 | 380136 | 0 | 15 | 4762 | 380930 | 6 | 0 | 3 (8) | 16 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.1.1.102 to 4.3.1.1.1.103	FFS

4.3.1.1.1.104	Reference test frequencies for NR operating band n104

Table 4.3.1.1.1.104-1: Test frequencies for NR operating band n104 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20 | 106 | Downlink | Low | 6435.00 | 829000 | 6425.46 | 828364 | 0 | 30 | 9882 | 828768 | 8 | 9 | 0 (4) | 13 |
|  |  | & | Mid | 6774.99 Note 4 | 851666 | 6747.09 | 849806 | 102 |  | 10120 | 851616 | 10 | 24 | 0 (4) | 130 |
|  |  | Uplink | High | 7114.98 Note 4 | 874332 | 7014.72 | 867648 | 504 |  | 10358 | 874464 | 0 | 40 | 0 (4) | 548 |
| 30 | 160 | Downlink | Low | 6440.01 | 829334 | 6425.61 | 828374 | 0 | 30 | 9882 | 828768 | 10 | 8 | 0 (4) | 12 |
|  |  | & | Mid | 6774.99 Note 4 | 851666 | 6742.23 | 849482 | 102 |  | 10120 | 851616 | 10 | 51 | 0 (4) | 157 |
|  |  | Uplink | High | 7110.00 | 874000 | 7004.88 | 866992 | 504 |  | 10351 | 873792 | 8 | 38 | 0 (4) | 546 |
| 40 | 216 | Downlink | Low | 6445.02 Note 3 | 829668 | 6425.58 | 828372 | 0 | 30 | 9882 | 828768 | -0 | 9 | 0 (4) | 13 |
|  |  | & | Mid | 6774.99 Note 4 | 851666 | 6737.19 | 849146 | 102 |  | 10113 | 850944 | 10 | 23 | 0 (4) | 129 |
|  |  | Uplink | High | 7104.99 | 873666 | 6994.83 | 866322 | 504 |  | 10344 | 873120 | 6 | 38 | 0 (4) | 546 |
| 50 | 270 | Downlink | Low | 6450.00 | 830000 | 6425.70 | 828380 | 0 | 30 | 9882 | 828768 | 4 | 8 | 0 (4) | 12 |
|  |  | & | Mid | 6774.99 Note 4 | 851666 | 6732.33 | 848822 | 102 |  | 10113 | 850944 | 10 | 50 | 0 (4) | 156 |
|  |  | Uplink | High | 7099.98 Note 4 | 873332 | 6984.96 | 865664 | 504 |  | 10337 | 872448 | 4 | 37 | 0 (4) | 545 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-5 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: Centre frequency shifted 15 kHz to align with the synchronisation raster (According to R1-1809879). Note 4: Centre frequency shifted -15 kHz to align with the synchronisation raster (According to R1-1809879). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.104-2: Test frequencies for NR operating band n104 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20 | 51 | Downlink | Low | 6435.00 | 829000 | 6425.82 | 828388 | 0 | 30 | 9882 | 828768 | 20 | 1 | 1 (4) | 10 |
|  |  | & | Mid | 6774.99 | 851666 | 6729.09 | 848606 | 102 |  | 10120 | 851616 | 10 | 9 | 1 (4) | 230 |
|  |  | Uplink | High | 7114.98 | 874332 | 6924.36 | 861624 | 504 |  | 10358 | 874464 | 0 | 17 | 1 (4) | 1050 |
| 30 | 78 | Downlink | Low | 6440.01 | 829334 | 6425.97 | 828398 | 0 | 30 | 9882 | 828768 | 10 | 1 | 1 (4) | 10 |
|  |  | & | Mid | 6774.99 | 851666 | 6724.23 | 848282 | 102 |  | 10120 | 851616 | 22 | 22 | 1 (4) | 256 |
|  |  | Uplink | High | 7110.00 | 874000 | 6914.52 | 860968 | 504 |  | 10351 | 873792 | 8 | 16 | 1 (4) | 1048 |
| 40 | 106 | Downlink | Low | 6445.02 | 829668 | 6425.94 | 828396 | 0 | 30 | 9882 | 828768 | 12 | 1 | 1 (4) | 10 |
|  |  | & | Mid | 6774.99 | 851666 | 6719.19 | 847946 | 102 |  | 10113 | 850944 | 22 | 8 | 1 (4) | 228 |
|  |  | Uplink | High | 7104.99 | 873666 | 6904.47 | 860298 | 504 |  | 10344 | 873120 | 6 | 16 | 1 (4) | 1048 |
| 50 | 133 | Downlink | Low | 6450.00 | 830000 | 6426.06 | 828404 | 0 | 30 | 9882 | 828768 | 4 | 1 | 1 (4) | 10 |
|  |  | & | Mid | 6774.99 | 851666 | 6714.33 | 847622 | 102 |  | 10113 | 850944 | 10 | 22 | 1 (4) | 256 |
|  |  | Uplink | High | 7099.98 | 873332 | 6894.60 | 859640 | 504 |  | 10337 | 872448 | 16 | 15 | 1 (4) | 1046 |
| 60 | 162 | Downlink | Low | 6455.01 | 830334 | 6425.85 | 828390 | 0 | 30 | 9882 | 828768 | 18 | 1 | 1 (4) | 10 |
|  |  | & | Mid | 6774.99 | 851666 | 6709.11 | 847274 | 102 |  | 10106 | 850272 | 22 | 8 | 1 (4) | 228 |
|  |  | Uplink | High | 7095.00 | 873000 | 6884.40 | 858960 | 504 |  | 10330 | 871776 | 0 | 16 | 1 (4) | 1048 |
| 70 | 189 | Downlink | Low | 6460.02 | 830668 | 6426.00 | 828400 | 0 | 30 | 9882 | 828768 | 8 | 1 | 1 (4) | 10 |
|  |  | & | Mid | 6774.99 | 851666 | 6704.25 | 846950 | 102 |  | 10106 | 850272 | 10 | 22 | 1 (4) | 256 |
|  |  | Uplink | High | 7089.99 | 872666 | 6874.53 | 858302 | 504 |  | 10323 | 871104 | 10 | 15 | 1 (4) | 1046 |
| 80 | 217 | Downlink | Low | 6465.00 | 831000 | 6425.94 | 828396 | 0 | 30 | 9882 | 828768 | 12 | 1 | 1 (4) | 10 |
|  |  | & | Mid | 6774.99 | 851666 | 6699.21 | 846614 | 102 |  | 10099 | 849600 | 10 | 8 | 1 (4) | 228 |
|  |  | Uplink | High | 7084.98 | 872332 | 6864.48 | 857632 | 504 |  | 10316 | 870432 | 8 | 15 | 1 (4) | 1046 |
| 90 | 245 | Downlink | Low | 6470.01 | 831334 | 6425.91 | 828394 | 0 | 30 | 9882 | 828768 | 14 | 1 | 1 (4) | 10 |
|  |  | & | Mid | 6774.99 | 851666 | 6694.17 | 846278 | 102 |  | 10099 | 849600 | 10 | 22 | 1 (4) | 256 |
|  |  | Uplink | High | 7080.00 | 872000 | 6854.46 | 856964 | 504 |  | 10309 | 869760 | 4 | 15 | 1 (4) | 1046 |
| 100 | 273 | Downlink | Low | 6475.02 | 831668 | 6425.88 | 828392 | 0 | 30 | 9882 | 828768 | 16 | 1 | 1 (4) | 10 |
|  |  | & | Mid | 6774.99 | 851666 | 6689.13 | 845942 | 102 |  | 10092 | 848928 | 10 | 8 | 1 (4) | 228 |
|  |  | Uplink | High | 7074.99 | 871666 | 6844.41 | 856294 | 504 |  | 10302 | 869088 | 2 | 15 | 1 (4) | 1046 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-6 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.1.1.105	Reference test frequencies for NR operating band n105

##### << End of changes>>