3GPP TSG-RAN5 Meeting #107	R5-253463

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 1165 | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to FR2 RF test spec Clause 6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Inc |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR2_req_Ph3-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Alignment needed in test spec with core specifications |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update clause in alignment with core specifications |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specifications will be misaligned with core specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Alignment with TS 38.101-2 V18.9.0 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1 still aligns with core spec but adds content not currently present in the test specification. |  |  |  |  |  |  |  |  |

<Unchanged sections skipped>

##### 6.2A.2.0.2	Maximum output power reduction for power class 1

For power class 1, MPR for intra-band contiguous UL CA with contiguous allocations within the cumulative aggregated bandwidth is defined as:

MPRC_CA = max(MPRWT_C_CA+∆MPR, MPRnarrow)

Where,

MPRnarrow = 14.4 dB, when BWalloc,RB is less than or equal to 1.44 MHz, MPRnarrow = 10 dB, when 1.44 MHz < BWalloc,RB ≤ 10.8 MHz, where BWalloc,RB is the bandwidth of the RB allocation size.

MPRWT_C_CA is the maximum power reduction due to modulation orders, transmit bandwidth configurations, and waveform types. MPRWT_C_CA is defined in Table 6.2A.2.0.2-1.

∆MPR for 256 QAM as specified in Table 6.2.2.3.1-3 applies.

Table 6.2A.2.0.2-1: Maximum power reduction (MPRWT_C_CA) for UE power class 1

| Waveform Type |  | Cumulative aggregated channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | < 400 MHz | ≥ 400 MHz and < 800 MHz | ≥ 800 MHz and ≤ 1400 MHz | > 1400 MHz and ≤ 2400 MHz |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 5.5 | ≤ 7.7 | ≤ 8.2 | ≤ 8.7 |
|  | QPSK | ≤ 6.5 | ≤ 8.7 | ≤ 9.7 | ≤ 9.7 |
|  | 16 QAM | ≤ 6.5 | ≤ 8.7 | ≤ 9.2 | ≤ 9.7 |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ 11.2 | ≤ 11.7 |
|  | 256 QAM2 | ≤ 12.5 | ≤ 14.2 | ≤ 14.7 | ≤ 15.7 |
| CP-OFDM | QPSK | ≤ 6.5 | ≤ 8.7 | ≤ 8.7 | ≤ 9.7 |
|  | 16 QAM | ≤ 6.5 | ≤ 8.7 | ≤ 8.7 | ≤ 9.7 |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ 11.2 | ≤ 11.7 |
|  | 256 QAM2 | ≤ 12.5 | ≤ 14.2 | ≤ 14.7 | ≤ 15.7 |
| NOTE 1: (Void) NOTE 2: Refer to clause 6.1 for 256 QAM applicability. |  |  |  |  |  |

In case of a contiguous RB, DFT-s-BPSK or DFT-s-QPSK UL allocation in a single CC of a CA configuration with contiguous CCs, and whose cumulative aggregated BW  400 MHz, MPRWT_C_CA shall be derived instead as MAX(MPR1, MPR2), where:

MPR1 shall be determined from Table 6.2.2.3.1-1 if CABW  200 MHz, from Table 6.2.2.3.1-2 if CABW > 200 MHz.

MPR2 shall be determined from Table 6.2.2.3.1-1 if UL BWchannel_CA  200 MHz, from Table 6.2.2.3.1-2 if UL BWchannel_CA > 200 MHz.

and assume all UL CCs use the same SCS for the purpose of determination of inner and outer RB allocations in Table 6.2.2.3.1-1 and Table 6.2.2.3.1-2:

NRB shall be chosen as the sum of NRB of all constituent UL CCs in the CA configuration.

LCRB shall be chosen as BWalloc,RB

RBstart shall be derived as: RBstart_allocatedCC+NRB_unallocatedCC_low

RBstart_allocatedCC is the index of the first allocated RB in the CC with allocation

NRB_unallocatedCC_low is the sum of NRB in all UL CCs lower in frequency compared to the CC with allocation

When different waveform types exist across CCs, the requirement is set by the waveform type used in the configuration with the largest MPRC_CA.

For intra-band contiguous UL CA with non-contiguous RB allocations, the following rule for MPR applies:

MPR = max(MPRC_CA, -10*A + 14.4)

Where:

A = NRB_alloc / NRB_agg_C.

NRB_alloc is the total number of allocated UL RBs

NRB_agg_C is the number of the aggregated RBs within the fully allocated cumulative aggregated channel bandwidth assuming lowest SCS among all configured CCs

For intra-band non-contiguous UL CA, the following rule for MPR applies:

MPR = max(MPRNC_CA+∆MPR, -10*A +  14.4)

Where:

MPRNC_CA is derived from table 6.2A.2.0.2-2

∆MPR as specified in Table 6.2.2.3.1-3 applies.

A = NRB_alloc / NRB_agg_C.

NRB_alloc is the total number of allocated UL RBs

NRB_agg_C is the number of the aggregated RBs within the fully allocated cumulative aggregated channel bandwidth assuming lowest SCS among all configured CCs

Table 6.2A.2.0.2-2: MPRNC_CA for UE power class 1

| Waveform Type |  | Cumulative aggregated channel bandwidth (CABW) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | < 400 MHz | ≥ 400 MHz and < 800 MHz | ≥ 800 MHz and ≤ 1400 MHz | > 1400 MHz and ≤ 2400 MHz |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 6 | ≤ 7.7 | ≤ 8.2 | ≤ 8.7 |
|  | QPSK | ≤ 7 | ≤ 8.7 | ≤ 9.2 | ≤ 9.7 |
|  | 16 QAM | ≤ 7 | ≤ 8.7 | ≤ 9.2 | ≤ 9.7 |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ 11.2 | ≤ 11.7 |
|  | 256 QAM1 | ≤ 12.5 | ≤ 14.2 | ≤ 14.7 | ≤ 15.7 |
| CP-OFDM | QPSK | ≤ 7 | ≤ 8.7 | ≤ 9.2 | ≤ 9.7 |
|  | 16 QAM | ≤ 7 | ≤ 8.7 | ≤ 9.2 | ≤ 9.7 |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ 11.2 | ≤ 11.7 |
|  | 256 QAM1 | ≤ 12.5 | ≤ 14.2 | ≤ 14.7 | ≤ 15.7 |
| NOTE 1: Refer to clause 6.1 for 256 QAM applicability. |  |  |  |  |  |

When different waveform types exist across CCs, the requirement is set by the waveform type used in the configuration with the largest MPRNC_CA.

For inter-band carrier aggregation with uplink assigned to two NR bands, the MPR for each configured UL band in the UL CA band combination is:

MPRinter-band_CA = max(MPRSingleBand, MPRPA-PA)

Where:

MPRSingleBand is the MPR specified in clause 6.2.2.1for the allocation and modulation type in that band

MPRPA-PA is MAX(MPR1, MPR2), where MPR1 and MPR2 are specified per band combination in Table 6.2A.2.2.2.3-1 and applies only when both bands have non-zero power UL RB allocations, 0 dB otherwise.

Table 6.2A.2.2.3-1: MPRPA-PA for Inter-band ULCA in FR2 for PC1

| NR CA Band | MPR | Value (dB) | Condition |
| --- | --- | --- | --- |
| CA_n260A-n261A | MPR1 | Max(0, 10 - 10*log10(Max(1.0, LRB,min*12*SCS/1e6))) | LRB,min = Min (LRB,n260 , LRB,n261 ), where LRB,n is the number of non-zero power UL RBs in band ‘n’ |
|  | MPR2 | 6.0 if condition satisfied, 0.0 otherwise | 47.2 GHz <= 2*fn260 - fn261 <= 48.2 GHz Where fn is any frequency inside the UL allocation in band ‘n’ |

<Unchanged sections skipped>

##### 6.2A.2.0.4	Maximum output power reduction for power class 3

For power class 3, MPR for intra-band contiguous UL CA with contiguous allocations within the cumulative aggregated bandwidth is denoted as MPRC_CA and is defined in Table 6.2A.2.0.4-1.

Table 6.2A.2.0.4-1: Maximum power reduction (MPRC_CA) for UE power class 3

|  |  | Cumulative aggregated bandwidth configuration (CABW) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | ≤ 400 MHz | > 400 MHz and < 800 MHz | ≥ 800 MHz and ≤ 1400 MHz | > 1400 MHz and ≤ 2400 MHz |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 5.0 | ≤ 7.7 | ≤ 8.2 | ≤ 8.7 |
|  | QPSK | ≤ 5.0 | ≤ 7.7 | ≤ 8.2 | ≤ 9.7 |
|  | 16 QAM | ≤ 6.5 | ≤ 8.7 | ≤ 9.3 | ≤ 9.7 |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ 11.2 | ≤ 11.7 |
| CP-OFDM | QPSK | ≤ 5.0 | ≤ 7.5 | ≤ 8.0 | ≤ 9.7 |
|  | 16 QAM | ≤ 6.5 | ≤ 8.7 | ≤ 9.2 | ≤ 9.7 |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ 11.2 | ≤ 11.7 |
| NOTE 1: void. |  |  |  |  |  |

In case of a contiguous RB, DFT-s-BPSK or DFT-s-QPSK UL allocation in a single CC of a CA configuration with contiguous CCs, and whose cumulative aggregated BW  400 MHz, MPRC_CA shall be derived instead as MAX(MPR1, MPR2), where:

MPR1 shall be determined from Table 6.2.2.3.3-1 if CABW  200 MHz, from Table 6.2.2.3.3-2 if CABW > 200 MHz.

MPR2 shall be determined from Table 6.2.2.3.3-1 if UL BWchannel_CA  200 MHz, from Table 6.2.2.3.3-2 if UL BWchannel_CA > 200 MHz.

and assume all UL CCs use the same SCS for the purpose of determination of inner and outer RB allocations in Table 6.2.2.3.3-1 and Table 6.2.2.3.3-2:

NRB shall be chosen as the sum of NRB of all constituent UL CCs in the CA configuration.

LCRB shall be chosen as BWalloc,RB

RBstart shall be derived as: RBstart_allocatedCC+NRB_unallocatedCC_low

RBstart_allocatedCC is the index of the first allocated RB in the CC with allocation

NRB_unallocatedCC_low is the sum of NRB in all UL CCs lower in frequency compared to the CC with allocation

When different waveform types exist across CCs, the requirement is set by the waveform type used in the configuration with the highest contiguous MPR.

For intra-band contiguous UL CA with non-contiguous RB allocations, the following rule for MPR applies:

MPR = max(MPRC_CA, -10*A +7.0)

Where:

A = NRB_alloc / NRB_agg_C.

NRB_alloc is the total number of allocated UL RBs

NRB_agg_C is the number of the aggregated RBs within the fully allocated cumulative aggregated channel bandwidth assuming lowest SCS among all configured CCs

For intra-band non-contiguous UL CA, the following rule for MPR applies:

MPR = max(MPRNC_CA, -8*A +10.0)

Where:

MPRNC_CA is derived from table 6.2A.2. 0.4-2

A = NRB_alloc / NRB_agg_C.

NRB_alloc is the total number of allocated UL RBs

NRB_agg_C is the number of the aggregated RBs within the fully allocated cumulative aggregated channel bandwidth assuming lowest SCS among all configured CCs

Table 6.2A.2.0.4-2: MPRNC_CA for UE power class 3

|  |  | Cumulative aggregated channel bandwidth (CABW) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | ≤ 400 MHz | > 400 MHz and < 800 MHz | ≥ 800 MHz and ≤ 1400 MHz | > 1400 MHz and ≤ 2400 MHz |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 5.5 | ≤ 7.7 | ≤ 8.2 | ≤ 8.7 |
|  | QPSK | ≤ 6 | ≤ 7.7 | ≤ 8.2 | ≤ 8.7 |
|  | 16 QAM | ≤ 7 | ≤ 8.7 | ≤ 9.3 | ≤ 9.8 |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ 11.2 | ≤ 11.7 |
| CP-OFDM | QPSK | ≤ 6 | ≤ 7.5 | ≤ 8.0 | ≤ 8.5 |
|  | 16 QAM | ≤ 7 | ≤ 8.7 | ≤ 9.2 | ≤ 9.7 |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ 11.2 | ≤ 11.7 |

When different waveform types exist across CCs, the requirement is set by the waveform type used in the configuration with the largest MPRNC_CA.

For inter-band carrier aggregation with uplink assigned to two NR bands, the MPR for each configured UL band in the UL CA band combination is:

MPRinter-band_CA = max(MPRSingleBand, MPRPA-PA)

Where:

MPRSingleBand is the MPR specified in clause 6.2.2.3 for the allocation and modulation type in that band.

MPRPA-PA is MAX(MPR1, MPR2), where MPR1 and MPR2 are specified per band combination in Table 6.2A.2.4.3-1 and applies only when both bands have non-zero power UL RB allocations, 0 dB otherwise.

Table 6.2A.2.0.4-3: MPRPA-PA for Inter-band ULCA in FR2 for PC3

| NR CA Band | MPR | Value (dB) | Condition |
| --- | --- | --- | --- |
|  | MPR2 | 0.0 | - |
| CA_n260A-n261A | MPR1 | Max(0, 6 - 10*log10(Max(1.0, LRB,min*12*SCS/1e6))) | LRB,min = Min (LRB,n260 , LRB,n261 ), where LRB,n is the number of non-zero power UL RBs in band ‘n’ |
|  | MPR2 | 2.0 if condition satisfied, 0.0 otherwise | 47.2 GHz <= 2*fn260 - fn261 <= 48.2 GHz Where fn is any frequency inside the UL allocation in band ‘n’ |

<End of changes>