3GPP TSG-RAN5 Meeting #107	R5-251875r3

# St Julian, Malta, 19 - 23 May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4922 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case 6.5.3.11 SNPN Selection in Automatic Mode / Access for localized services in SNPN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN_Ph2-NGRAN_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Addition of new test case 6.5.3.11 SNPN Selection in Automatic Mode / Access for localized services in SNPN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding new test case 6.5.3.11 SNPN Selection in Automatic Mode / Access for localized services in SNPN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification remains incomplete compared to work plan coverage |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.3.11(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.523-2 CR 0568 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Correct the release number in the coversheet. Remove the TP3 since it’s not testable.Correct the table number in table 6.5.3.11.3.2-3 and step number in table 6.5.3.11.3.3-1. R2: Adding the SIB1 value with condition SNPN and eNPN. R3: Modify the SIB1 table. |  |  |  |  |  |  |  |  |

<Start of Changed Section>

#### 6.5.3.11	SNPN Selection in Automatic Mode / Access for localized services in SNPN

6.5.3.11.1	Test Purpose (TP)

(1)

with { UE in Automatic SNPN selection mode and supports access to an SNPN providing access for localized services in SNPN }

ensure that {

when { an SNPN is available which broadcasts the indication that access using credentials from a credentials holder is supported and which is identified by an SNPN identity contained in an entry of the "credentials holder controlled prioritized list of preferred SNPNs for access for localized services in SNPN" and the validity information of the entry is met }

then { UE selects the SNPN and attempts a registration on the selected cell }

}

(2)

with { UE in Automatic SNPN selection mode and supports access to an SNPN providing access for localized services in SNPN }

ensure that {

when { SNPNs are available which broadcast the indication that access using credentials from a credentials holder is supported and which are identified by an SNPN identity contained in an entry of the "credentials holder controlled prioritized list of preferred SNPNs for access for localized services in SNPN" and the validity information of the entry is met }

then { UE selects a cell of last registered SNPN and attempts a registration on the selected cell }

}

6.5.3.11.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 23.122 clauses 4.9.3.0 and 4.9.3.1.1. Unless otherwise stated these are Rel-18 requirements.

[TS 23.122, clause 4.9.3.0]

The ME is configured with a "list of subscriber data" containing zero or more entries. Each entry of the "list of subscriber data" consists of:

…

g)	optionally, if the MS supports access to an SNPN using credentials from a credentials holder:

1)	the SNPN selection parameters, consisting of:

i)	a user controlled prioritized list of preferred SNPNs, where each entry contains an SNPN identity;

ii)	a credentials holder controlled prioritized list of preferred SNPNs, where each entry contains an SNPN identity;

iii)	a credentials holder controlled prioritized list of Group IDs for Network Selection (GINs); and

iv)	optionally, if the MS supports access to an SNPN providing access for localized services in SNPN, the SNPN selection parameters for access for localized services in SNPN, consisting of:

A)	a "credentials holder controlled prioritized list of preferred SNPNs for access for localized services in SNPN", where each entry contains:

-	an SNPN identity;

-	validity information consisting of time validity information and optionally, location validity information; and

-	optionally, location assistance information; and

B)	a "credentials holder controlled prioritized list of preferred GINs for access for localized services in SNPN", where each entry contains:

-	a GIN;

-	validity information consisting of time validity information and optionally, location validity information; and

-	optionally, location assistance information; and

NOTE 3C:	The location assistance information, if available, is used to aid the UE where to search for SNPN providing the Localized Services in SNPN.

NOTE 3D:	The location validity information, if available, is used to indicate the area where access to SNPN for accessing the Localized Services in SNPN is allowed.

NOTE 3E:	SNPN can provide both localized services in SNPN and services which are not localized services in SNPN, in the same location, for a UE.

[TS 23.122, clause 4.9.3.1.1]

If:

-	there is at least one entry in the "list of subscriber data"; or

-	there is zero or more entries in the "list of subscriber data", the MS has a USIM with a PLMN subscription and the ME is provisioned with the SNPN selection parameters associated with the PLMN subscription,

the MS shall select one entry in the "list of subscriber data", if any, or the PLMN subscription, if any, to be used for automatic SNPN selection. How the MS selects the entry in the "list of subscriber data" or the PLMN subscription is MS implementation specific.

The MS selects an SNPN, if available and allowable, in the following order:

…

a0)	if the MS supports access to an SNPN providing access for localized services in SNPN and access for localized services in SNPN is enabled, then, using the SNPN selection parameters for access for localized services in SNPN in the selected entry of the "list of subscriber data" or associated with the selected PLMN subscription:

1)	the SNPN previously selected as result of an entry of a list of bullet a0) 2) or a0) 3) with which the UE was last registered, if validity information of the entry is still met, or the equivalent SNPN if it is available and validity information of the entry of the SNPN previously selected as result of an entry of a list of bullet a0) 2) or a0) 3) with which the UE was last registered is still met;

2)	each SNPN which broadcasts the indication that access using credentials from a credentials holder is supported and which is identified by an SNPN identity contained in an entry of the "credentials holder controlled prioritized list of preferred SNPNs for access for localized services in SNPN" (in priority order), if the validity information of the entry is met; and

…

The MS shall limit its search for the SNPN to the NG-RAN access technology.

Once the MS selects the SNPN, the MS attempts registrations on the selected SNPN using the NG-RAN access technology, the subscriber identifier and the credentials from the selected entry of the "list of subscriber data" or from the USIM, if the PLMN subscription is selected.

If successful registration is achieved, the MS indicates the selected SNPN.

[TS 23.122, clause 4.9.3.3]

If the MS supports access to an SNPN providing access for localized services in SNPN, the UE is in automatic SNPN selection mode and:

a)	access for localized services in SNPN is changed between disabled and enabled; or

b)	access for localized services in SNPN is enabled; and:

1)	the selected SNPN is an SNPN selected for localized services in SNPN, and the validity information for the selected SNPN is no longer met; or

2)	the selected SNPN is not an SNPN selected for localized services in SNPN, and the validity information for an entry in the "credentials holder controlled prioritized list of preferred SNPNs for access for localized services in SNPN" or "credentials holder controlled prioritized list of preferred GINs for access for localized services in SNPN" changes from not met to met;

then:

-	if the MS does not have an emergency PDU session and is not registered for emergency services, the MS may perform SNPN selection according to clause 4.9.3.1.1; or

-	otherwise, if the validity information for the selected SNPN is no longer met the MS shall perform a local release of all PDU sessions except for the emergency PDU session. The MS may perform SNPN selection according to clause 4.9.3.1.1 after the emergency PDU session is released.

6.5.3.11.3	Test description

6.5.3.11.3.1	Pre-test conditions

System Simulator:

-	2 SNPN cells, NR Cell 1 and NR Cell 3, are configured broadcasting default SNPN IDs as indicated in TS 38.508-1 [4] Table 4.4.2-4.

-	System information combination NR-27 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

UE:

-	The UE is in Automatic SNPN selection mode.

-	The UE is provisioned with a “list of subscriber data” to allow access to SNPN identified by NR Cell 1 and NR Cell 3 providing access for localized services in SNPN.

Preamble:

-	Ensure that the UE has cleared the Registered SNPN. And the UE is in state Switched OFF (state 0-A) as defined in TS 38.508-1 [4] Table 4.4A.2-0.

6.5.3.11.3.2	Test procedure sequence

Table 6.5.3.11.3.2-1/2 shows the cell configurations used during the test. Subsequent configurations marked “T1” and “T2” are applied at the points indicated in the Main behaviour description in Table 6.5.3.11.3.2-3. Cell powers are chosen for a serving cell and a non-suitable “Off” cell as defined in TS 38.508-1 [4] Table 6.2.2.1-3 for FR1 and Table 6.2.2.2-2 for FR2.

Table 6.5.3.11.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 3 | Remarks |
| --- | --- | --- | --- | --- | --- |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | “Off” | -88 | Power level “Off” is defined in TS 38.508-1 [4] Table 6.2.2.1-3. |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -88 | Power level “Off” is defined in TS 38.508-1 [4] Table 6.2.2.1-3. |

Table 6.5.3.11.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 3 | Remarks |
| --- | --- | --- | --- | --- | --- |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | “Off” | -91 | Power level “Off” is defined in TS 38.508-1 [4] Table 6.2.2.1-3. |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -91 | -91 | Power level “Off” is defined in TS 38.508-1 [4] Table 6.2.2.1-3. |

Table 6.5.3.11.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS adjusts cell levels according to row T1 of table 6.5.3.11.3.2-1/2. | - | - | - | - |
| 2 | Power on the UE. | - | - | - | - |
| 3 | Check: Does the UE send a RRCSetupRequest on NR Cell 3? | --> | NR RRC: RRCSetupRequest | 1 | P |
| 4-21a1 | Steps 3 to 20a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed on NR Cell 3 with ‘connected without release’. | - | - | - | - |
| 22 | The UE is switched off by executing generic procedure in Table 4.9.6.3-1 in TS 38.508-1 [4] | - | - | - | - |
| 23 | SS adjusts cell levels according to row T2 of table 6.5.3.11.3.2-1/2. | - | - | - | - |
| 24 | The UE is Switched ON. | - | - | - | - |
| 25 | Check: Does the UE send a RRCSetupRequest on NR Cell 3? | --> | NR RRC: RRCSetupRequest | 2 | P |
| 26-43a1 | Steps 3 to 20a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed on NR Cell 3. | - | - | - | - |

6.5.3.11.3.3	Specific message contents

Table 6.5.3.11.3.3-1: SIB1 for NR Cell 1 and NR Cell 3 (all steps, Table 6.5.3.11.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB1:: = SEQUENCE { |  |  |  |
| cellAccessRelatedInfo | CellAccessRelatedInfo with conditions SNPN and eNPN |  |  |
| } |  |  |  |

Table 6.5.3.11.3.3-2: REGISTRATION REQUEST (step 5 and step 27, Table 6.5.3.11.3.2-3)

| Derivation path: TS 38.508-1 Table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| 5GS registration type value | ‘001’B | Initial registration |  |