3GPP TSG-RAN5 Meeting #107	R5-252307r1

# St Julian’s, Malta, May 19-23, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4973 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NES spatial domain Type 1 adaptation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netw_Energy_NR-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In check steps, the message to be checked should be CSI report on The default CSI reporting type is aperiodic, the message should be updated to use periodic CSI reporting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add periodic CSI reporting configuration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case can’t be performed properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.14.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | The 1st revision: - Remove the changes that overlap with Nokia’s R5-252609 |  |  |  |  |  |  |  |  |

<Start of Changes>

7.1.1.14.2.1		NES spatial domain Type 1 adaptation / Periodic CSI reporting

7.1.1.14.2.1.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state configured for Periodic CSI-RS measurement reporting }

ensure that {

when { UE receives RRCReconfiguration message to report periodical CSI measurements with CSI-ReportSubconfig along with minimum 2 CSI-ReportSubconfigID }

then { UE applies CSI-ReportConfig and sends periodical CSI measurement reports for all CSI-ReportSubconfig present in CSI-ReportConfig }

}

7.1.1.14.2.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.214, clauses 5.2.1.4.2 and 5.2.4. Unless otherwise stated these are Rel-18 requirements.

[TS 38.214, clause 5.2.1.4.2]

…

If the UE is configured with a CSI-ReportConfig that contains a list of sub-configurations, provided by csi-ReportSubConfigToAddModList:

-	The UE expects to be configured with the higher layer parameter codebookType set to 'typeI-SinglePanel' or 'typeI-MultiPanel'. If the UE indicates a capability for supporting mixed codebook combination in a slot with mixCodeBookSpatialAdaptation, each sub-configuration which is configured with portSubsetIndictor can be configured with the higher layer parameter codebookType set to 'typeI-SinglePanel' or 'typeI-MultiPanel'.

-	Each sub-configuration can be configured with an antenna port subset using the higher layer bitmap parameter portSubsetIndicator which contains the bit sequence p0,p1,...,pPm-1, where p0 is the MSB and pPm-1 is the LSB, bit pi corresponds to antenna port 3000+i, and Pm is the number of ports nrofPorts configured for the CSI-RS resources(s) within a NZP-CSI-RS-ResourceSet contained in the CSI-ResourceConfig for channel measurement that corresponds to the CSI-ReportConfig. A bit value 0 in portSubsetIndicator indicates that the corresponding antenna port is disabled for the sub-configuration, whereas bit value 1 indicates that the antenna port is enabled and belongs to the antenna port subset for the sub-configuration. For the derivation of PMI, antenna ports corresponding to all bits with value of 1 in portSubsetIndicator are mapped to consecutive antenna ports starting at CSI-RS antenna port 3000 in increasing order of the bit position in portSubsetIndicator.

-	If a sub-configuration is configured with an antenna port subset, then the sub-configuration can be configured with a ri-Restriction, codebookMode and, if the number of antenna ports of the subset greater than 2, with n1-n2 if the higher layer parameter codebookType is set to 'typeI-SinglePanel' or with ng-n1-n2 if the higher layer parameter codebookType is set to 'typeI-MultiPanel', and, if the corresponding number of antenna ports of the subset is 2, with twoTX-CodebookSubsetRestriction, where the parameters ri-Restriction, codebookMode, n1-n2, ng-n1-n2, twoTX-CodebookSubsetRestriction are as described in Clauses 5.2.2.2.1 and 5.2.2.2.2. If a sub-configuration is configured with an antenna port subset, and if higher layer parameter reportQuantity is set to 'cri-RI-i1-CQI', and if the higher layer parameter codebookType is set to 'typeI-SinglePanel', then the sub-configuration is configured with higher layer parameter typeI-SinglePanel-codebookSubsetRestriction-i2, where typeI-SinglePanel-codebookSubsetRestriction-i2 is as described in Clause 5.2.2.2.1.

-	If a sub-configuration is configured with an antenna port subset, and if the CSI-ReportConfig that contains a mix of sub-configuration(s) each corresponding to 'typeI-SinglePanel' and some other sub-configuration(s) each corresponding to 'typeI-MultiPanel', then the sub-configuration(s) which is configured with portSubsetIndictor is configured with the higher layer parameter codebookMode.

-	A sub-configuration can be configured with a power offset provided by powerOffset.

-	A sub-configuration can be configured with a list of NZP CSI-RS resources, provided by nzp-CSI-RS-ResourceList, which indicates one or more NZP CSI-RS resources, within a NZP-CSI-RS-ResourceSet contained in the CSI-ResourceConfig for channel measurement which corresponds to the CSI-ReportConfig. If there is no sub-configuration configured with a power offset provided by powerOffset, the list of NZP CSI-RS resources has no intersection with a list of NZP CSI-RS resources configured for any other sub-configuration(s) within the CSI-ReportConfig, otherwise, the list of NZP CSI-RS resources is identical to or has no intersection with a list of NZP CSI-RS resources configured for any other sub-configuration(s) within the CSI-ReportConfig.

-	If a sub-configuration is configured with a list of NZP CSI-RS resources with more than one resource, the UE shall derive the CSI parameters other than CRI conditioned on the reported CRI, where the CRI k (k ≥ 0) for the sub-configuration corresponds to the configured (k+1)-th entry of associated NZP-CSI-RS-Resource in the list of NZP CSI-RS resources.

-	If a sub-configurations is not configured with nzp-CSI-RS-ResourceList then the sub-configuration shall be associated with all the NZP CSI-RS resources within a NZP-CSI-RS-ResourceSet contained in the CSI-ResourceConfig for channel measurement which corresponds to the CSI-ReportConfig.

-	the UE reports CSI(s) for one or more sub-configurations according to Clauses 5.2.1.5.1, 5.2.1.5.2, 5.2.3 and 5.2.4, and according to the higher layer parameter reportQuantity configured for that CSI-ReportConfig.

-	The UE does not expect the higher layer parameter reportQuantity to be set to 'cri-RSRP', 'cri-SINR', 'cri-SINR- Index', 'cri-RSRP-Index', 'none', 'ssb-Index-RSRP', 'ssb-Index-SINR', 'ssb-Index-RSRP- Index', 'ssb-Index-SINR- Index', or 'tdcp'.

…

[TS 38.214, clause 5.2.4]

A UE is semi-statically configured by higher layers to perform periodic CSI Reporting on the PUCCH. A UE can be configured by higher layers for multiple periodic CSI Reports corresponding to multiple higher layer configured CSI Reporting Settings, where the associated CSI Resource Settings are higher layer configured. For a Reporting Setting for which the CSI-ReportConfig contains a list of sub-configurations provided by csi-ReportSubConfigToAddModList, CSI reporting is provided for all the sub-configurations in each corresponding reporting instance. Periodic CSI reporting on PUCCH formats 2, 3, 4 supports Type I CSI with wideband granularity.

7.1.1.14.2.1.3	Test description

7.1.1.14.2.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1

UE:

- None.

Preamble:

-	The UE is in state 3N-A as defined in TS 38.508-1 [4], subclause 4.4A on NR Cell 1.

7.1.1.14.2.1.3.2	Test procedure sequence

Table 7.1.1.14.2.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message on NR Cell 1 including CSI-MeasConfig to setup measurement and periodic reporting with CSI-ReportSubconfig. | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message on NR Cell 1. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 3 | Check: Does the UE transmit a MeasurementReport message on NR Cell 1 for all CSI-ReportSubconfig configured at first periodicity? | --> | NR RRC: MeasurementReport | 1 | P |
| 4 | Check: Does the UE transmit a MeasurementReport message on NR Cell 1 for all CSI-ReportSubconfig configured at second periodicity? | --> | NR RRC: MeasurementReport | 1 | P |
| 5 | The SS transmits an RRCReconfiguration including CSI-MeasConfig to remove measurements | <-- | NR RRC: RRCReconfiguration | - | - |
| 6 | The UE transmits an RRCReconfigurationComplete message to confirm the remove. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 7 | Check: Does the UE transmit a MeasurementReport message on NR Cell 1 for all CSI-ReportSubconfig configured within two periods? | --> | NR RRC: MeasurementReport | 1 | F |

7.1.1.14.2.1.3.3	Specific message contents

Table 7.1.1.14.2.1.3.3-1: RRCReconfiguration (Step 1 and Step 5, Table 7.1.1.14.2.1.3.2-1)

Derivation Path: TS 38.508-1 [4] table 4.6.1-13 with CellGroupConfig with condition NR_MEAS

Table 7.1.1.14.2.1.3.3-2: ServingCellConfig (Table 7.1.1.14.2.1.3.3-1)

| Derivation Path: TS 38.508-1 [4], clause 4.6.3-167 with condition MEAS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| csi-MeasConfig | CSI-MeasConfig |  |  |
| } |  |  |  |

Table 7.1.1.14.2.1.3.3-3: CSI-MeasConfig (Table 7.1.1.14.2.1.3.3-2)

Derivation Path: TS 38.508-1 [4], clause 4.6.3-38

Table 7.1.1.14.2.1.3.3-4: CSI-ReportConfig (Step 1, Table 7.1.1.14.2.1.3.3-3)

| Derivation Path: TS 38.508-1 [4], clause 4.6.3-39 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfig ::= SEQUENCE { |  |  |  |
| reportConfigType CHOICE { |  |  |  |
| periodic SEQUENCE { |  |  |  |
| reportSlotConfig | CSI-ReportPeriodicityAndOffset | Table 4.6.3-40D in TS 38.508-1 [4] |  |
| pucch-CSI-ResourceList SEQUENCE (SIZE (1.. maxNrofBWPs)) OF PUCCH-CSI-Resource { |  |  |  |
| PUCCH-CSI-Resource[1] | PUCCH-CSI-Resource | Table 4.6.3-113B in TS 38.508-1 [4] |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| csi-ReportSubConfigToAddModList-r18 SEQUENCE (SIZE (1..maxNrofCSI-ReportSubconfigPerCSI-ReportConfig-r18)) OF CSI-ReportSubConfig-r18 { | 2 entries |  |  |
| CSI-ReportSubConfig-r18[1] SEQUENCE{ |  | Entry 1 |  |
| reportSubConfigId-r18 | 0 |  |  |
| reportSubConfigParams-r18 CHOICE { |  |  |  |
| a1-parameters SEQUENCE { |  |  |  |
| codebookSubConfig-r18 | CodebookConfig |  |  |
| portSubsetIndicator-r18 CHOICE { | TBD |  |  |
| } |  |  |  |
| non-PMI-PortIndication-r18 SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerConfig)) OF PortIndexFor8Ranks { | 1 entry |  |  |
| PortIndexFor8Ranks[1] | TBD |  |  |
| } |  |  |  |
| } |  |  |  |
| a2-parameters SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceList-r18 SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceIndex-r18 { |  |  |  |
| NZP-CSI-RS-ResourceIndex-r18[1] | TBD |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| CSI-ReportSubConfig-r18[2] SEQUENCE{ |  | Entry 2 |  |
| reportSubConfigId-r18 | 1 |  |  |
| reportSubConfigParams-r18 CHOICE { |  |  |  |
| a1-parameters SEQUENCE { |  |  |  |
| codebookSubConfig-r18 | CodebookConfig |  |  |
| portSubsetIndicator-r18 CHOICE { | TBD |  |  |
| } |  |  |  |
| non-PMI-PortIndication-r18 SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerConfig)) OF PortIndexFor8Ranks { | 1 entry | PortIndexFor8Ranks |  |
| PortIndexFor8Ranks[1] | TBD |  |  |
| } |  |  |  |
| } |  |  |  |
| a2-parameters SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceList-r18 SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceIndex-r18 { |  |  |  |
| NZP-CSI-RS-ResourceIndex-r18[1] | TBD |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.14.2.1.3.3-5: CSI-ReportConfig (Step 5, Table 7.1.1.14.2.1.3.3-3)

| Derivation Path: TS 38.508-1 [4], clause 4.6.3-39 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfig ::= SEQUENCE { |  |  |  |
| csi-ReportSubConfigToReleaseList-r18 SEQUENCE (SIZE (1..maxNrofCSI-ReportSubconfigPerCSI-ReportConfig-r18)) OF CSI-ReportSubConfigId-r18 { | 1 entry |  |  |
| CSI-ReportSubConfigId-r18[1] | 0 |  |  |
| CSI-ReportSubConfigId-r18[2] | 1 |  |  |
| } |  |  |  |
| } |  |  |  |

<End of Changes>