3GPP TSG-RAN5 Meeting #107	R5-252402r2

# St Julian, Malta, 19th – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1601 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add test case for Emergency call add remove RTT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI12_Test |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add test case for Emergency call / add RTT and remove RTT. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A test case has been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test coverage for RTT according to EU directive 2019/882 remains incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New 19.1.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.508 CR 1522 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision R5-252402r1. Updated Method of test for call release. 2nd revision R5-252402r2. Specific message contents step 4 updated. |  |  |  |  |  |  |  |  |

<Start of first modified section>

## 19.1.8	Emergency call / add RTT and remove RTT

### 19.1.8.1	Definition

Test to verify that the UE is able to add a bidirectional text (RTT) component to an ongoing IMS emergency call. This process is described in 3GPP TS 24.229 [10], TS 24.173 [65] and TS 26.114 [66].

### 19.1.8.2	Conformance requirement

[TS 24.229, clause 5.1.2A.1.1]

After the dialog is established the UE may change the dialog capabilities (e.g. add a media or request a supplementary service) if defined for the IMS communication service as identified by the ICSI value using the same dialog. Otherwise, the UE shall initiate a new initial request to the other user.

[TS 26.114, clause 6.2.5]

The SDP shall include bandwidth information for each media stream and also for the session in total. The bandwidth information for each media stream and for the session is defined by the Application Specific (AS) bandwidth modifier as defined in RFC 4566.

[TS 26.114, clause 6.3]:

During session renegotiation for adding or removing media components, the SDP offerer should continue to use the same media (m=) line(s) from the previously negotiated SDP for the media components that are not being added or removed.

[TS 26.114, clause 7.3.1]

The bandwidth for RTCP traffic shall be described using the "RS" and "RR" SDP bandwidth modifiers at media level, as specified by RFC 3556.

### 19.1.8.3	Test purpose

1)	To verify that when adding an RTT component to an ongoing emergency call the UE performs correct exchange of SIP protocol signalling messages; and

2)	To verify that within SIP signalling the UE performs correct SDP offer/answer exchanges for negotiating media (as described by 3GPP TS 24.229 [10], clause 6.1); and

3)	To verify that when removing the RTT component from theemergency call the UE performs correct exchange of SIP and SDP protocol messages.

### 19.1.8.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC. UE has established an IMS emergency call according annex C.20 followed by annex C.22.

Test procedure

1)	Adding RTT to the emergency call is initiated on the UE.

2-10)	UE executes the procedure described in TS 36.508 [94] table 4.5A.pp.3-1, steps 2 to 6. In detail the following steps are done in IMS.

2)	UE to sends a re-INVITE request to the SS.

3)	SS responds to the re-INVITE request with a 100 Trying response.

4)	SS responds to the re-INVITE request with a valid 200 OK response.

5)	SS waits for the UE to send an ACK to acknowledge receipt of the 200 OK for INVITE.

6)	The UE is triggered to remove the RTT stream from the emergency call

7)	SS waits the UE to send a re-INVITE request with a SDP offer indicating the removal of the RTT stream.

8)	SS responds to the re-INVITE request with a 100 Trying response.

9)	SS deactivates the EPS bearer corresponding to the RTT stream and releases the associated radio resources by applying the procedure described in TS 36.508 [94] clause 4.5A.15

10)	 SS responds to the re-INVITE request with a valid 200 OK response.

11)	 SS waits for the UE to send an ACK to acknowledge receipt of the 200 OK for re-INVITE.

12-16)	 MO Call release according to procedure C.32.

Expected sequence

NOTE:	Only the IMS procedure relevant to the test purpose is described below.

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | Make UE add RTT to the voice call. |  |
| 2 |  |  | INVITE | UE sends re-INVITE with an SDP offer containing media lines for both voice and RTT |
| 3 |  |  | 100 Trying | The SS responds with a 100 Trying provisional response |
| 4 |  |  | 200 OK | The SS responds re-INVITE with 200 OK |
| 5 |  |  | ACK | The UE acknowledges the receipt of 200 OK for re-INVITE |
| 6 |  |  | Make UE release RTT from the media call |  |
| 7 |  |  | INVITE | UE sends re-INVITE with an SDP offer indicating that the RTT component is removed from the emergency call |
| 8 |  |  | 100 Trying | The SS responds with a 100 Trying provisional response |
| 9 |  |  | SS deactivates the EPS bearer for RTT |  |
| 10 |  |  | 200 OK | The SS responds re-INVITE with 200 OK |
| 11 |  |  | ACK | The UE acknowledges the receipt of 200 OK for re-INVITE |
| 12-16 |  |  | Steps defined in annex C.32 | MO Call release |

Specific Message Contents

INVITE (Step 2)

Use the default message “INVITE for MO Call” in annex A.2.1 with condition A5 (re-INVITE within a dialog) and the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Message-body | The following SDP types and values. Session description: v=0 o=(username) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) s=(session name) c=IN (addrtype) (connection-address for UE) [Note 1] Time description: t=(start-time) (stop-time) Media description: m=audio (transport port) [Note 2] c=IN (addrtype) (connection-address for UE) [Note 1] b=AS: (bandwidth-value) Media description: m=text (transport port) [Note 3] c=IN (addrtype) (connection-address for UE) [Note 1] b=AS: (bandwidth-value) Note 1: At least one "c=" field shall be present. Note 2: AMR-WB/16000 codec shall be present in the media attributes, optionally including channel number "/1". Note 3: t140/1000 name and rate shall be present in the media attributes. |

200 OK for INVITE (Step 4)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1 with condition A23 and the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Content-Type |  |
| media-type | application/sdp |
| Content-Length |  |
| value | length of message-body |
| Message-body | The following SDP types and values. Session description: v=0 o=- 1111111111 1111111111 IN (addrtype) (unicast-address for SS) s=- c=IN (addrtype) (connection-address for SS) b=AS:39 Time description: t=0 0 Media description: m=audio (transport port) RTP/AVP (fmt) [Note 1] b=AS:37 b=RS:0 b=RR:0 Attributes for media: a=rtpmap: (payload type) AMR-WB/16000/1 [Note 1] a=fmtp: (format) mode-change-capability=2; max-red=220 a=ptime:20 a=maxptime:240 Media description: m=text (transport port) RTP/AVP (fmt) [Note 1] b=AS:2 b=RS:0 b=RR:0 Attributes for media: a=rtpmap: (payload type) t140/1000 [Note 1] a=rtpmap: (payload type) red/1000 [Note 2 a=fmtp: (format) (format specific parameters) [Note 2] Note 1: The values for fmt, payload type and format are copied from step 2. Note 2: If red/1000 is present in step 1 the values for fmt, payload type, format specific parameters and format are copied from step1, otherwise the red/1000 codec is not present. |

ACK (Step 5)

Use the default message “ACK” in annex A.2.7 with condition A5.

INVITE (Step 7)

Use the default message “INVITE for MO Call” in annex A.2.1 with condition A5 (re-INVITE within a dialog) and the following exceptions:

| Header/param | Value/Remark |
| --- | --- |
| Supported |  |
| option-tag | precondition |
| Message-body | The following SDP types and values. Session description: - v=0 - o=(username) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) [Note 2] - s=(session name) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) Time description: - t= (start-time) (stop-time) Media description: - m=audio (transport port) [Note 3] - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) Attributes for media: - a=rtpmap: (payload type) EVS/16000 [Note 3] - a=fmtp: (format) Media description: - m=text 0 RTP/AVP (fmt) Note 1: At least one "c=" field shall be present. Note 2: "o=" line identical to previous SDP sent by UE except that sess-version is incremented by one. Note 3: AMR-WB/16000 codec shall be present in the media attributes, optionally including channel number "/1". |

200 OK for INVITE (Step 10)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1 with condition A23 (Response sent for re-INVITE within an established dialog) and the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Require option-tag | precondition |
| Content-Type |  |
| media-type | application/sdp |
| Content-Length |  |
| Value | length of message-body |
| Message-body | SDP body of the 200 response copied from the received INVITE and modified as follows: - "o=" line identical to previous SDP sent by SS except that sess-version is incremented by one - IP address on "c=" line and, for audio, transport port on "m=" line changed to indicate to which IP address and port the UE should start sending the media; |

ACK (Step 11)

Use the default message “ACK” in annex A.2.7 with condition A5.

### 19.1.8.5	Test requirements

The UE shall send requests and responses as described in clause 19.1.8.4

<End of modified section>