3GPP TSG-RAN WG5 Meeting #107	R5-252410r3

# St. Julian’s, Malta, 19th May 2025 – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-4 | CR | 0096 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to Clause 5 and addition of reference sensitivity test cases with variable Tx-Rx spacing for NB-IoT for NTN bands |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | EchoStar |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_Test |  |  |  |  |  | Date: |  |  | 2025-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN2 specifications allow configuration UL and DL center frequency independently. Normally RAN4 defines minimum requirements with a single default Tx-Rx separation. Satellite industry has a strong need to use different Tx-Rx spacing in different deployments. Therefore, RAN4 specified variable Tx-Rx spacing for NTN bands and added a requirement to validate those different Tx-Rx spacing values for reference sensitivity test case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adds new clauses for reference sensitivity test cases for variable Tx-Rx spacing to support conformance of variable Tx-Rx spacing for NB-IoT NTN UEs |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformance of variable Tx-Rx spacing for NTN UEs cannot be completed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | new clause 7.3B_1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Version R1: Removed changes to 5.4.A3, 5.4.B3, 7.1, 7.3A.3 and 7.3B.3 to remove overlap with R5-252070. ii) Added editors note indicating that TP analysis in TR 36.905 needs to be updated, iii) removed new test case for eMTC as test coverage for eMTC is deferred to a later time when needed per prior RAN5 agreement; iv) updated test case title; iii) Revised by referencing the aspects of baseline test case for default Tx-Rx separation that remain unchanged; iv) minor edit to the title to indicate changes are for only NB-IoT NTN bands. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 7.3B_1	Reference sensitivity power level for UE category NB1 and NB2 with flexible Tx-Rx frequency separation

Editor’s Note: This test is incomplete. The following aspects are not yet determined:

- TP analysis needs to be captured in TR 36.905

- Clause reference for the test frequencies is FFS

- Annex F updates are FFS

7.3B_1.1	Test purpose

To verify the UE's ability to receive data with a given average throughput for a specified reference measurement channel, under conditions of low signal level, ideal propagation and no added noise for Tx-Rx frequency separation other than the default value.

A UE unable to meet the throughput requirement under these conditions will decrease the effective coverage area.

7.3B_1.2	Test applicability

This test case applies to all types of NB-IoT UE release 17 and forward of category NB1 and NB2 that support satellite access operation and flexible Tx-Rx frequency separation.

This test case applies to all types of NB-IoT UE release 18 and forward of category NB1 and NB2 that support satellite access operation.

7.3B_1.3	Minimum conformance requirements

Same as in 7.3B.3.

7.3B_1.4	Test description

7.3B_1.4.1	Initial conditions

Same as in 7.3B.4.1 with test configuration as listed in Table 7.3B_1.4.1-1.

Table 7.3B_1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] clause 8.1.1 |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS36.508 [12] clause 8.1.3.1.X-2 |  |  | Low range, High range |  |  |
| Configuration ID | Downlink Configuration |  | Uplink Configuration |  |  |
|  | Modulation | Subcarriers | Modulation | Ntones | Subcarrier spacing |
| 1 | QPSK | 12 | BPSK | 1@0 | 15kHz |

7.3B_1.4.2	Test procedure

Same as in 7.3B.4.2.

7.3B_1.4.3	Message contents

Same as in 7.3B.4.3.

7.3B_1.5	Test requirement

Same as in 7.3B.1.5.

END OF CHANGES >>