# Title:	Test Tolerances for SS-SINR accuracy tests in FR1

Source:	Rohde & Schwarz, Huawei, HiSilicon

# 1.	Introduction

The purpose of this document is to present the methodology and principles used for the development of Test Tolerances for SS-SINR measurement accuracy tests for FR1.

The test cases in question, from TS 38.533, are:

- 4.7.3.1, EN-DC FR1 SS-SINR measurement accuracy

- 6.7.3.1, NR SA FR1 SS- SINR measurement accuracy

- 16.7.3.1, NR SA FR1 Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE

- 16.7.3.2, NR SA FR1 Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE

- 19.6.3.1, NR SA FR1 SS-SINR absolute measurement accuracy for ATG

This document describes the process to derive the Test Tolerances. The calculations are provided in the accompanying spreadsheet.

# 2. 	Test cases in TS 38.133

The test requirements are defined in the following extract from TS 38.133

<< Extract from TS 38.133 >>

#### A.4.7.3.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell

##### A.4.7.3.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-SINR measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.1.12.1.1.

##### A.4.7.3.1.2	Test Parameters

In this test case all cells are on the same carrier frequency. Supported test configuration are shown in Table A.4.7.3.1.2-1. The absolute accuracy of SS-SINR intra-frequency measurement is tested by using the parameters in Table A.4.7.3.1.2-2. The configuration of cell 1 (E-UTRA PCell) is specified in clause A.3.7.2.1. In all test cases, Cell 2 is the PSCell and Cell 3 is the target cell.

Table A.4.7.3.1.2-1: SS-SINR Intra frequency SS-SINR supported test configurations

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.4.7.3.1.2-2: SS-SINR Intra frequency test parameters

| Parameter |  |  |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Cell 2 | Cell 3 | Cell 2 | Cell 3 |
| SSB ARFCN |  |  |  |  |  | freq1 |  | freq1 |  |
| Duplex mode |  |  |  | Config 1,4 |  | FDD |  |  |  |
|  |  |  |  | Config 2,3,5,6 |  | TDD |  |  |  |
| TDD configuration |  |  |  | Config 1,4 |  | Not Applicable |  |  |  |
|  |  |  |  | Config 2,5 |  | TDDConf.1.1 |  |  |  |
|  |  |  |  | Config 3,6 |  | TDDConf.2.1 |  |  |  |
| Downlink initial BWP configuration |  |  |  |  |  | DLBWP.0.1 |  |  |  |
| Downlink dedicated BWP configuration |  |  |  |  |  | DLBWP.1.1 |  |  |  |
| Uplink initial BWP configuration |  |  |  |  |  | ULBWP.0.1 |  |  |  |
| Uplink dedicated BWP configuration |  |  |  |  |  | ULBWP.1.1 |  |  |  |
| DRX Cycle configuration |  |  |  |  | ms | Not Applicable |  |  |  |
| TRS configuration |  |  | Config 1, 4 |  |  | TRS.1.1 FDD |  |  |  |
|  |  |  | Config 2, 5 |  |  | TRS.1.1 TDD |  |  |  |
|  |  |  | Config 3, 6 |  |  | TRS.1.2 TDD |  |  |  |
| PDSCH Reference measurement channel |  |  |  | Config 1,4 |  | SR.1.1 FDD | - | SR.1.1 FDD | - |
|  |  |  |  | Config 2,5 |  | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  |  |  |  | Config 3,6 |  | SR.2.1 TDD |  | SR2.1 TDD |  |
| RMSI CORESET Reference Channel |  |  |  | Config 1,4 |  | CR.1.1 FDD | - | CR.1.1 FDD |  |
|  |  |  |  | Config 2,5 |  | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  |  |  | Config 3,6 |  | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel |  |  |  | Config 1,4 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - |
|  |  |  |  | Config 2,5 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  |  |  | Config 3,6 |  | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Patterns |  |  |  |  |  | OP.1 |  |  |  |
| SS-RSSI-Measurement |  |  |  |  |  | Not Applicable |  |  |  |
| SMTC configruation |  |  |  |  |  | SMTC.1 |  |  |  |
| SSB configuration |  |  |  | Config 1,2,4,5 |  | SSB.1 FR1 |  |  |  |
|  |  |  |  | Config 3,6 |  | SSB.2 FR1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  |  | Config 1,2,4,5 | kHz | 15 |  |  |  |
|  |  |  |  | Config 3,6 |  | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 |  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/15kHz | -93 |  | -116 |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  | -115.5 |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  | -115 |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -114.5 |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -114 |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  | -113 |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  | -112.5 |  |
| Note2 | Config 1,2,4,5 |  |  |  | dBm/SCS | -93 |  | Same as Noc for 15kHz |  |
|  | Config 3,6 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | -90 |  | -113 |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  | -112.5 |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  | -112 |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -111.5 |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -111 |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  | -110 |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  | -109.5 |  |
|  |  |  |  |  | dB | 0 | -3.19 | -5.46 | -5.46 |
|  |  |  |  |  | dB | 4.54 | 2.66 | -4 | -4 |
| SS-RSRPNote3 | Config 1,2,4,5 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A | dBm/SCS | -88.46 | -90.34 | -120 | -120 |
|  |  |  |  | NR_FDD_FR1_B |  |  |  | -119.5 | -119.5 |
|  |  |  |  | NR_TDD_FR1_C |  |  |  | -119 | -119 |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -118.5 | -118.5 |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -118 | -118 |
|  |  |  |  | NR_FDD_FR1_G |  |  |  | -117 | -117 |
|  |  |  |  | NR_FDD_FR1_H |  |  |  | -116.5 | -116.5 |
|  | Config 3,6 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | -85.46 | -87.34 | -117 | -117 |
|  |  |  |  | NR_FDD_FR1_B |  |  |  | -116.5 | -116.5 |
|  |  |  |  | NR_TDD_FR1_C |  |  |  | -116 | -116 |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5 | -115.5 |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115 | -115 |
|  |  |  |  | NR_FDD_FR1_G |  |  |  | -114 | -114 |
|  |  |  |  | NR_FDD_FR1_H |  |  |  | -113.5 | -113.5 |
| SS-SINR Note3 |  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dB | 0 | -3.19 | -5.46 | -5.46 |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |
| IoNote3 |  | Config 1,2,4,5 |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/ 9.36MHz | -57.5 |  | -85.51 |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  | -85.01 |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  | -84.51 |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -84.01 |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -83.51 |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  | -82.51 |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  | -82.01 |  |
|  |  | Config 3,6 |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/ 38.16MHz | -51.41 |  | -79.41 |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  | -78.91 |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  | -78.41 |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -77.91 |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -77.41 |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  | -76.41 |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  | -75.91 |  |
| Propagation condition |  |  |  |  | - | AWGN |  |  |  |
| Antenna configuration |  |  |  |  | - | 1x2 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-SINR, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-SINR, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in Clause 3.5.2. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification |  |  |  |  |  |  |  |  |  |

##### A.4.7.3.1.3	Test Requirements

The SS-SINR measurement accuracy shall fulfil the requirements in clause 10.1.12.1.1.

# 3. 	Discussion

The test case has 2 subtests: Test 1 and Test 2. To analyse the test case it is helpful to visualise these as pie-charts showing the distribution of power, as below:

The subtests are designed to test the UE at two points over which the SS-SINR accuracy requirement applies:

- Test 1 applies where the reporting accuracy is +/-3.5dB, in the region where Es/Iot is above -6 dB, and at the highest overall power with Io just below -50dBm

- Test 2 applies where the reporting accuracy is +/-3.5dB, in the region where Es/Iot is close to the lower limit of -6 dB, and with SS-RSRP just above Ref. sensitivity

The UE measures the Reported SS-SINR of Cell 2.

There are three test configurations according to Table A.4.7.3.1.1-1. Test Configurations 1 and 2 have identical power settings, so they will be analysed together. Test Configuration 3 has different power settings and it will be analysed in a separate tab in the accompanying excel sheet

# 4. 	Calculation of Test Tolerances

General approach

The general approach is given in the steps below:

- Copy the originally specified key parameters from the core requirements

- Where relevant, calculate derived parameters from the core requirements

- Define uncertainties for a minimum set of parameters

- Define controlled parameters (critical to the test verdict), calculate sensitivity factors and uncertainty

- Determine which original or derived parameters to offset (apply Test Tolerances to) and by how much

- Recalculate original or derived parameters including Test Tolerances

- Check that the controlled parameters meet requirements to get the correct test verdict

Each step is explained below, and the calculations are given in the accompanying spreadsheet.

a) Original specified key parameters

The key parameters are copied from Tables A.4.7.3.1-1 and 10.1.12.1.1-1 in TS 38.133. The key parameters are selected as the minimum set to define the cell power levels, which are Noc, Cell 1 Es/Noc and Cell 2 Es/Noc. All other parameters such as SSB_RP, SINR, SSB Es/Iot and Io are derived, and not independently settable by the Test Equipment.

The key parameters appear in section a) of the accompanying spreadsheet.

b) Derived parameters

A number of derived parameters are calculated, using the base information in a). The reason for deriving each additional parameter is given in the “Comment” column of section b) in the accompanying spreadsheet.

The derived parameter SS-SINR is calculated according to TS 38.215.

c) Uncertainties

The SS provides AWGN and 2 intra-frequency cells to the UE. We propose to control the following parameters:

- AWGN absolute power, Noc ±1.5 dB averaged over BWConfig

- AWGN absolute power, Noc ±1.5 dB for PRBs #RBJ-RBJ+19

- Ratio of cell 1 signal / AWGN, Ês1 / Noc ±0.3 dB averaged over BWConfig

- Ratio of cell 1 signal / AWGN, Ês1 / Noc ±0.8 dB for PRBs #RBJ-RBJ+19

- Ratio of cell 2 signal / AWGN, Ês2 / Noc ±0.3 dB averaged over BWConfig

- Ratio of cell 2 signal / AWGN, Ês2 / Noc ±0.8 dB for PRBs #RBJ-RBJ+19

In this test the UE measures the SS-SINR over specific Physical Resource Block (PRB) numbers # RBJ to # RBJ+19. The generic AWGN parameters values similar to those used in performance tests are therefore unsuitable, because the AWGN flatness specification would allow a large deviation for the absolute power in PRBs #RBJ-RBJ+19.

In addition, this test has separate constraints on the overall power in the configured bandwidth Io.

Two sets of parameters are therefore given. The set averaged over the configured bandwidth have similar values to those already used for other tests. The set averaged over PRBs # RBJ-RBJ+19 have wider values, but constraining the deviation enough not to widen the SS-SINR reporting range too much.

We note also that the outcome of this test is a range of allowed SS-SINR values reported by the UE. The UE reporting accuracy is therefore taken into account when determining the test limits, although it is not an uncertainty of the test system itself.

This choice forms a minimum set (separately for PRBs #RBJ-RBJ+19, and for “averaged over BWConfig“), so the superposition principle can be applied if necessary.

The absolute level of Noc directly affects the UE’s SS-SINR measurement, and is specified for frequency bands up to 6GHz.

The UE measures the SS-SINR and reports it. The absolute measurement accuracy is given in TS 38.133:

##### 10.1.12.1.1	Absolute SS-SINR Accuracy in FR1

Unless otherwise specified, the requirements for absolute accuracy of SS-SINR in this clause apply to a cell on the same frequency as that of the serving cell in FR1.

The accuracy requirements in Table 10.1.12.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band.

Table 10.1.12.1.1-1: SS-SINR Intra frequency absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 3 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 4 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| 3.0 | 4 | -3 dB | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
| 3.5 | 4 | -6 dB | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: The requirements apply for SSB Ês/Iot ≤ 25 dB. NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

Table 10.1A.10.1.1-1: SS-SINR Intra frequency absolute accuracy for 1Rx RedCap UE in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  | Note 3 | NR operating band groups Note 4 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
| 4.0 | 4 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | N/A | -50 |
| 4.5 | 4 | -6 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: The requirements apply for SSB Ês/Iot ≤ 25 dB under non-HST scenarios. NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 5: The requirements apply for SSB Ês/Iot ≤5 dB with SCS 15kHz or 30kHz under NR high speed scenarios. |  |  |  |  |  |  |  |

The Io is in different ranges for each subtest, so the accuracy levels is selected differently for each subtests as indicated in Section 3 of this document. The absolute accuracy requirement refers to SS-RSRP side conditions in TS 38.133 Annex B.2.2:

## B.2.2	Conditions for NR intra-frequency measurements

This section defines the following conditions for NR intra-frequency measurements and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.2-1 for FR1 NR cells.

The conditions are defined in Table B.2.2-2 for FR2 NR cells.

Table B.2.2-1: Conditions for intra-frequency measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A | -127 | -124 | -6 |
|  | NR_FDD_FR1_B | -126.5 | -123.5 |  |
|  | NR_TDD_FR1_C | -126 | -123 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -122.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -122 |  |
|  | NR_FDD_FR1_G | -124 | -121 |  |
|  | NR_FDD_FR1_H | -123.5 | -120.5 |  |
| NOTE 1: NR operating band groups are defined in Section 3.5.2. |  |  |  |  |

The SSB Ês/Iot side condition is ≥ -6dB in both tables. Band group NR_FDD_FR1_H gives the highest SSB_RP side condition of -123.5dBm/15kHz.

d) Controlled parameters critical to verdict

In many RRM test cases, and particularly intra-frequency multi-cell tests, there is not a simple one-to-one relationship between the parameters that can be set by the test equipment, and their effect on parameters determining the test verdict.

It is therefore essential to identify those parameters determining the test verdict. In this test case there are two aspects to consider:

- The uncertainties in the stimulus set by the Test system (Downlink signal)

- The uncertainties in the UE response (SS-SINR reports)

The stimulus set by the Test system should be within the constraints (side conditions) for the UE measurement. For each cell they are the SSB Es/Iot range, the SSB_RP power range, the Io power range, and for Cell 2 the SS-SINR range over which the UE meets the specified SS-SINR reporting accuracy. The 7 controlled parameters listed in the accompanying spreadsheet have been derived by study of the test case and by careful reading of the relevant clauses in TS 38.133. The reason for each parameter being critical to the test verdict is given briefly in the “Comment” column of section d) in the accompanying spreadsheet. Information about the value to be achieved is given later in the “Comment” column of section g) in the spreadsheet.

It may not be strictly necessary to control the Cell 1 parameters, but the original values are the same for each cell. For the avoidance of doubt the Cell 1 parameters are also kept within range.

The uncertainty in UE reporting accuracy is also listed in a separate row. The range of reported results is affected by both the Test system stimulus setting uncertainty and the UE reporting accuracy.

In general all the values are listed for Test 1 and all the variants of Test 2.

Having identified the parameters critical to the test verdict which need to be controlled, we now need to consider how they are affected by the parameters which can be set by the test equipment. This is done by working out “sensitivity factors”. A sensitivity factor is just the ratio (effect on a critical parameter y / a test equipment uncertainty x), and is usually in dB/dB. Often it can be derived by inspection as one or zero. For example, an error of 1dB in the absolute AWGN level Noc would cause 1dB error in the Cell 1 RSRP, so the sensitivity factor is 1.000 for all tests. However the same error of 1dB in the absolute AWGN level Noc would cause no change to Cell 2 Es/Iot, because all other powers are specified relative to Noc, so the sensitivity factor is zero.

In some cases, the sensitivity factor is an intermediate value. For example, the Cell 2 Es/Noc has an effect on Cell 1 Es/Iot which depends on ratios of the powers making up the total. In such cases the sensitivity factor value between 0 and 1 result. It is important to calculate these correctly to obtain the overall uncertainty.

For example,

- In Test 1, the effect of Cell 2 Es/Noc uncertainty on Cell 1 Es/Iot is x 0.32

The SS-SINR also has sensitivity factors with an intermediate value. In simple terms the SS-SINR is the ratio (Power in wanted cell / interference and noise on that frequency), then scaled to be in 15 kHz (one subcarrier) rather than 180 kHz (one resource block). This gives the following sensitivity factors:

- The effect of Noc absolute power on SS-SINR is zero, because all the cell powers are specified relative to Noc. The SS-SINR does not change so the sensitivity factor is 0.

- The effect of Cell 1 Es/Noc on Cell 2 SS-SINR is scaled according to the linear fraction of power in Cell 1.

- The effect of Cell 2 Es/Noc on Cell 2 SS-SINR is scaled according to the linear fraction of power and noise not from Cell 2.

It is quite difficult to deduce the sensitivity factors for SS-SINR intuitively. The figures given can be verified using the spreadsheet, by entering different figures into the “Original specified key parameters” in section a, and monitoring the effect on the “Derived parameters” in section b.

Having filled in the matrix of sensitivity factors, the accompanying spreadsheet calculates the overall uncertainty for each controlled parameter, taking into account the uncertainties and sensitivity factors for each parameter that can be set by the test equipment, and the UE SS-SINR absolute measurement accuracy where relevant. This process follows the superposition principle. More details and explanation can be found in section 4 of TR 38.903.

For the test system uncertainties the normal procedure of combining uncorrelated uncertainties root-sum-square is followed.

When the test system uncertainties and the UE SS-SINR reporting accuracy are combined, the root-sum square of test system uncertainties is added arithmetically to the UE SS-SINR reporting accuracy. If all the uncertainties were combined root-sum square, the resulting smaller test limits could cause a conformant test system to fail a conformant UE, which would be unacceptable.

e) Determine parameters to offset

For Test Configuration 3, we observe that the nominal Io for Test 1 is on the -50 dBm upper limit of the +/-3.5 dB UE reporting accuracy range, and that the test equipment uncertainties could take the UE outside the allowed range. The Noc for Test 1 is therefore decreased by an amount sufficient to achieve Io <= -50 dBm.

For all Test Configurations in Test 2 the test equipment uncertainties can cause the SSB Ês/Iot of Cell 2 to fall below the lower limit of -6 dB for ±3.5 dB UE reporting accuracy. The Es/Noc of Cell 2 is therefore increased by an amount sufficient to achieve Cell 2 SSB Es/Iot >=-6dB. Cell 1 is treated in the same way to ensure that it also remains within limits, although this may not be strictly necessary to meet the test purpose.

The SS-SINR values for Cell 2 remains inside the allowed range, and are checked in step g).

f) Parameters modified by Test Tolerances

Based on the decision in e), the set of parameters in a) and b) is reproduced in section f) of the accompanying spreadsheet, but this time modified by the Test Tolerances (applied offsets).

For all Test Configurations, for Test 3 the Es/Noc of Cell 1 and Cell 2 are each increased by an amount sufficient to achieve Es/Iot >= - 6dB. The actual offset required is +0.5 dB for all variants of Test 3 in all Test Configurations. These values are found empirically by observing the minimum/maximum parameter values in step g).

Fortunately the increase of Cell 1 and Cell 2 Es/Noc in Test 3 also increases the SS-SINR, ensuring that it remains above the band-dependent minimum value even with test equipment uncertainties.

Additionally, for Test Configuration 3 only, the Noc for Test 1 is decreased by an amount sufficient to achieve Io <= -50 dBm. The actual offset required is 0.2 dB. The value is set equal to the overall uncertainty for Io, or can be found empirically by observing the minimum/maximum parameter values in step g).

Re-derived parameters such as SS-SINR are calculated using the same methods as were used in step b).

g) Check controlled parameters Min/Max

Using a format similar to that in step d), the nominal value of each controlled parameter is recalculated, as at least some will have changed from the original due to the application of the Test Tolerances in step f).

The minimum and maximum values, due to variability from uncertainties, of controlled parameters is then calculated and compared against the requirements (SSB Es/Iot range, SS-SINR range, SS-RSRP power range, and the Io power range). It is not necessary to calculate all parameters during each test, so a selection is made of those critical to the test verdict. The critical requirement for each parameter is given briefly in the “Comment” column of section g) in the accompanying spreadsheet. The cases closest to limit (in these test cases, all limits are one-sided) are identified by turquoise cells in the spreadsheet. If all the stimulus requirements are met, then the chosen stimulus offsets are acceptable.

It can be seen that with the uncertainty values and Test Tolerances proposed, the stimulus requirements are met.

For this test, the verdict is based on the SS-SINR values reported by the UE. As stated earlier, the reported values are affected by:

- The uncertainties in the stimulus set by the Test system (Downlink signal)

- The uncertainties in the UE response (SS-SINR reports)

Having ensured that the stimulus set by the Test system remains within the side conditions, and knowing its uncertainty, we now need to calculate the range of SS-SINR values that a conformant UE could report.

# 5. Treatment of related test cases

38.533 also contains a similar SA test case in clauses 6.7.3.1, which have the same NR signal levels and the same signalled parameters related to level, so we propose to apply the same level uncertainties and Test Tolerances. There is no need to provide a tab on the spreadsheet as all the numeric values are identical.

38.533 also contains a similar ATG test case in clauses 19.6.3.1, which have the same NR signal levels and the same signalled parameters related to level. The only difference test parameters between 19.6.3.1 and 4.7.3.1 is the propagation conditions. In 19.6.3.1, additional 220/500Hz Doppler shift is added for 15kHz/30kHz test configurations, which has no impact to TT analysis. So, we propose to apply the same level uncertainties and Test Tolerances. There is no need to provide a tab on the spreadsheet as all the numeric values are identical.

38.533 also contains similar of intra-frequency SS-SINR absolute accuracy test case for RedCap UEs in clause 16.7.3.1 and 16.7.3.2. Same level of uncertainties applies to these test cases since Rel-15 legacy UE and Rel-17 RedCap UE with 1Rx or 2 Rx share the same requirements according to 38.133. For these RedCap test cases:

- Test parameters for config 1/2/4 are identical with that of 4.7.3.1 config 1/2/4/5 with the exception that HD-FDD duplex mode is used in config 4. Then the same test tolerances for 4.7.3.1 config 1/2/4/5 apply considering duplex mode bringing no any additional measurement uncertainties.

- Test parameters for config 3 are identical with that of 4.7.3.1 config 3/6 with the exception that the BW is only 20MHz (51 RBs). As the calculations use different values, a separate spreadsheet tab is provided. The following parameters are changed comparing with 4.7.3.1 config 3/6:

  - Channel bandwidth and RBs in Channel bandwidth, section a) and associated comment

From the analysis one can observe that the only difference is that Io value is 10*(log106/51) = 3.18dB lower. Then same test tolerances also apply to 16.7.3.1/16.7.3.2 config 3 sub-test 2.