3GPP TSG-RAN5 Meeting #108	R5-253800

# Bengaluru, India, 25th - 29th August, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 1172 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | FR2 MU - PC6 spherical Tx test update in 38.521-2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_HST_FR2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Progress has been made in R5-254048 for the MU analysis associated to PC6 1AoA spherical tests. Tx test should be updated accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test procedure updated according to proposal made in mentioned discussion paper. Table numbering of the minimum requirements updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.1, 6.2.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

###### 6.2.1.1.3.6	UE maximum output power for power class 6

The following requirements define the maximum output power radiated by the UE for any transmission bandwidth within the channel bandwidth for non-CA configuration, unless otherwise stated. The period of measurement shall be at least one sub frame (1ms). The minimum output power values for EIRP are found in Table 6.2.1.1.3.6-1. The requirement is verified with the test metric of EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.2.1.1.3.6-1: UE minimum peak EIRP for power class 6

| Operating band | Min peak EIRP (dBm) |
| --- | --- |
| n257 | 30 |
| n258 | 30.4 |
| n261 | 30 |
| NOTE 1: Minimum peak EIRP is defined as the lower limit without tolerance |  |

The maximum output power values for TRP and EIRP are found in Table 6.2.1.1.3.6-2 below. The maximum allowed EIRP is derived from regulatory requirements [8]. The requirements are verified with the test metrics of TRP (Link=TX beam peak direction, Meas=TRP grid) in beam locked mode and EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.2.1.1.3.6-2: UE maximum output power limits for power class 6

| Operating band | Max TRP (dBm) | Max EIRP (dBm) |
| --- | --- | --- |
| n257 | 23 | 43 |
| n258 | 23 | 43 |
| n261 | 23 | 43 |

The minimum EIRP measured over the spherical coverage evaluation areas specified below is defined as the spherical coverage requirement and is found in Table 6.2.1.1.3.6-3 below. UE spherical coverage evaluation areas are found in Table 6.2.1.1.3.6-3a below, by consisting of Area-1 and Area-2, in the reference coordinate system in Annex J.1. The requirement is verified with the test metric of EIRP (Link= Spherical coverage grid, Meas=Link angle).

Table 6.2.1.1.3.6-3: UE spherical coverage for power class 6

| Operating band | Min EIRP over UE spherical coverage evaluation areas (dBm) |
| --- | --- |
| n257 | 20 |
| n258 | 20.4 |
| n261 | 20 |
| NOTE 1: Minimum EIRP over UE spherical coverage evaluation areas is defined as the lower limit without tolerance. NOTE 2: The requirements in this table are verified only under normal temperature conditions as defined in TS 38.508-1 [10] subclause 4.1.1. NOTE 3: The requirements in this table are applicable to FR2 PC6 UE with the network signalling highSpeedMeasFlagFR2-r17 configured as set2. |  |

Table 6.2.1.1.3.6-3a: UE spherical coverage evaluation areas for power class 6

|  | θ range (degree) | ϕ range (degree) |
| --- | --- | --- |
| Area-1 | 90 to 60 | - 37.5 to + 37.5 |
| Area-2 | 90 to 60 | 142.5to 217.5 |
| NOTE 1: When testing power class 6 UEs, DUT orientation can be determined according to the UE spherical coverage evaluation areas, not necessarily following default alignment in Figure J.1-2 or positioning guidelines in clause J.3. NOTE 2: High speed train deployment is expected to be w.r.t. the reference coordination system: θ = 90 (degree) corresponds to the ground plane the train is running on, and ϕ= 0 or 180 with θ = 90 are the train track directions. |  |  |

For the UEs that support multiple FR2 bands, minimum requirement for peak EIRP and EIRP spherical coverage in Tables 6.2.1.1.3.6-1 and 6.2.1.1.3.6-3 shall be decreased per band, respectively, by the peak EIRP relaxation parameter MBP,n and EIRP spherical coverage relaxation parameter MBS,n, as defined in Table 6.2.1.1.3.6-4.

Table 6.2.1.1.3.6-4: UE multi-band relaxation factors for power class 6

| Band | MBP,n (dB) | MBS,n (dB) |
| --- | --- | --- |
| n257 | 0.7 | 0.7 |
| n258 | 0.7 | 0.7 |
| n261 | 0.7 | 0.7 |

## <<< Skip unchanged sections >>>

#### 6.2.1.2	UE maximum output power - Spherical coverage

Editor’s note: The following aspects are either missing or not yet determined:

-	Measurement Uncertainties and Test Tolerances are FFS for power class other than 1, 3, 5, 6 and 7.

6.2.1.2.1	Test purpose

To verify that the spatial coverage of the UE in expected directions is acceptable.

6.2.1.2.2	Test applicability

This test case applies to all types of release 15 NR UEs.

This test case also applies to all types of release 16 and forward NR Power Class 1, Power Class 2 and Power Class 4 UEs.

This test case also applies to all types of release 16 and forward NR Power Class 3 UEs not supporting CSI-RS based or SSB-based enhanced beam correspondence. This test case also applies to all types of release 17 and forward NR Power Class 7 UEs not supporting CSI-RS based or SSB-based enhanced beam correspondence.

6.2.1.2.3	Minimum conformance requirements

Minimum conformance requirements are defined in clause 6.2.1.1.3.

6.2.1.2.4	Test description

6.2.1.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and subcarrier spacing, are shown in Table 6.2.1.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2.1.2.4.1-1: Test Configuration Table

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, Mid Range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | 120 kHz |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |  |
|  |  | Default | - | Modulation |  | RB allocation (NOTE 1) |
| 1 | 50 |  |  | DFT-s-OFDM QPSK |  | Inner_Full for PC2, PC3, |
| 2 | 100 |  |  |  |  | PC4, PC5, PC6 and PC7 |
| 3 | 200 |  |  |  |  | Inner_Full_Region1 for |
| 4 | 400 |  |  |  |  | PC1 |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1 for PC2, PC3, PC4, PC5, PC6 and PC7 or Table 6.1-2 for PC1. |  |  |  |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.1 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C, and uplink signals according to Annex G.

4.	The UL Reference Measurement channels are set according to Table 6.2.1.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.2.1.2.4.3

6.2.1.2.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2.1.2.4.1-1. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. Messages to configure the appropriate uplink modulation in section 6.2.1.2.4.3.

1a.	The side conditions for SSB-based and CSI-RS based L1-RSRP measurements are applied as per clause 6.6.1.3.3.1.1 for PC3 and 6.6.1.3.6.1.1 for PC7.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200 msec to ensure that the UE transmits at its maximum output power. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

3.	Through its beam correspondence procedure, DUT refines its TX beam toward that direction depending on DUT’s beam correspondence capability which shall match OEM declaration:

3a	If the DUT’s beam correspondence capability beamCorrespondenceWithoutUL-BeamSweeping is supported, then DUT autonomously chooses the corresponding TX beam for PUSCH transmission using downlink reference signals to transmit in the direction of the incoming DL signal, which is based on beam correspondence without relying on UL beam sweeping;

3b	If the DUT’s beam correspondence capability beamCorrespondenceWithoutUL-BeamSweeping is not present, then DUT chooses the TX beam for PUSCH transmission which is based on beam correspondence with relying on both DL measurements on downlink reference signals and network-assisted uplink beam sweeping:

3b.1) DUT uses downlink reference signals to select proper RX beam and uses autonomous beam correspondence to select the TX beam.

3b.2) SS configures M=8 SRS resources to DUT, with the field spatialRelationInfo omitted and the field usage set as ‘beamManagement’. In case DUT supports less than 8 SRS resources, SS configures the number of SRS resources according to the maximum number of SRS resources indicated by UE capability signalling. Additionally, for codebook based PUSCH transmission, SS configures a semi-persistent SRS resource set with the field usage as 'codebook'.

3b.3) Based on the TX beam autonomously selected by DUT, DUT chooses TX beams to transmit SRS-resources configured by SS.

3b.4) Based on measurement of the received beamManagement SRS, SS chooses the best SRS beam and, if needed, updates the spatial relation information between the semi-persistent codebook SRS resources and the SS selected beamManagement SRS resource in the activation MAC CE of the semi-persistent SRS resource. The SS indicates in the SRS Resource Indicator (SRI) field in the scheduling grant for PUSCH, if present, the SRS resource within the semi-persistent SRS resource set whose spatial relation is linked to the best detected SRS beam.

3b.5) DUT transmits PUSCH corresponding to the SRS resource indicated by the SRI.

For power class UEs other than PC6, steps 4 and 5 apply:

4.	Measure UE EIRP value for each grid point according to the EIRP spherical coverage procedure defined in Annex K.1.5, and obtain a cumulative distribution function (CDF) of all EIRP dBm values. Alternatively, UE EIRP measurement for each grid point could be done according to Tx Fast spherical coverage procedure defined in Annex K.1.5.1. After a rotation, allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for UE to find the best beam to use. The measuring duration is one active uplink subframe. EIRP is calculated considering both polarizations, theta and phi.

5.	Identify the EIRP dBm value corresponding to %-tile (UE power class dependent) value in the applicable test requirement table in section 6.2.1.2.5.

For power class 6 UEs, steps 6 and 7 apply:

6.	Measure UE EIRP value for each grid point within the UE spherical coverage evaluation areas, Area-1 and Area-2, per Table 6.2.1.1.3.6-3a. After a rotation, allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for UE to find the best beam to use. The measuring duration is one active uplink subframe. EIRP is calculated considering both polarizations, theta and phi and EIRP needs to be recorded separately for both link polarizations EIRP(PolLink=q), EIRP(PolLink=f).

7.	Identify the minimum EIRP dBm value corresponding to value in the applicable test requirement table in clause 6.2.1.2.5.

NOTE 1:	The BEAM_SELECT_WAIT_TIME default value is defined in Annex K.

6.2.1.2.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6 with TRANSFORM_PRECODER_ENABLED condition in Table 4.6.3-118 PUSCH-Config.

6.2.1.2.5	Test requirement

The defined %-tile EIRP in measurement distribution derived in step 5 shall exceed the values specified in Table 6.2.1.2.5-1 to Table 6.2.1.2.5-4.

Table 6.2.1.2.5-1: UE spherical coverage for power class 1

| Operating band | Min EIRP at 85%-tile CDF (dBm) |
| --- | --- |
| n257 | 32.0-TT |
| n258 | 32.0-TT |
| n260 | 30.0-TT |
| n261 | 32.0-TT |

Table 6.2.1.2.5-1a: Test Tolerance (UE spherical coverage for Power class 1)

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| Max device size ≤ 30 cm | 2.69 dB | 2.69 dB |

Table 6.2.1.2.5-2: UE spherical coverage for power class 2

| Operating band | Min EIRP at 60%-tile CDF (dBm) |
| --- | --- |
| n257 | 18.0-TT |
| n258 | 18.0-TT |
| n260 |  |
| n261 | 18.0-TT |

Table 6.2.1.2.5-3: UE spherical coverage for power class 3 for single band UE or multiband UE declaring MBs = 0 in all FR2 bands

| Operating band | Min EIRP at 50t%-tile CDF (dBm) |
| --- | --- |
| n257 | 11.5-TT |
| n258 | 11.5-TT |
| n259 | 5.8-TT |
| n260 | 8-TT |
| n261 | 11.5-TT |

Table 6.2.1.2.5-3a: UE spherical coverage for power class 3 for multi band UE declaring MBs>0 in any FR2 band (Rel-15)

| ID | Supported FR2 bands set | Test requirement (dB) (Note 1) |  |  |  | Maximum sum of MBs, ∑MBs (dB) (Note 3) | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | n257 | n258 | n260 | n261 |  |  |
| 1 | n257, n258 | 11.5-TT-MBs | 11.5-TT-MBs |  |  | 1.25 | Maximum 0.75 dB relaxation allowed for each band |
| 2 | n257, n260 | 11.5-TT-MBs |  | 8-TT-MBs |  | 0.75 | Maximum 0.4 dB relaxation allowed for n260 and 0.75 dB relaxation allowed for all other bands |
| 3 | n258, n260 |  | 11.5-TT-MBs | 8-TT-MBs |  | 0.75 | Maximum 0.4 dB relaxation allowed for n260 and 0.75 dB relaxation allowed for all other bands |
| 4 | n258, n261 |  | 11.5-TT-MBs |  | 11.5-TT-MBs | 1.25 | Maximum 0.75 dB relaxation allowed for each band |
| 5 | n260, n261 |  |  | 8-TT-MBs | 11.5-TT-MBs | 0.75 | No relaxation allowed for n260 and 0.75 dB relaxation allowed for all other bands |
| 6 | n257, n258, n260 | 11.5-TT-MBs | 11.5-TT-MBs | 8-TT-MBs |  | 1.75 | Maximum 0.4 dB relaxation allowed for n260 and 0.75 dB relaxation allowed for all other bands |
| 7 | n257, n258, n261 | 11.5-TT-MBs | 11.5-TT-MBs |  | 11.5-TT-MBs | 1.75 | Maximum 0.75 dB relaxation allowed for each band |
| 8 | n257, n260, n261 | 11.5-TT-MBs |  | 8-TT-MBs | 11.5-TT-MBs | 1.25 | Maximum 0.4 dB relaxation allowed for n260 and 0.75 dB relaxation allowed for all other bands |
| 9 | n258, n260, n261 |  | 11.5-TT-MBs | 8-TT-MBs | 11.5-TT-MBs | 1.25 | Maximum 0.4 dB relaxation allowed for n260 and 0.75 dB relaxation allowed for all other bands |
| 10 | n257, n258, n260, n261 | 11.5-TT-MBs | 11.5-TT-MBs | 8-TT-MBs | 11.5-TT-MBs | 1.75 | Maximum 0.4 dB relaxation allowed for n260 and 0.75 dB relaxation allowed for all other bands |
| Note 1: MBs is the Multiband Relaxation factor declared by the UE for the tested band in table A.4.3.9-3 of TS38.508-2 [11]. This declaration shall fulfil the requirements in Table 6.2.1.1.3.3-4. Note 2: All UE supported bands needs to be tested to ensure the multiband relaxation declaration is compliant Note 3: Max allowed sum of MBs over all supported FR2 bands as defined in clause 6.2.1.1.3.3. Note 4: For a Rel-15 UE supporting FR2 bands set not defined in Table 6.2.1.1.3.3-4, Table 6.2.1.2.5-3c applies. |  |  |  |  |  |  |  |

Table 6.2.1.2.5-3b: Test Tolerance (UE spherical coverage for Power class 3)

| Test Metric | FR2a | FR2b | FR2c |
| --- | --- | --- | --- |
| Max device size ≤ 30 cm | 2.69 dB | 2.69 dB | 3.50 dB |

Table 6.2.1.2.5-3c: UE spherical coverage for power class 3 (Rel-16 and forward)

| ID | FR2 bands/set | Test requirement (dB) (Note 1) |  |  |  |  | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | n257 | n258 | n259 | n260 | n261 |  |
| 1 | n257 | 11.5-TT-MBs,n |  |  |  |  |  |
| 2 | n258 |  | 11.5-TT-MBs,n |  |  |  |  |
| 3 | n259 |  |  | 5.8-TT-MBs,n |  |  |  |
| 4 | n260 |  |  |  | 8-TT-MBs,n |  |  |
| 5 | n261 |  |  |  |  | 11.5-TT-MBs,n |  |
| 6 | n257, n261 | 11.5-TT-MBs,n |  |  |  | 11.5-TT-MBs,n | MBs,n relaxation is 0 dB |
| 7 | n260, n261 |  |  |  | 8-TT-MBs,n | 11.5-TT-MBs,n | MBs,n relaxation is 0 dB for n260 |
| Note 1: MBs,n is the Multiband Relaxation factor for the tested band. This shall fulfil the requirements in Table 6.2.1.1.3.3-5. |  |  |  |  |  |  |  |

Table 6.2.1.2.5-4: UE spherical coverage for power class 4

| Operating band | Min EIRP at 20%-tile CDF (dBm) |
| --- | --- |
| n257 | 25 |
| n258 | 25 |
| n260 | 19 |
| n261 | 25 |

Table 6.2.1.2.5-5: UE spherical coverage for power class 5

| Operating band | Min EIRP at 85%-tile CDF (dBm) |
| --- | --- |
| n257 | 22.0-TT-MBs,n |
| n258 | 22.4-TT-MBs,n |
| Note 1: MBs,n = 0 for single band UE. For multi-band UEs, MBs,n is defined in table 6.2.1.1.3.5-5. |  |

Table 6.2.1.2.5-5a: Test Tolerance (UE spherical coverage for Power class 5)

| Test Metric | FR2a |
| --- | --- |
| Max device size ≤ 30 cm | 2.69 dB |

Table 6.2.1.2.5-6: UE spherical coverage for power class 6

| Operating band | Min EIRP at 85%-tile CDF (dBm) |
| --- | --- |
| n257, n261 | 20.0-TT-MBs,n |
| n258 | 20.4-TT-MBs,n |
| Note 1: MBs,n = 0 for single band UE. For multi-band UEs, MBs,n is defined in table 6.2.1.1.3.6-4. |  |

Table 6.2.1.2.5-6a: Test Tolerance (UE spherical coverage for Power class 6)

| Test Metric | FR2a |
| --- | --- |
| Max device size ≤ 30 cm | 3.11 dB |

Table 6.2.1.2.5-7: UE spherical coverage for power class 7

| Operating band | Min EIRP at 50 %-tile CDF (dBm) |
| --- | --- |
| n257 | 5.5 |
| n258 | 5.5 |
| n261 | 5.5 |
| NOTE 1: Minimum EIRP at 50 %-tile CDF is defined as the lower limit without tolerance NOTE 2: The requirements in this table are verified only under normal temperature conditions as defined in TS 38.508-1 [10] subclause 4.1.1. |  |

Table 6.2.1.2.5-7a: Test Tolerance (UE spherical coverage for Power class 7)

| Test Metric | FR2a |
| --- | --- |
| Max device size ≤ 30 cm | 2.69 dB |

Table 6.2.1.2.5-7b: UE spherical coverage for power class 7 (Rel-16 and forward)

| ID | FR2 bands/set | Test requirement (dB) (Note 1) |  |  |  |  | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | n257 | n258 | n259 | n260 | n261 |  |
| 1 | n257 | 5.5-TT-MBs,n |  |  |  |  |  |
| 2 | n258 |  | 5.5-TT-MBs,n |  |  |  |  |
| 3 | n261 |  |  |  |  | 5.5-TT-MBs,n |  |
| 4 | n257, n261 | 5.5-TT-MBs,n |  |  |  | 5.5-TT-MBs,n | MBs,n relaxation is 0 dB |
| Note 1: MBs,n is the Multiband Relaxation factor for the tested band. This shall fulfil the requirements in Table 6.2.1.1.3.3-5. |  |  |  |  |  |  |  |

## <<< END OF CHANGES >>>