3GPP TSG-RAN5 Meeting #108	R5-253804

# Bengaluru, India, 25th - 29th August, 2025

Agenda item:	5.4.20

Source:	Keysight Technologies UK Ltd

Title:	FR2 MU - PC3 analysis for OBW UL MIMO test

Document for:	Discussion and Endorsement

# Introduction

This contribution aims to continue the MU analysis for PC3 in OBW UL MIMO test essentially resubmitting the proposals made in [1].

# Discussion

As per analysis made in [1] and associated agreements, currently the MU is undefined for the following cases in the OBW UL MIMO test.

| Frequency sub-range | CBW | UL level | Noise Floor assumption | SNR | OBW MTSU | MBW/CBW |
| --- | --- | --- | --- | --- | --- | --- |
| FR2b | 400 MHz | 11.6 dBm | -5.5dBm | 17.1 dB | [1.7] %CBW | [1.3] |
| FR2c | 200 MHz | 11.7 dBm | -7.5dBm | 19.2 dB | [1.2] %CBW | [1.3] |
|  | 400 MHz | 9.7 dBm | -4.5dBm | 14.2 dB | TBD | TBD |

Table 2.1. Test points with undefined MU in OBW UL MIMO test for PC3.

For the values in square brackets, it is proposed to accept them by removing the [ ].

Proposal 1. For OBW UL MIMO test in FR2b, CBW = 400MHz and FR2c, CBW = 200MHz, accept current MTSU and MBW defined for PC3 by removing the [ ].

Regarding FR2c, CBW = 400MHz case, analysis in [1] concluded that to keep MTSU under control, about 1.2%CBW, the required MBW would be 1.1*CBW. It is still our understanding that such a MBW is not acceptable considering the general spurious emission mask is permitted to the UE. Therefore, our proposal again is to declare this case as not testable.

Proposal 2. For OBW UL MIMO test in FR2c, for CBW = 400MHz, declare the test as not testable.

# Conclusion

The following observations and proposals were made in this contribution.

Proposal 1. For OBW UL MIMO test in FR2b, CBW = 400MHz and FR2c, CBW = 200MHz, accept current MTSU and MBW defined for PC3 by removing the [ ].

Proposal 2. For OBW UL MIMO test in FR2c, for CBW = 400MHz, declare the test as not testable.

# References

- R5-253603, “FR2 MU - PC3 analysis for OBW UL MIMO test”, Keysight, RAN5#107

- R5-247165, “MU discussion on OBW for FR2 UL MIMO”, Anritsu, RAN5#105

- R5-247337, “On the MU and TT for SISO and MIMO test cases”, R&S, RAN5#105

- R5-242998, “On testability issue of FR2c OBW”, Anritsu, RAN5#103

- R5-244167, “Discussion on MU and TT analysis for FR2c and FR2b”, Keysight, RAN5#104