3GPP TSG RAN WG5 #108	R5-254427

# Bangalore, India 25th - 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3456 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to N_RB_alloc numbers in Table 6.1A-1a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI-18 |  |  |  |  |  | Date: |  |  | 2025-08-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Some N_RB_alloc numbers are incorrect |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the N_RB_alloc numbers. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect N_RB_alloc numbers are in the spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

Table 6.1A-1a: Common uplink configuration for intra-band contiguous 2UL CA (contiguous RB allocation)

| BWchannel_CA (MHz) | SCS for all CC (kHz) | Channel bandwidth combination (MHz) | OFDM | RB allocation (Inner Full) |  |  | RB allocation (Outer Full) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 |
|  | 15 | 5+15 | CP | 52 | 0@0 | 52@1 | 104 | 25@0 | 79@0 |
|  |  |  | DFT-s | 50 | 0@0 | 50@0 | 100 | 25@0 | 75@0 |
| 20 |  | 10+10 | CP | 52 | 26@26 | 26@0 | 104 | 52@0 | 52@0 |
|  |  |  | DFT-s | 50 | 25@27 | 25@0 | 100 | 50@2 | 50@0 |
|  |  | 15+5 | CP | 52 | 52@26 | 0@0 | 104 | 79@0 | 25@0 |
|  |  |  | DFT-s | 50 | 50@29 | 0@0 | 100 | 75@4 | 25@0 |
|  | 15 | 5+20 | CP | 654 | 0@0 | 65@7 | 131 | 25@0 | 106@0 |
|  |  |  | DFT-s | 64 | 0@0 | 64@7 | 125 | 25@0 | 100@0 |
|  |  | 10+15 | CP | 654 | 20@32 | 45@0 | 131 | 52@0 | 79@0 |
|  |  |  | DFT-s | 65 | 20@32 | 45@0 | 125 | 50@2 | 75@0 |
|  |  | 15+10 | CP | 654 | 47@32 | 18@0 | 131 | 79@0 | 52@0 |
|  |  |  | DFT-s | 65 | 45@34 | 20@0 | 125 | 75@4 | 50@0 |
|  |  | 20+5 | CP | 654 | 65@32 | 0@0 | 131 | 106@0 | 25@0 |
| 25 |  |  | DFT-s | 64 | 64@35 | 0@0 | 125 | 100@6 | 25@0 |
|  | 30 | 10+15 | CP | 304 | 9@15 | 21@0 | 62 | 24@0 | 38@0 |
|  |  |  | DFT-s | 29 | 9@15 | 20@0 | 60 | 24@0 | 36@0 |
|  |  | 15+10 | CP | 304 | 23@15 | 7@0 | 62 | 38@0 | 24@0 |
|  |  |  | DFT-s | 29 | 20@18 | 9@0 | 60 | 36@2 | 24@0 |
|  | 15 | 10+20 | CP | 79 | 13@39 | 66@0 | 158 | 52@0 | 106@0 |
|  |  |  | DFT-s | 76 | 12@40 | 64@0 | 150 | 50@2 | 100@0 |
|  |  | 15+15 | CP | 79 | 40@39 | 39@0 | 158 | 79@0 | 79@0 |
|  |  |  | DFT-s | 76 | 40@39 | 36@0 | 150 | 75@4 | 75@0 |
|  |  | 20+10 | CP | 79 | 67@39 | 12@0 | 158 | 106@0 | 52@0 |
|  |  |  | DFT-s | 76 | 64@42 | 12@0 | 150 | 100@6 | 50@0 |
|  | 30 | 10+20 | CP | 37 | 5@19 | 32@0 | 75 | 24@0 | 51@0 |
|  |  |  | DFT-s | 37 | 5@19 | 32@0 | 74 | 24@0 | 50@0 |
| 30 |  | 15+15 | CP | 38 | 19@19 | 19@0 | 76 | 38@0 | 38@0 |
|  |  |  | DFT-s | 36 | 20@18 | 16@0 | 72 | 36@2 | 36@0 |
|  |  | 20+10 | CP | 37 | 32@19 | 5@0 | 75 | 51@0 | 24@0 |
|  |  |  | DFT-s | 37 | 32@19 | 5@0 | 74 | 50@1 | 24@0 |
|  | 15 | 15+20 | CP | 93 | 33@46 | 60@0 | 185 | 79@0 | 106@0 |
|  |  |  | DFT-s | 92 | 32@47 | 60@0 | 175 | 75@4 | 100@0 |
|  |  | 20+15 | CP | 93 | 60@46 | 33@0 | 185 | 106@0 | 79@0 |
|  |  |  | DFT-s | 92 | 60@46 | 32@0 | 175 | 100@6 | 75@0 |
|  | 30 | 15+20 | CP | 44 | 16@22 | 28@0 | 89 | 38@0 | 51@0 |
|  |  |  | DFT-s | 43 | 16@22 | 27@0 | 86 | 36@2 | 50@0 |
| 35 |  | 20+15 | CP | 44 | 29@22 | 15@0 | 89 | 51@0 | 38@0 |
|  |  |  | DFT-s | 43 | 27@24 | 16@0 | 86 | 50@1 | 36@0 |

| BWchannel_CA (MHz) | SCS for all CC (kHz) | Channel bandwidth combination (MHz) | OFDM | RB allocation (Inner Full) |  |  | RB allocation (Outer Full) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 |
|  | 15 | 10+30 | CP | 106 | 0@0 | 106@1 | 212 | 52@0 | 160@0 |
|  |  |  | DFT-s | 100 | 0@52 | 100@0 | 210 | 50@2 | 160@0 |
| 40 |  | 20+20 | CP | 106 | 53@53 | 53@0 | 212 | 106@0 | 106@0 |
|  |  |  | DFT-s | 104 | 54@52 | 50@0 | 200 | 100@6 | 100@0 |
|  |  | 30+10 | CP | 106 | 106@53 | 0@0 | 212 | 160@0 | 52@0 |
|  |  |  | DFT-s | 100 | 100@60 | 0@0 | 210 | 160@0 | 50@0 |
|  | 30 | 10+30 | CP | 504 | 0@0 | 50@1 | 102 | 24@0 | 78@0 |
|  |  |  | DFT-s | 50 | 0@0 | 50@1 | 99 | 24@0 | 75@0 |
|  |  | 20+20 | CP | 504 | 26@25 | 24@0 | 102 | 51@0 | 51@0 |
|  |  |  | DFT-s | 50 | 25@26 | 25@0 | 100 | 50@1 | 50@0 |
|  |  | 30+10 | CP | 504 | 50@25 | 0@0 | 102 | 78@0 | 24@0 |
|  |  |  | DFT-s | 50 | 50@27 | 0@0 | 99 | 75@3 | 24@0 |
|  | 15 | 15+30 | CP | 1064 | 26@53 | 80@0 | 239 | 79@0 | 160@0 |
|  |  |  | DFT-s | 118 | 18@61 | 100@0 | 235 | 75@4 | 160@0 |
| 45 | 30 | 15+30 | CP | 58 | 9@29 | 49@0 | 116 | 38@0 | 78@0 |
|  |  |  | DFT-s | 57 | 9@29 | 48@0 | 111 | 36@2 | 75@0 |
|  | 15 | 10+40 | CP | 134 | 0@0 | 134@15 | 268 | 52@0 | 216@0 |
|  |  |  | DFT-s | 128 | 0@0 | 128@12 | 266 | 50@2 | 216@0 |
|  |  | 20+30 | CP | 133 | 40@66 | 93@0 | 266 | 106@0 | 160@0 |
|  |  |  | DFT-s | 130 | 40@66 | 90@0 | 260 | 100@6 | 160@0 |
|  |  | 30+20 | CP | 133 | 94@66 | 39@0 | 266 | 160@0 | 106@0 |
|  |  |  | DFT-s | 130 | 90@70 | 40@0 | 260 | 160@0 | 100@0 |
| 50 |  | 40+10 | CP | 134 | 134@67 | 0@0 | 268 | 216@0 | 52@0 |
|  |  |  | DFT-s | 128 | 128@76 | 0@0 | 266 | 216@0 | 50@0 |
|  | 30 | 10+40 | CP | 644 | 0@0 | 64@8 | 130 | 24@0 | 106@0 |
|  |  |  | DFT-s | 64 | 0@0 | 64@8 | 124 | 24@0 | 100@0 |
|  |  | 20+30 | CP | 64 | 19@32 | 45@0 | 129 | 51@0 | 78@0 |
|  |  |  | DFT-s | 63 | 18@33 | 45@0 | 125 | 50@1 | 75@0 |
|  |  | 30+20 | CP | 64 | 46@32 | 18@0 | 129 | 78@0 | 51@0 |
|  |  |  | DFT-s | 63 | 45@33 | 18@0 | 125 | 75@3 | 50@0 |
|  |  | 40+10 | CP | 644 | 64@32 | 0@0 | 130 | 106@0 | 24@0 |
|  |  |  | DFT-s | 64 | 64@34 | 0@0 | 124 | 100@6 | 24@0 |
|  | 15 | 15+40 | CP | 1474 | 6@73 | 141@0 | 295 | 79@0 | 216@0 |
|  |  |  | DFT-s | 143 | 8@71 | 135@0 | 291 | 75@4 | 216@0 |
| 55 | 30 | 15+40 | CP | 72 | 2@36 | 70@0 | 144 | 38@0 | 106@0 |
|  |  |  | DFT-s | 68 | 4@34 | 64@0 | 136 | 36@2 | 100@0 |

| BWchannel_CA (MHz) | SCS for all CC (kHz) | Channel bandwidth combination (MHz) | OFDM | RB allocation (Inner Full) |  |  | RB allocation (Outer Full) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 |
|  | 15 | 10+50 | CP | 161 | 0@0 | 161@28 | 322 | 52@0 | 270@0 |
|  |  |  | DFT-s | 160 | 0@0 | 160@28 | 320 | 50@2 | 270@0 |
|  |  | 20+40 | CP | 161 | 26@80 | 135@0 | 322 | 106@0 | 216@0 |
|  |  |  | DFT-s | 160 | 25@81 | 135@0 | 316 | 100@6 | 216@0 |
|  |  | 30+30 | CP | 160 | 80@80 | 80@0 | 320 | 160@0 | 160@0 |
|  |  |  | DFT-s | 160 | 80@80 | 80@0 | 320 | 160@0 | 160@0 |
|  |  | 40+20 | CP | 161 | 136@80 | 25@0 | 322 | 216@0 | 106@0 |
|  |  |  | DFT-s | 160 | 135@81 | 25@0 | 316 | 216@0 | 100@0 |
|  |  | 50+10 | CP | 161 | 161@80 | 0@0 | 322 | 270@0 | 52@0 |
|  |  |  | DFT-s | 160 | 160@82 | 0@0 | 320 | 270@0 | 50@0 |
|  | 30 | 10+50 | CP | 78 | 0@0 | 78@15 | 157 | 24@0 | 133@0 |
|  |  |  | DFT-s | 75 | 0@0 | 75@14 | 152 | 24@0 | 128@0 |
| 60 |  | 20+40 | CP | 78 | 12@39 | 66@0 | 157 | 51@0 | 106@0 |
|  |  |  | DFT-s | 76 | 12@39 | 64@0 | 150 | 50@1 | 100@0 |
|  |  | 30+30 | CP | 78 | 39@39 | 39@0 | 156 | 78@0 | 78@0 |
|  |  |  | DFT-s | 76 | 40@38 | 36@0 | 150 | 75@3 | 75@0 |
|  |  | 40+20 | CP | 78 | 67@39 | 11@0 | 157 | 106@0 | 51@0 |
|  |  |  | DFT-s | 76 | 64@42 | 12@0 | 150 | 100@6 | 50@0 |
|  |  | 50+10 | CP | 78 | 78@39 | 0@0 | 157 | 133@0 | 24@0 |
|  |  |  | DFT-s | 75 | 75@44 | 0@0 | 152 | 128@5 | 24@0 |
|  | 15 | 20+50 | CP | 188 | 12@94 | 176@0 | 376 | 106@0 | 270@0 |
|  |  |  | DFT-s | 181 | 1@105 | 180@0 | 370 | 100@6 | 270@0 |
|  |  | 30+40 | CP | 188 | 66@94 | 122@0 | 376 | 160@0 | 216@0 |
|  |  |  | DFT-s | 184 | 64@96 | 120@0 | 376 | 160@0 | 216@0 |
|  |  | 40+30 | CP | 188 | 122@94 | 66@0 | 376 | 216@0 | 160@0 |
|  |  |  | DFT-s | 184 | 120@96 | 64@0 | 376 | 216@0 | 160@0 |
|  |  | 50+20 | CP | 188 | 176@94 | 12@0 | 376 | 270@0 | 106@0 |
|  |  |  | DFT-s | 181 | 180@90 | 1@0 | 370 | 270@0 | 100@0 |
|  | 30 | 20+50 | CP | 92 | 5@46 | 87@0 | 184 | 51@0 | 133@0 |
|  |  |  | DFT-s | 87 | 6@45 | 81@0 | 178 | 50@1 | 128@0 |
|  |  | 30+40 | CP | 92 | 32@46 | 60@0 | 184 | 78@0 | 106@0 |
| 70 |  |  | DFT-s | 92 | 32@46 | 60@0 | 175 | 75@3 | 100@0 |
|  |  | 40+30 | CP | 92 | 60@46 | 32@0 | 184 | 106@0 | 78@0 |
|  |  |  | DFT-s | 92 | 60@46 | 32@0 | 175 | 100@6 | 75@0 |
|  |  | 50+20 | CP | 92 | 87@46 | 5@0 | 184 | 133@0 | 51@0 |
|  |  |  | DFT-s | 87 | 81@52 | 6@0 | 178 | 128@5 | 50@0 |

| BWchannel_CA (MHz) | SCS for all CC (kHz) | Channel bandwidth combination (MHz) | OFDM | RB allocation (Inner Full) |  |  | RB allocation (Outer Full) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 |
|  | 15 | 30+50 | CP | 215 | 53@107 | 162@0 | 430 | 160@0 | 270@0 |
|  |  |  | DFT-s | 210 | 50@110 | 160@0 | 430 | 160@0 | 270@0 |
|  |  | 40+40 | CP | 216 | 108@108 | 108@0 | 432 | 216@0 | 216@0 |
|  |  |  | DFT-s | 216 | 108@108 | 108@0 | 432 | 216@0 | 216@0 |
|  |  | 50+30 | CP | 215 | 163@107 | 52@0 | 430 | 270@0 | 160@0 |
|  |  |  | DFT-s | 210 | 162@108 | 48@0 | 430 | 270@0 | 160@0 |
|  | 30 | 30+50 | CP | 105 | 25@53 | 80@0 | 211 | 78@0 | 133@0 |
| 80 |  |  | DFT-s | 105 | 25@53 | 80@0 | 203 | 75@3 | 128@0 |
|  |  | 40+40 | CP | 106 | 53@53 | 53@0 | 212 | 106@0 | 106@0 |
|  |  |  | DFT-s | 104 | 54@52 | 50@0 | 200 | 100@6 | 100@0 |
|  |  | 50+30 | CP | 105 | 80@53 | 25@0 | 211 | 133@0 | 78@0 |
|  |  |  | DFT-s | 105 | 80@53 | 25@0 | 203 | 128@5 | 75@0 |
|  | 15 | 40+50 | CP | 243 | 95@121 | 148@0 | 486 | 216@0 | 270@0 |
| 90 |  |  | DFT-s | 240 | 96@120 | 144@0 | 486 | 216@0 | 270@0 |
|  |  | 50+40 | CP | 243 | 149@121 | 94@0 | 486 | 270@0 | 216@0 |
|  |  |  | DFT-s | 240 | 150@120 | 90@0 | 486 | 270@0 | 216@0 |
|  | 30 | 40+50 | CP | 119 | 46@60 | 73@0 | 239 | 106@0 | 133@0 |
|  |  |  | DFT-s | 117 | 45@61 | 72@0 | 228 | 100@6 | 128@0 |
|  |  | 50+40 | CP | 119 | 73@60 | 46@0 | 239 | 133@0 | 106@0 |
|  |  |  | DFT-s | 117 | 72@61 | 45@0 | 228 | 128@5 | 100@0 |
|  | 15 | 50+50 | CP | 270 | 135@135 | 135@0 | 540 | 270@0 | 270@0 |
|  |  |  | DFT-s | 270 | 135@135 | 135@0 | 540 | 270@0 | 270@0 |
| 100 | 30 | 50+50 | CP | 1324 | 67@66 | 65@0 | 266 | 133@0 | 133@0 |
|  |  |  | DFT-s | 128 | 64@69 | 64@0 | 256 | 128@5 | 128@0 |
|  | 15 | N/A |  |  |  |  |  |  |  |
|  | 30 | 15+90 | CP | 141 | 0@0 | 141@33 | 283 | 38@0 | 245@0 |
|  |  |  | DFT-s | 135 | 0@0 | 135@30 | 279 | 36@2 | 243@0 |
| 105 |  | 25+80 | CP | 1404 | 0@0 | 140@5 | 282 | 65@0 | 217@0 |
|  |  |  | DFT-s | 135 | 0@0 | 135@3 | 280 | 64@1 | 216@0 |

| BWchannel_CA (MHz) | SCS for all CC (kHz) | Channel bandwidth combination (MHz) | OFDM | RB allocation (Inner Full) |  |  | RB allocation (Outer Full) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 |
|  | 15 | N/A |  |  |  |  |  |  |  |
|  | 30 | 10+100 | CP | 148 | 0@0 | 148@50 | 297 | 24@0 | 273@0 |
|  |  |  | DFT-s | 144 | 0@0 | 144@48 | 294 | 24@0 | 270@0 |
|  |  | 20+90 | CP | 148 | 0@0 | 148@23 | 296 | 51@0 | 245@0 |
|  |  |  | DFT-s | 144 | 0@0 | 144@21 | 293 | 50@1 | 243@0 |
|  |  | 30+80 | CP | 147 | 4@74 | 143@0 | 295 | 78@0 | 217@0 |
|  |  |  | DFT-s | 147 | 3@74 | 144@0 | 291 | 75@3 | 216@0 |
|  |  | 40+70 | CP | 147 | 32@74 | 115@0 | 295 | 106@0 | 189@0 |
|  |  |  | DFT-s | 140 | 32@74 | 108@0 | 280 | 100@6 | 180@0 |
|  |  | 50+60 | CP | 147 | 59@74 | 88@0 | 295 | 133@0 | 162@0 |
|  |  |  | DFT-s | 141 | 60@73 | 81@0 | 290 | 128@5 | 162@0 |
|  |  | 60+50 | CP | 147 | 88@74 | 59@0 | 295 | 162@0 | 133@0 |
|  |  |  | DFT-s | 141 | 81@81 | 60@0 | 290 | 162@0 | 128@0 |
|  |  | 70+40 | CP | 147 | 115@74 | 32@0 | 295 | 189@0 | 106@0 |
|  |  |  | DFT-s | 147 | 108@74 | 32@0 | 280 | 180@9 | 100@0 |
|  |  | 80+30 | CP | 147 | 143@74 | 4@0 | 295 | 217@0 | 78@0 |
|  |  |  | DFT-s | 146 | 144@73 | 2@0 | 291 | 216@1 | 75@0 |
| 110 |  | 90+20 | CP | 148 | 148@74 | 0@0 | 296 | 245@0 | 51@0 |
|  |  |  | DFT-s | 144 | 144@80 | 0@0 | 293 | 243@2 | 50@0 |
|  |  | 100+10 | CP | 148 | 148@74 | 0@0 | 297 | 273@0 | 24@0 |
|  |  |  | DFT-s | 144 | 144@81 | 0@0 | 294 | 270@3 | 24@0 |

| BWchannel_CA (MHz) | SCS for all CC (kHz) | Channel bandwidth combination (MHz) | OFDM | RB allocation (Inner Full) |  |  | RB allocation (Outer Full) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 |
|  | 15 | N/A |  |  |  |  |  |  |  |
|  | 30 | 15+100 | CP | 155 | 0@0 | 155@40 | 311 | 38@0 | 273@0 |
|  |  |  | DFT-s | 150 | 0@0 | 150@37 | 306 | 36@2 | 270@0 |
|  |  | 25+90 | CP | 1544 | 0@0 | 154@12 | 310 | 65@0 | 245@0 |
|  |  |  | DFT-s | 150 | 0@0 | 150@10 | 307 | 64@1 | 243@0 |
|  |  | 90+25 | CP | 1544 | 154@77 | 0@0 | 310 | 245@0 | 65@0 |
| 115 |  |  | DFT-s | 150 | 150@85 | 0@0 | 307 | 243@2 | 64@0 |
|  |  | 100+15 | CP | 155 | 155@78 | 0@0 | 311 | 273@0 | 38@0 |
|  |  |  | DFT-s | 150 | 150@86 | 0@0 | 306 | 270@3 | 36@0 |
|  | 15 | N/A |  |  |  |  |  |  |  |
|  | 30 | 20+100 | CP | 162 | 0@0 | 162@30 | 324 | 51@0 | 273@0 |
|  |  |  | DFT-s | 162 | 0@0 | 162@30 | 320 | 50@1 | 270@0 |
|  |  | 30+90 | CP | 161 | 0@0 | 161@3 | 323 | 78@0 | 245@0 |
|  |  |  | DFT-s | 160 | 0@0 | 160@2 | 318 | 75@3 | 243@0 |
|  |  | 40+80 | CP | 161 | 25@81 | 136@0 | 323 | 106@0 | 217@0 |
|  |  |  | DFT-s | 160 | 25@81 | 135@0 | 316 | 100@6 | 216@0 |
|  |  | 50+70 | CP | 1604 | 53@80 | 107@0 | 322 | 133@0 | 189@0 |
|  |  |  | DFT-s | 160 | 50@81 | 108@0 | 308 | 128@5 | 180@0 |
|  |  | 60+60 | CP | 162 | 81@81 | 81@0 | 324 | 162@0 | 162@0 |
|  |  |  | DFT-s | 162 | 81@81 | 81@0 | 324 | 162@0 | 162@0 |
|  |  | 70+50 | CP | 1604 | 109@80 | 51@0 | 322 | 189@0 | 133@0 |
|  |  |  | DFT-s | 158 | 108@81 | 50@0 | 308 | 180@9 | 128@0 |
|  |  | 80+40 | CP | 161 | 136@81 | 25@0 | 323 | 217@0 | 106@0 |
|  |  |  | DFT-s | 159 | 135@82 | 24@0 | 316 | 216@1 | 100@0 |
|  |  | 90+30 | CP | 161 | 161@81 | 0@0 | 323 | 245@0 | 78@0 |
|  |  |  | DFT-s | 160 | 160@83 | 0@0 | 318 | 243@2 | 75@0 |
| 120 |  | 100+20 | CP | 162 | 162@81 | 0@0 | 324 | 273@0 | 51@0 |
|  |  |  | DFT-s | 162 | 162@81 | 0@0 | 320 | 270@3 | 50@0 |

| BWchannel_CA (MHz) | SCS for all CC (kHz) | Channel bandwidth combination (MHz) | OFDM | RB allocation (Inner Full) |  |  | RB allocation (Outer Full) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 |
|  | 15 | N/A |  |  |  |  |  |  |  |
|  | 30 | 25+100 | CP | 1684 | 0@0 | 168@19 | 338 | 65@0 | 273@0 |
|  |  |  | DFT-s | 162 | 0@0 | 162@16 | 334 | 64@1 | 270@0 |
|  |  | 100+25 | CP | 1684 | 168@84 | 0@0 | 338 | 273@0 | 65@0 |
| 125 |  |  | DFT-s | 162 | 162@95 | 0@0 | 334 | 270@3 | 64@0 |
|  | 15 | N/A |  |  |  |  |  |  |  |
|  | 30 | 30+100 | CP | 175 | 0@0 | 175@10 | 351 | 78@0 | 273@0 |
|  |  |  | DFT-s | 162 | 0@0 | 162@3 | 345 | 75@3 | 270@0 |
|  |  | 40+90 | CP | 175 | 18@88 | 157@0 | 351 | 106@0 | 245@0 |
|  |  |  | DFT-s | 170 | 20@86 | 150@0 | 343 | 100@6 | 243@0 |
|  |  | 50+80 | CP | 1744 | 46@87 | 128@0 | 350 | 133@0 | 217@0 |
|  |  |  | DFT-s | 173 | 45@88 | 128@0 | 344 | 128@5 | 216@0 |
|  |  | 60+70 | CP | 175 | 74@88 | 101@0 | 351 | 162@0 | 189@0 |
|  |  |  | DFT-s | 174 | 72@88 | 100@0 | 342 | 162@0 | 180@0 |
|  |  | 70+60 | CP | 175 | 101@88 | 74@0 | 351 | 189@0 | 162@0 |
|  |  |  | DFT-s | 173 | 100@88 | 72@0 | 342 | 180@9 | 162@0 |
|  |  | 80+50 | CP | 1744 | 130@87 | 44@0 | 350 | 217@0 | 133@0 |
|  |  |  | DFT-s | 173 | 128@89 | 45@0 | 344 | 216@1 | 128@0 |
| 130 |  | 90+40 | CP | 175 | 157@88 | 18@0 | 351 | 245@0 | 106@0 |
|  |  |  | DFT-s | 170 | 160@85 | 10@0 | 343 | 243@2 | 100@0 |
|  |  | 100+30 | CP | 175 | 175@88 | 0@0 | 351 | 273@0 | 78@0 |
|  |  |  | DFT-s | 162 | 162@108 | 0@0 | 345 | 270@3 | 75@0 |

| BWchannel_CA (MHz) | SCS for all CC (kHz) | Channel bandwidth combination (MHz) | OFDM | RB allocation (Inner Full) |  |  | RB allocation (Outer Full) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 |
|  | 15 | N/A |  |  |  |  |  |  |  |
|  | 30 | 40+100 | CP | 189 | 11@95 | 178@0 | 379 | 106@0 | 273@0 |
|  |  |  | DFT-s | 186 | 6@100 | 180@0 | 370 | 100@6 | 270@0 |
|  |  | 50+90 | CP | 1884 | 39@94 | 149@0 | 378 | 133@0 | 245@0 |
|  |  |  | DFT-s | 186 | 36@97 | 150@0 | 371 | 128@5 | 243@0 |
|  |  | 60+80 | CP | 189 | 67@95 | 122@0 | 379 | 162@0 | 217@0 |
|  |  |  | DFT-s | 184 | 64@98 | 120@0 | 378 | 162@0 | 216@0 |
|  |  | 70+70 | CP | 1884 | 95@94 | 93@0 | 378 | 189@0 | 189@0 |
|  |  |  | DFT-s | 184 | 90@95 | 90@0 | 360 | 180@9 | 180@0 |
|  |  | 80+60 | CP | 189 | 122@95 | 67@0 | 379 | 217@0 | 162@0 |
|  |  |  | DFT-s | 184 | 120@97 | 64@0 | 378 | 216@1 | 162@0 |
|  |  | 90+50 | CP | 1884 | 151@94 | 37@0 | 378 | 245@0 | 133@0 |
| 140 |  |  | DFT-s | 186 | 150@95 | 36@0 | 371 | 243@2 | 128@0 |
|  |  | 100+40 | CP | 189 | 178@95 | 11@0 | 379 | 273@0 | 106@0 |
|  |  |  | DFT-s | 186 | 180@93 | 6@0 | 370 | 270@3 | 100@0 |
|  | 15 | N/A |  |  |  |  |  |  |  |
|  | 30 | 50+100 | CP | 2024 | 32@101 | 170@0 | 406 | 133@0 | 273@0 |
|  |  |  | DFT-s | 194 | 32@101 | 162@0 | 398 | 128@5 | 270@0 |
|  |  | 60+90 | CP | 203 | 60@102 | 143@0 | 407 | 162@0 | 245@0 |
|  |  |  | DFT-s | 198 | 54@108 | 144@0 | 405 | 162@0 | 243@0 |
|  |  | 70+80 | CP | 2024 | 88@101 | 114@0 | 406 | 189@0 | 217@0 |
|  |  |  | DFT-s | 187 | 81@102 | 100@0 | 396 | 180@9 | 216@0 |
|  |  | 80+70 | CP | 2024 | 116@101 | 86@0 | 406 | 217@0 | 189@0 |
|  |  |  | DFT-s | 196 | 100@102 | 81@0 | 396 | 216@1 | 180@0 |
|  |  | 90+60 | CP | 203 | 143@102 | 60@0 | 407 | 245@0 | 162@0 |
|  |  |  | DFT-s | 198 | 144@101 | 54@0 | 405 | 243@2 | 162@0 |
|  |  | 100+50 | CP | 2024 | 172@101 | 30@0 | 406 | 273@0 | 133@0 |
| 150 |  |  | DFT-s | 194 | 162@111 | 32@0 | 398 | 270@3 | 128@0 |

| BWchannel_CA (MHz) | SCS for all CC (kHz) | Channel bandwidth combination (MHz) | OFDM | RB allocation (Inner Full) |  |  | RB allocation (Outer Full) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 |
|  | 15 | N/A |  |  |  |  |  |  |  |
|  | 30 | 60+100 | CP | 217 | 53@109 | 164@0 | 435 | 162@0 | 273@0 |
|  |  |  | DFT-s | 216 | 54@108 | 162@0 | 432 | 162@0 | 270@0 |
|  |  | 70+90 | CP | 2164 | 81@108 | 135@0 | 434 | 189@0 | 245@0 |
|  |  |  | DFT-s | 215 | 80@109 | 135@0 | 423 | 180@9 | 243@0 |
|  |  | 80+80 | CP | 2164 | 109@108 | 107@0 | 432 | 216@1 | 216@0 |
|  |  |  | DFT-s | 216 | 108@109 | 108@0 | 432 | 216@1 | 216@0 |
|  |  | 90+70 | CP | 2164 | 137@108 | 79@0 | 434 | 245@0 | 189@0 |
|  |  |  | DFT-s | 216 | 135@109 | 80@0 | 423 | 243@2 | 180@0 |
| 160 |  | 100+60 | CP | 217 | 164@109 | 53@0 | 435 | 273@0 | 162@0 |
|  |  |  | DFT-s | 216 | 162@111 | 54@0 | 432 | 270@3 | 162@0 |
|  | 15 | N/A |  |  |  |  |  |  |  |
|  | 30 | 70+100 | CP | 2304 | 74@115 | 156@0 | 462 | 189@0 | 273@0 |
|  |  |  | DFT-s | 223 | 72@116 | 150@0 | 450 | 180@9 | 270@0 |
|  |  | 80+90 | CP | 2304 | 102@115 | 128@0 | 462 | 217@0 | 245@0 |
|  |  |  | DFT-s | 228 | 100@117 | 128@0 | 459 | 216@1 | 243@0 |
|  |  | 90+80 | CP | 2304 | 130@115 | 100@0 | 462 | 245@0 | 217@0 |
|  |  |  | DFT-s | 228 | 128@117 | 100@0 | 459 | 243@2 | 216@0 |
|  |  | 100+70 | CP | 2304 | 158@115 | 72@0 | 462 | 273@0 | 189@0 |
| 170 |  |  | DFT-s | 229 | 150@116 | 72@0 | 450 | 270@3 | 180@0 |
|  | 15 | N/A |  |  |  |  |  |  |  |
|  | 30 | 80+100 | CP | 2444 | 95@122 | 149@0 | 490 | 217@0 | 273@0 |
|  |  |  | DFT-s | 240 | 96@121 | 144@0 | 486 | 216@1 | 270@0 |
|  |  | 90+90 | CP | 2444 | 123@122 | 121@0 | 490 | 245@0 | 245@0 |
|  |  |  | DFT-s | 240 | 120@125 | 120@0 | 486 | 243@2 | 243@0 |
|  |  | 100+80 | CP | 2444 | 151@122 | 93@0 | 490 | 273@0 | 217@0 |
| 180 |  |  | DFT-s | 240 | 150@123 | 90@0 | 486 | 270@3 | 216@0 |

| BWchannel_CA (MHz) | SCS for all CC (kHz) | Channel bandwidth combination (MHz) | OFDM | RB allocation (Inner Full) |  |  | RB allocation (Outer Full) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 |
|  | 15 | N/A |  |  |  |  |  |  |  |
|  | 30 | 90+100 | CP | 258 | 116@129 | 142@0 | 518 | 245@0 | 273@0 |
|  |  |  | DFT-s | 252 | 108@137 | 144@0 | 513 | 243@2 | 270@0 |
|  |  | 100+90 | CP | 2584 | 144@129 | 114@0 | 518 | 273@0 | 245@0 |
| 190 |  |  | DFT-s | 252 | 144@129 | 108@0 | 513 | 270@3 | 243@0 |
|  | 15 | N/A |  |  |  |  |  |  |  |
|  | 30 | 100+100 | CP | 272 | 137@136 | 135@0 | 546 | 273@0 | 273@0 |
| 200 |  |  | DFT-s | 270 | 135@138 | 135@0 | 540 | 270@3 | 270@0 |
| NOTE 1: BWchannel_CA is defined in clause 5.3A.3 NOTE 2: NRB_alloc for intra-band contiguous UL CA is defined in 6.2A.2.0 NOTE 3: Void NOTE 4: The allocated RB number NRB_alloc is ceil(NRB_agg/2) -1∙2^μ in order to meet Inner RB allocation definition (RBStart,Low ≤ RBStart ≤ RBStart,High) described in subclause 6.2A.2.0. |  |  |  |  |  |  |  |  |  |

## <<< END OF CHANGES >>>