3GPP TSG-RAN5 Meeting #108	R5-254443

# Bengaluru, India, Aug 25th – 29th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-1 | CR | 0654 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introducing new test case NR RSTD measurement reporting delay for PRS aggregation in FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | NR RSTD measurement reporting delay test case for PRS aggregation in FR2 is absent in the specification |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introducing new test case: NR RSTD measurement reporting delay test case for PRS aggregation in FR2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Necessary test case is absent in specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.2.12(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Please implement this CR after CR#0653 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

Table 14.2.10.5-6: value of Teffect and Tlast based on durationOfPRS-ProcessingSymbolsInEveryTms

| durationOfPRS-ProcessingSymbolsInEveryTms | Teffect | Tlast |
| --- | --- | --- |
| n8 | 160 | 168 |
| n16 | 160 | 176 |
| n20 | 160 | 180 |
| n30 | 160 | 190 |
| n40 | 160 | 200 |
| n80 | 160 | 240 |
| n160 | 160 | 320 |
| n320 | 320 | 480 |
| n640 | 640 | 800 |
| n1280 | 1280 | 1440 |

The test tolerances are defined in clauses C.1.6 and C.2.5.

The rate of successful tests during repeated tests shall be at least 90% with a confidence level of 95%.

14.2.12	NR RSTD measurement reporting delay test case for PRS aggregation in FR2

Editor's Note: This test case is incomplete. Following aspects are either missing or FFS:

-	Test requirement is incomplete and MU/TT analysis is missing

14.2.12.1	Test purpose

The purpose of the test is to verify that the RSTD measurement performed by UE by aggregating PRS resources from multiple positioning frequency layers (PFLs) meets the requirements specified in TS 38.133 [50] clause 9.9.2.10 in an environment with AWGN propagation conditions in FR2 in standalone scenario when multiple positioning frequency layer is configured.

#### 14.2.12.2	Test applicability

This test applies to all types of NR UE release 18 onwards that supports DL-TDOA positioning and supportOfDL-PRS-BWA-RRC-Connected-r18.

#### 14.2.12.3	Minimum conformance requirements

Same as in clause 14.2.11.3.

14.2.12.4	Test description

14.2.12.4.1	Initial conditions

Table 14.2.12.4-1: Test Configurations

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 200 MHz bandwidth per PFL, TDD duplex mode |

Test Environment: Normal, as defined in TS 38.508-1 [45] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [45] clause 4.3.1.

Channel bandwidth to be tested: are specified in Table 14.2.12.4-1.

1.	Connect the SS and AWGN noise sources to the UE antenna connector or antenna connectors as shown in Annex A, Figure A.14.

2.	The general test parameter settings are set up according to Table 14.2.12.5-1, Table 14.2.12.5-2 and Table 14.2.12.5-3.

3.	Propagation conditions are set according to clause 4.15.2.

4.	Message contents are defined in clause 14.2.12.4.3.

5.	In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the reference as well as the PCell. Cell 2 and Cell 3 are the neighbour cells. All 3 cells are on the same RF channel in FR1.

14.2.12.4.2	Test procedure

Same test procedure as defined in clause 14.2.11.4.2 with following exceptions:

Table 14.2.12.4-1 replaces Table 14.2.11.4-1; Table 14.2.12.5-1 replaces 14.2.11.5-1; Table 14.2.12.5-2 repalces Table 14.2.11.5-2; Table 14.2.12.5-3 replaces Table 14.2.11.5-3;

14.2.12.4.3	Message contents

Same message contents as defined in clause 14.2.11.4.3, with the following exceptions:

Table 14.2.12.4.3-1 replaces Table 14.2.11.4.3-5; Table 14.2.12.4.3-2 replacesTable 14.2.11.4.3-6.

Table 14.2.12.4.3-1: NR-DL-PRS-AssistanceData

| Derivation Path: 37.355 clause 6.4.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NR-DL-PRS-AssistanceData-r16 ::= SEQUENCE { |  |  |  |
| nr-DL-PRS-ReferenceInfo-r16 SEQUENCE { |  |  |  |
| dl-PRS-ID-r16 | 0 |  |  |
| nr-DL-PRS-ResourceID-List-r16 | Not present |  |  |
| nr-DL-PRS-ResourceSetID-r16 | Not present |  |  |
| } |  |  |  |
| nr-DL-PRS-AssistanceDataList-r16 SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF NR-DL-PRS-AssistanceDataPerFreq-r16 { | 2 entries |  |  |
| NR-DL-PRS-AssistanceDataPerFreq-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-PositioningFrequencyLayer-r16 SEQUENCE { |  |  |  |
| dl-PRS-SubcarrierSpacing-r16 | kHz120 |  |  |
| dl-PRS-ResourceBandwidth-r16 | 3 | 32 PRBs |  |
| dl-PRS-StartPRB-r16 | 0 |  |  |
| dl-PRS-PointA-r16 | absoluteFrequencyPointA + offsetToCarrier, as defined for the DL frequency of the Cell 1 |  |  |
| dl-PRS-CombSizeN-r16 | n2 |  |  |
| dl-PRS-CyclicPrefix-r16 | normal |  |  |
| } |  |  |  |
| nr-DL-PRS-AssistanceDataPerFreq-r16 SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF NR-DL-PRS-AssistanceDataPerTRP-r16{ | 3 entries |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[1] SEQUENCE { |  | entry 1 | Cell 1 |
| dl-PRS-ID-r16 | 0 |  |  |
| nr-PhysCellID-r16 | Cell 1 |  |  |
| nr-CellGlobalID-r16 | Cell 1 |  |  |
| nr-ARFCN-r16 | Cell 1 |  |  |
| nr-DL-PRS-SFN0-Offset-r16 SEQUENCE { |  |  |  |
| sfn-Offset-r16 | 0 |  |  |
| integerSubframeOffset-r16 | 0 |  |  |
| } |  |  |  |
| nr-DL-PRS-ExpectedRSTD-r16 | 0 |  |  |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | 0 |  |  |
| nr-DL-PRS-Info-r16 | As specified in Table 14.2.12.4.3-2 |  |  |
| } |  |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[2] SEQUENCE { |  | entry 2 | Cell 2 |
| dl-PRS-ID-r16 | 1 |  |  |
| nr-PhysCellID-r16 | Cell 2 |  |  |
| nr-CellGlobalID-r16 | Cell 2 |  |  |
| nr-ARFCN-r16 | Cell 2 |  |  |
| nr-DL-PRS-SFN0-Offset-r16 SEQUENCE { |  |  |  |
| sfn-Offset-r16 | 0 |  |  |
| integerSubframeOffset-r16 | 0 |  |  |
| } |  |  |  |
| nr-DL-PRS-ExpectedRSTD-r16 | 23 | About 3 s |  |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | 39 | About 5 s |  |
| nr-DL-PRS-Info-r16 | As specified in Table 14.2.12.4.3-2 |  |  |
| } |  |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[3] SEQUENCE { |  | entry 3 | Cell 3 |
| dl-PRS-ID-r16 | 2 |  |  |
| nr-PhysCellID-r16 | Cell 3 |  |  |
| nr-CellGlobalID-r16 | Cell 3 |  |  |
| nr-ARFCN-r16 | Cell 3 |  |  |
| nr-DL-PRS-SFN0-Offset-r16 SEQUENCE { |  |  |  |
| sfn-Offset-r16 | 0 |  |  |
| integerSubframeOffset-r16 | 0 |  |  |
| } |  |  |  |
| nr-DL-PRS-ExpectedRSTD-r16 | 23 | About 3 s |  |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | 39 | About 5 s |  |
| nr-DL-PRS-Info-r16 | As specified in Table 14.2.12.4.3-2 |  |  |
| } |  |  |  |
| } |  |  |  |
| NR-DL-PRS-AssistanceDataPerFreq-r16[2] SEQUENCE { |  | entry 2 |  |
| nr-DL-PRS-PositioningFrequencyLayer-r16 SEQUENCE { |  |  |  |
| dl-PRS-SubcarrierSpacing-r16 | kHz120 |  |  |
| dl-PRS-ResourceBandwidth-r16 | 3 | 32 PRBs |  |
| dl-PRS-StartPRB-r16 | 0 |  |  |
| dl-PRS-PointA-r16 | absoluteFrequencyPointA + offsetToCarrier, as defined for the DL frequency of the Cell 2 |  |  |
| dl-PRS-CombSizeN-r16 | n2 |  |  |
| dl-PRS-CyclicPrefix-r16 | normal |  |  |
| } |  |  |  |
| nr-DL-PRS-AssistanceDataPerFreq-r16 SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF NR-DL-PRS-AssistanceDataPerTRP-r16{ | 3 entries |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[1] SEQUENCE { |  | entry 1 | Cell 1 |
| dl-PRS-ID-r16 | 0 |  |  |
| nr-PhysCellID-r16 | Cell 1 |  |  |
| nr-CellGlobalID-r16 | Cell 1 |  |  |
| nr-ARFCN-r16 | Cell 1 |  |  |
| nr-DL-PRS-SFN0-Offset-r16 SEQUENCE { |  |  |  |
| sfn-Offset-r16 | 0 |  |  |
| integerSubframeOffset-r16 | 0 |  |  |
| } |  |  |  |
| nr-DL-PRS-ExpectedRSTD-r16 | 0 |  |  |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | 0 |  |  |
| nr-DL-PRS-Info-r16 | As specified in Table 14.2.12.4.3-2 |  |  |
| } |  |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[2] SEQUENCE { |  | entry 2 | Cell 2 |
| dl-PRS-ID-r16 | 1 |  |  |
| nr-PhysCellID-r16 | Cell 2 |  |  |
| nr-CellGlobalID-r16 | Cell 2 |  |  |
| nr-ARFCN-r16 | Cell 2 |  |  |
| nr-DL-PRS-SFN0-Offset-r16 SEQUENCE { |  |  |  |
| sfn-Offset-r16 | 0 |  |  |
| integerSubframeOffset-r16 | 0 |  |  |
| } |  |  |  |
| nr-DL-PRS-ExpectedRSTD-r16 | 23 | About 3 s |  |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | 39 | About 5 s |  |
| nr-DL-PRS-Info-r16 | As specified in Table 14.2.12.4.3-2 |  |  |
| } |  |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[2] SEQUENCE { |  | entry 3 | Cell 3 |
| dl-PRS-ID-r16 | 2 |  |  |
| nr-PhysCellID-r16 | Cell 3 |  |  |
| nr-CellGlobalID-r16 | Cell 3 |  |  |
| nr-ARFCN-r16 | Cell 3 |  |  |
| nr-DL-PRS-SFN0-Offset-r16 SEQUENCE { |  |  |  |
| sfn-Offset-r16 | 0 |  |  |
| integerSubframeOffset-r16 | 0 |  |  |
| } |  |  |  |
| nr-DL-PRS-ExpectedRSTD-r16 | 23 | About 3 s |  |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | 39 | About 5 s |  |
| nr-DL-PRS-Info-r16 | As specified in Table 14.2.12.4.3-2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nr-SSB-Config-r16 | Not present |  |  |
| nr-DL-PRS-AggregationInfo-r18 SEQUENCE (SIZE (1..nrMaxNumPRS-BandWidthAggregation-r18)) OF NR-linkedDL-PRS-ResourceSetID-PRS-AggregationList-r18 { | 3 entries |  |  |
| NR-linkedDL-PRS-ResourceSetID-PRS-AggregationList-r18[1] SEQUENCE (SIZE (2..3)) OF NR-DL-PRS-AggregationElement-r18 { | 2 entries | entry 1 |  |
| NR-DL-PRS-AggregationElement-r18[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-FrequencyLayerIndex-r18 | 0 |  |  |
| nr-DL-PRS-TRP-Index-r18 | 0 |  |  |
| nr-DL-PRS-ResourceSetIndex-r18 | 0 |  |  |
| } |  |  |  |
| NR-DL-PRS-AggregationElement-r18[2] SEQUENCE { |  | entry 2 |  |
| nr-DL-PRS-FrequencyLayerIndex-r18 | 1 |  |  |
| nr-DL-PRS-TRP-Index-r18 | 0 |  |  |
| nr-DL-PRS-ResourceSetIndex-r18 | 0 |  |  |
| } |  |  |  |
| NR-linkedDL-PRS-ResourceSetID-PRS-AggregationList-r18[2] SEQUENCE (SIZE (2..3)) OF NR-DL-PRS-AggregationElement-r18 { | 2 entries | entry 2 |  |
| NR-DL-PRS-AggregationElement-r18[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-FrequencyLayerIndex-r18 | 0 |  |  |
| nr-DL-PRS-TRP-Index-r18 | 1 |  |  |
| nr-DL-PRS-ResourceSetIndex-r18 | 0 |  |  |
| } |  |  |  |
| NR-DL-PRS-AggregationElement-r18[2] SEQUENCE { |  | entry 2 |  |
| nr-DL-PRS-FrequencyLayerIndex-r18 | 1 |  |  |
| nr-DL-PRS-TRP-Index-r18 | 1 |  |  |
| nr-DL-PRS-ResourceSetIndex-r18 | 0 |  |  |
| } |  |  |  |
| NR-linkedDL-PRS-ResourceSetID-PRS-AggregationList-r18[3] SEQUENCE (SIZE (2..3)) OF NR-DL-PRS-AggregationElement-r18 { | 2 entries | entry 3 |  |
| NR-DL-PRS-AggregationElement-r18[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-FrequencyLayerIndex-r18 | 0 |  |  |
| nr-DL-PRS-TRP-Index-r18 | 2 |  |  |
| nr-DL-PRS-ResourceSetIndex-r18 | 0 |  |  |
| } |  |  |  |
| NR-DL-PRS-AggregationElement-r18[2] SEQUENCE { |  | entry 2 |  |
| nr-DL-PRS-FrequencyLayerIndex-r18 | 1 |  |  |
| nr-DL-PRS-TRP-Index-r18 | 2 |  |  |
| nr-DL-PRS-ResourceSetIndex-r18 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.12.4.3-2: NR-DL-PRS-Info

| Derivation Path: 37.355 clause 6.4.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NR-DL-PRS-Info-r16 ::= SEQUENCE { |  |  |  |
| nr-DL-PRS-ResourceSetList-r16 SEQUENCE (SIZE (1..nrMaxSetsPerTrp-r16)) OF NR-DL-PRS-ResourceSet-r16 { | 1 entry |  |  |
| NR-DL-PRS-ResourceSet-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-ResourceSetID-r16 | 0 |  |  |
| dl-PRS-Periodicity-and-ResourceSetSlotOffset-r16 CHOICE { |  |  |  |
| scs120-r16 CHOICE { |  |  |  |
| n160-r16 | 80 |  |  |
| } |  |  |  |
| dl-PRS-ResourceRepetitionFactor-r16 | n2 |  |  |
| dl-PRS-ResourceTimeGap-r16 | s1 |  |  |
| dl-PRS-NumSymbols-r16 | n4 |  |  |
| dl-PRS-MutingOption1-r16 SEQUENCE { |  |  |  |
| dl-prs-MutingBitRepetitionFactor-r16 | Not present |  |  |
| nr-option1-muting-r16 CHOICE { |  |  |  |
| po2-r16 | 10 |  | Cell 1 |
|  | 01 |  | Cell 2 |
|  | 10 |  | Cell 3 |
| } |  |  |  |
| } |  |  |  |
| dl-PRS-MutingOption2-r16 | Not present |  |  |
| dl-PRS-ResourcePower-r16 | 27 |  |  |
| dl-PRS-ResourceList-r16 SEQUENCE (SIZE (1..nrMaxResourcesPerSet-r16)) OF NR-DL-PRS-Resource-r16 { | 1 entry |  |  |
| NR-DL-PRS-Resource-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-ResourceID-r16 | 0 |  | NR-DL-PRS-AssistanceDataPerFreq-r16[1] |
|  | 1 |  | NR-DL-PRS-AssistanceDataPerFreq-r16[2] |
| dl-PRS-SequenceID-r16 | 0 |  | NR-DL-PRS-AssistanceDataPerTRP-r16[1] |
|  | 1 |  | NR-DL-PRS-AssistanceDataPerTRP-r16[2] |
|  | 2 |  | NR-DL-PRS-AssistanceDataPerTRP-r16[3] |
| dl-PRS-CombSizeN-AndReOffset-r16 CHOICE { |  |  |  |
| n2-r16 | 0 |  | NR Cell 1 |
|  | 0 |  | NR Cell 2 |
|  | 1 |  | NR Cell 3 |
| } |  |  |  |
| dl-PRS-ResourceSlotOffset-r16 | 0 |  |  |
| dl-PRS-ResourceSymbolOffset-r16 | 0 |  |  |
| dl-PRS-QCL-Info-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

14.2.12.5	Test requirement

Table 14.2.12.5-1 to Table 14.2.12.5-3 define the primary level settings including test tolerances for the test.

Table 14.2.12.5-1: General test parameters for RSTD measurement reporting delay

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  |  | Cell 1 | Reference cell is the cell in the DL-TDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 38.215 [57] and TS 37.355 [49]. The reference cell is the PCell in this test case. |
| Neighbor cells |  |  | Cell 2 and Cell 3 | Cell 2 and Cell 3 appear at the first and second places in the neighbour cell list in the DL-TDOA assistance data. |
| SSB configuration | Config 1 |  | SSB.3 FR2 |  |
| SMTC configuration | Config 1 |  | SMTC.1 |  |
| PDSCH RMC configuration | Config 1 |  | SR.1.1 FDD |  |
| RMSI CORESET RMC configuration | Config 1 |  | CR.3.1 TDD | As specified in clause A.3.1.2.1 |
| Dedicated CORESET RMC configuration | Config 1 |  | CCR.1.1 FDD |  |
| PRS Configuration per PFL | Config 1 |  | PRS.1.1 FR2 | As specified in TS 38.133 [50] clause A.3.31 |
| Physical cell ID PCI |  |  | (PCI of Cell 1 – PCI of Cell 2)mod6=0 and (PCI of Cell 1 – PCI of Cell 3)mod6=0 | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF |  |
| Measurement gap |  |  | GP#24 or GP#13 | GP#24 is configured if UE supports MG#24, otherwise GP#13 is configured |
| Radio frame receive time offset between the cells at the UE antenna connector |  | s | Cell 2 to Cell 1: 0 Cell 3 to Cell 1: 3 | PRS are transmitted from synchronous cells |
| Expected RSTD |  | s | Cell 2: 3 Cell 3: 3 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the DL-TDOA assistance data specified in TS 37.355 [49] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells |  | s | 5 | The corresponding parameter in the DL-TDOA assistance data specified in TS 37.355 [49] is the expectedRSTD-Uncertainty index |
| Number of cells provided in DL-TDOA assistance data |  |  | 3 | Including the reference cell |
| PRS muting info |  |  | Cell 1: ‘10’ Cell 2: ‘01’ Cell 3: ‘10’ | Correponds to prs-MutingInfo defined in TS 37.355 [49] |
| PRS resource RE offset |  |  | Cell 1: 0 Cell 2: 0 Cell 3: 1 | Cell 1 and Cell 3 are configured with different resource offsets |
| T1 |  | s | 3 | The length of the time interval from the beginning of each test |
| T2 |  | s | 1.28 | The length of the time interval that follows immediately after time interval T1 |
| AoA setup |  |  | Setup 1 | As defined in TS 38.133 [50] A.3.15.1 |
| Beam assumption |  |  | Rough | Information about types of UE beam is given in clause TS 38.133 [50] B.2.1.3, and does not limit UE implementation or test system implementation |

Table 14.2.12.5-2: Cell-specific test parameters for RSTD measurement reporting delay during T1

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | 1 | 1 |
| Positiong frequency layer |  |  | 1 | 1 | 1 |
| BWchannel |  | MHz | 200: NRB,c = 132 | 200: NRB,c = 132 | 200: NRB,c = 132 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 1x2 Low | 1x2 Low |
| OCNG patterns defined in A.3.2.1 |  |  | OP.1 | N/A | N/A |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| Note 3 | Config 1 | dBm/SCS | -89 |  |  |
| PRS |  | dB | -Infinity | -Infinity | -Infinity |
| Io Note 4 | Config 1 | dBm/ 190.08MHz | -53.99 | -53.99 | -53.99 |
| SSB_RP Note4 | Config 1 | dBm/SCS | -89 | -Infinity | -Infinity |
| SSB | Config 1 | dB | 0 | -Infinity | -Infinity |
| Propagation Condition |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the slots with transmitted PRS. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: SSB_RP and Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |  |

Table 14.2.12.5-3: Cell-specific test parameters for RSTD measurement reporting delay during T2

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T2 | T2 | T2 |
| RF Channel Number NOTE 6 |  |  | 1,2 | 1,2 | 1,2 |
| Positioning frequency layer (PFL) |  |  | 1,2 | 1,2 | 1,2 |
| BWchannel per PFL |  | MHz | 200: NRB,c = 132 | 200: NRB,c = 132 | 200: NRB,c = 132 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 1x2 Low | 1x2 Low |
| OCNG patterns defined in A.3.2.1 |  |  | OP.1 | OP.1 | OP.1 |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| EPRE ratio of PRS to SSS |  |  |  |  |  |
| PRACH configuration |  |  | FR2 PRACH configuration 1 | FR2 PRACH configuration 1 | FR2 PRACH configuration 1 |
| Note 3 | Config 1 | dBm/SCS | -89 + TT | -89 + TT | -89 + TT |
| PRS EsNoc | Config 1 | dB | -5.45 + TT | -11.67 + TT | -11.67 + TT |
| Io Note4 | Config 1 | dBm/ 190.08MHz per PFL | -55.48 + TT | -55.48 + TT | -55.48 + TT |
| PRS EsIot |  | dB | -6 + TT | -13 + TT | -13 + TT |
| PRP Note 4 |  | dBm/SCS | -94.45 + TT | -100.67 + TT | -100.67 + TT |
| Propagation Condition |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that active cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the slots with transmitted PRS. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: PRP and Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. The Io is calculated based only on the symbols in which PRS is transmitted. Note 5: Calculation of Es/Iot includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [55], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [55] table 6.2.1.3-4. Note 6: Carriers are intra-band and contiguous to each other. |  |  |  |  |  |

The RSTD measurement time fulfils the requirements specified in 14.2.12.3.

The UE shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the DL-TDOA assistance data, Cell 1, within the time duration specified in TS 38.133 [50] clause 9.9.2.10 starting from the beginning of time interval T2.

NOTE:	The actual overall delays measured in the test may be up to 2×TTIDCCH higher than the time duration above because of TTI insertion uncertainty of the measurement report in DCCH.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in TS 38.133 [50] clause 10.1.23A.2, i.e., between RSTD_000000000 and RSTD_126083073.

< End of Changes >