# Title:	Test tolerance methodology for inter-frequency SSB-based L1-RSRP measurement test in FR2

# Source:	Ericsson

# 1.	Introduction

The purpose of this document is to present the methodology and principles used for the development of test tolerances for inter-frequency SSB-based L1-RSRP measurement test in FR2.

The test case in question, from TS 38.533, is:

- 7.6.22.1	NR SA FR2 Inter-frequency SSB based L1-RSRP measurement without measurement gap in FR2

The applicability of the analysis is:

- UE power class 3, f ≤40.8GHz

This document describes the process to derive the test tolerances. The calculations are provided in the accompanying spreadsheet.

# 2.	Prototype test case in Annex A of TS 38.133

The test requirements are defined in the following extract from TS 38.133 v18.10.0.

#### A.7.6.22.1	Inter-frequency SSB based L1-RSRP measurement without measurement gap in FR2

##### A.7.6.22.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE supporting interFreqSSB-L1-MeasWithoutGaps-r18 makes correct reporting of inter-frequency L1-RSRP measurement without gap for LTM. This test will partly verify the L1-RSRP measurement requirements in clause 9.15.6, with the testing configurations in tables A.7.6.22.1.1-1, A.7.6.22.1.2-1, A.7.6.22.1.2-2, A.7.6.22.1.2-3 and A.7.6.22.1.2-4.

The AoA setup of FR2 cell for this test is Setup 1 as defined in clause A.3.15.

Table A.7.6.22.1.1-1: Applicable NR configurations for SSB based inter-frequency L1-RSRP LTM measurement without gap in FR2

| Config | Description |
| --- | --- |
| 1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | NR 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations NOTE 2: Target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

##### A.7.6.22.1.2	Test parameters

There are two cells in the test, Cell 1 is the PCell on NR RF channel number 1 and Cell 2 is a neighbor cell NR RF channel number 2. The SSB of Cell 2 is completely within UE’s active BWP BW. The PRBs containing SSB from Cell 1 and Cell 2 should be different in frequency location within the cell bandwidth. The test parameters for Cell 1 are given in table A.7.6.22.1.2-1. The test parameters for Cell 2 are given in table A.7.6.22.1.2-2 and table A.7.6.22.1.2-3.

There are two tests in the test case, test 1 and test 2:

-	In test 1, time offset between cells is within CP length.

-	In test 2, time offset between cells is larger than CP length.

UE not capable of multiCellL1-measRTD-greaterThan-CP-r18 is only required to pass test 1. Otherwise, it is only required to pass test 2.

The test consists of two successive time periods, with time duration of T1 and T2 respectively. SSB_RP of Cell 2 in T1 and T2 are different. No measurement gap is configured in the test.

Prior to the start of the time duration T1,

-	UE is connected to Cell 1 (PCell) on RF channel 1 (PCC).

-	A measurement object is configured for the RF channel 2, and it is indicated to the UE that event-triggered reporting with Event A3 is used. Before the start of the T1, event is triggered, and UE has sent a measurement report for the Cell 2 with SSB Index.

-	UE is provided with LTM-Candidate-r18 for Cell 2.

-	UE is configured with SSB-based L1-RSRP measurements and periodic L1-RSRP measurement reports on candidate cell (Cell 2) in PUCCH format 2.

At the beginning of T2, SSB_RP of Cell 2 changes to a different value from T1. T2 starts at the beginning of a frame with an odd SFN.

Table A.7.6.22.1.2-1: General test parameters for SSB based inter-frequency L1-RSRP LTM measurement test in FR2

| Parameter |  | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| Active cell |  |  | Cell 1 |  |  |
| Neighbouring cell |  |  | Cell 2 |  | Cell 2 is the candidate cell |
| RF Channel Number |  |  | 1: Cell 1 2: Cell 2 |  |  |
| A3-Offset |  | dB | -15 |  |  |
| Hysteresis |  | dB | 0 |  |  |
| Time To Trigger |  | ms | 0 |  |  |
| Filter coefficient |  |  | 0 |  | L3 filtering is not used |
| maxNrofRS-IndexesToReport |  |  | 1 |  |  |
| includeBeamMeasurements |  |  | True |  |  |
| DRX |  |  | OFF |  | DRX is not used |
| Time offset between cells |  |  | 0.2 s | 2s | The timing of Cell 2 is later than the timing of Cell 1 |
| LTM-CSI-ReportConfig | L1-RSRP reporting period | slot | 320 |  | Periodic L1-RSRP reporting configured |
|  | nrOfReportedCells |  | n1 |  | Report candidate cell’s (Cell 2) L1-RSRP measurement results. |
|  | nrOfReportedRS-PerCell |  | n1 |  |  |
|  | spCellInclusion |  | N/A |  |  |
| ltm-ConfigComplete |  |  | True |  | Candidate cell’s configuration is complete configuration |
| T1 |  | s | 1.5 |  |  |
| T2 |  | s | 2 |  |  |

Table A.7.6.22.1.2-2: Cell specific test parameters for SSB based inter-frequency L1-RSRP LTM measurement test in FR2

| Parameter | Config | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| SSB ARFCN | 1~2 |  | freq1 | freq2 |
| Duplex mode | 1~2 |  | TDD |  |
| TDD Configuration | 1~2 |  | TDDConf.3.1 |  |
| BWchannel | 1~2 | MHz | 100: NPRB,c = 66 |  |
| Data PRBs allocated | 1~2 |  | 66 |  |
| PDSCH/PDCCH subcarrier spacing | 1~2 | KHz | 120 |  |
| PUCCH/PUSCH subcarrier spacing | 1~2 | KHz | 120 |  |
| PDSCH Reference measurement channel | 1 |  | SR.3.2 TDD | N/A |
|  | 2 |  | SR.3.3 TDD | N/A |
| RMSI CORESET Reference Channel | 1 |  | CR.3.1 TDD | N/A |
|  | 2 |  | CR.3.2 TDD | N/A |
| Dedicated CORESET Reference Channel | 1 |  | CCR.3.1 TDD | N/A |
|  | 2 |  | CCR.3.7 TDD | N/A |
| SSB configuration | 1 |  | SSB.3 FR2 |  |
|  | 2 |  | SSB.4 FR2 |  |
| OCNG Patterns | 1~2 |  | OP.4 | N/A |
| Initial BWP Configuration | 1~2 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP configuration | 1~2 |  | DLBWP.1.3 ULBWP.1.3 |  |
| SMTC configuration | 1~2 |  | SMTC.1 |  |
| TRS Configuration | 1~2 |  | TRS.2.1 TDD | N/A |
| PDCCH/PDSCH TCI Configuration | 1~2 |  | TCI.State.2 | N/A |
| EPRE ratio of PSS to SSS | 1~2 | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Propagation condition | 1~2 |  | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |  |

Table A.7.6.22.1.2-3: NR OTA Cell specific test parameters for SSB based inter-frequency L1-RSRP LTM measurement without measurement gap in FR2

| Parameter | Config | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Angle of arrival configuration | 1~2 |  | Setup 1 according to A.3.15.1 |  |  |  |
| Beam AssumptionNote 3 | 1~2 |  | Rough |  | Rough |  |
| BBNote 4 | 1~2 | dB | -1.5 | 16.5 | -1.5 | 16.5 |
| SSB_RP Note2 | 1 | dBm/SSB SCS | -103 | -85 | -103 | -85 |
|  | 2 |  | -100 | -82 | -100 | -82 |
| Io Note2 | 1 | dBm/95.04 MHz | -74 | -56 | -74 | -56 |
|  | 2 |  | -74 | -56 | -74 | -56 |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Es/Iot, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 3: Information about types of UE beam is given in clause B.2.1.3, and does not limit UE implementation or test system implementation NOTE 4: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1 dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |  |  |

##### A.7.6.22.1.3	Test Requirements

The UE shall send L1-RSRP report every 320 slots in T2. No later than 960 ms plus 320 slots from the beginning of time period T2, UE shall send L1-RSRP report including the valid results for Cell 2 while meeting the accuracy requirements defined in clause 10.1.20B.

The reported L1-RSRP value shall include the Rx antenna gain in the range of -10 to +20 dB.

The rate of correct events observed during repeated tests shall be at least 90 %.

# 3. 	Discussion

The test case consists of 2 test configurations. Each test configuration has 2 time periods: T1 and T2.

To analyse the test case, it is helpful to show the values of Ês/IotBB for NR Cell 1 and NR Cell 2 versus time on Figure 1.

Figure 1: Ês/IotBB NR Cell 1 and NR Cell 2

Observations

In the test, there is no external noise. Interference to NR Cell 1 and NR Cell 2 only comes from UE internal noise.

During T1, NR Cell 1 and NR Cell 2 are both present. For NR Cell 1, Ês/IotBB value is -1.5 dB for all configurations 1 and 2. For NR Cell 2, Ês/IotBB value is -1.5dB for all configurations 1 and 2.

During T2, NR Cell 1 and NR Cell 2 are both present, and the Ês/IotBB value is 16.5dB for all configurations 1 and 2. The UE shall send L1-RSRP report every 320 slots including results of Cell 2 while meeting the L1-RSRP absolute accuracy requirement. L1-RSRP relative accuracy can’t be verified because only Cell 2 with 1 SSB is configured to report. The effect of uncertainties on the test verdict needs to be checked. A decision can then be made on whether Test Tolerances need to be applied.

A detailed analysis of the test case is given in section 4 of this document.

# 4. 	Calculation of test tolerances

General approach

The general approach is given in the steps below:

- Copy the originally specified key parameters from the core requirements.

- Where relevant, calculate derived parameters from the core requirements.

- Define uncertainties for a minimum set of parameters.

- Define controlled parameters (critical to the test verdict), calculate sensitivity factors and uncertainty.

- Determine which original or derived parameters to offset (apply Test Tolerances to) and by how much.

- Recalculate original or derived parameters including Test Tolerances.

- Check that the controlled parameters meet requirements to get the correct test verdict.

Each step is explained below, and the calculations are given in the accompanying spreadsheet.

a) Original specified key parameters

The key parameters are copied from Table A.7.6.22.1.1-1, Table A.7.6.22.1.2-1, Table A.7.6.22.1.2-2 and Table A.7.6.22.1.2-3 in TS 38.133. The key parameters are selected as the minimum set to define the cell power levels, and which are subject to a test system uncertainty which may affect the verdict of the test.

b) Derived parameters

Several derived parameters are calculated, using the base information in a). The reason for deriving each additional parameter is given in the “Comment” column of section b) in the accompanying spreadsheet.

In this test case, only Ês/IotBB is configured, and there is no external noise. Only UE internal noise contributes the interference and it’s a fixed value. Then the applied parameters at reference point can be calculated directly.

Before T1 start, NR Cell 1 and NR Cell 2 already become detectable. UE is expected to send an event triggered measurement report. The criterion is Event A3: Neighbour becomes amount of offset better than SpCell. The UE makes a measurement of neighbour Cell 2 relative to serving Cell 1 and compares it to the signalled A3 offset. And the entering condition of event A3 specified in 38.331 clause 5.5.4.4 inequality A3-1 shall be satisfied:

Mn + Ofn + Ocn – Hys > Mp + Ofp + Ocp + Off

where,

- Mp and Mn are the SS-RSRP of Cell 1 and Cell 2 UE measured respectively before T2.

- Ofn is the MO-specific offset offsetMO for Cell 2, specified as 0dB by default.

- Ofp is the MO-specific offset offsetMO for Cell 1. Its default value is specified as 0dB in TS 38.508-1 Table 4.6.3-76.

- Ocp and Ocn are the cell-specific offset cellIndividualOffset for Cell 1 and Cell 2 respectively. It’s set to 0dB if not configured.

- Hys is the hysteresis parameter for this event, specified as 0dB in TS 38.133 Table A.7.6.22.1.2-1.

- Off is the threshold parameter for this event, specified as -15dB in TS 38.133 Table A.7.6.22.1.2-1.

The equation therefore reduces to:

Mn > Mp + Off

So further parameter to calculate SS-RSRP power difference including the offset is simplified to:

- (NR Cell 2 SS-RSRP – NR Cell 1 SS-RSRP) – Off

During T1, the power levels for NR Cell 1 and NR Cell 2 are unchanged and the same as power settings before T1. Therefore, the power levels in T1 for both cells also satisfy the event A3 triggered criterion.

During T1 and T2, NR Cell 2 L1-RSRP is measured and reported using mapping Table 10.1.6.1-1. In T2 period, the reported value should be within the absolute accuracy requirements as defined in TS 38.133 clause 10.1.20A.1.1.

c) Uncertainties

The SS provides NR Cell 1 with one SSB, and NR Cell 2 with one SSB to the UE. Propose to control the following parameters:

- Ês1 of NR Cell 1, averaged over BWConfig, Ês1 ±5.65* dB

- Ês1 of NR Cell 1, averaged over measurement PRB, Ês1 ±5.65* dB

- Ês2 of NR Cell 2, averaged over BWConfig, Ês2 ±5.65* dB

- Ês2 of NR Cell 2, averaged over measurement PRB, Ês2 ±5.65* dB

- Relative accuracy of inter-frequency SSB measurement for normal test environment, ±6 dB

- Absolute accuracy of inter-frequency SSB based L1-RSRP measurement for normal test environment, ±8.5 dB

- UE gain G variation, ±15 dB

*These values are applicable for frequencies <= 40.8 GHz.

In this test the UE measures the SS-RSRP over specific Physical Resource Block (PRB) numbers # RBJ to # RBJ+19. In addition, this test has separate constraints on the overall power in the configured bandwidth Io.

This choice forms a minimum set, so the superposition principle can be applied if necessary.

Before T1, UE is required to send A3 measurement report. UE makes the (NR Cell 2 SS-RSRP-NR Cell 1 SS-RSRP) comparison with the A3 offset, and it has some inaccuracy which could affect the test verdict, so the SS-RSRP relative measurement accuracy needs to be considered. The SS-RSRP inter frequency relative accuracy is defined in the following extract from TS 38.133:

##### 10.1.5.1.2	Relative SS-RSRP Accuracy

The relative SS-RSRP accuracy is defined as the SS-RSRP measured from one cell on a frequency in FR2 compared to the SS-RSRP measured from another cell on another frequency in FR2.

The accuracy requirements in table 10.1.5.1.2-1 are valid under the following conditions:

-	Conditions defined in TS 38.101-2 [19] clause 7.3 for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to annex B.2.3 for a corresponding Band for each relevant SSB.

--	|SSB_RP1dBm - SSB_RP2dBm|  27dB

-	|Channel 1_Io -Channel 2_Io |  20 dB

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.5.1.2-1: SS-RSRP Inter-frequency relative accuracy in FR2

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 2 range |  |  |
|  |  |  | Minimum Io |  | Maximum Io |
| dB | dB | dB | dBm / SCSSSB Note 1 |  | dBm/BWChannel |
|  |  |  | SCSSSB = 120 kHz | SCSSSB = 240 kHz |  |
| 6 | 9 | ≥-4 | Same value as SSB_RP in Table B.2.3-2, according to UE Power class, operating band and angle of arrival |  | -50 |
| NOTE 1: Values based on REFSENS and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 2: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. NOTE 4: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. |  |  |  |  |  |

To ensure this relative SS-RSRP accuracy, the conditions of SSB_RP and SSB Ês/Iot in TS 38.133 Annex B.2.3 need to be fulfilled.

## B.2.3	Conditions for NR inter-frequency measurements

This clause defines the following conditions for NR inter-frequency measurements and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in table B.2.3-2 for FR2 NR cells.

Table B.2.3-2: Conditions for inter-frequency measurements in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum SSB_RP Note 2, Note 3 |  |  |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSSB |  |  |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  |  |  | SCSSSB = 240 kHz |  |
|  |  |  | UE Power class |  |  |  |  |  | UE Power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 5 | 6 | 1, 2, 3, 4, 5, 6 |  |
| Conditions | Rx Beam Peak | n257 | -108.3 | -111.8 | -110.1 | -125.8+Y4 | -121.4+Y5 | -105.9 | (Value for SCSSSB = 120 kHz) +3 dB | ≥-4 |
|  |  | n258 | -108.3 | -111.8 | -110.1 | -125.8+Y4 | -121.6+Y5 | -106.1 |  |  |
|  |  | n259 |  |  | -106.5 |  | -118.5+Y5 |  |  |  |
|  |  | n260 | -105.3 |  | -107.5 | -123.8+Y4 |  |  |  |  |
|  |  | n261 | -108.3 | -111.8 | -110.1 | -125.8+Y4 |  | -105.9 |  |  |
|  |  | n262 | -103.3 | -106.6 | -104.6 | -119.8+Y4 |  |  |  |  |
|  | Spherical coverage Note 1 | n257 | -100.3 | -100.8 | -99.2 | -116.8+Z4 | -113.4+Z5 | -97.9 | (Value for SCSSSB = 120 kHz) +3 dB | ≥-4 |
|  |  | n258 | -100.3 | -100.8 | -99.2 | -116.8+Z4 | -113.6+Z5 | -98.1 |  |  |
|  |  | n259 |  |  | -93.7 |  |  |  |  |  |
|  |  | n260 | -97.3 |  | -94.9 | -111.8+Z4 |  |  |  |  |
|  |  | n261 | -100.3 | -100.8 | -99.2 | -116.8+Z4 |  | -97.9 |  |  |
|  |  | n262 | -95.1 | -94.7 | -91.5 | -107.7+Z4 |  |  |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and Spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |  |  |

Editor’s notes for Table B.2.3-2:

- The value of Y for power classes 4 and 5 is FFS, where Y4 and Y5 are the rough/fine beam gain differences in Rx beam peak direction for power classes 4 and 5 respectively

- The value of Z for power classes 4 and 5 is FFS, where Z4 and Z5 are the rough/fine beam gain differences in spherical coverage directions for power classes 4 and 5 respectively

It’s noted that band n262 has the tightest SSB_RP requirement, which is -104.6 dBm/120kHz (-101.6 dBm/240kHz) for UE PC3. But band n262 is not considered. There are two reasons. Firstly, the calculated SSB_RP and Io in section b) based on n259 are the same as parameters specified in TS 38.133 Table A.7.6.22.1.2-3. In other words, the test parameters are designed based on band n259 assumption in TS 38.133. Secondly, MU values specified in TS 38.533 Annex Table F.1.1.2-2 are only applicable for frequencies <=40.8GHz and frequencies in n262 are not less than 47.2GHz. Band n259 is therefore selected to do test tolerance analysis. Band n259 SSB_RP requirement is -106.5 dBm/120kHz (-103.5 dBm/240kHz) for UE PC3 including the UE multi-band relaxation factor DMBP,n specified in clause 6.2.1 of TS 38.101-2. And side conditions of SSB Ês/Iot is already considered.

During T2, the UE L1-RSRP absolute measurement accuracy needs to be considered, because when the UE makes the L1-RSRP measurements it has some inaccuracy which could affect the test verdict. The UE L1-RSRP measurement accuracy is defined in the following extract from TS 38.133:

### 10.1.20B			LTM Inter-frequency L1-RSRP accuracy requirements for FR2

#### 10.1.20B.1	SSB based inter-frequency L1-RSRP accuracy requirements

##### 10.1.20B.1.1	Absolute Accuracy

Unless otherwise specified, the requirements for absolute SSB based L1-RSRP accuracy in this clause apply to all SSBs of candidate neighbour cell(s) on a frequency in FR2 that is on a different frequency than the serving cell.

The accuracy requirements in table 10.1.20B.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for L1-RSRP measurements are fulfilled according to annex B.2.4.1 for a corresponding Band for each relevant SSB.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.20B.1.1-1: SSB based L1-RSRP absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | Minimum Io |  |  | Maximum Io |
| dB | dB | dB | dBm / SCSSSB Note 2 |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCSSSB = 120 kHz | SCSSSB = 240 kHz |  |  |
| ±6.5 | ±9.5 | ≥-3 | Same value as SSB_RP in Table B.2.4.1-2, according to UE Power class, operating band and angle of arrival |  | N/A | -70 |
| 8.5 | 11.5 | ≥-3 | N/A |  | -70 | -50 |
| NOTE 1: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 2: Values based on REFSENS and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |

To ensure absolute L1-RSRP accuracy, the conditions of SSB_RP and SSB Ês/Iot in TS 38.133 Annex B.2.4, need to be fulfilled.

## B.2.4	Conditions for NR L1-RSRP reporting

### B.2.4.1	Conditions for SSB based L1-RSRP reporting

This clause defines the following conditions for NR L1-RSRP measurement reporting and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.4.1-2 for FR2 NR cells.

Table B.2.4.1-2: Conditions for SSB based L1-RSRP measurements in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum SSB_RP Note 2, Note 3 |  |  |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSSB |  |  |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  |  |  | SCSSSB = 240 kHz |  |
|  |  |  | UE Power class |  |  |  |  |  | UE Power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 5 | 6 | 1, 2, 3, 4, 5 |  |
| Conditions | Rx Beam Peak | n257 | -107.3 | -110.8 | -109.1 | -124.8+Y4 | -120.4+Y5 | -120.4+ Y6 | (Value for SCSSSB = 120 kHz) +3 dB | ≥-3 |
|  |  | n258 | -107.3 | -110.8 | -109.1 | -124.8+Y4 | -120.6+Y5 | -120.6+ Y6 |  |  |
|  |  | n259 |  |  | -105.5 |  | -117.5+Y5 |  |  |  |
|  |  | n260 | -104.3 |  | -106.5 | -122.8+Y4 |  |  |  |  |
|  |  | n261 | -107.3 | -110.8 | -109.1 | -124.8+Y4 |  | -120.4+ Y6 |  |  |
|  |  | n262 | -102.3 | -105.6 | -103.6 | -118.8+Y4 |  |  |  |  |
|  | Spherical coverage Note 1 | n257 | -99.3 | -99.8 | -98.2 | -115.8+Z4 | -112.4+Z5 | -112.4+Z6 | (Value for SCSSSB = 120 kHz) +3 dB | ≥-3 |
|  |  | n258 | -99.3 | -99.8 | -98.2 | -115.8+Z4 | -112.6+Z5 | -112.6+Z6 |  |  |
|  |  | n259 |  |  | -92.7 |  |  |  |  |  |
|  |  | n260 | -96.3 |  | -93.9 | -110.8+Z4 |  |  |  |  |
|  |  | n261 | -99.3 | -99.8 | -98.2 | -115.8+Z4 |  | -112.4+Z6 |  |  |
|  |  | n262 | -94.3 | -93.7 | -90.5 | -106.7+Z4 |  |  |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and Spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |  |  |

Editor’s notes for Table B.2.4.1-2:

- The value of Y for power classes 4 and 5 is FFS, where Y4 and Y5 are the rough/fine beam gain differences in Rx beam peak direction for power classes 4 and 5 respectively

- The value of Z for power classes 4 and 5 is FFS, where Z4 and Z5 are the rough/fine beam gain differences in spherical coverage directions for power classes 4 and 5 respectively

In addition, UE gain is considered. UE gain variation is defined in 38.133 Annex B.2.1.5:

#### B.2.1.5.1	Gain to SS-RSRP and CSI-RSRP measurement point for Rx Beam Peak angle of arrival

In clause 5.1.1 of TS 38.215 [4] SS-RSRP and CSI-RSRP is defined to be measured based on the combined signal from antenna elements corresponding to a given receiver branch. The reference point for requirement parameters from the UE perspective is the input of the UE antenna array. The gain “G” relates the combined signal from antenna elements corresponding to a given receiver branch to the reference point for requirement parameters.

The gain “G” affects absolute signal level values reported by the UE.

Figure B.2.1.5.1-1: Gain and Reference point for requirement parameters

Table B.2.1.5.1-1: UE gain G, Rx beam peak direction

|  | UE Power class |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| Minimum, dBi | 0 | FFS | -10 | FFS | -5 | -5 | FFS |
| Maximum, dBi | +50 | FFS | +20 | FFS | +44 | +44 | FFS |

Gain range in spherical coverage directions may be lower than in Rx beam peak direction, according to the difference between the EIS spherical coverage value specified in TS 38.101-2 [19] clause 7.3.4 and the Reference sensitivity level specified in TS 38.101-2 [19] clause 7.3.2.

The UE gain G variation can be expressed as (maximum UE gain - minimum UE gain)/2 according to 38.903 annex A.2.6.2. For UE PC3, it's 15dB.

In this test, power levels of both Cell 1 and Cell 2 are the same and are only changed when T2 starts. Before T1, UE is configured to send A3 report.

During T1, UE needs to send the event triggered measurement, the a3 reporting side conditions needs to be checked.

During T2, NR Cell 2 continuously sends L1-RSRP report. If NR Cell 2’s power levels meet inter-frequency L1-RSRP absolute accuracy requirement during T2, NR Cell 1’s power levels can also meet the same absolute accuracy requirement.

In summary, during T1 the following conditions shall be satisfied:

- SSB Ês/Iot of Cell 1/Cell 2 at baseband shall be no less than -4dB.

- SSB_RP of Cell 1/Cell 2 shall be no less than -105.5 dBm/120kHz (-102.5 dBm/240kHz) including the UE multi-band relaxation factor ΔMBP specified in clause 6.2.1 of TS 38.101-2.

- Io shall be no less than -105.5 dBm/120kHz (-102.5 dBm/240kHz) and no larger than -50dBm/Ch BW including the UE multi-band relaxation factor ΔMBP specified in clause 6.2.1 of TS 38.101-2.

- UE measured (Cell 2 RSRP - Cell 1 RSRP - A3-Offset) shall be larger than 0 dB.

And during T2 the following condition shall be satisfied:

- SSB Ês/Iot of Cell 1 and Cell 2 at baseband shall be no less than -3dB.

- SSB_RP of Cell 1 and Cell 2 shall be no less than -104.5 dBm/120kHz (-101.5 dBm/240kHz) including the UE multi-band relaxation factor ΔMBP specified in clause 6.2.1 of TS 38.101-2.

- Io shall be no less than -70dBm/Ch BW and no larger than -50dBm/Ch BW including the UE multi-band relaxation factor ΔMBP specified in clause 6.2.1 of TS 38.101-2.

d) Controlled parameters critical to verdict

In many RRM test cases, there is not a simple one-to-one relationship between the parameters that can be set by the test equipment, and their effect on parameters determining the test verdict.

It is therefore essential to identify those parameters determining the test verdict. In this test case there are two aspects to consider:

- The uncertainties in the stimulus set by the test system (downlink signals)

- The uncertainties in the UE response (A3 report, L1-RSRP reports)

The stimulus set by the test system should be within the constraints (side conditions) for the UE measurement. For each cell, they are the SSB Ês/Iot range, the SSB_RP power range, and the Io power range, over which the UE meets the related requirements.

The 10 controlled parameters listed in the accompanying spreadsheet have been derived by study of the test case and by careful reading of the relevant clauses in TS 38.133. The reason for each parameter being critical to the test verdict is given briefly in the “Comment” column of section d) in the accompanying spreadsheet. Information about the value to be achieved is given in the “Requirement” column of section g) in the spreadsheet.

Having identified the parameters critical to the test verdict which need to be controlled, now need to consider how they are affected by the parameters which can be set by the test equipment. This is done by working out “sensitivity factors”. A sensitivity factor is the ratio (effect on a critical parameter y / a test equipment uncertainty x) and is usually in dB/dB.

Having filled in the matrix of sensitivity factors, the accompanying spreadsheet calculates the overall uncertainty for each controlled parameter, considering the uncertainties and sensitivity factors for each parameter that can be set by the test equipment. This process follows the superposition principle. More details and explanation can be found in section 4 of TS 38.903. Uncertainties are calculated separately for T1 and T2.

For the test system uncertainties, the normal procedure of combining uncorrelated uncertainties root-sum-square is followed.

When the test system uncertainties and the UE reporting accuracies are combined, the (root-sum square of test system uncertainties) is added arithmetically to the UE reporting accuracies. If all the uncertainties were combined root-sum square, the resulting smaller test limits could cause a conformant test system to fail a conformant UE, which would be unacceptable.

e) Determine parameters to offset

During T1, the test equipment uncertainties take SSB Ês/Iot, SSB_RP and Io for both NR Cell 1 and NR Cell 2 outside the allowed ranges. The Ês/Iot at UE baseband of both cells is therefore increased by an amount sufficient (3.35dB) to achieve SSB Ês/Iot, SSB_RP and Io values within the requirements.

Before T1, the event A3 measurement report is triggered. In section b), the A3 report entering condition has already reduced to:

Mn + ofn > Mp + Off

Putting in values and rearranging gives:

Test configuration 1: (-103dBm) – (-103dBm) > -15dB

Test configuration 2: (-100dBm) – (-100dBm) > -15dB

This inequality is satisfied by 15dB under nominal conditions. Taking the Ês/Iot at UE baseband of NR Cell 1 and NR Cell 2 increased 3.35dB into account, the inequality is still satisfied by the same 15 dB. The overall uncertainty for UE measured (Cell 2 SS-RSRP - Cell 1 SS-RSRP), considering both test system and UE measurement uncertainties, is ±13.99dB as calculated in step d) of the accompanying spreadsheet. The inequality defined above is therefore also satisfied when uncertainties are included. No offsets are needed for this requirement.

f) Parameters modified by Test Tolerances

Based on the decision in e), the set of parameters in a) and b) is reproduced in section f) of the accompanying spreadsheet, but this time modified by the Test Tolerances (applied offsets).

Re-derived parameters are calculated using the same methods as were used in step b).

g) Check controlled parameters Min/Max

Using a format like that in step d), the nominal value of each controlled parameter is recalculated.

The minimum and maximum values, due to variability from uncertainties, of controlled parameters is then calculated and compared against the requirements (SSB Ês/Iot, SSB_RP and Io). It is not necessary to calculate all parameters during each time interval T1 and T2, so a selection is made of those critical to the test verdict. The critical requirement for each parameter is given briefly in the “Comment” column of section g) in the accompanying spreadsheet. The cases closest to limit are identified by turquoise cells in the spreadsheet. If all requirements are met, then the exercise is complete.

With the uncertainty values and test tolerances proposed, all the requirements are met.

For this test, A3 report is the precondition, and the final verdict is based on the L1-RSRP values reported by the UE. As stated earlier, the reported values are affected by:

- The uncertainties in the stimulus set by the test system (downlink signals)

- The uncertainties in the UE response (A3 report, L1-RSRP reports)

Having ensured that the stimulus set by the test system remains within the side conditions, and knowing its uncertainty, now need to calculate the range of L1-RSRP values that a conformant UE could report.

# 5. Treatment of other configurations within the same test case

TS 38.533 test case 7.6.22.1 also contains configurations with different SSB subcarrier spacings. As the calculations use different values, a separate spreadsheet tab is provided. The shaded cells show what changes for different configurations. The following parameters are changed:

- Subcarrier spacing, section a)

- SSB_RP minimum requirement, section g) and associated Source/Reference

- Io minimum requirement, section g) and associated Source/Reference