Title:	Discussion on test point selection for NS_18 A-MPR and A-SE in FR1

Document for:	Discussion and approval

Background

The purpose of this paper is to provide an analysis of the parameters for the A-MPR test configuration table with a selection of test points that minimize testing and still achieve good test coverage for NS_18. The intention with NS_18 is to protect 692-698 MHz (TV bands) as indicated in Table 1 below.

Table 1: Additional requirement for"NS_18"

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |  |
| --- | --- | --- | --- |
|  | 3,5, 10, 15, 20, 25, 30, 40 MHz |  |  |
| 692-698 | -26.2 | 6 MHz |  |

The general agreement in RAN5 is that FR1 testing should follow what is being tested for LTE. This is also valid for A-MPR FR1 test point selection.

The selection of test points below is based on approved test point analyzes for SEM [2, 3] for FR1. However, for NS_18 the lower part of operating band n28, n83 and n109 (UL 703 - 733 MHz) has specific A-MPR requirements to protect 692-698 MHz (TV bands), see Table 2. There are no specific A-MPR requirements for upper part of operating band n28, n83 and n109.

The applicable AMPR requirements for different channel bandwidth and power class could be found in Table 6.2.3.1-1 in [4].

Table 1a: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_18 | 6.5.3.3.3 | n28, n8313, n109 | 3, 5 |  | Table 6.2.3.13-1, A1 Table 6.2.3.13-3, A1 |
|  |  |  | 10, 15, 20 |  | Table 6.2.3.13-1, A2 Table 6.2.3.13-3, A2 |
|  |  |  | 25, 30, 40 |  | Table 6.2.3.13-1, A3, A4, A5 Table 6.2.3.13-3, A3, A4, A5, A6 |

The specific AMPR value for A1~A6 are specified in Table 6.2.3.13-1 for power class 3 and Table 6.2.3.13-3 for power class 2 in [4]

Table 2: A-MPR regions for protection of 692-698 MHz (NS_18) (power class 3) (from [4])

| Modulation/Waveform |  | A1 (dB) |  | A2 (dB) | A3 | A4 | A5 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Inner/Outer | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 2 | N/A | ≤ 5 | 3 | 8 | 3 |
|  | QPSK | ≤ 2 |  | ≤ 5 | 3 | 8 | 3 |
|  | 16 QAM | ≤ 3 |  | ≤ 6 | 3 | 8 | 3 |
|  | 64 QAM | ≤ 4 |  | ≤ 7 | 3 | 8 | 4.5 |
|  | 256 QAM | ≤ 6 |  | ≤ 9 | 3 | 8 | 5.5 |
| CP-OFDM | QPSK | ≤ 5 |  | ≤ 6.5 | 4.5 | 9.5 | 5 |
|  | 16 QAM | ≤ 5 |  | ≤ 7 | 4.5 | 9.5 | 5 |
|  | 64 QAM | ≤ 5.5 |  | ≤ 8.5 | 4.5 | 9.5 | 5.5 |
|  | 256 QAM | ≤ 8.5 |  | ≤ 11.5 | 4.5 | 9.5 | 7.5 |
| NOTE 1: Void NOTE 2: Void |  |  |  |  |  |  |  |

Table 2a: A-MPR regions for protection of 692-698 MHz (NS_18) (power class 2) (from [4])

| Modulation/Waveform |  | A1 (dB) |  | A2 (dB) | A3 (dB) | A4 (dB) | A5 (dB) | A6 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Inner/Outer | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | N/A | ≤ 8 | 3.5 | 10 | 5 | 2 |
|  | QPSK | ≤ 3.5 |  | ≤ 8 | 4 | 11 | 5 | 3 |
|  | 16 QAM | ≤ 4 |  | ≤ 9 | 4 | 11 | 5.5 | 4 |
|  | 64 QAM | ≤ 4.5 |  | ≤ 10 | 4 | 11 | 6 | 4.5 |
|  | 256 QAM | ≤ 6.5 |  | ≤ 11 | 4 | 11 | 6 | 4.5 |
| CP-OFDM | QPSK | ≤ 5.5 |  | ≤ 9.5 | 5.5 | 12.5 | 7 | 4.5 |
|  | 16 QAM | ≤ 5.5 |  | ≤ 10 | 5.5 | 12.5 | 7 | 5 |
|  | 64 QAM | ≤ 6 |  | ≤ 11.5 | 5.5 | 12.5 | 7 | 5 |
|  | 256 QAM | ≤ 9 |  | ≤ 11.5 | 5.5 | 12.5 | 8 | 6.5 |

The A-MPR requirements in Table 2 and Table 3 are split into 3 regions, A1 for 3, 5MHz, A2 for 10, 15 and 20MHz and A3, A4, A5 and A6 for 25, 30MHz and 40MHz. The UL RB allocation for testing of these regions should be allocated to the lower part of operating band n28, n83 and n109.

Test Environment

Based on the baseline agreed in [2], the test environment for NR A-MPR measurement for FR1 can be defined as in LTE.

PROPOSAL 1: Select Test Environment as Normal Conditions (NC) for A-MPR.

Test Frequencies

UL test carrier centre frequencies should be located to the lower part of operating band n28, n83 and n109, since this frequency range is closest to the band to be protected.

PROPOSAL 2: Select Low Range as test frequency for NS_18 A-MPR tests.

Test Channel Bandwidth

The minimum requirements for NS_18 apply for channel bandwidths 3, 5, 10, 15, 20, 25, 30MHz and 40MHz. The requirements are different for groups {3, 5MHz}, {10, 15, 20MHz} and {25, 30, 40MHz}. Therefore, it is enough to test with maximum supported channel bandwidth in each group.

PROPOSAL 3: Select the maximum supported channel bandwidth in each group of {3, 5MHz}, {10, 15, 20MHz} and {25, 30, 40MHz}.

  - Test Subcarrier Spacing

The selection of test subcarrier spacing can be based on the agreed proposals for SEM [2], i.e. Lowest and Highest supported subcarrier spacing.

PROPOSAL 4: Select Lowest and Highest supported subcarrier spacing for NS_18 A-MPR in FR1.

  - UL Modulation

The selection of UL modulation can be based on the agreed proposals for SEM [2], i.e. all defined UL waveforms (5 DFT-s OFDM and 4 CP-OFDM waveforms). For the same AMPR value, the lower order modulation could be skipped.

PROPOSAL 5: Test all UL modulation waveforms supported by the UE for NS_18. For the same AMPR value, the lower order modulation could be skipped.

  - Uplink RB allocation

The UL RB allocation can be based on the agreed proposals for SEM [2], i.e. Edge_1RB_left, Edge_1RB_Right and Outer_Full. These common uplink configurations are listed in [5].

For 3MHz, 5MHz, 10MHz, 15MHz, 20MHz, A-MPR for NS_18 has some region-specific requirements in the lower part of operating band n28, n83 and n109. Edge_1RB_left and Outer_Full can be considered as a worst-case scenario. No need to test with Edge_1RB_Right as it is not close to the lower part of the operating band.

For channel bandwidth 25MHz, 30MHz and 40MHz, some Region-specific RB allocations are specified in terms of RBend × 12 × SCS and LCRB × 12 × SCS.

Table 3: A-MPR regions for NS_18 (power class 3)

| Channel Bandwidth, MHz | Frequency range of UL channel bandwidth, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 25 | 703~748 | >(LCRB*12*SCS)/2+3.6 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+3.6 | ≥5.4 | A4 |
|  |  | ≤6.3 | <5.4 | A5 |
| 30 | 703~748 | >(LCRB*12*SCS)/2+5.22 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+5.22 | ≥5.4 | A4 |
| 40 | 703~743.04 | ≤7.92 | <5.4 | A5 |
|  |  | >(LCRB*12*SCS)/2+8.46 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+8.46 | ≥5.4 | A4 |
|  |  | ≤11.16 | <5.4 | A5 |

Table 4 A-MPR regions for NS_18 (power class 2)

| Channel Bandwidth, MHz | Frequency range of UL channel bandwidth, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 25 | 703~748 | >(LCRB*12*SCS)/2+3.6 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+3.6 | ≥5.4 | A4 |
|  |  | ≤6.3 | <5.4 | A5 |
|  |  | >(LCRB*12*SCS)/2+3.6 ≤(LCRB*12*SCS)/2+5.76 | <Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) ≥5.4 | A6 |
| 30 | 703~748 | >(LCRB*12*SCS)/2+5.22 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+5.22 | ≥5.4 | A4 |
|  |  | ≤7.92 | <5.4 | A5 |
|  |  | >(LCRB*12*SCS)/2+5.22 ≤(LCRB*12*SCS)/2+7.38 | <Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) ≥5.4 | A6 |
| 40 | 703~743.04 | >(LCRB*12*SCS)/2+8.46 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+8.46 | ≥5.4 | A4 |
|  |  | ≤11.16 | <5.4 | A5 |
|  |  | >(LCRB*12*SCS)/2+8.46 ≤(LCRB*12*SCS)/2+11.7 | <Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) ≥5.4 | A6 |

Table 5 and 6 shows the corresponding RB ranges for 15/30 SCS and proposed allocation expressed as LCRB@RBstart. Edge_1RB_left, Edge_1RB_Right and Outer_Full are selected to align with SEM. One additional RB allocation for A3 and A6 respectively is based on allocating as many RBs as possible located to the lower part of the channel bandwidth, i.e. select lowest RBstart.

Table 5: UL RB allocation for NS_18 with 25MHz, 30MHz and 40MHz bandwidth for PC3

Table 6: UL RB allocation for NS_18 with 25MHz, 30MHz and 40MHz bandwidth for PC2

PROPOSAL 6: Select Outer_Full and Edge_1RB_Left for testing of channel bandwidth 3MHz, 5MHz, 10MHz, 15MHz, 20MHz; Select the RBs as defined in Table 5 and Table 6 for 25MHz, 30MHz and 40MHz.

1.7.	Number of Test Points

The maximum number of test points for A-MPR tests for NS_18:

| Power Class | Environment conditions | Maximum Number of Steps (Freq. X Ch BW. X SCS) | Maximum Number of (Modulations X RB alloc.) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- |
| PC3 | 1 (NC) | 5MHz and 20MHz: 1 X 2 X 2 25MHz, 30MHz, 40MHz: 1 x 1 x 2 | 5MHz and 20MHz: 16 (8x2) 25MHz, 30MHz, 40MHz: 12 (A3: 2x2, A4: 2x1, A5: 6x1) | All: 88 5MHz and 20MHz: 64 25MHz, 30MHz, 40MHz: 24 |
| PC2 | 1 (NC) | 5MHz and 20MHz: 1 X 2 X 2 25MHz, 30MHz, 40MHz: 1 x 1 x 2 | 5MHz and 20MHz: 16 (8x2) 25MHz, 30MHz, 40MHz: 20 (A3: 3x2, A4: 3x1, A5: 5x1, A6: 6x1) | All: 104 5MHz and 20MHz: 64 25MHz, 30MHz, 40MHz: 40 |

Proposal

It is proposed that RAN5 discusses and agrees the following proposals for open areas described in section 2 of this document to progress on SA FR1 NS_18 A-MPR test case definition:

PROPOSAL 1: Select Test Environment as Normal Conditions (NC) for A-MPR.

PROPOSAL 2: Select Low Range as test frequency for NS_18 A-MPR tests.

PROPOSAL 3: Select the maximum supported channel bandwidth in each group of {3, 5MHz}, {10, 15, 20MHz} and {25, 30, 40MHz}.

PROPOSAL 4: Select Lowest and Highest supported subcarrier spacing for NS_18 A-MPR in FR1.

PROPOSAL 5: Test all UL modulation waveforms supported by the UE for NS_18. For the same AMPR value, the lower order modulation could be skipped.

PROPOSAL 6: Select Outer_Full and Edge_1RB_Left for testing of channel bandwidth 3MHz, 5MHz, 10MHz, 15MHz, 20MHz; Select the RBs as defined in Table 5 and Table 6 for 25MHz, 30MHz and 40MHz.

Handling of Additional spurious emission

Given the test point analysis of A-MPR for NS_18 has already taken Additional spurious emission requirement into consideration, the test point of A-SE could reuse that of A-MPR as much as possible. The only difference could be SCS. As per the TP analysis of general spurious emission, higher spectrum utilization would be a more demanding case which can be provided by Lowest SCS.

PROPOSAL 7: A-SE for NS_18 reuses the test points selected for A-MPR for NS_18, except that Lowest SCS is selected.

The maximum number of test points for A-SE tests for NS_18:

| Power Class | Environment conditions | Maximum Number of Steps (Freq. X Ch BW X SCS) | Maximum Number of (Modulations X RB alloc.) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- |
| PC3 | 1 (NC) | 5MHz and 20MHz: 1 X 2 X 1 25MHz, 30MHz, 40MHz: 1 x 1 x 1 | 5MHz and 20MHz: 16 (8x2) 25MHz, 30MHz, 40MHz: 12 (A3: 2x2, A4: 2x1, A5: 6x1) | All: 44 5MHz and 20MHz: 32 25MHz, 30MHz, 40MHz: 12 |
| PC2 | 1 (NC) | 5MHz and 20MHz: 1 X 2 X 1 25MHz, 30MHz, 40MHz: 1 x 1 x 1 | 5MHz and 20MHz: 16 (8x2) 25MHz, 30MHz, 40MHz: 20 (A3: 3x2, A4: 3x1, A5: 5x1, A6: 6x1) | All: 52 5MHz and 20MHz: 32 25MHz, 30MHz, 40MHz: 20 |

References

- R5-180065, "Discussion about Test parameters for NR TRx TCs " (NTT DOCOMO).

- R5-180886, " Discussion on test point selection for NR SEM in FR1” (Keysight Technologies).

- R5-182019, “Discussion of NR FR1 Test Point for TX Spurious Emission test cases”, (Qualcomm Inc.)

- 3GPP TS 38.101-1, V18.7.0 (2024-09), Table 6.2.3.6-1

- 3GPP TS 38.521-1, V18.4.0 (2024-09), Table 6.1-1