3GPP TSG-RAN5 Meeting #108	R5-254243r1

# Bengaluru, India, 25th - 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 4046 | rev |  | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to RedCap BWP configurations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to Table A.8.1-1 and Table A.8.2-1, the BWP BW shall be same as the RMSI CORESET (CORESET#0) definied in the test. Implicitely it is also expected that the SSBs – which the UE needs to read – are included in that BW as well. However for certain bands (e.g. n70) there is an additional offset (kSSB) for the starting ressource of SSB, which pushes the SSB-end beyond the BW of the CORESET#0 (though just some subcarriers). Since the reading of the SSB is required, the BW of the BWP needs to be adopted accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In Table A.8.1-1 and Table A.8.2-1 the following note has been added: Note 2: In case CORESET#0 and SSBs defined in the tests do not completely fit into the BWP bandwidth, the BWP bandwidth is extended to the next smallest size which can fully accommodate CORESERT#0 and SSBs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE will not be able to read the SSB for some bands and fail the test unfairly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.8.1, A.8.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Depends on R4-2512323 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: RAN4 Tdoc number has ben updated in cover sheet. Editorial corrections to Note 2, to align to revised RAN4 CR |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

# A.8	Reference BWP configurations

This clause provides the typical BWP configurations used for RRM test cases defined in this test specification. For downlink BWP, both initial BWP and dedicated BWP configurations are specified in clause A.8.1 and for uplink BWP, dedicated BWP configurations are specified in clause A.8.2. Parameters not listed in this clause can be derived from the configuration of each test.

## A.8.1	Downlink BWP configurations

Table A.8.1-1 defines the different downlink initial BWP configurations. Table A.8.1-2 defines the different downlink dedicated BWP configurations.

Table A.8.1-1: Downlink BWP patterns for initial BWP configuration

| BWP Parameters | Unit | Values |  |  |
| --- | --- | --- | --- | --- |
| DL BWP |  | DLBWP.0.1 | DLBWP.0.2 |  |
| Starting PRB index |  | 0 | RBc Note 1 |  |
| Bandwidth |  | Same as RF channel defined in each test | same as RMSI CORSET(CORESET #0) defined in each test Note 2 |  |
| NOTE 1: RBc is the lowest PRB index to guarantee the BWP including CORESET #0 which is defined in Clause A.1.2. NOTE 2: In case CORESET#0 and SSBs defined in the test do not completely fit into the bandwidth of the reference BWP bandwidth, the latter one is extended to the next smallest size which can fully accommodate the respective CORESET#0 and SSBs. |  |  |  |  |

Table A.8.1-2: Downlink BWP patterns for dedicated BWP configuration

| BWP Parameters | Unit | Values |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL BWP |  | DLBWP.1.1 | DLBWP.1.2 |  |  |  | DLBWP.1.3 |  |  |  | DLBWP.1.4 |  | DLBWP.1.7 |  |  |  |
| Starting PRB index |  | 0 | RBb Note 1 |  |  |  | RBa Note 2 |  |  |  | 0 |  | RBc Note 3 |  |  |  |
| SSB SCS | kHz |  | 15 | 30 | 120 | 240 | 15 | 30 | 120 | 240 | 120 | 240 | 15 | 30 | 120 | 240 |
| Bandwidth | RB | Same as RF channel defined for the serving cell in each test | 25 | 51 | 32 | 48 | 25 | 51 | 32 | 48 | 24 | 24 | 25 | 51 | 32 | 48 |
| NOTE 1: RBb is the lowest PRB index to guarantee the BWP not fully overlapped with SSB PRB index (RBJ, RBJ+1,.…, RBJ+19) which is defined in Clause A.3. NOTE 2: RBa is the lowest PRB index to guarantee the BWP including SSB PRB index (RBJ, RBJ+1,.…, RBJ+19) which is defined in Clause A.3. NOTE 3: RBc is the PRB index having an offset of 6 PRBs with respect to the start of SSB PRB index (RBJ, RBJ+1,.…, RBJ+19) which is defined in Clause A.3. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## A.8.2	Uplink BWP configurations

Table A.8.2-1 defines the uplink initial BWP configurations. Table A.8.2-2 defines the uplink dedicated BWP configurations.

Table A.8.2-1: Uplink BWP patterns for initial BWP configurations

| BWP Parameters | Values |  |
| --- | --- | --- |
| UL BWP | ULBWP.0.1 | ULBWP.0.2 |
| Staring PRB index | 0 | RBc Note 1 |
| Bandwidth | Same as RF channel defined in each test | same as RMSI CORESET(CORSET #0) defined in each test Note 2 |
| NOTE 1: RBc is the lowest PRB index to guarantee the BWP including CORESET #0 which is defined in Clause A.1.2. NOTE 2: In case CORESET#0 and SSBs defined in the test do not completely fit into the bandwidth of the reference BWP bandwidth, the latter one is extended to the next smallest size which can fully accommodate the respective CORESET#0 and SSBs. |  |  |

Table A.8.2-2: Uplink BWP patterns for dedicated BWP configurations

| BWP Parameters | Unit | Values |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL BWP |  | ULBWP.1.1 | ULBWP.1.2 |  |  |  | ULBWP.1.3 |  |  |  | ULBWP.1.4 |  | ULBWP.1.7 |  |  |  |
| Staring PRB index |  | 0 | RBb Note 1 |  |  |  | RBa Note 2 |  |  |  | 0 |  | RBc Note 3 |  |  |  |
| SSB SCS | kHz |  | 15 | 30 | 120 | 240 | 15 | 30 | 120 | 240 | 120 | 240 | 15 | 30 | 120 | 240 |
| Bandwidth | RB | Same as RF channel defined for the serving cell in each test | 25 | 51 | 32 | 48 | 25 | 51 | 32 | 48 | 24 | 24 | 25 | 51 | 32 | 48 |
| NOTE 1: RBb is the lowest PRB index to guarantee the BWP not fully overlapped with SSB PRB index (RBJ, RBJ+1,.…, RBJ+19) which is defined in Clause A.3. NOTE 2: RBa is the lowest PRB index to guarantee the BWP including SSB PRB index (RBJ, RBJ+1,.…, RBJ+19) which is defined in Clause A.3. NOTE 3: RBc is the PRB index having an offset of 6 PRBs with respect to the start of SSB PRB index (RBJ, RBJ+1,.…, RBJ+19) which is defined in Clause A.3. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

< End of changes >