3GPP TSG-RAN5 Meeting #108	R5-254283r1

# Bengaluru, India, 25th Aug 2025 – 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 5115 | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to Test case 8.1.3.1.27 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | QUALCOMM JAPAN LLC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test case 8.1.3.1.27 needs modification to accommodate for InterFreq measurements |  |  |  |  |  |  |  |  |
| Summary of change: |  | - MeasObjNR defined in a separate table 8.1.3.1.27.3.3-8 - Meas Object/Meas ObjectId (in MeasConfig) for NR Cell1 added - referenceLocation IE removed from Table 8.1.3.1.27.3.3-2 - System Information combination updated from NR-29 to NR-32 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This test case will not be implementable |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.3.1.27 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Meeting Location, Meeting number corrected |  |  |  |  |  |  |  |  |

<Unchanged Sections Omitted>

##### 8.1.3.1.27	NR NTN Enhancements / Measurement configuration control and reporting / Event D2

8.1.3.1.27	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED and measurement configured for event D2 with event based periodical reporting }

ensure that {

when { Entering condition for event D2 is satisfied }

then { UE sends MeasurementReport message at regular intervals while entering condition for event D2 is satisfied }

}

(2)

with { UE in NR RRC_CONNECTED and measurement configured for event D2 with event based periodical reporting }

ensure that {

when { Leaving condition for event D2 is satisfied }

then { UE stops sending MeasurementReport message }

}

8.1.3.1.27.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 38.331 clause 5.5.5.1, 5.5.4.15a, and clause 5.5.4.1. Unless otherwise stated these are Rel-18 requirements.

[TS 38.331, clause 5.5.5.1]

Figure 5.5.5.1-1: Measurement reporting

The purpose of this procedure is to transfer measurement results from the UE to the network. The UE shall initiate this procedure only after successful AS security activation.

The UE shall, for each entry in the VarMeasReportList:

...

1>	if there is at least one applicable neighbouring cell or candidate L2 U2N Relay UE to report:

2>	if the reportType is set to eventTriggered or periodical:

3>	if the measurement report concerns the candidate L2 U2N Relay UE:

...

3>	else:

4>	set the measResultNeighCells to include the best neighbouring cells up to maxReportCells in accordance with the following:

5>	if the reportType is set to eventTriggered and eventId is not set to eventD1 or eventD2 or eventH1 or eventH2:

6>	include the cells included in the cellsTriggeredList as defined within the VarMeasReportList for this measId;

...

[TS 38.331, clause 5.5.4.1]

If AS security has been activated successfully, the UE shall:

1>	for each measId included in the measIdList within VarMeasConfig:

2>	if the corresponding reportConfig includes a reportType set to eventTriggered or periodical:

...

2>	else if the reportType is set to eventTriggered and if the eventId is set to eventD1 or eventD2 or eventH1 or eventH2 and if the entering condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled during timeToTrigger defined for this event within the VarMeasConfig, while the VarMeasReportList does not include a measurement reporting entry for this measId:

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	else if the reportType is set to eventTriggered and if the eventId is set to eventD1 or eventD2 or eventH1 or eventH2 and if the leaving condition applicable for this event is fulfilled for the associated VarMeasReport within the VarMeasReportList for this measId during timeToTrigger defined within the VarMeasConfig for this event:

3>	if reportOnLeave is set to true for the corresponding reporting configuration:

4>	initiate the measurement reporting procedure, as specified in 5.5.5;

3>	remove the measurement reporting entry within the VarMeasReportList for this measId;

3>	stop the periodical reporting timer for this measId, if running;

...

[TS 38.331, Clause 5.5.4.15a]

The UE shall:

1>	consider the entering condition for this event to be satisfied when both condition D2-1 and condition D2-2, as specified below, are fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition D2-3 or condition D2-4, i.e. at least one of the two, as specified below, are fulfilled;

Inequality D2-1 (Entering condition 1)

Ml1 - Hys > Thresh1

Inequality D2-2 (Entering condition 2)

Ml2 + Hys < Thresh2

Inequality D2-3 (Leaving condition 1)

Ml1+Hys<Thresh1

Inequality D2-4 (Leaving condition 2)

Ml2 - Hys > Thresh2

The variables in the formula are defined as follows:

Ml1 is the distance between UE and a moving reference location for this event, not taking into account any offsets. The moving reference location is determined based on movingReferenceLocation and the corresponding epoch time and satellite ephemeris for the serving cell broadcast in SIB19.

Ml2 is the distance between UE and a moving reference location for this event, not taking into account any offsets. The moving reference location is determined based on the parameter referenceLocation and the corresponding epoch time and satellite ephemeris configured within the MeasObjectNR associated to this event.

Hys is the hysteresis parameter for this event (i.e. hysteresisLocation as defined within reportConfigNR for this event).

Thresh1 is the threshold for this event defined as a distance, configured with parameter distanceThreshFromReference1 in reportConfigNR for this event, from a moving reference location determined based on the parameter movingReferenceLocation and the corresponding epoch time and satellite ephemeris for the serving cell broadcast in SIB19.

Thresh2 is the threshold for this event defined as a distance, configured with parameter distanceThreshFromReference2 in reportConfigNR for this event, from a moving reference location determined based on the parameter referenceLocation and the corresponding epoch time and satellite ephemeris configured within the MeasObjectNR associated to this event.

Ml1 is expressed in meters.

Ml2 is expressed in the same unit as Ml1.

Hys is expressed in the same unit as Ml1.

Thresh1 is expressed in the same unit as Ml1.

Thresh2 is expressed in the same unit as Ml1.

[]NOTE:	The definition of Event D2 also applies to CondEvent D2.

[38.331 Clause 5.4.4.1]

If AS security has been activated successfully, the UE shall:

1>	for each measId included in the measIdList within VarMeasConfig:

2>	if the corresponding reportConfig includes a reportType set to eventTriggered or periodical:

3>	if the corresponding measObject concerns NR:

....

4>	if corresponding reportConfig includes reportType set to periodical; or

4>	for measurement events other than eventA1, eventA2, eventD1, eventD2, eventX2, eventH1 or eventH2:

5>	if useAllowedCellList is set to true:

6>	consider any neighbouring cell detected based on parameters in the associated measObjectNR to be applicable when the concerned cell is included in the allowedCellsToAddModList defined within the VarMeasConfig for this measId;

5>	else:

6>	consider any neighbouring cell detected based on parameters in the associated measObjectNR to be applicable when the concerned cell is not included in the excludedCellsToAddModList defined within the VarMeasConfig for this measId;

...

2>	else if the reportType is set to eventTriggered and if the eventId is set to eventD1 or eventD2 or eventH1 or eventH2 and if the entering condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled during timeToTrigger defined for this event within the VarMeasConfig, while the VarMeasReportList does not include a measurement reporting entry for this measId:

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	else if the reportType is set to eventTriggered and if the eventId is set to eventD1 or eventD2 or eventH1 or eventH2 and if the leaving condition applicable for this event is fulfilled for the associated VarMeasReport within the VarMeasReportList for this measId during timeToTrigger defined within the VarMeasConfig for this event:

3>	if reportOnLeave is set to true for the corresponding reporting configuration:

4>	initiate the measurement reporting procedure, as specified in 5.5.5;

3>	remove the measurement reporting entry within the VarMeasReportList for this measId;

3>	stop the periodical reporting timer for this measId, if running;

NOTE 1:	Void.

8.1.3.1.27.3	Test description

8.1.3.1.27.3.1	Pre-test conditions

System Simulator:

NR Cell 1 is serving cell and NR Cell 3 is suitable inter-frequency cell [SIB19 of NR Cell 1 and NR Cell 3 are FFS]

System information combination NR-32 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

UE:

-	The pre-configured UE location is defined in TS 38.508-1 [4] Clause 4.5C.

Preamble:

-	The UE is in 5GS 3N-A on NR Cell 1 according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3.

8.1.3.1.27.3.2	Test procedure sequence

Table 8.1.3.1.27.3.2-1 illustrates the downlink power levels to be applied for NR Cell 1 and NR Cell 3 at various time instants of the test execution. Row marked "T0" denotes the conditions after the preamble and at the point indicated in the Main behaviour description in Table 8.1.3.1.27.3.2-2.

Table 8.1.3.1.27.3.2-1: Time instances of cell power level and parameter changes for NR Cell 1 and NR Cell 3 in conducted test environment

|  | Parameter | Unit | NR Cell 1 | NR Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/ SCS | -79 | -85 |  |

Table 8.1.3.1.27.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message including MeasConfig to setup NR measurement and reporting for event D2 (measId 1). | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| - | EXCEPTION: In parallel to events described in step 3, the steps specified in table 8.1.3.1.27.3.2-3 shall take place. | - | - | - | - |
| 3 | Wait for 30 seconds to ensure that the UE performs a periodical inter-frequency reporting for NR Cell 3. | - | - | - | - |
| 4 | The SS transmits Update UE location information message. (Note1) | <-- | UPDATE UE LOCATION INFORMATION | - | - |
| 5 | Wait and ignore MeasurementReport messages for 10s to allow change of UE location and UE measurement. | - | - | - | - |
| 6 | Check: Does the UE transmit a MeasurementReport message within the next 10s? | --> | NR RRC: MeasurementReport | 2 | F |
| Note 1: UE location information is configured such that the event D2 is not satisfied. |  |  |  |  |  |

Table 8.1.3.1.27.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: After the 1st message is received, step 1 below shall be repeated every time the duration indicated in the IE reportInterval has elapsed | - | - | - | - |
| 1 | Check: Does the UE transmit a MeasurementReport message to report event D2 (measId 1) with the measured RSRP value for NR Cell 3? | --> | MeasurementReport | 1 | P |

8.1.3.1.27.3.3	Specific message contents

Table 8.1.3.1.27.3.3-1: SIB19 for NR Cell 1 (All steps, Table 8.1.3.1.27.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-18C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| distanceThresh-r17 | FFS |  |  |
| movingReferenceLocation-r18 | FFS |  |  |
| } |  |  |  |

Table 8.1.3.1.27.3.3-2: SIB19 for NR Cell 3 (All steps, Table 8.1.3.1.27.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-18C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| distanceThresh-r17 | FFS |  |  |
| movingReferenceLocation-r18 | FFS |  |  |
| } |  |  |  |

Table 8.1.3.1.27.3.3-3: RRCReconfiguration (step 1, 8.1.3.1.27.3.2-2)

Derivation Path: TS 38.508-1 [4] Table 4.6.1-13 with condition NR_MEAS

Table 8.1.3.1.27.3.3-4: MeasConfig (Table 8.1.3.1.27.3.3-3)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-69 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 1 entry |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 | MeasObjectIdNR-f1 |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-f1 | Table 8.1.3.1.27.3.3-8 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR- eventD2-r18 | Table 8.1.3.1.27.3.3-5 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 1 entry |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 1 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.1.27.3.3-5: ReportConfigNR-EventD2-r18 (Table 8.1.3.1.27.3.3-4)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-142 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventD2-r18 SEQUENCE { |  |  |  |
| distanceThreshFromReference1-r18 | FFS |  |  |
| distanceThreshFromReference2-r18 | FFS |  |  |
| reportOnLeave-r18 | false |  |  |
| hysteresisLocation-r18 | 0 |  |  |
| timeToTrigger-r18 | ms2560 |  |  |
| } |  |  |  |
| } |  |  |  |
| reportInterval | ms2048 |  |  |
| reportAmount | infinity |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.1.27.3.3-6: UPDATE UE LOCATION INFORMATION (Step 5, Table 8.1.3.1.27.3.2-2)

| Derivation Path: TS 36.509 [8] Table 6.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ellipsoidPointWithAltitude | FFS |  |  |
| horizontalVelocity | FFS |  |  |
| gnss-TOD-msec | FFS |  |  |

Table 8.1.3.1.27.3.3-7: MeasurementReport (step 1, Table 8.1.3.1.27.3.2-3)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-5A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| measurementReport SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServingMOList SEQUENCE (SIZE (1..maxNrofServingCells)) OF MeasResultServMO { | 1 entry | Report NR Cell 1 |  |
| MeasResultServMO[1] SEQUENCE { |  | entry 1 |  |
| servCellId | ServCellIndex of NR Cell 1 |  |  |
| measResultServingCell SEQUENCE { |  |  |  |
| physCellId | Physical layer cell identity of NR Cell 1 |  |  |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListNR SEQUENCE (SIZE (1.. maxCellReport)) OF MeasResultNR { | 1 entry | Report NR neighbour cell |  |
| MeasResultNR[1] SEQUENCE { |  | entry 1 |  |
| physCellId | Physical layer cell identity of NR Cell 3 |  |  |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | Not present |  |  |
| sinr | Not present |  |  |
| } |  |  |  |
| resultsCSI-RS-Cell | Not present |  |  |
| } |  |  |  |
| rsIndexResults | Not present |  |  |
| } |  |  |  |
| cgi-Info | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.1.27.3.3-8: MeasObjectNR-f1 (Table 8.1.3.1.27.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR for SSB of NR Cell 1 |  |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| cellsToAddModListExt-v1800 SEQUENCE { | 1 entry |  |  |
| cellsToAddModExt-v1800[1] SEQUENCE { |  | entry1 |  |
| ntn-NeighbourCellInfo-r18 SEQUENCE { |  |  |  |
| epochTime-r18 | FFS |  |  |
| ephemerisInfo-r18 | FFS |  |  |
| referenceLocation-r18 | FFS |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<Unchanged Sections Omitted>