3GPP TSG-RAN5 Meeting #108	R5-254515r1

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5486 | rev | - | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Inter-RAT cell reselection test case 6.2.3.37 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_Test |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on the outcome of RAN5#107 discussion paper R5-253041, there is industry interest for test coverage related to protection against improper reselection to GERAN/UTRAN feature. Hence, new test case needs to be introduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new IRAT reselection E-UTRA - GERAN test case 6.2.3.37. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test coverage will not exist for protection against improper reselection to GERAN/UTRAN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.37 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 36.523-2 CR 1509 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-254515r1: 1. Added Note 1 in Table 6.2.3.37.3.2-2. 2. TP3 updated and t320 is set to "Not present” in specific message content Table 6.2.3.37.3.3-5. |  |  |  |  |  |  |  |  |

<Start of modified section>

#### 6.2.3.37	Inter-RAT cell reselection / From E-UTRA RRC_IDLE to GSM_Idle/GPRS Packet_Idle / Protection against improper reselection to GERAN/UTRAN

6.2.3.37.1	Test Purpose (TP)

(1)

with { UE supporting protection against improper reselection to GERAN/UTRAN in E-UTRA RRC_IDLE state }

ensure that {

when { UE detects the cell which belongs to the higher priority inter-RAT(GERAN frequency }

then { UE does not reselect the cell which belongs to the higher priority inter-RAT(GERAN) frequency }

}

(2)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { UE supporting protection against improper reselection to GERAN/UTRAN detects the cell re-selection criteria are not met for the cell which belongs to the lower priority inter-RAT(GERAN) frequency }

then { UE does not reselect the cell which belongs to the lower priority inter-RAT(GERAN) frequency }

}

(3)

with { UE supporting protection against improper reselection to GERAN/UTRAN in E-UTRA RRC_IDLE state having received a RRCCconnectionRelease message including a cellReselectionPriority }

ensure that {

when { UE detects the cell re-selection criteria are met for the cell which belongs to the higher priority inter-RAT(GERAN) frequency }

then { UE reselects the cell which belongs to the higher priority inter-RAT(GERAN) frequency }

}

6.2.3.37.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304, clause 5.2.4.1 and 5.2.4.5. Unless otherwise stated these are Rel-18 requirements.

[TS 36.304, clause 5.2.4.1]

Absolute priorities of different E-UTRAN frequencies or inter-RAT frequencies may be provided to the UE in the system information, in the RRCConnectionRelease or RRCEarlyDataComplete message, or by inheriting from another RAT at inter-RAT cell (re)selection. In the case of system information, an E-UTRAN frequency or inter-RAT frequency may be listed without providing a priority (i.e. the field cellReselectionPriority is absent for that frequency). If priorities are provided in dedicated signalling, the UE shall ignore all the priorities provided in system information. If UE is in camped on any cell state, UE shall only apply the priorities (i.e. cellReselectionPriority and/or cellReselectionSubPriority) provided by system information from current cell, and the UE preserves priorities provided by dedicated signalling, deprioritisationReq received in RRCConnectionReject and altFreqPriorities provided by dedicated signalling unless specified otherwise. When the UE in camped normally state, has only dedicated priorities other than for the current frequency, the UE shall consider the current frequency to be the lowest priority frequency (i.e. lower than any of the network configured values).

…

The UE shall delete priorities or altFreqPriorities provided by dedicated signalling when:

-	the UE enters a different RRC state; or

-	the optional validity time of dedicated priorities (T320) expires; or

-	the optional validity time of altFreqPriorities (T323) expires; or

-	a PLMN selection is performed on request by NAS TS 23.122 [5].

NOTE 6:	Equal priorities between RATs are not supported.

The UE shall only perform cell reselection evaluation for E-UTRAN frequencies and inter-RAT frequencies that are given in system information and for which the UE has a priority provided.

…

The UE shall not consider any exclude-listed cells as candidate for cell reselection.

For cell reselection to NR operating with shared spectrum channel access, the UE shall consider only the allow-listed cells, if configured in SIB24, as candidates for cell reselection.

The UE shall inherit the priorities provided by dedicated signalling and the remaining validity time (i.e., T320 in E-UTRA and NR, T322 in UTRA and T3230 in GERAN), if configured, at inter-RAT cell (re)selection. The UE shall delete altFreqPriorities provided by dedicated signalling, if configured, at inter-RAT cell (re)selection.

NOTE 7:	The network may assign dedicated cell reselection priorities for frequencies not configured by system information.

…

[TS 36.304, clause 5.2.4.5]

If threshServingLowQ is provided in SystemInformationBlockType3 and more than 1 second has elapsed since the UE camped on the current serving cell and if the measurements are not performed using RSS as specified in [10], cell reselection to a cell on a higher priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	A cell of a higher priority EUTRAN, NR or UTRAN FDD RAT/ frequency fulfils Squal > ThreshX, HighQ during a time interval TreselectionRAT; or

-	A cell of a higher priority UTRAN TDD, GERAN or CDMA2000 RAT/ frequency fulfils Srxlev > ThreshX, HighP during a time interval TreselectionRAT.

Otherwise, cell reselection to a cell on a higher priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	A cell of a higher priority RAT/ frequency fulfils Srxlev > ThreshX, HighP during a time interval TreselectionRAT; and

-	More than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a cell on an equal priority E-UTRAN frequency shall be based on ranking for Intra-frequency cell reselection as defined in clause 5.2.4.6.

If threshServingLowQ is provided in SystemInformationBlockType3 and more than 1 second has elapsed since the UE camped on the current serving cell and if the measurements are not performed using RSS as specified in [10], cell reselection to a cell on a lower priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	The serving cell fulfils Squal < ThreshServing, LowQ and a cell of a lower priority EUTRAN, NR or UTRAN FDD RAT/ frequency fulfils Squal > ThreshX, LowQ during a time interval TreselectionRAT; or

-	The serving cell fulfils Squal < ThreshServing, LowQ and a cell of a lower priority UTRAN TDD, GERAN or CDMA2000 RAT/ frequency fulfils Srxlev > ThreshX, LowP during a time interval TreselectionRAT.

Otherwise, cell reselection to a cell on a lower priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	The serving cell fulfils Srxlev < ThreshServing, LowP and a cell of a lower priority RAT/ frequency fulfils Srxlev > ThreshX, LowP during a time interval TreselectionRAT; and

-	More than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a higher priority RAT/ frequency shall take precedence over a lower priority RAT/ frequency, if multiple cells of different priorities fulfil the cell reselection criteria.

If the UE supports the protection against improper reselection to GERAN/UTRAN then:

-	When the UE evaluates serving cell conditions for the purpose of cell reselection to GERAN/UTRAN FDD/TDD by applying the reselection priorities provided in AS security protected dedicated signalling with a GERAN/UTRAN FDD/TDD frequency as lower priority compared to the current E-UTRAN frequency, or when the UE applies GERAN/UTRAN FDD/TDD priorities provided in system information, the UE shall:

-	treat GERAN and/or UTRAN FDD and/or UTRAN TDD frequencies as the lower priority compared to E-UTRAN;

-	set the value of ThreshServing, LowP to 6 dB if the value received in the system information is higher than 6 dB;

-	set the value of Q-RxLevMin to -116 dBm if the value received in SIB1 is higher than -116 dBm;

-	set the values of Pcompensation and Qoffsettemp to 0.

…

6.2.3.37.3	Test description

6.2.3.37.3.1	Pre-test conditions

System Simulator:

-	Cell 1, Cell 24 and Cell 25.

-	System information combination 5 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

-	None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1 (serving cell) according to [18].

6.2.3.37.3.2	Test procedure sequence

Table 6.2.3.37.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 6.2.3.37.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 24 | Cell 25 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -70 | - | - |  |
|  | Srxlev* | dB | 36 | - | - | SrxlevCell 1 > 0 |
|  | RF Signal level | dBm | - | -80 | OFF | SrxlevCell 24 > 4 [threshX-High] |
|  | C1* | - | - | 21 | N/A |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -100 | - | - | SrxlevCell 1 < 20 [threshServingLow] |
|  | RF Signal level | dBm | - | OFF | -80 | SrxlevCell 25 > 4 [threshX-Low] |
| Note: Srxlev and C1 are calculated in the UE. |  |  |  |  |  |  |

Table 6.2.3.37.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Wait for 6s for UE to receive system information. | - | - | - | - |
| 2 | SS adjusts the cell power levels according to the row "T1" in table 6.2.3.37.3.2-1. | - | - | - | - |
| 3 | Check: Does the UE send a CHANNEL REQUEST within the next 60s on Cell 24? | --> | CHANNEL REQUEST | 1 | F |
| 4 | SS adjusts the cell power levels according to the row "T2" in table 6.2.3.37.3.2-1. (Note 1) | - | - | - | - |
| 5 | Check: Does the UE send a CHANNEL REQUEST within the next 60s on Cell 25? | --> | CHANNEL REQUEST | 2 | F |
| 6 | The SS transmits a Paging message including matched identity on Cell 1. | <-- | Paging | - | - |
| 7 | UE transmit an RRCConnectionRequest message on Cell 1. | --> | RRCConnectionRequest | - | - |
| 8 | The SS transmits an RRCConnectionSetup message on Cell 1. | <-- | RRCConnectionSetup | - | - |
| 9 | The UE transmits an RRCConnectionSetupComplete on Cell 1. | --> | RRCConnectionSetupComplete | - | - |
| 10 | The SS transmits a SecurityModeCommand message. | <-- | SecurityModeCommand | - | - |
| 11 | The UE transmits a SecurityModeComplete message. | --> | SecurityModeComplete | - | - |
| 12 | The SS transmits an RRCConnectionRelease message with an idleModeMobilityControlInfo including a cellReselectionPriority on Cell 1. | <-- | RRCConnectionRelease | - | - |
| 13 | SS adjusts cell levels according to row "T1" of table 6.2.3.37.3.2-1. | - | - | - | - |
| 14 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.9 indicate that the UE is camped on Cell 24? NOTE: The UE performs a RAU procedure. | - | - | 3 | P |
| Note 1: Power levels are such that low priority inter RAT cell reselection criteria is satisfied for UEs not supporting protection against improper reselection to GERAN/UTRAN. |  |  |  |  |  |

6.2.3.37.3.3	Specific message contents

Table 6.2.3.37.3.3-1:  SystemInformationBlockType3 for cell 1 (preamble and all steps, Table 6.2.3.37.3.2-2)

| Derivation Path: 36.508[18] table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| s-NonIntraSearch | Not present |  |  |
| threshServingLow | 10 | Actual value =20 dB |  |
| cellReselectionPriority | 4 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.37.3.3-2: SystemInformationBlockType7 for cell 1 (preamble and all steps, Table 6.2.3.37.3.2-2)

| Derivation Path: 36.508[18] table 4.4.3.3-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType7 ::= SEQUENCE { |  |  |  |
| t-ReselectionGERAN | 7 |  |  |
| speedDependentScalingParameters SEQUENCE {} | Not present |  |  |
| carrierFreqsInfoList SEQUENCE (SIZE (1..maxGNFG)) OF SEQUENCE { |  |  |  |
| carrierFreqsGERAN SEQUENCE [n] { |  |  |  |
| startingARFCN | Same as cell 24 |  |  |
| bandIndicator | Same as cell 24 |  |  |
| followingARFCNs CHOICE { |  |  |  |
| explicitListOfARFCNs | Empty |  |  |
| } |  |  |  |
| } |  |  |  |
| commonInfo SEQUENCE { |  |  |  |
| cellReselectionPriority | 5 | Greater than EUTRA Freq |  |
| ncc-Permitted | ‘01000000’B |  |  |
| q-RxLevMin | 2 | The actual value of Qrxlevmin in dBm = (IE value * 2) 115. |  |
| threshX-High | 2 | Actual value =4 dB |  |
| threshX-Low | 2 | Actual value =4 dB |  |
| } |  |  |  |
| carrierFreqsGERAN SEQUENCE [m] { |  |  |  |
| startingARFCN | Same as cell 25 |  |  |
| bandIndicator | Same as cell 25 |  |  |
| followingARFCNs CHOICE { |  |  |  |
| explicitListOfARFCNs | Empty |  |  |
| } |  |  |  |
| } |  |  |  |
| commonInfo SEQUENCE { |  |  |  |
| cellReselectionPriority | 3 | Less than EUTRA Freq |  |
| ncc-Permitted | ‘01000000’B |  |  |
| q-RxLevMin | 2 | The actual value of Qrxlevmin in dBm = (IE value * 2) 115. |  |
| threshX-High | 2 | Actual value =4 dB |  |
| threshX-Low | 2 | Actual value =4 dB |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.37.3.3-3: System Information 2 Quater for Cell 24 (preamble and all steps, Table 6.2.3.37.3.2-2)

| Derivation Path: 36.508[18] 4.4.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Serving Cell Priority Parameters Description::= { |  |  |  |
| GERAN_PRIORITY | 5 | Greater than EUTRA freq |  |
| } |  |  |  |
| Repeated E-UTRAN Neighbour Cells { | 1 | Present |  |
| EARFCN | Same as cell 1 |  |  |
| EARFCN | 65535 |  | Band > 64 |
| E-UTRAN_QRXLEVMIN | 21 (-98 dBm) |  |  |
| } ** 0 |  |  |  |
| Extended EARFCNs Description | 1 |  | Band > 64 |
| Repeated EARFCN extended { | 1 | Present |  |
| EARFCN_extended | Same as cell 1 | This field specifies the E-UTRA Absolute Radio Frequency Channel Number as defined in 3GPP TS 36.104. |  |
| } ** 0 |  | End of Repeated EARFCN_extended |  |

Table 6.2.3.37.3.3-4: System Information 2 Quater for Cell 25 (preamble and all steps, Table 6.2.3.37.3.2-2)

| Derivation Path: 36.508[18] 4.4.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Serving Cell Priority Parameters Description::= { |  |  |  |
| GERAN_PRIORITY | 3 | Less than EUTRA freq |  |
| } |  |  |  |
| Repeated E-UTRAN Neighbour Cells { | 1 | Present |  |
| EARFCN | Same as cell 1 |  |  |
| EARFCN | 65535 |  | Band > 64 |
| E-UTRAN_QRXLEVMIN | 21 (-98 dBm) |  |  |
| } ** 0 |  |  |  |
| Extended EARFCNs Description | 1 |  | Band > 64 |
| Repeated EARFCN extended { | 1 | Present |  |
| EARFCN_extended | Same as cell 1 | This field specifies the E-UTRA Absolute Radio Frequency Channel Number as defined in 3GPP TS 36.104. |  |
| } ** 0 |  | End of Repeated EARFCN_extended |  |

Table 6.2.3.37.3.3-5: RRCConnectionRelease (step 12, Table 6.2.3.37.3.2-2)

| Derivation Path: 36.508[18], Table 4.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |
| idleModeMobilityControlInfo SEQUENCE { |  |  |  |
| freqPriorityListEUTRA SEQUENCE (SIZE (1.. maxFreq)) OF SEQUENCE { | 1 entry |  |  |
| carrierFreq[1] | Same downlink EARFCN as used for Cell 1 |  |  |
| carrierFreq[1] | maxEARFCN |  | Band > 64 |
| cellReselectionPriority[1] | 4 |  |  |
| } |  |  |  |
| FreqPriorityListGERAN SEQUENCE (SIZE (1..maxCNFG)) OF SEQUENCE { | 1 entry |  |  |
| carrierFreqs[1] | Same downlink ARFCN as used for Cell 24 |  |  |
| cellReselectionPriority[1] | 5 |  |  |
| } |  |  |  |
| bandClassPriorityListHRPD | Not present |  |  |
| bandClassPriorityList1XRTT | Not present |  |  |
| t320 | Not present |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | Band > 64 |
| lateNonCriticalExtension SEQUENCE { |  |  |  |
| idleModeMobilityControlInfo-v9e0 SEQUENCE { |  |  |  |
| freqPriorityListEUTRA-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| carrierFreq-v9e0[1] | Same downlink EARFCN as used for Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<End of modified section>