3GPP TSG-RAN5 Meeting #109	R5-255631

# Dallas, US, 17th – 21st November, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.903 | CR | 1095 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | FR2 MU - PC3 update for OBW UL MIMO test in 38.903 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Certain progress has been made for PC3 MU and TT analysis in discussion R5-255558. Impacted test cases should be updated accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated OBW UL MIMO test for PC3 defining MU and MBW for the FR2c 400MHz case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification will remain incomplete for PC3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | B.15 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

# B.15	Occupied bandwidth

Following tables summarize the MU threshold for EIRP measurements for Occupied bandwidth. The origin MU values for different test setups can be found in following subclauses.

Table B.15-1: MU threshold for beam peak measurement for Occupied bandwidth (SISO)

| Power Class | Frequency | Power | BW | MBW/BW | Threshold MU value (NOTE1) [%CBW] |
| --- | --- | --- | --- | --- | --- |
| PC3, PC1 | 23.45 GHz ≤ f ≤ 32.125 GHz | P = Max Output Power | 50 MHz | 1.5 | ±0.4 |
|  |  |  | 100 MHz | 1.5 | ±0.4 |
|  |  |  | 200 MHz | 1.5 | ±1.2 |
|  |  |  | 400 MHz | 1.5 | ±1.2 |
|  | 32.125 GHz < f ≤ 40.8 GHz | P = Max Output Power | 50 MHz | 1.5 | ±0.4 |
|  |  |  | 100 MHz | 1.5 | ±0.4 |
|  |  |  | 200 MHz | 1.5 | ±1.3 |
|  |  |  | 400 MHz | 1.5 | ±1.3 |
| PC3 | 40.8GHz < f <= 44.3GHz | P = Max Output Power | 50 MHz | 1.5 | ±0.65 |
|  |  |  | 100 MHz | 1.5 | ±0.65 |
|  |  |  | 200 MHz | 1.5 | ±1.3 |
|  |  |  | 400 MHz | 1.3 | ±1.5 |
| PC5, PC6 | 23.45 GHz ≤ f ≤ 32.125 GHz | P = Max Output Power | 50 MHz | 1.5 | ±0.4 |
|  |  |  | 100 MHz | 1.5 | ±0.4 |
|  |  |  | 200 MHz | 1.5 | ±1.2 |
|  |  |  | 400 MHz | 1.5 | ±1.2 |
| NOTE 1: Total Expanded MU for IFF for Quiet Zone size ≤ 30cm. |  |  |  |  |  |

Table B.15-2: MU threshold for beam peak measurement for Occupied bandwidth (MIMO)

| Power Class | Frequency |  | Power | BW | MBW/BW | Threshold MU value (NOTE1) [%CBW] |
| --- | --- | --- | --- | --- | --- | --- |
| PC3 | 23.45 GHz ≤ f ≤ 32.125 GHz |  | P = Max Output Power | 50 MHz | 1.5 | ±0.4 |
|  |  |  |  | 100 MHz | 1.5 | ±0.4 |
|  |  |  |  | 200 MHz | 1.5 | ±1.2 |
|  |  |  |  | 400 MHz | 1.5 | ±1.3 |
|  | 32.125 GHz < f ≤ 40.8 GHz |  | P = Max Output Power | 50 MHz | 1.5 | ±0.4 |
|  |  |  |  | 100 MHz | 1.5 | ±0.4 |
|  |  |  |  | 200 MHz | 1.5 | ±1.3 |
|  |  |  |  | 400 MHz | 1.3 | ±1.7 |
|  | 40.8GHz < f <= 44.3GHz |  | P = Max Output Power | 50 MHz | 1.5 | ±0.65 |
|  |  |  |  | 100 MHz | 1.5 | ±0.65 |
|  |  |  |  | 200 MHz | 1.3 | ±1.3 |
|  |  |  |  | 400 MHz | 1.2 | ±2.0 |
|  |  | NOTE 1: Total Expanded MU for IFF for Quiet Zone size ≤ 30cm. |  |  |  |  |

## <<< END OF CHANGES >>>