3GPP TSG-RAN WG5 Meeting #109	R5-256164

# Dallas, US, 17 - 21 Nov, 2025

Title:	Discussion on work plan for NR_channel_raster_enh-UEConTest

Agenda item:	6.3.35.3

WI/SI:	NR_channel_raster_enh-UEConTest

Release:	Rel-18

Source:	China Telecom

Document for:	Discussion

# 1 Background

This contribution will discuss the work plan for WI NR_channel_raster_enh-UEConTest.

# 2 Discussion

The major change in the parent WI (NR_channel_raster_enh-Core) under RAN4 is the introduction of the enhanced channel raster for FR1 bands that currently use a 100 kHz channel raster. Also, a new UE capability signalling is introduced to indicate the support of this new feature.

As for the work in RAN5, we believe most of the tasks relate to the modifications in 38.508-1. Given the large number of bands involved, which makes it like a "basket WI". So, we have the work plan like this:

Observation 1: The primary RAN5 task entails updating the test frequency table in TS 38.508-1 to incorporate the enhanced channel raster.

Proposal 1: To this end, we should enumerate all impacted bands in the work plan, as illustrated in the prior example.

Two primary methods are available for implementing this feature in Tx and Rx test cases. Option 1 involves creating a new variant for the affected test cases. While comprehensive, this would create significant additional work given the scope of impact. Conversely, Option 2 addresses the requirement by adding a note to the initial condition table in existing test cases. This method fulfils the objective efficiently, maintaining clarity while minimizing effort, and is therefore recommended.

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1[x] |  |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration |  | Uplink Configuration |  |
|  | Modulation | RB allocation | Modulation | RB allocation |
| 1 | CP-OFDM QPSK | Full RB (NOTE 1) | DFT-s-OFDM QPSK | REFSENS (NOTE 2) |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2. NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. NOTE 3: In a band where UE supports 2Rx but not supports 4Rx, the test needs to be repeated with only 2Rx antennas connected and the other antennas terminated. NOTE x: In a band where UE supports enhanced channel raster, the test needs to be repeated using enhanced channel raster specified in TS 38.508-1 subclause 4.3.1 |  |  |  |  |

Proposal 2: It is proposed to address this feature by adding a note to the existing test cases, thereby minimizing implementation overhead.

# 3 Conclusion

Observation 1: The primary RAN5 task entails updating the test frequency table in TS 38.508-1 to incorporate the enhanced channel raster.

Proposal 1: To this end, we should enumerate all impacted bands in the work plan.

Proposal 2: It is proposed to address this feature by adding a note to the existing test cases, thereby minimizing implementation overhead.