3GPP TSG-RAN5 Meeting #110	R5-261457

# Gothenburg Metropolitan Area, Sweden, 9th Feb 2026 - 13th Feb 2026

| CR-Form-v12.5 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3700 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of RF test frequencies for NTN NR band n252 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_lic_bands_BW_R19-UEConTest |  |  |  |  |  | Date: |  |  | 2026-01-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | RF test frequencies for NTN NR band n252 are not defined yet. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Defined test frequencies for band n252 in clause 4.3.1.9.1.5. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.9.1.5 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 1: -To help CRs implementation, removed clause 4.3.1.9.1.4 placeholder and clarified instead that clause 4.3.1.9.1.4 will be added by R5-260676 (CR 3730). |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

4.3.1.9.1.3	Reference test frequencies for NR NTN operating band n254

Table 4.3.1.9.1.3-1: Test frequencies for NR operating band n254 with SCS 15 kHz and SSB SCS 15 kHz

| UL/DL Bandwidth combination [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5/5 | 25 | Downlink | Low | 2486 | 497200 | 2483.75 | 496750 | 0 | 15 | 6215 | 497290 | 0 | 1 | 2(4) | 5 |
|  |  |  | Mid | 2491.8 | 498360 | 2471.19 | 494238 | 102 |  | 6228 | 498270 | 0 | 0 | 0(0) | 102 |
|  |  |  | High | 2497.5 | 499500 | 2404.53 | 480906 | 504 |  | 6243 | 499470 | 8 | 1 | 0(0) | 505 |
|  |  | Uplink | Low | 1612.5 | 322500 | 1610.25 | 322050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1618.3 | 323660 | 1525.33 | 305066 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1624 | 324800 | 1620.67 | 324134 | 6 |  | - | - | - | - | - | - |
| 5/10 | 52 | Downlink | Low | 2488.5 | 497700 | 2483.82 | 496764 | 0 | 15 | 6216 | 497310 | 2 | 1 | 2(4) | 5 |
|  |  |  | Mid | 2491.8 | 498360 | 2468.76 | 493752 | 102 |  | 6222 | 497790 | 2 | 0 | 0(0) | 102 |
|  |  |  | High | 2495 | 499000 | 2399.6 | 479920 | 504 |  | 6233 | 498730 | 6 | 4 | 2(4) | 512 |
|  | 25 | Uplink | Low | 1612.5 | 322500 | 1610.25 | 322050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1618.3 | 323660 | 1525.33 | 305066 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1624 | 324800 | 1620.67 | 324134 | 6 |  | - | - | - | - | - | - |
| 5/15 | 79 | Downlink | Low | 2491 | 498200 | 2483.89 | 496778 | 0 | 15 | 6217 | 497330 | 4 | 1 | 2(4) | 5 |
|  |  |  | Mid | 2491.8 | 498360 | 2466.33 | 493266 | 102 |  | 6216 | 497310 | 4 | 0 | 0(0) | 102 |
|  |  |  | High | 2492.5 | 498500 | 2394.67 | 478934 | 504 |  | 6220 | 497570 | 8 | 1 | 1(2) | 507 |
|  | 25 | Uplink | Low | 1612.5 | 322500 | 1610.25 | 322050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1618.3 | 323660 | 1525.33 | 305066 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1624 | 324800 | 1620.67 | 324134 | 6 |  | - | - | - | - | - | - |
| 10/10 | 52 | Downlink | Low | 2488.5 | 497700 | 2483.82 | 496764 | 0 | 15 | 6216 | 497310 | 2 | 1 | 2(4) | 5 |
|  |  |  | Mid | 2491.8 | 498360 | 2468.76 | 493752 | 102 |  | 6222 | 497790 | 2 | 0 | 0(0) | 102 |
|  |  |  | High | 2495 | 499000 | 2399.6 | 479920 | 504 |  | 6233 | 498730 | 6 | 4 | 2(4) | 512 |
|  |  | Uplink | Low | 1615 | 323000 | 1610.32 | 322064 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1618.3 | 323660 | 1522.9 | 304580 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1621.5 | 324300 | 1615.74 | 323148 | 6 |  | - | - | - | - | - | - |
| 10/15 | 79 | Downlink | Low | 2491 | 498200 | 2483.89 | 496778 | 0 | 15 | 6217 | 497330 | 4 | 1 | 2(4) | 5 |
|  |  |  | Mid | 2491.8 | 498360 | 2466.33 | 493266 | 102 |  | 6216 | 497310 | 4 | 0 | 0(0) | 102 |
|  |  |  | High | 2492.5 | 498500 | 2394.67 | 478934 | 504 |  | 6220 | 497570 | 8 | 1 | 1(2) | 507 |
|  | 52 | Uplink | Low | 1615 | 323000 | 1610.32 | 322064 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1618.3 | 323660 | 1522.9 | 304580 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1621.5 | 324300 | 1615.74 | 323148 | 6 |  | - | - | - | - | - | - |
| 15/15 | 79 | Downlink | Low | 2491 | 498200 | 2483.89 | 496778 | 0 | 15 | 6217 | 497330 | 4 | 1 | 2(4) | 5 |
|  |  |  | Mid | 2491.8 | 498360 | 2466.33 | 493266 | 102 |  | 6216 | 497310 | 4 | 0 | 0(0) | 102 |
|  |  |  | High | 2492.5 | 498500 | 2394.67 | 478934 | 504 |  | 6220 | 497570 | 8 | 1 | 1(2) | 507 |
|  |  | Uplink | Low | 1617.5 | 323500 | 1610.39 | 322078 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1618.3 | 323660 | 1520.47 | 304094 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1619 | 323800 | 1610.81 | 322162 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: 20/35 MHz UL/DL Bandwidth combination is tested with Low range test frequency only. Low range test frequency shall be used instead of Mid range and High range test frequencies. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.9.1.3-2: Test frequencies for NR operating band n254 with SCS 30 kHz and SSB SCS 15 kHz

| UL/DL Bandwidth combination [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10/10 | 24 | Downlink | Low | 2488.5 | 497700 | 2484.18 | 496836 | 0 | 15 | 6222 | 497790 | 6 | 0 | 3 (8) | 16 |
|  |  |  | Mid | 2491.8 | 498360 | 2450.76 | 490152 | 102 |  | 6228 | 498270 | 18 | 0 | 0 (5) | 214 |
|  |  |  | High | 2495 | 499000 | 2309.24 | 461848 | 504 |  | 6236 | 498970 | 14 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 1615 | 323000 | 1610.68 | 322136 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1618.3 | 323660 | 1432.54 | 286508 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1621.5 | 324300 | 1615.02 | 323004 | 6 |  | - | - | - | - | - | - |
| 10/15 | 38 | Downlink | Low | 2491 | 498200 | 2484.16 | 496832 | 0 | 15 | 6220 | 497570 | 6 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 2491.8 | 498360 | 2448.24 | 489648 | 102 |  | 6222 | 497790 | 2 | 0 | 1 (6) | 216 |
|  |  |  | High | 2492.5 | 498500 | 2304.22 | 460844 | 504 |  | 6226 | 497810 | 18 | 2 | 0 (5) | 1022 |
|  | 24 | Uplink | Low | 1615 | 323000 | 1610.68 | 322136 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1618.3 | 323660 | 1432.54 | 286508 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1621.5 | 324300 | 1615.02 | 323004 | 6 |  | - | - | - | - | - | - |
| 15/15 | 38 | Downlink | Low | 2491 | 498200 | 2484.16 | 496832 | 0 | 15 | 6220 | 497570 | 6 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 2491.8 | 498360 | 2448.24 | 489648 | 102 |  | 6222 | 497790 | 2 | 0 | 1 (6) | 216 |
|  |  |  | High | 2492.5 | 498500 | 2304.22 | 460844 | 504 |  | 6226 | 498050 | 18 | 2 | 0 (5) | 1022 |
|  |  | Uplink | Low | 1617.5 | 323500 | 1610.66 | 322132 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1618.3 | 323660 | 1430.02 | 286004 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1619 | 323800 | 1610 | 322000 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: 20/35 MHz UL/DL Bandwidth combination is tested with Low range test frequency only. Low range test frequency shall be used instead of Mid range and High range test frequencies. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

r

Table 4.3.1.9.1.3-3: Test frequencies for NR operating band n254 with SCS 60 kHz and SSB SCS 15 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 2488.5 | 497700 | 2484.54 | 496908 | 0 | 15 | - | 497268 |
|  |  |  | Mid | 2491.8 | 498360 | 2414.4 | 482880 | 102 |  | - | 497928 |
|  |  |  | High | 2495 | 499000 | 2128.16 | 425632 | 504 |  | - | 498568 |
|  |  | Uplink | Low | 1615 | 323000 | 1611.04 | 322208 | 0 | - | - | - |
|  |  |  | Mid | 1618.3 | 323660 | 1251.46 | 250292 | 504 |  | - | - |
|  |  |  | High | 1621.5 | 324300 | 1613.22 | 322644 | 6 |  | - | - |
| 15 | 18 | Downlink | Low | 2491 | 498200 | 2484.52 | 496904 | 0 | 15 | - | 497264 |
|  |  |  | Mid | 2491.8 | 498360 | 2411.88 | 482376 | 102 |  | - | 497424 |
|  |  |  | High | 2492.5 | 498500 | 2123.14 | 424628 | 504 |  | - | 497564 |
|  |  | Uplink | Low | 1617.5 | 323500 | 1611.02 | 322204 | 0 | - | - | - |
|  |  |  | Mid | 1618.3 | 323660 | 1248.94 | 249788 | 504 |  | - | - |
|  |  |  | High | 1619 | 323800 | 1608.2 | 321640 | 6 |  | - | - |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

## <<< new clause 4.3.1.9.1.4 will be added by R5-260676 (CR 3730) >>>

4.3.1.9.1.5	Reference test frequencies for NR NTN operating band n252

Table 4.3.1.9.1.5-1: Test frequencies for NR operating band n252 with SCS 15 kHz and SSB SCS 15 kHz

| UL/DL Bandwidth combination [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5/5 | 25 | Downlink | Low | 2182.5 | 436500 | 2180.25 | 436050 | 0 | 15 | 5457 | 436590 | 0 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 2190 | 438000 | 2169.39 | 433878 | 102 |  | 5475 | 438030 | 4 | 1 | 1 (2) | 105 |
|  |  |  | High | 2197.5 | 439500 | 2104.53 | 420906 | 504 |  | 5493 | 439470 | 8 | 1 | 0 (0) | 505 |
|  |  | Uplink | Low | 2002.5 | 400500 | 2000.25 | 400050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2010 | 402000 | 1917.03 | 383406 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2017.5 | 403500 | 2014.17 | 402834 | 6 |  | - | - | - | - | - | - |
| 5/10 | 52 | Downlink | Low | 2185 | 437000 | 2180.32 | 436064 | 0 | 15 | 5458 | 436610 | 2 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 2190 | 438000 | 2166.93 | 433392 | 102 |  | 5469 | 437550 | 6 | 1 | 1 (2) | 105 |
|  |  |  | High | 2195 | 439000 | 2099.6 | 419920 | 504 |  | 5480 | 438490 | 10 | 1 | 0 (0) | 505 |
|  | 25 | Uplink | Low | 2005 | 401000 | 2002.75 | 400550 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2010 | 402000 | 1971.03 | 383406 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2015 | 403000 | 2011.67 | 402334 | 6 |  | - | - | - | - | - | - |
| 5/15 | 79 | Downlink | Low | 2187.5 | 437500 | 2180.39 | 436078 | 0 | 15 | 5456 | 436570 | 8 | 1 | 1 (2) | 3 |
|  |  |  | Mid | 2190 | 438000 | 2164.53 | 432906 | 102 |  | 5463 | 437070 | 8 | 1 | 1 (2) | 105 |
|  |  |  | High | 2192.5 | 438500 | 2094.67 | 418934 | 504 |  | 5470 | 437570 | 8 | 1 | 1 (2) | 507 |
|  | 25 | Uplink | Low | 2007.5 | 401500 | 2005.25 | 401050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2010 | 402000 | 1917.03 | 383406 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2012.5 | 402500 | 2009.17 | 401834 | 6 |  | - | - | - | - | - | - |
| 5/20 | 106 | Downlink | Low | 2190 | 438000 | 2180.46 | 436092 | 0 | 15 | 5457 | 436590 | 10 | 1 | 1 (2) | 3 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 25 | Uplink | Low | 2010 | 402000 | 2007.75 | 401550 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| 10/10 | 52 | Downlink | Low | 2185 | 437000 | 2180.32 | 436064 | 0 | 15 | 5458 | 436610 | 2 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 2190 | 438000 | 2166.96 | 433392 | 102 |  | 5469 | 437550 | 6 | 1 | 1 (2) | 105 |
|  |  |  | High | 2195 | 439000 | 2099.6 | 419920 | 504 |  | 5480 | 438490 | 10 | 1 | 0 (0) | 505 |
|  |  | Uplink | Low | 2005 | 401000 | 2000.32 | 400064 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2010 | 402000 | 1914.6 | 382920 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2015 | 403000 | 2009.24 | 401848 | 6 |  | - | - | - | - | - | - |
| 10/15 | 79 | Downlink | Low | 2187.5 | 437500 | 2180.39 | 436078 | 0 | 15 | 5456 | 436570 | 8 | 1 | 1 (2) | 3 |
|  |  |  | Mid | 2190 | 438000 | 2164.53 | 432906 | 102 |  | 5463 | 437070 | 8 | 1 | 1 (2) | 105 |
|  |  |  | High | 2192.5 | 438500 | 2094.67 | 418934 | 504 |  | 5470 | 437570 | 8 | 1 | 1 (2) | 507 |
|  | 52 | Uplink | Low | 2007.5 | 401500 | 2002.82 | 400564 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2010 | 402000 | 1914.6 | 382920 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2012.5 | 402500 | 2006.74 | 401348 | 6 |  | - | - | - | - | - | - |
| 10/20 | 106 | Downlink | Low | 2190 | 438000 | 2180.46 | 436092 | 0 | 15 | 5457 | 436590 | 10 | 1 | 1 (2) | 3 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 52 | Uplink | Low | 2010 | 402000 | 2005.32 | 401064 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| 15/15 | 79 | Downlink | Low | 2187.5 | 437500 | 2180.39 | 436078 | 0 | 15 | 5456 | 436570 | 8 | 1 | 1 (2) | 3 |
|  |  |  | Mid | 2190 | 438000 | 2164.53 | 432906 | 102 |  | 5463 | 437070 | 8 | 1 | 1 (2) | 105 |
|  |  |  | High | 2192.5 | 438500 | 2094.67 | 418934 | 504 |  | 5470 | 437570 | 8 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 2007.5 | 401500 | 2000.39 | 400078 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2010 | 402000 | 1912.17 | 382434 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2012.5 | 402500 | 2004.31 | 400862 | 6 |  | - | - | - | - | - | - |
| 15/20 | 106 | Downlink | Low | 2190 | 438000 | 2180.46 | 436092 | 0 | 15 | 5457 | 436590 | 10 | 1 | 1 (2) | 3 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 79 | Uplink | Low | 2010 | 402000 | 2002.89 | 400578 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| 20/20 | 106 | Downlink | Low | 2190 | 438000 | 2180.46 | 436092 | 0 | 15 | 5457 | 436590 | 10 | 1 | 1 (2) | 3 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | Low | 2010 | 402000 | 2000.46 | 400092 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: 5/20 MHz, 10/20 MHz, 15/20 MHz and 20/20 MHz UL/DL Bandwidth combination is tested with Low range test frequency only. Low range test frequency shall be used instead of Mid range and High range test frequencies. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.9.1.5-2: Test frequencies for NR operating band n252 with SCS 30 kHz and SSB SCS 15 kHz

| UL/DL Bandwidth combination [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10/10 | 24 | Downlink | Low | 2185 | 437000 | 2180.68 | 436136 | 0 | 15 | 5464 | 437090 | 6 | 0 | 3 (8) | 16 |
|  |  |  | Mid | 2190 | 438000 | 2148.96 | 429792 | 102 |  | 5475 | 438030 | 10 | 0 | 2 (7) | 218 |
|  |  |  | High | 2195 | 439000 | 2009.24 | 401848 | 504 |  | 5486 | 438970 | 14 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 2005 | 401000 | 2000.68 | 400136 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2010 | 402000 | 1824.24 | 364848 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2015 | 403000 | 2008.52 | 401704 | 6 |  | - | - | - | - | - | - |
| 10/15 | 38 | Downlink | Low | 2187.5 | 437500 | 2180.66 | 436132 | 0 | 15 | 5462 | 437050 | 18 | 0 | 2 (7) | 14 |
|  |  |  | Mid | 2190 | 438000 | 2146.44 | 429288 | 102 |  | 5469 | 437550 | 18 | 0 | 2 (7) | 218 |
|  |  |  | High | 2192.5 | 438500 | 2004.22 | 400844 | 504 |  | 5476 | 438050 | 18 | 0 | 2 (7) | 1022 |
|  | 24 | Uplink | Low | 2007.5 | 401500 | 2003.18 | 400636 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2010 | 402000 | 1824.24 | 364848 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2012.5 | 402500 | 2006.02 | 401204 | 6 |  | - | - | - | - | - | - |
| 10/20 | 51 | Downlink | Low | 2190 | 438000 | 2180.82 | 436164 | 0 | 15 | 5463 | 437070 | 14 | 0 | 2 (7) | 14 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 24 | Uplink | Low | 2010 | 402000 | 2005.68 | 401136 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| 15/15 | 38 | Downlink | Low | 2187.5 | 437500 | 2180.66 | 436132 | 0 | 15 | 5462 | 437050 | 18 | 0 | 2 (7) | 14 |
|  |  |  | Mid | 2190 | 438000 | 2146.44 | 429288 | 102 |  | 5469 | 437550 | 18 | 0 | 2 (7) | 18 |
|  |  |  | High | 2192.5 | 438500 | 2004.22 | 400844 | 504 |  | 5476 | 438050 | 18 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 2007.5 | 401500 | 2000.66 | 400132 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2010 | 402000 | 1821.72 | 364344 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2012.5 | 402500 | 2003.5 | 400700 | 6 |  | - | - | - | - | - | - |
| 15/20 | 51 | Downlink | Low | 2190 | 438000 | 2180.46 | 436092 | 0 | 15 | 5457 | 436590 | 10 | 1 | 1 (2) | 3 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 38 | Uplink | Low | 2010 | 402000 | 2002.89 | 400578 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| 20/20 | 51 | Downlink | Low | 2190 | 438000 | 2180.46 | 436092 | 0 | 15 | 5457 | 436590 | 10 | 1 | 1 (2) | 3 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | Low | 2010 | 402000 | 2000.46 | 400092 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: 10/20 MHz, 15/20 MHz and 20/20 MHz UL/DL Bandwidth combination is tested with Low range test frequency only. Low range test frequency shall be used instead of Mid range and High range test frequencies. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.9.1.5-3: Test frequencies for NR operating band n252 with SCS 60 kHz and SSB SCS 15 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 2185 | 437000 | 2181.04 | 436208 | 0 | 15 | - | 436568 |
|  |  |  | Mid | 2190 | 438000 | 2112.6 | 422520 | 102 |  | - | 437568 |
|  |  |  | High | 2195 | 439000 | 1828.16 | 365632 | 504 |  | - | 438568 |
|  |  | Uplink | Low | 2005 | 401000 | 2001.04 | 400208 | 0 | - | - | - |
|  |  |  | Mid | 2010 | 402000 | 1643.16 | 328632 | 504 |  | - | - |
|  |  |  | High | 2015 | 403000 | 2006.72 | 401344 | 6 |  | - | - |
| 15 | 18 | Downlink | Low | 2187.5 | 437500 | 2181.02 | 436204 | 0 | 15 | - | 436564 |
|  |  |  | Mid | 2190 | 438000 | 2110.08 | 422016 | 102 |  | - | 437064 |
|  |  |  | High | 2192.5 | 438500 | 1823.14 | 364628 | 504 |  | - | 437564 |
|  |  | Uplink | Low | 2007.5 | 401500 | 2001.02 | 400204 | 0 | - | - | - |
|  |  |  | Mid | 2010 | 402000 | 1640.64 | 328128 | 504 |  | - | - |
|  |  |  | High | 2012.5 | 402500 | 2001.7 | 400340 | 6 |  | - | - |
| 20 | 24 | Downlink | Low | 2190 | 438000 | 2181.36 | 436272 | 0 | 15 | - | 436632 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |
|  |  | Uplink | Low | 2010 | 402000 | 2001.36 | 400272 | 0 | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |
| Note 1: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). Note 2: 20/20 MHz UL/DL Bandwidth combination is tested with Low range test frequency only. Low range test frequency shall be used instead of Mid range and High range test frequencies. |  |  |  |  |  |  |  |  |  |  |  |

## <<< END OF CHANGES >>>