3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210510

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4606 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE test case 8.6.7.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI11_Test |  |  |  |  |  | Date: |  |  | 2021-04-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current TTCN implementation, the following assignment: v_MeasurementReport_vb50ext_IEs := cr_MeasurementReport_vb50ext_EUTRA_EventResults_EARFCN_ext(v_EUTRASysInfo.Eutra[0].Arfcn); is done at the beginning of the function f_TC_8_6_7_4_UTRAN. However, this should be done only for the case when Band > 64 as input parameter of cr_MeasurementReport_vb50ext_EUTRA_EventResults_EARFCN_ext is of type EARFCNExtension which according to ASN.1 definition should be within range ::= INTEGER (65536..262143), only achieved by Bands > 64. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Moved v_MeasurementReport_vb50ext_IEs assigment to statement only executed when Band > 64. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may FAIL the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.6.7.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | R5s210231 was raised earlier but this testcase got missed in that CR |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	3

1.1	Correction to function f_TC_8_6_7_4_UTRAN	3

# Corrections required

## Correction to function f_TC_8_6_7_4_UTRAN

| Function name | f_TC_8_6_7_4_UTRAN |
| --- | --- |
| Reason for change | In the current TTCN implementation, the following assignment: v_MeasurementReport_vb50ext_IEs := cr_MeasurementReport_vb50ext_EUTRA_EventResults_EARFCN_ext(v_EUTRASysInfo.Eutra[0].Arfcn); is done at the beginning of the function f_TC_8_6_7_4_UTRAN. However, this should be done only for the case when Band > 64 as input parameter of cr_MeasurementReport_vb50ext_EUTRA_EventResults_EARFCN_ext is of type EARFCNExtension which according to ASN.1 definition should be within range ::= INTEGER (65536..262143), only achieved by Bands > 64. |
| Summary of change | Moved v_MeasurementReport_vb50ext_IEs assigment to statement only executed when Band > 64. |
| TTCN module | LTE_A_IRAT/8_6/RRC_Connection_Establishment_Failure_UG_UTRAN.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> //Get Cell Parameters for EUTRA cell1 v_EUTRASysInfo := f_IRAT_WaitForCoOrd_SysInfo(EUTRA); v_MeasurementReport_vb50ext_IEs := cr_MeasurementReport_vb50ext_EUTRA_EventResults_EARFCN_ext(v_EUTRASysInfo.Eutra[0].Arfcn); //@sic R5s180225 sic@ if (v_EUTRASysInfo.Eutra[0].Arfcn < tsc_EUTRA_MaxEARFCN ) { // EUTRA Band < 64 @sic R5-173007 sic@ v_EUTRA_FrequencyAndPriorityInfoList[0] := cs_EUTRA_FrequencyAndPriorityInfo(v_EUTRASysInfo.Eutra[0].Arfcn, f_UTRAN_ConvertIntegerToEUTRA_MeasurementBandwidth(v_EUTRASysInfo.Eutra[0].DL_Bandwidth)); } else { // EUTRA Band > 64 @sic R5-174500, R5-173007 sic@ v_EUTRA_FrequencyAndPriorityInfoList_ext[0] := cs_EUTRA_FrequencyAndPriorityInfoExtension_Def(v_EUTRASysInfo.Eutra[0].Arfcn, f_UTRAN_ConvertIntegerToEUTRA_MeasurementBandwidth(v_EUTRASysInfo.Eutra[0].DL_Bandwidth)); } //Get Cell Parameters for EUTRA cell12 v_EUTRASysInfo := f_IRAT_WaitForCoOrd_SysInfo (EUTRA); if (v_EUTRASysInfo.Eutra[0].Arfcn < tsc_EUTRA_MaxEARFCN ) { // EUTRA Band < 64 @sic R5-173007 sic@ v_EUTRA_FrequencyAndPriorityInfoList[1] := cs_EUTRA_FrequencyAndPriorityInfo(v_EUTRASysInfo.Eutra[0].Arfcn, f_UTRAN_ConvertIntegerToEUTRA_MeasurementBandwidth(v_EUTRASysInfo.Eutra[0].DL_Bandwidth)); v_EUTRA_FrequencyInfoList_ext := cs_EUTRA_FrequencyInfoList_1Entry_ext(v_EUTRASysInfo.Eutra[0].Arfcn, omit, v_EUTRA_MeasurementBandwidth); //@sic R5s180248, R5s180225 sic@ v_MeasurementControl_InterRATMeasurement_NoCompMode := cs_MeasurementControl_InterRATMeasurement_NoCompMode(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth); //@sic R5s180248, R5s180225, R5s150833 sic@ v_MeasurementControl_InterRATMeasurement_TDD_r9 := cs_MeasurementControl_InterRATMeasurement_TDD_r9(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth, cs_CellDCHMeasOccasionInfo_TDD128_r9); //@sic R5s180248, R5s180225, R5s150833 sic@ v_MeasurementReport := cr_UTRAN_MeasurementReport_EUTRACell(v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRASysInfo.Eutra[0].HO.PhysCellId,?,?); //@sic R5s180248, R5s180225 sic@ } else { // EUTRA Band > 64 @sic R5-174500, R5-173007 sic@ v_EUTRA_FrequencyAndPriorityInfoList_ext[1] := cs_EUTRA_FrequencyAndPriorityInfoExtension_Def(v_EUTRASysInfo.Eutra[0].Arfcn, f_UTRAN_ConvertIntegerToEUTRA_MeasurementBandwidth(v_EUTRASysInfo.Eutra[0].DL_Bandwidth)); v_EUTRA_FrequencyInfoList_ext := cs_EUTRA_FrequencyInfoList_1Entry_ext(tsc_EUTRA_MaxEARFCN, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth); //@sic R5s180248, R5s180225 sic@ v_MeasurementControl_InterRATMeasurement_NoCompMode := cs_MeasurementControl_InterRATMeasurement_NoCompMode_BandExt(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRA_FrequencyInfoList_ext, v_EUTRA_MeasurementBandwidth); //@sic R5s180248, R5s180225, R5s150833 sic@ v_MeasurementControl_InterRATMeasurement_TDD_r9 := cs_MeasurementControl_InterRATMeasurement_TDD_r9_BandExt(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, tsc_EUTRA_MaxEARFCN, v_EUTRA_FrequencyInfoList_ext, v_EUTRA_MeasurementBandwidth, cs_CellDCHMeasOccasionInfo_TDD128_r9); //@sic R5s180248, R5s180225, R5s150833 sic@ v_MeasurementReport := cdr_UTRAN_MeasurementReport_EUTRACell_Band64(tsc_EUTRA_MaxEARFCN, v_EUTRASysInfo.Eutra[0].HO.PhysCellId,?,?, v_MeasurementReport_vb50ext_IEs); //@sic R5s180248, R5s180225 sic@ } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> //Get Cell Parameters for EUTRA cell1 v_EUTRASysInfo := f_IRAT_WaitForCoOrd_SysInfo(EUTRA); <<MOVED CODE>> if (v_EUTRASysInfo.Eutra[0].Arfcn < tsc_EUTRA_MaxEARFCN ) { // EUTRA Band < 64 @sic R5-173007 sic@ v_EUTRA_FrequencyAndPriorityInfoList[0] := cs_EUTRA_FrequencyAndPriorityInfo(v_EUTRASysInfo.Eutra[0].Arfcn, f_UTRAN_ConvertIntegerToEUTRA_MeasurementBandwidth(v_EUTRASysInfo.Eutra[0].DL_Bandwidth)); } else { // EUTRA Band > 64 @sic R5-174500, R5-173007 sic@ v_EUTRA_FrequencyAndPriorityInfoList_ext[0] := cs_EUTRA_FrequencyAndPriorityInfoExtension_Def(v_EUTRASysInfo.Eutra[0].Arfcn, f_UTRAN_ConvertIntegerToEUTRA_MeasurementBandwidth(v_EUTRASysInfo.Eutra[0].DL_Bandwidth)); } //Get Cell Parameters for EUTRA cell12 v_EUTRASysInfo := f_IRAT_WaitForCoOrd_SysInfo (EUTRA); if (v_EUTRASysInfo.Eutra[0].Arfcn < tsc_EUTRA_MaxEARFCN ) { // EUTRA Band < 64 @sic R5-173007 sic@ v_EUTRA_FrequencyAndPriorityInfoList[1] := cs_EUTRA_FrequencyAndPriorityInfo(v_EUTRASysInfo.Eutra[0].Arfcn, f_UTRAN_ConvertIntegerToEUTRA_MeasurementBandwidth(v_EUTRASysInfo.Eutra[0].DL_Bandwidth)); v_EUTRA_FrequencyInfoList_ext := cs_EUTRA_FrequencyInfoList_1Entry_ext(v_EUTRASysInfo.Eutra[0].Arfcn, omit, v_EUTRA_MeasurementBandwidth); //@sic R5s180248, R5s180225 sic@ v_MeasurementControl_InterRATMeasurement_NoCompMode := cs_MeasurementControl_InterRATMeasurement_NoCompMode(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth); //@sic R5s180248, R5s180225, R5s150833 sic@ v_MeasurementControl_InterRATMeasurement_TDD_r9 := cs_MeasurementControl_InterRATMeasurement_TDD_r9(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth, cs_CellDCHMeasOccasionInfo_TDD128_r9); //@sic R5s180248, R5s180225, R5s150833 sic@ v_MeasurementReport := cr_UTRAN_MeasurementReport_EUTRACell(v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRASysInfo.Eutra[0].HO.PhysCellId,?,?); //@sic R5s180248, R5s180225 sic@ } else { // EUTRA Band > 64 @sic R5-174500, R5-173007 sic@ v_EUTRA_FrequencyAndPriorityInfoList_ext[1] := cs_EUTRA_FrequencyAndPriorityInfoExtension_Def(v_EUTRASysInfo.Eutra[0].Arfcn, f_UTRAN_ConvertIntegerToEUTRA_MeasurementBandwidth(v_EUTRASysInfo.Eutra[0].DL_Bandwidth)); v_EUTRA_FrequencyInfoList_ext := cs_EUTRA_FrequencyInfoList_1Entry_ext(tsc_EUTRA_MaxEARFCN, v_EUTRASysInfo.Eutra[0].Arfcn, v_EUTRA_MeasurementBandwidth); //@sic R5s180248, R5s180225 sic@ v_MeasurementControl_InterRATMeasurement_NoCompMode := cs_MeasurementControl_InterRATMeasurement_NoCompMode_BandExt(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, v_EUTRA_FrequencyInfoList_ext, v_EUTRA_MeasurementBandwidth); //@sic R5s180248, R5s180225, R5s150833 sic@ v_MeasurementControl_InterRATMeasurement_TDD_r9 := cs_MeasurementControl_InterRATMeasurement_TDD_r9_BandExt(v_SecurityInfo.dl_IntegrityCheckInfo, v_RRC_TI, tsc_EUTRA_MaxEARFCN, v_EUTRA_FrequencyInfoList_ext, v_EUTRA_MeasurementBandwidth, cs_CellDCHMeasOccasionInfo_TDD128_r9); //@sic R5s180248, R5s180225, R5s150833 sic@ v_MeasurementReport_vb50ext_IEs := cr_MeasurementReport_vb50ext_EUTRA_EventResults_EARFCN_ext(v_EUTRASysInfo.Eutra[0].Arfcn); //@sic R5s180225 sic@ //8.6.7.4 v_MeasurementReport := cdr_UTRAN_MeasurementReport_EUTRACell_Band64(tsc_EUTRA_MaxEARFCN, v_EUTRASysInfo.Eutra[0].HO.PhysCellId,?,?, v_MeasurementReport_vb50ext_IEs); //@sic R5s180248, R5s180225 sic@ } <<SKIPPED CODE>>