3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221182

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.123-3 | CR | 3665 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-17 Sep'22 baseline upgrade for UTRA Test Suites |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test |  |  |  |  |  | Date: |  |  | 2022-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In order to develop the Rel-17 test cases in TTCN-3, the current baseline shall be upgraded. RAN5 agreed to move baseline to September 2022 in Rel-17 to all RAN5 TTCN-3 test suites. This CR is implementing this decision on 34.123-3 Test Suites. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CR lists all changes affecting the messages and IEs type definitions caused by the baseline moving to the September 2022 Rel-17. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-17 test cases cannot be implemented and verified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All 34.123-3 TTCN-3 ATSs |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

Overview	2

1	Baseline Moving – 25.331 ASN.1	2

1.1	Type and field names change	3

2	Baseline Moving – 25.321	3

3	Baseline Moving – 25.322	3

4	Baseline Moving – 24.008	4

4.1	Modified Type Definitions	4

5	Baseline Moving – 24.080	4

5.1	Modified Type Definitions	4

6	Baseline Moving – 29.002	4

7	Baseline Moving – 44.018	5

8	Baseline Moving – 44.060	5

9	Other changes	5

# Overview

This document lists all the changes needed for baseline moving based on iwd-TTCN3-B2020-09_D22wk37.

Contact:	Olivier Genoud
	olivier.genoud@etsi.org

# Baseline Moving – 25.331 ASN.1

The ASN.1 module is upgraded to 25.331-h10.

## Type and field names change

| ASN.1 type affected | Renaming | Impact on TTCN |
| --- | --- | --- |
| EUTRA-FrequencyAndPriorityInfo | Field eutra-blackListedCellList renamed to eutra-exclude-ListedCellList | cs_EUTRA_FrequencyAndPriorityInfo cs_Eutra_FreqAndPriList_OneEntry_Common |
| EUTRA-FrequencyAndPriorityInfoExtension-vb50ext | Field eutra-blackListedCellList renamed to eutra-exclude-listedCellList | cs_EUTRA_FrequencyAndPriorityInfoExtension_Common |
| EUTRA-FrequencyInfo | eutra-blackListedCellList renamed to eutra-exclude-listedCellList | cs_EUTRA_FrequencyInfo |
| EUTRA-FrequencyInfo-r11 | eutra-blackListedCellList renamed to eutra-exclude-listedCellList | cs_EUTRA_FrequencyInfo_r11 cs_EUTRA_FrequencyInfo_ext |
| EUTRA-FrequencyInfo-r12 | eutra-blackListedCellList renamed to eutra-exclude-listedCellList | No impact |
| InterFreqBlacklistedCellsList-r12 | Type renamed to InterFreqExclude-listedCellsList-r12 | No impact |
| InterFreqBlacklistedCells-r12 | Type renamed to InterFreqExclude-listedCells-r12 | No impact |
| InterFreqReportingCriteria-r12 | Field interFreqBlacklistedCellsList renamed to interFreqExclude-listedCellsList | No impact |
| EUTRA-BlacklistedCell | Type renamed to EUTRA-Exclude-listedCell | No impact |
| EUTRA-BlacklistedCellPerFreqList | Type renamed to EUTRA-Exclude-listedCellPerFreqList | cs_Eutra_FreqAndPriList_OneEntry_Common cs_EUTRA_FrequencyAndPriorityInfoExtension_Common cs_EUTRA_FrequencyInfo_r11 |
| EUTRA-TargetFreqInfo | Field eutraBlacklistedCellPerFreqList renamed to eutraExclude-listedCellPerFreqList | cs_EUTRA_TargetFreqInfo cs_RRCConnectionReject_NonCriticalExtensions_EARFCN |

# Baseline Moving – 25.321

25.321 is upgraded to 25321-h00.

No change is required in TTCN.

# Baseline Moving – 25.322

25.322 is upgraded to 25322-h00.

No change is required in TTCN.

# Baseline Moving – 24.008

24.008 is upgraded to 24008-h80.

## Modified Type Definitions

| IE Definition | Changes Required | Ref: 3GPP 24.008 clause |
| --- | --- | --- |
| Extended DRX Parameters | New optional field: Extended Paging Time Window | 10.5.5.32 |

# Baseline Moving – 24.080

24.080 is upgraded to 24080-h30.

## Modified Type Definitions

| ASN.1 type affected | Change Required | Ref: 3GPP 24.080 clause |
| --- | --- | --- |
| LCS-MOLRArg | Additional field: scheduledLocTime | 4.4.2 |
| LCS-MOLRRes | Additional field: timestampOfLocationEstimate | 4.4.2 |
| DateTime | Changed size: from 8 octets to 30 octets | 4.4.2 |

# Baseline Moving – 29.002

29.002 is upgraded to 29002-h20.

No change is required in TTCN.

# Baseline Moving – 44.018

44.018 is upgraded to 44018-h00.

No change is required in TTCN.

# Baseline Moving – 44.060

44.060 is upgraded to 44060-h00.

No change is required in TTCN.

# Other changes

The following duplicated templates are resolved:

- cs_EUTRA_FrequencyInfo_r11 and cs_EUTRA_FrequencyInfo_ext:  cs_EUTRA_FrequencyInfo_ext is deleted

- cs_EUTRA_FrequencyInfoList_r11_1Entry and cs_EUTRA_FrequencyInfoList_1Entry_ext: cs_EUTRA_FrequencyInfoList_1Entry_ext is deleted