3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221216

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4696 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE testcase 9.2.3.1.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2022-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | At Step 8, UE moves from Cell B (PLMN1) to Cell C (PLMN2) and at Step 11, SS transmits TAU Accept message including PLMN1 in the list of equivalent PLMNs. Hence UE would not perform IMS reregistration post Step 12A and Step 12Aa1 will be redundant. Whereas at step 16, UE moves from Cell C (PLMN2) to Cell D (PLMN3) and at Step 19, SS transmits TAU Accept message without the list of equivalent PLMNs. Hence Cell D would be considered as VPLMN and UE would like to perform IMS reregistration post Step 20A. |  |  |  |  |  |  |  |  |
| Summary of change: |  | IMS Reregistration procedure moved from Step12Aa1 to Step20Aa1 (new step after Step20A) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 36.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## 2.1 Change 1

| Function name | fl_TC_9_2_3_1_4_Body |
| --- | --- |
| Reason for change | At Step 8, UE moves from Cell B (PLMN1) to Cell C (PLMN2) and at Step 11, SS transmits TAU Accept message including PLMN1 in the list of equivalent PLMNs. Hence UE would not perform IMS reregistration post Step 12A and Step 12Aa1 would not be required. Whereas at step 16, UE moves from Cell C (PLMN2) to Cell D (PLMN3) and at Step 19, SS transmits TAU Accept message without the list of equivalent PLMNs. Hence Cell D would be considered as VPLMN and UE would like to perform IMS reregistration post Step 20A. |
| Summary of change | IMS Reregistration procedure moved from Step12Aa1 to Step20Aa1 (new step after Step20A) |
| TTCN module | LTE\9_2\NAS_TrackingArea_EOnly.ttcn |
| MCC160 Comment |  |

Before change:

function fl_TC_9_2_3_1_4_Body() runs on EUTRA_PTC

{

//<=== SKIPPED ===>

//@siclog "Step 11" siclog@

//+   The SS transmits a TRACKING AREA UPDATE ACCEPT message including PLMN1

//+   and PLMN3 in the list of equivalent PLMNs. <-- TRACKING AREA UPDATE

//+   ACCEPT

SRB.send(cas_SRB1_NasPdu_REQ(eutra_CellC,  // @sic R5-115627 sic@

cs_TimingInfo_Now,

cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered,

cs_TAU_Accept_EPLMN(v_UpdateType,

f_GutiParameters2MobileIdentity(tsc_IEI_Guti, v_Guti3_Params),

cds_TAIListNonConsecutive_tlv(f_Asn2Nas_PlmnId(v_Guti3_Params.PLMN_Identity),

{ bit2oct(f_EUTRA_CellInfo_GetTAC(eutra_CellC)) }),  // @sic R5-115627 sic@

v_EpsBearerCtxtStatus,

v_LAI,

v_MsId,

f_GetAdditionalUpdateResult(v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.addUpdateType), // @sic R5-103681 sic@,

cs_PLMN_List_2PLMNs(tsc_PLMN1, tsc_PLMN3)))));

//@siclog "Step 12" siclog@

//+   Check: Does the UE transmit a TRACKING AREA UPDATE COMPLETE message?

//+   --> TRACKING AREA UPDATE COMPLETE [1,P]

SRB.receive(car_SRB1_NasPdu_IND(eutra_CellC,  // @sic R5-115627 sic@

cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered,

cr_508_TAU_Complete))); // @sic R5-130644 sic@

//@siclog "Step 12A" siclog@

//+  SS releases rrc connection <-- RRC_CONNECTION_RELEASE

f_EUTRA_RRC_ConnectionRelease(eutra_CellC); // @sic R5-113810 sic@  @sic R5-115627 sic@

//@sic "Step 12Aa1" sic@

//The UE may attempt the generic test procedure for IMS Re-registration on EPS VPLMN as specified in TS 36.508 [18] subclause 4.5A.30.

f_EUTRA_WaitForIMSReRegister(eutra_CellC); //@sic R5-204345 sic@

//@siclog "Step 13" siclog@

//+   Set the cell type of cell C to the "Non-Suitable cell"

//+   Set the cell type of cell D to the "Serving cell"

//+ Note: Cell D (PLMN3) belongs to the forbidden PLMN.

v_CellPowerList := {

cs_CellPower ( eutra_CellC, tsc_NonSuitableCellRS_EPRE ),

cs_CellPower ( eutra_CellD, tsc_ServingCellRS_EPRE )  // @sic R5-130644 sic@

};

f_EUTRA_SetCellPowerList ( v_CellPowerList );  // @sic R5-115627 sic@

//@siclog "Step 14" siclog@

//+   Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message on

//+   cell D (PLMN3) in next 30 seconds?  --> TRACKING AREA UPDATE REQUEST

//+   [1,F]

v_TimerValue := f_EUTRA_SetTimerToleranceMax(eutra_CellD, nasTTCNTimer, 30.0);  // @sic R5-106675 sic@ @sic R5-115627 sic@

if (f_EUTRA_RRC_RRCConnectionRequest_Check(eutra_CellD, v_TimerValue)) {        // @sic R5s110507 Additional Changes sic@ @sic R5s120463 sic@

// even though prose says TAU, as soon as UE sends RRC ConnReq, we know something is wrong

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 14");

}

//@siclog "Step 15" siclog@

//+   The UE is switched to manual PLMN selection mode and is made to select

//+   PLMN3 in order to remove PLMN3 in the forbidden PLMN list in the UE.

f_UT_ManualPLMN_Select (UT, f_Asn2Nas_PlmnId(v_Guti4_Params.PLMN_Identity)); // @sic R5-106675, R5s100875 sic@

//@siclog "Step 16" siclog@

//+   The UE transmit a TRACKING AREA UPDATE REQUEST message on cell D

//+   (PLMN3)? --> TRACKING AREA UPDATE REQUEST

v_NasInd := f_EUTRA_RRC_ConnEst_DefWithNas(eutra_CellD,  // @sic R5-115627 sic@

tsc_RRC_TI_Def,

cr_EstablishmentCause_Any,

cr_NAS_Indication(tsc_SHT_IntegrityProtected,

cr_508_TAU_Request(v_UpdateType,

f_EUTRA_SecurityKSIasme_Get(),

cr_DRXparameter_Any ('5C'O) ifpresent, // @sic R5s120210, R5-123117 sic@

v_AdditionalUpdateType)));

v_EpsBearerCtxtStatus := v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.epsBearerContextStatus;

//@siclog "Step 17" siclog@

//+   VOID

//@siclog "Step 18" siclog@

//+   VOID

//@siclog "Step 19" siclog@

//+   The SS transmits a TRACKING AREA UPDATE ACCEPT message without the list

//+   of equivalent PLMNs.  <-- TRACKING AREA UPDATE ACCEPT

SRB.send(cas_SRB1_NasPdu_REQ(eutra_CellD,  // @sic R5-115627 sic@

cs_TimingInfo_Now,

cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered,

cs_508_TAU_Accept(v_UpdateType,

f_GutiParameters2MobileIdentity(tsc_IEI_Guti, v_Guti4_Params),

cds_TAIListNonConsecutive_tlv(f_Asn2Nas_PlmnId(v_Guti4_Params.PLMN_Identity),

{ bit2oct(f_EUTRA_CellInfo_GetTAC(eutra_CellD)) }),  // @sic R5-115627 sic@

v_EpsBearerCtxtStatus,

f_EUTRA_GetCellLAI(eutra_CellI, NORMAL), // @sic R5s110176 sic@

omit,

f_GetAdditionalUpdateResult(v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.addUpdateType)) ) ) );

//@siclog "Step 20" siclog@

//+   Check: Does the UE transmit a TRACKING AREA UPDATE COMPLETE message?

//+   --> TRACKING AREA UPDATE COMPLETE [1,P]

SRB.receive (car_SRB1_NasPdu_IND(eutra_CellD, // @sic R5-115627 sic@

cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered,

cr_508_TAU_Complete)));

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 20");

//@siclog "Step 20A" siclog@

//+  SS releases rrc connection <-- RRC_CONNECTION_RELEASE

f_EUTRA_RRC_ConnectionRelease(eutra_CellD);  // @sic R5-115627 sic@

//@siclog "Step 21" siclog@

//+   The UE is switched back to automatic PLMN selection mode.

f_UT_AutomaticPLMN_Select(UT);

//<=== SKIPPED ===>

} // function tc_9_2_3_1_4_EUTRA

After change:

function fl_TC_9_2_3_1_4_Body() runs on EUTRA_PTC

{

//<=== SKIPPED ===>

//@siclog "Step 11" siclog@

//+   The SS transmits a TRACKING AREA UPDATE ACCEPT message including PLMN1

//+   and PLMN3 in the list of equivalent PLMNs. <-- TRACKING AREA UPDATE

//+   ACCEPT

SRB.send(cas_SRB1_NasPdu_REQ(eutra_CellC,  // @sic R5-115627 sic@

cs_TimingInfo_Now,

cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered,

cs_TAU_Accept_EPLMN(v_UpdateType,

f_GutiParameters2MobileIdentity(tsc_IEI_Guti, v_Guti3_Params),

cds_TAIListNonConsecutive_tlv(f_Asn2Nas_PlmnId(v_Guti3_Params.PLMN_Identity),

{ bit2oct(f_EUTRA_CellInfo_GetTAC(eutra_CellC)) }),  // @sic R5-115627 sic@

v_EpsBearerCtxtStatus,

v_LAI,

v_MsId,

f_GetAdditionalUpdateResult(v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.addUpdateType), // @sic R5-103681 sic@,

cs_PLMN_List_2PLMNs(tsc_PLMN1, tsc_PLMN3)))));

//@siclog "Step 12" siclog@

//+   Check: Does the UE transmit a TRACKING AREA UPDATE COMPLETE message?

//+   --> TRACKING AREA UPDATE COMPLETE [1,P]

SRB.receive(car_SRB1_NasPdu_IND(eutra_CellC,  // @sic R5-115627 sic@

cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered,

cr_508_TAU_Complete))); // @sic R5-130644 sic@

//@siclog "Step 12A" siclog@

//+  SS releases rrc connection <-- RRC_CONNECTION_RELEASE

f_EUTRA_RRC_ConnectionRelease(eutra_CellC); // @sic R5-113810 sic@  @sic R5-115627 sic@

//@siclog "Step 13" siclog@

//+   Set the cell type of cell C to the "Non-Suitable cell"

//+   Set the cell type of cell D to the "Serving cell"

//+ Note: Cell D (PLMN3) belongs to the forbidden PLMN.

v_CellPowerList := {

cs_CellPower ( eutra_CellC, tsc_NonSuitableCellRS_EPRE ),

cs_CellPower ( eutra_CellD, tsc_ServingCellRS_EPRE )  // @sic R5-130644 sic@

};

f_EUTRA_SetCellPowerList ( v_CellPowerList );  // @sic R5-115627 sic@

//@siclog "Step 14" siclog@

//+   Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message on

//+   cell D (PLMN3) in next 30 seconds?  --> TRACKING AREA UPDATE REQUEST

//+   [1,F]

v_TimerValue := f_EUTRA_SetTimerToleranceMax(eutra_CellD, nasTTCNTimer, 30.0);  // @sic R5-106675 sic@ @sic R5-115627 sic@

if (f_EUTRA_RRC_RRCConnectionRequest_Check(eutra_CellD, v_TimerValue)) {        // @sic R5s110507 Additional Changes sic@ @sic R5s120463 sic@

// even though prose says TAU, as soon as UE sends RRC ConnReq, we know something is wrong

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 14");

}

//@siclog "Step 15" siclog@

//+   The UE is switched to manual PLMN selection mode and is made to select

//+   PLMN3 in order to remove PLMN3 in the forbidden PLMN list in the UE.

f_UT_ManualPLMN_Select (UT, f_Asn2Nas_PlmnId(v_Guti4_Params.PLMN_Identity)); // @sic R5-106675, R5s100875 sic@

//@siclog "Step 16" siclog@

//+   The UE transmit a TRACKING AREA UPDATE REQUEST message on cell D

//+   (PLMN3)? --> TRACKING AREA UPDATE REQUEST

v_NasInd := f_EUTRA_RRC_ConnEst_DefWithNas(eutra_CellD,  // @sic R5-115627 sic@

tsc_RRC_TI_Def,

cr_EstablishmentCause_Any,

cr_NAS_Indication(tsc_SHT_IntegrityProtected,

cr_508_TAU_Request(v_UpdateType,

f_EUTRA_SecurityKSIasme_Get(),

cr_DRXparameter_Any ('5C'O) ifpresent, // @sic R5s120210, R5-123117 sic@

v_AdditionalUpdateType)));

v_EpsBearerCtxtStatus := v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.epsBearerContextStatus;

//@siclog "Step 17" siclog@

//+   VOID

//@siclog "Step 18" siclog@

//+   VOID

//@siclog "Step 19" siclog@

//+   The SS transmits a TRACKING AREA UPDATE ACCEPT message without the list

//+   of equivalent PLMNs.  <-- TRACKING AREA UPDATE ACCEPT

SRB.send(cas_SRB1_NasPdu_REQ(eutra_CellD,  // @sic R5-115627 sic@

cs_TimingInfo_Now,

cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered,

cs_508_TAU_Accept(v_UpdateType,

f_GutiParameters2MobileIdentity(tsc_IEI_Guti, v_Guti4_Params),

cds_TAIListNonConsecutive_tlv(f_Asn2Nas_PlmnId(v_Guti4_Params.PLMN_Identity),

{ bit2oct(f_EUTRA_CellInfo_GetTAC(eutra_CellD)) }),  // @sic R5-115627 sic@

v_EpsBearerCtxtStatus,

f_EUTRA_GetCellLAI(eutra_CellI, NORMAL), // @sic R5s110176 sic@

omit,

f_GetAdditionalUpdateResult(v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.addUpdateType)) ) ) );

//@siclog "Step 20" siclog@

//+   Check: Does the UE transmit a TRACKING AREA UPDATE COMPLETE message?

//+   --> TRACKING AREA UPDATE COMPLETE [1,P]

SRB.receive (car_SRB1_NasPdu_IND(eutra_CellD, // @sic R5-115627 sic@

cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered,

cr_508_TAU_Complete)));

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 20");

//@siclog "Step 20A" siclog@

//+  SS releases rrc connection <-- RRC_CONNECTION_RELEASE

f_EUTRA_RRC_ConnectionRelease(eutra_CellD);  // @sic R5-115627 sic@

//@sic "Step 20Aa1" sic@

//The UE may attempt the generic test procedure for IMS Re-registration on EPS VPLMN as specified in TS 36.508 [18] subclause 4.5A.30.

f_EUTRA_WaitForIMSReRegister(eutra_CellD); //@sic R5-204345 sic@

//@siclog "Step 21" siclog@

//+   The UE is switched back to automatic PLMN selection mode.

f_UT_AutomaticPLMN_Select(UT);

//<=== SKIPPED ===>

} // function tc_9_2_3_1_4_EUTRA