3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221312

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2744 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to bwp_SwitchingDelay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-09-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to 38306 bwp-SwitchingDelay Defines whether the UE supports DCI and timer based active BWP switching delay type1 or type2 specified in clause 8.6.2 of TS 38.133 [5]. It is mandatory to report type 1 or type 2 when bwp-SameNumerology or bwp-DiffNumerology is supported on at least one band. This capability is not applicable to IAB-MT bwp-SameNumerology / bwp-DiffNumerology are not mandatory ones as per 38306 so Following statement :: bwp_SwitchingDelay := f_NR_Derive_Phy_ParametersCommon_bwp_SwitchingDelay(), might result in set verdict if (not (pc_bwp_SwitchingDelay_Type1 xor pc_bwp_SwitchingDelay_Type2)){ f_SetVerdict(fail, __FILE__, __LINE__, "Acc. to 38.306, 4.2.7.10, It is mandatory to report type 1 or type 2"); } for the UEs which are not reporting bwp-SwitchingDelay Also 38306 says , reporting bwp-SwitchingDelay as “ conditional yes “ / not mandatory Yes |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to have any or omit for bwp-SwitchingDelay |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All NR5GC and ENDC testcases |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Correction to the template cr_Phy_ParametersCommon	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to the template cr_Phy_ParametersCommon

| Template name | cr_Phy_ParametersCommon |
| --- | --- |
| Reason for change | According to 38306 bwp-SwitchingDelay Defines whether the UE supports DCI and timer based active BWP switching delay type1 or type2 specified in clause 8.6.2 of TS 38.133 [5]. It is mandatory to report type 1 or type 2 when bwp-SameNumerology or bwp-DiffNumerology is supported on at least one band. This capability is not applicable to IAB-MT bwp-SameNumerology / bwp-DiffNumerology are not mandatory ones as per 38306 so Following statement :: bwp_SwitchingDelay := f_NR_Derive_Phy_ParametersCommon_bwp_SwitchingDelay(), might result in set verdict if (not (pc_bwp_SwitchingDelay_Type1 xor pc_bwp_SwitchingDelay_Type2)){ f_SetVerdict(fail, __FILE__, __LINE__, "Acc. to 38.306, 4.2.7.10, It is mandatory to report type 1 or type 2"); } for the UEs which are not reporting bwp-SwitchingDelay Also 38306 says , reporting bwp-SwitchingDelay as “ conditional yes “ / not mandatory Yes |
| Summary of change | It is proposed to have any or omit for bwp-SwitchingDelay |
| TTCN module | NR_AuxiliaryCapCheckFunctions.ttcn |
| MCC160 Comment |  |

Before Change:

template (present) Phy_ParametersCommon cr_Phy_ParametersCommon := { /* @status APPROVED (ENDC, NR5GC) */ << SKIPPED CODE >> bwp_SwitchingDelay := f_NR_Derive_Phy_ParametersCommon_bwp_SwitchingDelay(), //@sic R5-225384 sic@ dummy := *, << SKIPPED CODE >> };

After Change:

template (present) Phy_ParametersCommon cr_Phy_ParametersCommon := { /* @status APPROVED (ENDC, NR5GC) */ << SKIPPED CODE >> bwp_SwitchingDelay := *, /*f_NR_Derive_Phy_ParametersCommon_bwp_SwitchingDelay(),*/ //@sic R5-225384 sic@ //WA#WI=1007394 dummy := *, << SKIPPED CODE >> };