3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230761

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4767 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on selection expressions for test cases with options FDD/TDD in EUTRA and NB-IOT Test Suites |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2023-10-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Inconsistencies between TTCN and prose regarding test case selection expressions in EUTRA and NB-IOT Test Suites remain. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction of conditions C33F_C33T, C238 and C254d in the selection expressions of several EUTRA ATSs. In NB-IOT corrected C349_C403 and created function f_NBIOT_FrequencyBandIsFDD. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent conditions in the selection expressions for several EUTRA and NB-IOT test cases will remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | LTE, LTE_A_PRO, LTE_A_R12 and NB-IoT Test Suites |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

Overview	2

1	List of changes	3

1.1	On EUTRA Test Suites	3

1.2	On NBIOT Test Suite	5

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk37 related to the title of this CR.

Contact:	Olivier Genoud
	olivier.genoud@etsi.org

# 1	List of changes

## 1.1	On EUTRA Test Suites

- in module LTE_36523_SelectionExpressions.ttcn

Before Change:

.. } case (C32F_C32T) { if (v_FrequencyBandIsFDD) { v_ApplCond := pc_eFDD and pc_FeatrGrp_7_F and pc_FeatrGrp_20_F; } else { v_ApplCond := pc_eTDD and pc_FeatrGrp_7_T and pc_FeatrGrp_20_T; } } case (C33F_C33T) { if (v_FrequencyBandIsFDD) { v_ApplCond := pc_eFDD and pc_FeatrGrp_20_F; } else {//@sic R5-168641 sic@ } } case (C34) { v_ApplCond := (pc_eFDD or pc_eTDD) and pc_UTRA and pc_GERAN; } case (C35) { v_ApplCond := (pc_eFDD or pc_eTDD) and pc_UTRA; } case (C36F_C36T) { if (v_FrequencyBandIsFDD) { v_ApplCond := pc_eFDD and pc_UTRA and pc_FeatrGrp_8_F and pc_FeatrGrp_22_F and not pc_ue_CategoryDL_M1; } else { v_ApplCond := pc_eTDD and pc_UTRA and pc_FeatrGrp_8_T and pc_FeatrGrp_22_T and not pc_ue_CategoryDL_M1; } } .. case(C237){ v_ApplCond := (pc_eFDD or pc_eTDD) and pc_MultimediaTelephonyService and pc_InitiateSession and pc_MTSI_Speech and pc_MTSI_Video and not pc_ue_CategoryDL_M1; } case(C238){ v_ApplCond := pc_eFDD and pc_commSupportedBands; } case(C240){ v_ApplCond := (pc_eFDD or pc_eTDD) and pc_disc_public_safety; } .. case(C254c) { v_ApplCond := (pc_eFDD or pc_eTDD) and pc_CEmodeA and pc_IntraFreqA3_CE_ModeA and pc_IntraFreqHO_CE_ModeA; } case (C254d) { v_ApplCond := (pc_eFDD or pc_eTDD) and pc_CEmodeA and pc_IntraFreqA3_CE_ModeA and (not pc_ue_CategoryDL_M2); } case(C255){ v_ApplCond := (pc_eFDD or pc_eTDD) and pc_CEmodeB; } ..

After Change:

.. case (C32F_C32T) { if (v_FrequencyBandIsFDD) { v_ApplCond := pc_eFDD and pc_FeatrGrp_7_F and pc_FeatrGrp_20_F; } else { v_ApplCond := pc_eTDD and pc_FeatrGrp_7_T and pc_FeatrGrp_20_T; } } case (C33F_C33T) { if (v_FrequencyBandIsFDD) { v_ApplCond := pc_eFDD and pc_FeatrGrp_20_F; } else { v_ApplCond := pc_eTDD and pc_FeatrGrp_20_T; } } case (C36F_C36T) { if (v_FrequencyBandIsFDD) { v_ApplCond := pc_eFDD and pc_UTRA and pc_FeatrGrp_8_F and pc_FeatrGrp_22_F and not pc_ue_CategoryDL_M1; } else { v_ApplCond := pc_eTDD and pc_UTRA and pc_FeatrGrp_8_T and pc_FeatrGrp_22_T and not pc_ue_CategoryDL_M1; } } .. case (C237) { v_ApplCond := (pc_eFDD or pc_eTDD) and pc_MultimediaTelephonyService and pc_InitiateSession and pc_MTSI_Speech and pc_MTSI_Video and not pc_ue_CategoryDL_M1; } case (C238) { v_ApplCond := (pc_eFDD or pc_eTDD) and pc_commSupportedBands; } case (C240) { v_ApplCond := (pc_eFDD or pc_eTDD) and pc_disc_public_safety; } .. case (C254c) { v_ApplCond := (pc_eFDD or pc_eTDD) and pc_CEmodeA and pc_IntraFreqA3_CE_ModeA and pc_IntraFreqHO_CE_ModeA; } case (C254d) { v_ApplCond := (pc_eFDD or pc_eTDD) and pc_CEmodeA and not pc_ue_CategoryDL_M2; } case (C255) { v_ApplCond := (pc_eFDD or pc_eTDD) and pc_CEmodeB; } ..

## 1.2	On NBIOT Test Suite

- in module NBIOT_36523_SelectionExpressions.ttcn

Before Change:

function f_NBIOT_SelectionExpr(NBIOT_36523_Condition_Type p_Condition) return boolean { var boolean v_ApplCond; select (p_Condition) { case(C266){ v_ApplCond := pc_NB_FDD or pc_NB_TDD; } case(C266a){ v_ApplCond := (pc_NB_FDD or pc_NB_TDD) and pc_PLMN_Manual_Mode_Exception; } case(C271){ v_ApplCond := (pc_NB_FDD or pc_NB_TDD) and pc_NB_User_Plane_CIoT_Optimisation; } case(C272){ v_ApplCond := (pc_NB_FDD or pc_NB_TDD) and pc_ZUC; } case(C273){ v_ApplCond := (pc_NB_FDD or pc_NB_TDD) and pc_edrx; } case(C275){ v_ApplCond := (pc_NB_FDD or pc_NB_TDD) and pc_LAP and pc_LAP_override; } case(C277){ v_ApplCond := (pc_NB_FDD or pc_NB_TDD) and pc_Multiple_PDN and pc_LAP and pc_LAP_override; } case(C288){ v_ApplCond := (pc_NB_FDD or pc_NB_TDD) and pc_NB_MultiCarrier; } case(C290) { v_ApplCond := (pc_NB_FDD or pc_NB_TDD) and (pc_User_Plane_CIoT_Optimisation or pc_S1_U_DataTransfer); } case(C291) { v_ApplCond := (pc_NB_FDD or pc_NB_TDD) and (pc_User_Plane_CIoT_Optimisation or pc_S1_U_DataTransfer) and pc_ZUC; } case(C293){ v_ApplCond := (pc_NB_FDD or pc_NB_TDD) and pc_IP_PDN and pc_ESM_MO_Bearer_Modification; } case(C322){ v_ApplCond := (pc_NB_FDD or pc_NB_TDD) and pc_RRC_re_establishment_CP_CIoT; } case(C323){ v_ApplCond := (pc_eFDD or pc_eTDD) and (pc_NB_FDD or pc_NB_TDD); } case(C339){ v_ApplCond := (pc_NB_FDD or pc_NB_TDD) and pc_NB_TwoHARQ_Processes and pc_ue_Category_NB2; } case(C342) { v_ApplCond := (pc_NB_FDD or pc_NB_TDD) and pc_APN_RateControl and pc_Additional_APN_RateControl; } case(C347) { v_ApplCond := (pc_NB_FDD or pc_NB_TDD) and pc_ue_Category_NB2; } case(C348) { v_ApplCond := (pc_NB_FDD or pc_NB_TDD) and pc_NB_MultiCarrier_NPRACH; } case(C349_C403) { v_ApplCond := pc_NB_FDD and pc_NB_MultiCarrier_Paging and pc_NB_TDD and pc_NB_MultiCarrier_Paging_TDD; } case(C349) { v_ApplCond := pc_NB_FDD and pc_NB_MultiCarrier_Paging; } case(C350) { v_ApplCond := pc_NB_FDD and pc_SCPTM_IDLE; } case(C351){ v_ApplCond := pc_NB_FDD and pc_SCPTM and (pc_FeatrGrp_3_F or pc_FeatrGrp_3_T) and (pc_FeatrGrp_7_F or pc_FeatrGrp_7_T); } case(C377){ v_ApplCond := (pc_NB_FDD or pc_NB_TDD) and pc_NB_RLC_UM and (pc_User_Plane_CIoT_Optimisation or pc_S1_U_DataTransfer); } case(C390){ v_ApplCond := pc_NB_FDD and pc_wakeUpSignal; } case(C391){ v_ApplCond := pc_NB_FDD and pc_edrx and pc_wakeUpSignal; } case(C392){ v_ApplCond := pc_NB_FDD and pc_SR_WithoutHARQ_ACK; } case(C396){ v_ApplCond := (pc_NB_FDD or pc_NB_TDD) and pc_NB_User_Plane_CIoT_Optimisation and (pc_ROHC_profile0x0002 or pc_ROHC_profile0x0003 or pc_ROHC_profile0x0004 or pc_ROHC_profile0x0006 or pc_ROHC_profile0x0102 or pc_ROHC_profile0x0103 or pc_ROHC_profile0x0104); } case(C400){ v_ApplCond := pc_NB_FDD and pc_NB_mixedOperationMode; } case(C402){ v_ApplCond := pc_NB_FDD and pc_NB_nprach_Format2; } case(C403){ v_ApplCond := pc_NB_TDD and pc_NB_MultiCarrier_Paging_TDD; } case(C412) { v_ApplCond := pc_NB_FDD and (pc_NB_ntn_Connectivity_EPC or pc_NB_ntn_only_Connectivity_EPC) and (pc_NB_ntn_GSO_ScenarioSupport or pc_NB_ntn_NGSO_ScenarioSupport); } case(C413) { v_ApplCond := pc_NB_FDD and (pc_NB_ntn_Connectivity_EPC or pc_NB_ntn_only_Connectivity_EPC) and (pc_NB_ntn_GSO_ScenarioSupport or pc_NB_ntn_NGSO_ScenarioSupport) and pc_NB_ntn_TA_Report and pc_NB_ntn_OffsetTimingEnh; } case else { v_ApplCond := false; } } return v_ApplCond; }

After Change:

function f_NBIOT_FrequencyBandIsFDD(integer p_FrequencyBand) return boolean { return (p_FrequencyBand < 41 or p_FrequencyBand >= 44); } function f_NBIOT_SelectionExpr(NBIOT_36523_Condition_Type p_Condition) return boolean { var boolean v_ApplCond; var boolean v_FrequencyBandIsFDD := f_NBIOT_FrequencyBandIsFDD(px_ePrimaryFrequencyBand); select (p_Condition) { case(C266){ v_ApplCond := pc_NB_FDD or pc_NB_TDD; } case(C266a){ v_ApplCond := (pc_NB_FDD or pc_NB_TDD) and pc_PLMN_Manual_Mode_Exception; } case(C271){ v_ApplCond := (pc_NB_FDD or pc_NB_TDD) and pc_NB_User_Plane_CIoT_Optimisation; } case(C272){ v_ApplCond := (pc_NB_FDD or pc_NB_TDD) and pc_ZUC; } case(C273){ v_ApplCond := (pc_NB_FDD or pc_NB_TDD) and pc_edrx; } case(C275){ v_ApplCond := (pc_NB_FDD or pc_NB_TDD) and pc_LAP and pc_LAP_override; } case(C277){ v_ApplCond := (pc_NB_FDD or pc_NB_TDD) and pc_Multiple_PDN and pc_LAP and pc_LAP_override; } case(C288){ v_ApplCond := (pc_NB_FDD or pc_NB_TDD) and pc_NB_MultiCarrier; } case(C290) { v_ApplCond := (pc_NB_FDD or pc_NB_TDD) and (pc_User_Plane_CIoT_Optimisation or pc_S1_U_DataTransfer); } case(C291) { v_ApplCond := (pc_NB_FDD or pc_NB_TDD) and (pc_User_Plane_CIoT_Optimisation or pc_S1_U_DataTransfer) and pc_ZUC; } case(C293){ v_ApplCond := (pc_NB_FDD or pc_NB_TDD) and pc_IP_PDN and pc_ESM_MO_Bearer_Modification; } case(C322){ v_ApplCond := (pc_NB_FDD or pc_NB_TDD) and pc_RRC_re_establishment_CP_CIoT; } case(C323){ v_ApplCond := (pc_eFDD or pc_eTDD) and (pc_NB_FDD or pc_NB_TDD); } case(C339){ v_ApplCond := (pc_NB_FDD or pc_NB_TDD) and pc_NB_TwoHARQ_Processes and pc_ue_Category_NB2; } case(C342) { v_ApplCond := (pc_NB_FDD or pc_NB_TDD) and pc_APN_RateControl and pc_Additional_APN_RateControl; } case(C347) { v_ApplCond := (pc_NB_FDD or pc_NB_TDD) and pc_ue_Category_NB2; } case(C348) { v_ApplCond := (pc_NB_FDD or pc_NB_TDD) and pc_NB_MultiCarrier_NPRACH; } case (C349_C403) { if (v_FrequencyBandIsFDD) { v_ApplCond := pc_NB_FDD and pc_NB_MultiCarrier_Paging; } else { v_ApplCond := pc_NB_TDD and pc_NB_MultiCarrier_Paging_TDD; } } case(C349) { v_ApplCond := pc_NB_FDD and pc_NB_MultiCarrier_Paging; } case(C350) { v_ApplCond := pc_NB_FDD and pc_SCPTM_IDLE; } case(C351){ v_ApplCond := pc_NB_FDD and pc_SCPTM and (pc_FeatrGrp_3_F or pc_FeatrGrp_3_T) and (pc_FeatrGrp_7_F or pc_FeatrGrp_7_T); } case(C377){ v_ApplCond := (pc_NB_FDD or pc_NB_TDD) and pc_NB_RLC_UM and (pc_User_Plane_CIoT_Optimisation or pc_S1_U_DataTransfer); } case(C390){ v_ApplCond := pc_NB_FDD and pc_wakeUpSignal; } case(C391){ v_ApplCond := pc_NB_FDD and pc_edrx and pc_wakeUpSignal; } case(C392){ v_ApplCond := pc_NB_FDD and pc_SR_WithoutHARQ_ACK; } case(C396){ v_ApplCond := (pc_NB_FDD or pc_NB_TDD) and pc_NB_User_Plane_CIoT_Optimisation and (pc_ROHC_profile0x0002 or pc_ROHC_profile0x0003 or pc_ROHC_profile0x0004 or pc_ROHC_profile0x0006 or pc_ROHC_profile0x0102 or pc_ROHC_profile0x0103 or pc_ROHC_profile0x0104); } case(C400){ v_ApplCond := pc_NB_FDD and pc_NB_mixedOperationMode; } case(C402){ v_ApplCond := pc_NB_FDD and pc_NB_nprach_Format2; } case(C403){ v_ApplCond := pc_NB_TDD and pc_NB_MultiCarrier_Paging_TDD; } case(C412) { v_ApplCond := pc_NB_FDD and (pc_NB_ntn_Connectivity_EPC or pc_NB_ntn_only_Connectivity_EPC) and (pc_NB_ntn_GSO_ScenarioSupport or pc_NB_ntn_NGSO_ScenarioSupport); } case(C413) { v_ApplCond := pc_NB_FDD and (pc_NB_ntn_Connectivity_EPC or pc_NB_ntn_only_Connectivity_EPC) and (pc_NB_ntn_GSO_ScenarioSupport or pc_NB_ntn_NGSO_ScenarioSupport) and pc_NB_ntn_TA_Report and pc_NB_ntn_OffsetTimingEnh; } case else { v_ApplCond := false; } } return v_ApplCond; }